3GPP TSG-RAN WG4 Meeting # 116	R4-2509769

Bengaluru, India, August 25th – 29th, 2025

Source: 	Xiaomi

Title: 	Discussion on RRM performance requirement for Rel-19 MIMO phase 5

Agenda Item:	7.20.5

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the RRM performance requirement of Rel-19 MIMO phase 5.

- Discussion

Performance metric for legacy L3 event reporting

For legacy event triggered L3 reporting, there will be test to make sure that UE correctly detect the channel variation and send L3 report within required delay. Precision = 90% is used. Precision considers the correct reporting ratio for all triggered events, which is shown below:

##### A.4.6.1.1.3 Test Requirements The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

Observation 1: In legacy RAN4, there will be performance test to make sure that UE correctly detect the channel variation and report within required delay. Precision = 90% is used.

For event triggered L1 reporting, similar, it also needs to verify UE can correctly detect the channel variation and send L1 report within required delay.

Reporting quantity CMI

In the context of event-triggered L1-RSRP reporting, for M>1 case, 3GPP RAN1 has introduced a new reporting quantity: the Condition Met Indicator (CMI). This 1-bit indicator is designed to provide an efficient and reliable mechanism for the UE to inform the gNodeB about whether a specific beam has met the event-triggering conditions within a defined time window. The interpretation of the CMI is as follows:

Agreement On UE-initiated/event-driven beam reporting, support the following interpretation on each of the codepoints of the ‘1-bit’ condition met indicator as follows: - ‘0’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window doesn’t reach the configured number M. - ‘1’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window reach the configured number M. FFS: whether/how to introduce the ‘1-bit’ condition met indicator for Event 7.

Observation 2: RAN1 introduce a new reporting quantity: the Condition Met Indicator (CMI).

The current technical framework lacks a mechanism to verify the correlation between the CMI and the reporting. Without such a mechanism, it is challenging to ensure that the CMI accurately reflects the conditions over the M measurement periods. Introducing performance metrics for the CMI will enable a systematic validation of the indicator's reliability, ensuring that it truly represents the sustained triggering conditions.

M>1 ensures that the CMI reflects a consistent and stable condition over a longer period. This is particularly important in dynamic environments where beam conditions can change rapidly. By verifying the CMI's performance with M > 1, we can ensure that the indicator provides a reliable and stable representation of the beam's performance, which is essential for making informed network decision. K>=2 can ensure that CMI for different beam index can be verified.

Proposal 1: RAN4 to define requirement to verify the performance of condition met indicator(CMI) in the report.

How to consider the performance metric of CMI

Key considerations include:

1. Configuration Requirement: M>1 in test configurations to validate CMI’s ability to reflect sustained conditions.

2. Accuracy Metric: Define a "CMI Correct Rate" (≥90%) where CMI values align with actual Event-2 fulfillment/non-fulfillment.

3. Stability Validation: Ensure CMI=1 is reported only when M consecutive Event-2 instances are met.

4. Beam Requirement: Mandate K>=2 to validate CMI across different beams.

Defining the Correct Rate for Event-Triggered L1 Reporting

With the introduction of CMI, it is necessary to reconsider how the correct rate for event-triggered L1 reporting is defined. The legacy L3 event reporting correct rate is based on the UE correctly reporting the event for 90% of the cases, with the report being within the required delay. For L1 reporting with CMI, we need to determine whether the correct rate should be defined independently or in conjunction with the CMI.

Observation 3: With the introduction of CMI, it is necessary to reconsider how the correct rate for event-triggered L1 reporting is defined.

The following options are proposed:

1. Option 1: Hybrid Event Reporting Correct Rate

- Define a hybrid event reporting correct rate that involves the correctness of both the event reporting and the CMI.

- For each report, it should be considered correct if:

- The report is within the required reporting delay.

- The CMI for all K beams or a subset of beams is correct.

2. Option 2: Independent Performance Metrics

- Design independent performance metrics for event reporting and CMI.

- For event reporting:

- The rate of correct event reports observed during repeated tests should be at least 90%.

- The report should be within the required reporting delay.

- For CMI:

- The rate of correct CMI reports observed during repeated tests should be at least 90%.

- The CMI should correctly indicate whether the event-triggering conditions have been met for the specified number of instances (M) within the defined time window.

Option 1 Combines reporting delay and CMI correctness into a single metric, ensuring comprehensive validation. Option 2 Treats reporting delay and CMI as separate metrics, offering simplicity and flexibility. We suggest RAN4 to discuss how to define correct rate for event triggered reporting, whether to include CMI together or independently.

Proposal 2: RAN4 to discuss how to define performance metric for event triggered reporting, whether to include CMI together or independently.

- Conclusion

In this contribution, we provide the following proposals：

Observation 1: In legacy RAN4, there will be performance test to make sure that UE correctly detect the channel variation and report within required delay. Precision = 90% is used.

Observation 2: RAN1 introduce a new reporting quantity: the Condition Met Indicator (CMI).

Proposal 1: RAN4 to define requirement to verify the performance of condition met indicator(CMI) in the report.

Observation 3: With the introduction of CMI, it is necessary to reconsider how the correct rate for event-triggered L1 reporting is defined.

Proposal 2: RAN4 to discuss how to define performance metric for event triggered reporting, whether to include CMI together or independently.

- Reference