3GPP TSG-RAN WG4 Meeting #116															R4-2509771

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:			7.6.3.3

Source: 	Xiaomi

Title: 						IoT-NTN HPUE RF requirements

Document for:	Discussion

- Introduction

Rel-19 NTN agreed to introduce HPUE for NR-NTN/Iot-NTN operation:

High power UE (HPUE) for NTN - Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

In this contribution, views on remaining open issues were provided.

- Discussion

ACLR requirements

In last RAN4 meeting, initial conclusion was reached for co-existence study as following:

Issue 1-1-6: ACLR for eMTC based Iot-NTN for PC1; - Agreement: - [34dB]

On isolation distance, we prefer to align with previous assumption the deployment scenario and restriction shall be consistency independent to device power classes which belongs to UE capability.

On NTN operation, the critical issue is coverage due to long distance between satellite over the space to terminal. With reasonable ACLR, it will bring benefits to improve uplink coverage with less MPR and or A-MPR.

During the RAN4#115 meeting, ACLR value for PC1 eMTC based Iot-NTN HPUE has been fully discussed by the meeting. A comprise value 34dB ACLR agreed by most of the companies.

Based on above consideration, we suggest to specify following ACLR values:

Proposal 1: For PC1 eMTC based Iot-NTN HPUE ACLR can be determine as 34dBc

In TS 36.101, P-MPR isn't exclusive to handheld UEs. It's a power management mechanism ensuring the UE's transmit power meets electromagnetic radiation safety and regulations across various scenarios.

While P-MPR is common in handheld devices, especially when users hold them close to their heads or bodies due to SAR issue, it also applies to other UE types like Vehicle-mounted UEs and Fixed-installed UEs.

Thus, P-MPR is a general power management parameter for various UE devices, not just handheld ones.

Proposal 2: RAN4 should consider P-MPR for both handheld UEs and non-handheld UEs.

SAR Related issues

Issue 2-1-1: Whether and how to define SAR scheme other than already agreed P-MPR (e.g. duty cycle related scheme): - Background (for information): - P-MPR was already agreed to be the baseline SAR solution for all NTN HPUE cases. - The current status for duty-cycle based SAR scheme in TN - NR: - Defined in MOP section; - Applied in ΔPPowerClass in configured transmitted power clause; - whether can apply to FDD band has some different understanding. - eMTC: - Not defined in MOP section; - Reference general LTE configurated transmitted power clause, no separate definition - For general LTE, the duty cycle related power backoff explicitly defined only for TDD band whose frame configuration is 0 or 6 - NB-Iot: - Not defined in MOP section; - Not defined in configured transmitted power clause which use dedicated equations (Not reference LTE general equation) - Options for Selection: - Option 1: Keep the same status as TN for respective base RAT - NR-NTN: Reuse NR TN, - Including ΔPPowerClass in configured transmitted power clause - eMTC based Iot-NTN: Reuse eMTC - Not precluding ΔPPowerClass in configurated transmitted power clause - NB-Iot based Iot-NTN: Reusing NB-Iot - No duty cycle related scheme defined - Option 2: Introduce further revision / new method - NR-NTN sub-options: - A. Introduce clarification for FDD band support; (Nokia, Qualcomm) - B. changing the ΔPPowerClass from mandatory to optional;(Qualcomm) - C. UE voluntarily dropping uplink transmissions by self-assessment to control the SAR (vivo) - D. differentiate UE working status in terms of whether SAR is required in a time duration. (OPPO) - E. To have the network to notify/signal its upper bound of uplink ratio for a certain time duration (Samsung) - F. Energy headroom reporting (Qualcomm) - G. ΔPPowerClass and P-MPR reporting (Ericsson, SONY, Nokia) - H. Send information LS to RAN2 support PC1 NTN HPUE for FDD use cases. (Nokia) - I. inform the RAN that it is close to the SAR limit (ViaSet, Inmarsat) - Iot-NTN (including eMTC & NB-Iot based): - Introduce NR like scheme - Introduce other revision / new method as proposed for NR-NTN - Option3: Do not introduce for NTN - NR NTN : Explicitly preclude ΔPPowerClass and no other duty cycle related reference - eMTC-based Iot-NTN: Explicitly preclude ΔPPowerClass and no other duty cycle related reference - NB-Iot based Iot-NTN: No duty cycle or other new scheme defined

The P-MPR scheme remains preferable due to its mature standardization in 3GPP since Rel-15, ensuring direct NTN compatibility. Its implementation simplicity leverages UE-local sensor data (e.g., proximity/temperature) and autonomous power adjustments, eliminating network coordination needs—critical for NTN’s high-latency FDD environments. The framework supports dynamic power optimization (e.g., per-slot backoff) for coverage enhancement. Additionally, existing SAR/MPE testing standards (TS 37.145) already validate P-MPR, avoiding costly redefinition of test cases required by alternative approaches.

Basic on the current options, there is no perfect way to replace the P-MPR for FDD SAR issue by duty cycle solutions. Duty cycle need to determine the UL time percentage of UE, but how to determine the time percentage would be a big issue for 3GPP. Duty cycle solution can reduce the average SAR, however, the peak SAR or maximum SAR still same with previous. It means during time slots involving high-power transmission over a short period, the SAR in local areas (such as the antenna position close to the head) may still exceed the standard. Only P-MPR solution can cover both average and peak SAR at the same time.

Proposal 3: To not define any duty-cycle based SAR solution, and to use the baseline P-MPR method.

- Conclusion

In this contribution, views on remaining open issues were provided.

Proposal 1: For PC1 eMTC based Iot-NTN HPUE ACLR can be determine as 34dBc

Proposal 2: RAN4 should consider P-MPR for both handheld UEs and non-handheld UEs

Proposal 3: To not define any duty cycle based SAR solution, and to use the baseline P-MPR method.

Reference

[1] R4-2508122, WF on IoT-NTN HPUE, vivo