3GPP TSG-RAN WG4 Meeting #116	R4-2509775

# Bengaluru, India, August 25th – 29th, 2025

Source: 	Xiaomi

Title: 	Discussion on RRM core requirements for LP-WUS/WUR

Agenda Item:	7.24.5.2

Document for:	Discussion

# 1Introduction

In RAN4#114 meeting, the RRM impacts on low-power wake-up signal and receiver for NR (LP-WUS/WUR) was discussed and a WF was approved [1]. In this contribution, we provide our further analysis on RRM impacts of LP-WUS/WUR features.

- Discussion

Issue 1-1-10: Higher priority frequency layer measurement requirements

Agreement: For case 1: - MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60 - Note: RAN4 assumes Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is always met for case 1. For case 3: - When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60 - When the condition of Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is NOT met, the same requirement for higher priority, equal priority and lower priority carriers: - 16 times of Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter are applied

In last meeting, RAN4 reached agreements on the UE requirements of higher priority frequency layer measurement for different cases.

For case 1, the conclusion in last meeting was reached based on assumption that Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is always met for case 1. Based on RAN2 progress, RAN4’s assumption is aligned with RAN2 agreements that “the threshold of the MR based entry condition for serving cell RRM offloading should be higher than or equal to the threshold to stop neighboring cell RRM measurement, which is the maximum of {SIntraSearchP, SnonIntraSearchP}”. So, last meeting agreement on higher priority frequency layer measurement requirements should be confirmed.

Proposal 1: RAN4 to confirm that, for case 1, MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60.

For case 3, RAN4 reached agreements on UE behavior on neighbor cells. In RAN4#114 meeting, RAN4 discussed on the MR RRM relaxation for serving cell/neighbour cell for case 3, and agreed on the scaling factor 16 for both serving cell and neighbor cell. To make RAN4 conclusion clearer, we proposal to clarify the UE requirement for serving cell and neighbor cell under case3 as follow:

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ,

- MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60;

- MR is expected to perform relaxed serving xell measurement with scaling factor 16.

when the condition of Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is NOT met:

- The same relaxed requirements for higher priority, equal priority and lower priority carriers are applied, i.e. 16 times of Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter;

- The relaxed requirement for serving cell is applied, i.e. 16 times of Nserv.

Proposal 2: RAN4 to clarify for case 3:

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ,

- MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60, if configured;

- MR is expected to perform relaxed serving cell measurement with scaling factor 16.

when the condition of Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is NOT met:

- The same relaxed requirements for higher priority, equal priority and lower priority carriers are applied, i.e. 16 times of Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter;

- The relaxed requirement for serving cell is applied, i.e. 16 times of Nserv.

Issue 1-2-2-2: Upper bound on SSB-based LP-WUR measurement periodicity

During previous discussion, the measurement delay requirement in IDLE/Inactive mode for LP-WUR was divided into two cases. One is for LP-WUR based on LP-SS, the other one is for LP-WUR based on SSB. For the former case, agreement was achieved that measurement delay requirements for LP-WUR based on LP-SS shall be defined based on LP-SS periodicity. For the later case, i.e. LP-WUR based on SSB, it was agreed that the measurement delay requirements are defined based on LO periodicity.

For the case LP-SS is not configured, we understand only OFDM-based LP-WUR can work in this case. The OFDM-based LP-WUR obtains synchronization based on SSB. Basically, we think it is quite similar as MR. Also, the current agreement, i.e. based on LO periodicity, also aligns with measurement delay requirements for MR. As no upper bound has been defined for MR, we prefer not to define upper bound for LR either.

Proposal 3: RAN4 not to define upper bound for the measurement delay requirements for LP-WUR when LP-SS is not configured.

Based on the RAN4 discussion progress, RAN4 agreed to define single set of LR evaluation requirement for the transition between different cases, while the applicability of LR evaluation requirements is still under discussion.

The agreements on entry and exit conditions reached under current RAN2 discussions are summarized below.

| Case | Entry conditions | Exit conditions |
| --- | --- | --- |
| LP-WUS monitoring | MR threshold and optional LR threshold If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). | LR threshold |
| RRM measurement fully offloading (Case 1) | MR threshold and optional LR threshold | LR threshold |
| RRM measurement relaxation (Case 3) | - MR threshold and optional LR threshold - If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). | FFS |

Taken legacy case into consideration, the transition between cases including:

- from RRM measurement fully offloading case to legacy case

- from RRM measurement fully offloading case to RRM measurement relaxation case

- from RRM measurement relaxation case to legacy case

- from legacy case to RRM measurement relaxation case

- from legacy case to RRM measurement fully offloading case

- from RRM measurement relaxation case to RRM measurement fully offloading case

- from LP-WUR monitoring case to legacy case

- from legacy case to LP-WUR monitoring case

We think it is straightforward that the LR evaluation requirements should be applicable for ①, ②, ⑦, as the corresponding exit condition is based on LR threshold.

For ③ and ⑥, RAN2 has not decided the corresponding exit condition and the corresponding entry condition can optionally include LR threshold. Based on this current discussion status, we think at least the LR evaluation requirements should be applicable when NW configure LR threshold.

For ④, ⑤ and ⑧, We understand LR evaluation requirements applicability is related to the LR status. In previous meeting, RAN4 achieved the agreements that when and how to turn on LR for serving cell measurement is up to UE implementation. We understand the LR evaluation requirement should be applicable for the case UE evaluate whether the LR measurement result is above the configured threshold. And the LP-WUR status has to be “on” to obtain the LR measurement result. If RAN4 agreed not to restrict when and how UE to turn on LR, we think it is reasonable that the LR evaluation requirement is not applicable for the case LR is “ON” at legacy state.

Proposal 4: No LR measurement and evaluation requirements apply at the legacy state, i.e., for the following cases: from legacy case to LP-WUR monitoring, from legacy case to RRM measurement fully offloading (case 1), and from legacy case to RRM measurement relaxation (case 3).

Issue 1-1-15 RedCap related

In previous meeting, companies proposed to consider LP-WUR operation for RedCap UE. In our view, it is reasonable to for RedCap UE to operate with LP-WUR in pursuit of power saving. In this case, the MR can be differentiated into non-RedCap and RedCap, and we understand our previous discussion is focus on non-RedCap. As it is already the last meeting for core part discussion, we support to take the agreement we achieved as baseline for RedCap.

Proposal 5: RAN4 to define LP-WUR related requirements for RedCap UE.

### Accuracy

Issue 1-2-1: General on LR accuracy requirement Agreement: No dedicated accuracy requirement is defined in the performance section for LP-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin relative to the threshold in the core requirement. Issue 1-2-1-1: Detail on LR accuracy and side conditions requirements Agreement: For FR1: - [3.5] dB can be used for core requirements for LP-RSRQ accuracy and [5.5 or 6] dB can be used for core requirements for LP-RSRP accuracy, under the side conditions Ês/Iot [x, x = -3] dB - [3.5] dB can be used for core requirements of SSB based RSRQ accuracy and [5.5 or 6] dB can be used for core requirements for SSB based RSRP accuracy, under the side conditions Ês/Iot [y, y = -3] dB - Note: [2 or 2.5] dB RF impairment margin is assumed

RAN4 discussed the LR accuracy and side conditions requirements in last meeting and the above agreement was achieved. For the specific numbers of LR accuracy used for core requirements were in the square brackets. In our understanding, the main issue here is the number of RF impairment margin.

In RAN4#114 meeting, RAN4 RF has reached agreement related to the NF:

Issue 2-1-1: SNR, NF and IM value for FR1 LP-WUR Agreement: - For the FR1 requirements targeting at bands <2.5GHz - The IM+NF values are - Set 1: 18dB - Set 2: 13.5dB - Send an LS to RAN1 with RAN4 decision of two sets of : NF (assume IM is 2.5dB)

Based on the above agreements, the two set of NF can be 15.5dB and 11dB for LR based on the assumption the IM is 2.5dB. For normal NR UE, which is the MR in our discussion, the NF is assumed as 9.0 dB. We can see that the value for LR NF is extremely larger than the MR NF. We prefer to assume a larger number for the RF impairment margin of LR to account for the LR NF deterioration. In our understanding, the RF impairment margin for MR is assumed as 2.5 dB during R15 accuracy discussion. For LR, we propose to consider extra 1 dB for the RF impairment margin.

Proposal 6: RAN4 to assume the 3.5 dB RF impairment margin for LR.

- Conclusion

Proposal 1: RAN4 to confirm that, for case 1, MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60.

Proposal 2: RAN4 to clarify for case 3:

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ,

- MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60, if configured;

- MR is expected to perform relaxed serving cell measurement with scaling factor 16.

when the condition of Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is NOT met:

- The same relaxed requirements for higher priority, equal priority and lower priority carriers are applied, i.e. 16 times of Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter;

- The relaxed requirement for serving cell is applied, i.e. 16 times of Nserv.

Proposal 3: RAN4 not to define upper bound for the measurement delay requirements for LP-WUR when LP-SS is not configured.

Proposal 4: No LR measurement and evaluation requirements apply at the legacy state, i.e., for the following cases: from legacy case to LP-WUR monitoring, from legacy case to RRM measurement fully offloading (case 1), and from legacy case to RRM measurement relaxation (case 3).

Proposal 5: RAN4 to define LP-WUR related requirements for RedCap UE.

Proposal 6: RAN4 to assume the 3.5 dB RF impairment margin for LR.

- Reference

- R4-2505572, WF on RRM requirements for NR_LPWUS, Vivo