3GPP TSG-RAN WG4 Meeting #116	R4-2509777

# Bengaluru, India, August 25th – 29th, 2025

Source: 	Xiaomi

Title: 	Discussion on (e)RedCap RRM requirements in NTN for NR Phase 3

Agenda Item:	7.27.5.1

Document for:	Discussion

# 1Introduction

In the last RAN4#115 meeting, further discussion on RRM requirements for Rel-19 NR NTN phase3 took place and a WF was approved as detailed in [1]. In this contribution, we would like to provide our further discussion on (e)RedCap RRM requirements in NTN for NR Phase 3 and provide our proposals.

- Discussion

Issue 1-3-5: The impact on NR Handover due to HD-FDD <Way Forward>: - Proposal 1: For the impact on NR handover for HD-FDD, the principle in option 1 is accepted. - Proposal 1a: Regarding Issue 3-2-1 (The impact on NR Handover due to HD-FDD), adopt Option 1 with 'availability' defined as 'no collision with UL transmission at the UE'. - Proposal 2: Adopt option 2 (no spec impact) for HO, CHO and satellite switch for HD-FDD UE.

Issue of the impact on NR handover due to HD-FDD is still open. From our side, the two listed proposals are not conflicted with each other. Proposal 1 does not state the priorities, but define applicability rules to preclude the case when there is an overlap on the SSB used for HO and the UL transmission. Basically, we do not have strong view, but slightly prefer Proposal 1 to align with the conclusion reached in RedCap dicsussion.

Proposal 1: For the impact on NR handover for HD-FDD, the principle in option 1 is accepted.

Issue 1-4-1: General principle for designing the UE capabilities for (e)RedCap UE with NTN <Way Forward>: - Option 1: Declare the capa1bilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). (CATT, CMCC, ZTE, Xiaomi, E///, Samsung - compromise, Apple, QC) - FFS: The Rel-19 requirement can be release independent from Rel-17. - Option 2: RAN4 to introduce new capability to declare support over RedCap over NTN. (vivo, Samsung) - Option 3: to wait for RAN1 to confirm the working assumption (UE that supports RedCap and NTN shall support RedCap over NTN). (Nokia, QC, MTK)

In last meeting, RAN4 discussed whether to design the UE capabilities for (e)RedCap UE with NTN, while no consensus was reached. In our view, we support option 1 that UE can declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of (e)RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). (e)RedCap UE and NR NTN UE are two UE features that 3GPP already introduced. With the declaration of these two feature, it is clear for UE behavior. Additional UE capability would introduce unnecessary signalling overhead. Also, as stated in the WID [2], the objective is to “Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands”. We do not see the necessity of introducing a new capability to declare the support of (e)RedCap UE with NTN.

Proposal 2: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of (e)RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17)

- Conclusion

Proposal 1: For the impact on NR handover for HD-FDD, the principle in option 1 is accepted.

Proposal 2: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of (e)RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17)

- Reference

- R4-2504912, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT