3GPP TSG-RAN WG4 Meeting #116			                                R4-2509783

Bengaluru, India, 25th – 29th August, 2025

Agenda Item:	7.10.3.2

Source:	Xiaomi

Title:	Discussion on test methodology and radiated performance metric for FR1 NTN devices

Document for:	Discussion

- Introduction

The test performance metric for FR1 NTN devices has been extensively discussed in previous meetings. The latest progress was captured in the WF [1]. This contribution continues to provide our views on these remaining issues.

- Discussion

Performance metric for scenario 2

Issue 2-1-5: What is the angle range of the partial sphere-after concluding center of the sphere Agreements: - RAN4 further discuss the theta angle range of partial sphere metric, from ±30°, ±60°, ±90°. - Option 1: single range is selected and captured in TR - Option 2: multi-range in TR but based on UE declaration, details are FFS - Note1: the phi range should be 360 degrees - Note2: if multiple ranges are captured in TR, the metric applicability rule should also be defined. The target is selecting one for NTN scenario 2.

For those UEs supporting scenario 2, RAN4 agreed to define partial sphere metrics as the performance metric, and FFS the angle range. From our perspective, the directional antenna design is more likely to be deployed in order to guarantee the UL link-budget, thus, it is reasonable to consider a certain angular range which is declared by OEMs, e.g. ±30°, ±60°, and the angle range is based on UE declaration. In addition, RAN4 agreed to adopt integration of EIRP/EIS or CDF, if the test angle is upper hemisphere, while the real implementation of UE antenna is radiated between ±30°, the degradation of measurement is expected which will cause the testing is not accuracy. Thus, from our perspective, it is preferred to define multi angle ranges for partial sphere metric which is based on UE declaration.

Proposal 1: RAN4 to define multi range for theta angle of partial sphere metric based on UE declaration.

3	Conclusions

In this paper, we provided our views on the angle range of the partial sphere and our proposal as follows:

Proposal 1: RAN4 to define multi range for theta angle of partial sphere metric based on UE declaration.

4 References

[1] R4-2508782, Way Forward for [115][337] TRP_TRS_MIMO_OTA_Ph3, Vivo