3GPP TSG-RAN WG4 Meeting #116				                                R4-2509792

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.6.4.5

Title: 	Discussion on the performance part requirements for less than 5MHz NTN

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In the previous RAN4 meeting, the RRM core requirements [1] related to the less than 5MHz NTN were completed [2, R4-2508261]. Accordingly, RAN4 is required to define the corresponding test cases to verify the RRM requirements in TS38.133 [3].

In the last RAN4 meeting, the agreements on the common testing configuration and part of test cases needed were agreed. In this paper we provide further consideration on the open issues below.

- Methodology to introduce the test cases for NTN less than 5MHz

- The detailed test configuration

- Discussion

- Issue 1: Methodology to introduce the test cases for NTN less than 5MHz

According to RAN4’s discussion on the core part, for NTN less than 5MHz, only part of new requirements were introduced in Rel19. For examples, for the requirements in IDLE/inactive mode, the core requirements for NTN in Rel18 can be reused.

Table 1. RRM impacts summary due to spectrum less than 5MHz

| RRM requirements | NTN requirements in TS38.133 v18.5.0 | Need the new requirements in Rel19 |
| --- | --- | --- |
| IDLE/inactive mode mobility | 4.2C Cell Re-selection for NR UE for Satellite Access | None |
| RRC connection mobility control | 6.2C RRC Connection Mobility Control for Satellite Access | None |
| Handover | 6.1C Handover for SAN | Yes |
| UE Tx timing, MTTD/ MRTD, timer accuracy, TA accuracy | 7.1C, 7.2C, 7.3C | None |
| RLM | 8.1C Radio Link Monitoring for Satellite Access 8.5C Link Recovery Procedures for Satellite Access | Yes |
| Measurement | 9.2C NR intra-frequency measurements for SAN 9.3C NR inter-frequency measurements for SAN | Yes |

And in the last meeting, RAN4 reached agreements on whether need to define the test cases for these core requirements reusing NTN as below.

Issue 2-2-2: Test cases to verify the core requirements reusing NTN Agreement: The new test cases of cell reselection for NTN less than 5MHz can be defined. And the detailed testing configuration and requirements (e.g. TSI-NR) can be further discussed: - Option 1: captured in the new sub clause in NTN clause - Option 2: an additional test configuration in the exsiting sub clause for NTN

However, for the test cases themselves, in our views in Rel18 test case for NTN [3, TS38.133 v18.6.0] the test configuration for 10MHz bandwidth was introduced only.

Observation 1: in Rel18 test case for NTN [3, TS38.133 v19.1.0] the test configuration for less than 5MHz was not included.

Therefore, in order to verify the performance of NTN less than 5MHz in which the legacy requirements were leveraged (e.g. the requirements for IDLE/Inactive mode, timing) the additional test configuration for NTN less than 5MHz needed be introduced. For UE support NTN less than 5MHz only, this test configuration shall be applicable. Otherwise, the test configuration with 10MHz BW needed to be pass.

On the other hand, for Option 2 it is true the more clear testing cases can be applied for the NTN less than 5MHz. However, this will increase the specification complexity a little bit. And especially considering UE supporting NTN less than 5MHz needs to pass these new test cases only, we slightly prefer to Option 1. Thus we can propose that:

Proposal 1: RAN4 can define the additional testing configuration with less than 5MHz for NTN RRC IDLE and Inactive mode test cases. And UE supporting ONLY NTN less than 5MHz shall pass such test.

- Issue 2: Detailed test case list and configuration for NTN less than 5MHz

In the last meeting, the detailed configurations for the test cases for NTN less than 5MHz core requirement were discussed. The further views on the open issues from our side can be updated in the table below [2,WF]

Proposal 2: The configuration for NTN less than 5MHz testing cases are provide in the table below.

| Test case | Requirements | Configurations | Companies views |
| --- | --- | --- | --- |
| Common configuration |  | 15KHz, 3MHz BW, 12 PRB SSB, GSO/NGSO | The SSB configuration for less than 5MHz can be used (SSB.13 FR1). |
| HO-1: | 6.1C.1.2 NR SAN FR1 – NR SAN FR1 Handover | unknown cell |  |
|  |  | Intra-frequency |  |
|  |  | Same testing procedure as NTN can be reused |  |
|  |  | Other aspects | Discussed in the draft CR directly. e.g. the other BWP/CORESET configuration for less than 5MHz bandwidth |
| CHO-1 | 6.1C.2.2 NR SAN FR1 – NR SAN FR1 conditional handover or 6.1C.2.3 NR SAN FR1 – NR SAN FR1 conditional handover without L3 measurements | With L3 measurement/ without L3 measurements | without L3 measurements |
|  |  | Time based |  |
|  |  | Unknown target cell |  |
|  |  | Intra-frequency |  |
|  |  | Other aspects | Discussed in the draft CR directly. e.g. the other BWP/ CORESET configuration for less than 5MHz bandwidth |
| SatSwitch-1 | 6.1C.3 NR SAN Satellite switching with re-synchronization | Hard switch |  |
|  |  | Unknown target cell |  |
|  |  | Other aspects: e.g. RACH based | Discussed in the draft CR directly. We can define the test case for RACH based ONLY. |
| RLM-1 | 8.1C Radio Link Monitoring for Satellite Access | RLM OOS |  |
|  |  | FR1 PCell only |  |
|  |  | non-DRX/DRX | Non-DRX test case can be prioritized. |
|  |  | Other aspects | Discussed in the draft CR directly. e.g. the CORESET configuration for less than 5MHz bandwidth |
| RLM-2 | 8.1C Radio Link Monitoring for Satellite Access | RLM IS |  |
|  |  | FR1 PCell only |  |
|  |  | non-DRX/DRX | Non-DRX test case can be prioritized. |
| BFR-1 | 8.5C Link Recovery Procedures for Satellite Access | SSB based BFD |  |
|  |  | FR1 PCell only |  |
|  |  | non-DRX/DRX | Non-DRX test case can be prioritized. |
|  |  | Other aspects | Can be further discussed when drafting CR (e.g. the gap pattern ID) |
| MR-1 | 9.2C NR intra-frequency measurements for SAN | with SSB index detection |  |
|  |  | Non-DRX/DRX | Non-DRX test case can be prioritized. |
|  |  | No gap | No gap test cases only. |
|  |  | Other aspects | Discussed in the draft CR directly. e.g. the other BWP/CORESET configuration for less than 5MHz bandwidth |
| MR-2 | 9.3C NR inter-frequency measurements for SAN | With SSB index detection |  |
|  |  | Non-DRX/DRX | Non-DRX test case can be prioritized. |
|  |  | No gap | No gap test cases only. |
|  |  | Other aspects | Discussed in the draft CR directly. e.g. the other BWP/CORESET configuration for less than 5MHz bandwidth |
| IDLE-1 | 4.2C Cell Re-selection for NR UE for Satellite Access |  | FFS upon issue 1 |

- Summary

This paper proposes the main configurations parameters and test cases list to verify the RRM requirements related to NTN less than 5MHz.

Observation 1: in Rel18 test case for NTN [3, TS38.133 v19.1.0] the test configuration for less than 5MHz was not included.

Proposal 1: RAN4 can define the additional testing configuration with less than 5MHz for NTN RRC IDLE and Inactive mode test cases. And UE supporting ONLY NTN less than 5MHz shall pass such test.

Proposal 2: The configuration for NTN less than 5MHz testing cases are provide in the table below.

| Test case | Requirements | Configurations | Companies views |
| --- | --- | --- | --- |
| Common configuration |  | 15KHz, 3MHz BW, 12 PRB SSB, GSO/NGSO | The SSB configuration for less than 5MHz can be used (SSB.13 FR1). |
| HO-1: | 6.1C.1.2 NR SAN FR1 – NR SAN FR1 Handover | unknown cell |  |
|  |  | Intra-frequency |  |
|  |  | Same testing procedure as NTN can be reused |  |
|  |  | Other aspects | Discussed in the draft CR directly. e.g. the other BWP/CORESET configuration for less than 5MHz bandwidth |
| CHO-1 | 6.1C.2.2 NR SAN FR1 – NR SAN FR1 conditional handover or 6.1C.2.3 NR SAN FR1 – NR SAN FR1 conditional handover without L3 measurements | With L3 measurement/ without L3 measurements | without L3 measurements |
|  |  | Time based |  |
|  |  | Unknown target cell |  |
|  |  | Intra-frequency |  |
|  |  | Other aspects | Discussed in the draft CR directly. e.g. the other BWP/ CORESET configuration for less than 5MHz bandwidth |
| SatSwitch-1 | 6.1C.3 NR SAN Satellite switching with re-synchronization | Hard switch |  |
|  |  | Unknown target cell |  |
|  |  | Other aspects: e.g. RACH based | Discussed in the draft CR directly. We can define the test case for RACH based ONLY. |
| RLM-1 | 8.1C Radio Link Monitoring for Satellite Access | RLM OOS |  |
|  |  | FR1 PCell only |  |
|  |  | non-DRX/DRX | Non-DRX test case can be prioritized. |
|  |  | Other aspects | Discussed in the draft CR directly. e.g. the CORESET configuration for less than 5MHz bandwidth |
| RLM-2 | 8.1C Radio Link Monitoring for Satellite Access | RLM IS |  |
|  |  | FR1 PCell only |  |
|  |  | non-DRX/DRX | Non-DRX test case can be prioritized. |
| BFR-1 | 8.5C Link Recovery Procedures for Satellite Access | SSB based BFD |  |
|  |  | FR1 PCell only |  |
|  |  | non-DRX/DRX | Non-DRX test case can be prioritized. |
|  |  | Other aspects | Can be further discussed when drafting CR (e.g. the gap pattern ID) |
| MR-1 | 9.2C NR intra-frequency measurements for SAN | with SSB index detection |  |
|  |  | Non-DRX/DRX | Non-DRX test case can be prioritized. |
|  |  | No gap | No gap test cases only. |
|  |  | Other aspects | Discussed in the draft CR directly. e.g. the other BWP/CORESET configuration for less than 5MHz bandwidth |
| MR-2 | 9.3C NR inter-frequency measurements for SAN | With SSB index detection |  |
|  |  | Non-DRX/DRX | Non-DRX test case can be prioritized. |
|  |  | No gap | No gap test cases only. |
|  |  | Other aspects | Discussed in the draft CR directly. e.g. the other BWP/CORESET configuration for less than 5MHz bandwidth |
| IDLE-1 | 4.2C Cell Re-selection for NR UE for Satellite Access |  | FFS upon issue 1 |

- References

- RP-240857

- R4-2508261

- 3GPP TS 38.133 v19.1.0