3GPP TSG-RAN WG4 Meeting #116	R4-2509806

Bengaluru, India, August 25th – 29th, 2025

Source: 	Xiaomi

Title: 	Discussion on AIoT system parameters

Agenda item:	7.22.2

Document for:	Discussion

- Introduction

This contribution is to discuss the remaining issues for D2R BW and channel raster according the last meeting’s agreements and open issues [1].

- Discussion

  - D2R bandwidth

The high level principles for D2R BW were agreed in last RAN4 meeting [1]. It was agreed that separate definitions will be used for reader side and device side. The details need more discussion. The followings are copied from the WF [1], confirmation for the equations is needed.

====================================================

- Using following equation for BS as starting point:

D2R CBW for BS (KHz)=(2SB Transmission BW*(1/2)+2* Small frequency shift)/0.9

=(2000*(1+R)/Tb)* (1+∣SFO∣)/0.9

=(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)/0.9

  - Define [200kHz]，[3.54MHz] D2R CBW for AIoT BS. Other D2R CBWs for BS are FFS

- Using following equation for device as starting point:

D2R CBW for device (KHz)=2SB Transmission BW*(1/2)+2* Small frequency shift

=(2000*(1+R)/Tb)* (1+∣SFO∣)

=(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)

====================================================

In order to decide the equations, some basic relationships between the parameters need to be clarified. The followings are the concepts.

Tb: time duration corresponding to a bit (that is after FEC if applied)

Tc: D2R chip length, for Manchester line code, Tc=Tb/2

R: Repetition number, R= Tb/(2 × Tc)

SFS (Small Frequency Shift): +/- R/Tb Hz

SFO (Sampling Frequency Offset): It is assumed 10% for device.

If there’s no SFS and SFO, 2SB CBW occupied by the data is 1/Tc=2/Tb which is the same with the 2SB TBW. When there’s SFO, it is agreed that CBW needs to consider the SFO. Then the maximum 2SB CBW is 1/Tc *1.1 = 2/Tb*1.1. When each Manchester codeword is repeated by a codeword repetition number R, within the same time duration Tb corresponding to a bit (that is before Manchester line code), where R = Tb/(2 × D2R chip length), D2R transmission with frequency location of small frequency shift R/Tb Hz. So the 2SB CBW is increased to 2/Tb*1.1+2*R/Tb*1.1 because the SFS should also take SFO into account. Finally, the D2R CBW for device side is (2+2R)/Tb*1.1= (1+R)/Tc*1.1. If extra guard band is need to be added for BS side, then the D2R CBW for BS side is (2+2R)/Tb*1.1/0.9= (1+R)/Tc*1.1/0.9.

Proposal 1: The D2R CBW for device side is corrected to (2+2R)/Tb*1.1= (1+R)/Tc*1.1. The D2R CBW for BS side is corrected to (2+2R)/Tb*1.1/0.9= (1+R)/Tc*1.1/0.9.

Regarding how to write the spec, our proposal is to capture both the CBW equation and the final values to the spec.

Proposal 2: Both the CBW equations and the final values are captured to the specs.

  - Channel raster

In last RAN4 meeting, it’s agreed that 10kHz channel raster is used for AIoT to align with NR system. The offset is FFS.

In [3], filter center frequency problem is raised.

Figure1: Diagram of channel raster offset, copied from [3]

We’re not sure if the filter refers to digital filter in BB or the analog filter in RF chain. If it refers the digital filter, it’s true that the digital filter’s center frequency doesn’t align with the center frequency of the BB carrier, which is naturally because of the IFFT operation of OFDM modulation. Setting 1/2 SCS frequency offset to the center frequency of BB carrier before it enters the digital filter can solve the issue of misalignment of the two center frequencies but it doesn’t mean the channel raster needs to be shifted. The LO in the following path of the Tx path need to set the final center frequency back to guarantee the orthogonality between the NR and AIoT. The same logic applies to analog filter, but there may no way to set the raster 1/2 SCS back if the LO is already set to NR raster + offset. There’s no way to set it back then non-orthogonality exist for NR and AIoT. So the solution in [3] seems an implementation solution without need to set offset to channel raster.

Observation: The frequency offset solution in [3] seems an implementation solution to solve the filter problem, but the channel raster shouldn’t be changed.

From the above analysis, it seems there’s some concept confusion of channel raster concept. In our understanding, channel raster is always in the center of the center RE as clarified in spec TS 38.101-1,

From this understanding, if the AIoT channel raster is changed, then the orthogonality between AIoT and NR doesn’t exist. So we think no offset is needed for the AIoT channel raster compared with NR.

Proposal 3: No offset is needed for the AIoT channel raster compared with NR.

- Conclusion

This contribution provides our views on the remaining issues of AIoT system parameters. We have the following proposals.

Proposal 1: The D2R CBW for device side is corrected to (2+2R)/Tb*1.1= (1+R)/Tc*1.1. The D2R CBW for BS side is corrected to (2+2R)/Tb*1.1/0.9= (1+R)/Tc*1.1/0.9.

Proposal 2: Both the CBW equations and the final values are captured to the specs.

Proposal 3: No offset is needed for the AIoT channel raster compared with NR.

- References

[1] R4-2508116, “WF on A-IoT_device”, CMCC, RAN4#115

[2] R4-2507573, “Topic summary for [115][134] A-IoT_device”, CMCC, RAN4#115

[3] R4-2505541, “A-IoT general aspects”, Huawei, HiSilicon, RAN4#115

[4] R4-2506229, “Discussion on system parameters for ambient IoT”, Spreadtrum,UNISOC, RAN4#115