3GPP TSG-RAN WG4 Meeting #116	R4-2509808

Bengaluru, India, August 25th – 29th, 2025

Source: 	Xiaomi

Title: 	Discussion on AIoT CW RF requirements

Agenda item:	7.22.3.3

Document for:	Discussion

- Introduction

This contribution provides our views for CW OFF power and the phase noise requirements based on the last RAN4 meeting’s progress [1].

- Discussion

2.1 ON/OFF power

The agreement in last RAN4 meeting is that OFF power is defined as -50dBm, RBW is FFS. Our understanding is that CW node is like an equipment node not a device node. So if there’s no implementation difficulty, use a smaller RBW is beneficial for the system.

Proposal 1: CW off power MBW is defined as 1MHz.

2.2 Phase noise

If BS doesn’t have phase cancellation capability, CW phase noise contribution to the noise is negligible that RF requirement should be defined. The methodology to derive CW phase noise is not discussed in detail. We used the link budget to see phase noise impact in our contribution [2]. It was observed that -120 dBc/Hz @ 7.5kHz offset to CW signal is needed to achieve 3 dB SNR for 20m distance for D2R, CW2D, CW2R. It also can be seen that If BS doesn’t have phase noise cancellation capability, phase noise becomes the limitation factor of D2R coverage.

Observation 1: If BS doesn’t have phase noise cancellation capability, phase noise becomes the limitation factor of D2R coverage.

In the D2R LLS simulation, SNR threshold is not that big, ~ -7 dB may be a reasonable assumption. The deployment also can help to handle the phase noise impact. The following figure shows D1T1-B scenario. If the active CW node is far away from reader, the

Figure 1: D1T1-B scenario

The following link budgets are the updates to [2] with the assumption that the CW2R distance is larger than CW2D and D2R distances.

Table 1: CW2R link budget

| CW2R link |  |
| --- | --- |
| CW Tx power (dBm) | 33 |
| CW Tx antenna gain | 6 |
| CW phase noise (dBc/Hz) | -110 |
| CW2R distance (m) | 30 |
| FSPL CW2R (dB) | 62.9 |
| Reader Rx antenna gain (dBi) | 6 |
| Reader Rx phase noise power at antenna port (dBm/Hz) | -127.9 |

Table 2: CW2D link budget

| CW2D link |  |
| --- | --- |
| Reader CW Tx power (dBm) | 33 |
| Reader CW Tx antenna gain | 6 |
| CW2D distance (m) | 10 |
| FSPL CW2D (dB) | 53.3 |
| Device Rx antenna gain (dBi) | 0 |
| Device Rx CW power at antenna port (dBm) | -14.3 |

Table 3: D2R link budget

| D2R link |  |
| --- | --- |
| Device antenna gain (dBi) | 0 |
| Device backscattering loss (dB) | 10 |
| Device Tx radiated power (dBm) | -24.3 |
| D2R distance (m) | 20 |
| FSPL D2R (dB) | 59.4 |
| Reader Rx antenna gain (dBi) | 6 |
| Reader Rx device power at antenna port (dBm) | -77.7 |
| Reader Rx device power at antenna port (dBm/Hz) | -129.5 |

From the above analysis, if CW node deployment is optimized or the proper CW nodes are activated, the phase noise impact to the D2R signal can be reduced.

Observation 2: Optimizing the deployment of CW nodes or carefully activating the proper CW nodes can help to reduce the phase noise impact.

For the exact requirement, there’re two aspects need to be decided. Frequency offset and the phase noise requirement are tradeoff for the CW node implementation. From system requirement, as there’re not clear guidance on the criteria to make a decision. Then our proposal is to define the requirement according to the current state of art of the oscillator.

Proposal 2: Define the phase noise requirement according to the state of art of oscillator.

For the frequency offset, 7.5kHz offset can be considered to make the channel on the first side lobe of 1/7.5kHz bit rate data usable.

Proposal 3: 7.5kHz frequency offset to CW frequency can be considered as the start point of the frequency offset of CW phase noise requirement.

For the requirement, -110 dBc/Hz from 7.5kHz frequency offset to CW frequency can be considered if it’s not very difficult with the state of the art.

Proposal 4: Phase noise can be defined as -110 dBc/Hz from 7.5kHz frequency offset to CW frequency if it’s not very difficult with the state of the art.

- Conclusion

This contribution provides our further analysis for AIoT CW requirement. We have the following observations and proposals.

Proposal 1: CW off power MBW is defined as 1MHz.

Observation 1: If BS doesn’t have phase noise cancellation capability, phase noise becomes the limitation factor of D2R coverage.

Observation 2: Optimizing the deployment of CW nodes or carefully activating the proper CW nodes can help to reduce the phase noise impact.

Proposal 2: Define the phase noise requirement according to the state of art of oscillator.

Proposal 3: 7.5kHz frequency offset to CW frequency can be considered as the start point of the frequency offset of CW phase noise requirement.

Proposal 4: Phase noise can be defined as -110 dBc/Hz from 7.5kHz frequency offset to CW frequency if it’s not very difficult with the state of the art.

- References

[1] R4-2508101, “WF on A-IoT BS and CW requirements”, Huawei, RAN4#115

[2] R4-2505756, “Discussion on AIoT CW RF requirements”, Xiaomi, RAN4#115