3GPP TSG-RAN WG4 Meeting #116	R4-2509810

Bengaluru, India, August 25th – 29th, 2025

Source: 	Xiaomi

Title: 	Discussion on ModifiedMPR-Behavior and spec

Agenda item:	4.2.1

Document for:	Discussion

- Introduction

A WF [1] on ModifiedMPR-Behavior was approved that some general issues are identified to be further checked to solve the issues.

- Discussion

We provided some analysis in [2]. Generally, our views are not changed much in this meeting. The followings are the views according the open issues identified in the WF [1].

- Check the difference between bit ‘0’ and absent from BS and UE perspective.

From UE perspective, we don’t see much difference between bit ‘0’ and absent if the spec is written clearly. Set the bit to ‘0’ may need more processing but it’s very limited because if there’s possibility to set this bit to ‘1’, the memory space for this bit should be reserved. So from our understanding, it’s ok for both approaches technically. Considering changing the approach to absent may cause NBC issue, we prefer to keep current approach, i.e. if UE follows legacy requirements, this bit is set to ‘0’.

Observation 1: Not much difference is seen between bit ‘0’ and absent from UE perspective.

Proposal 1: Current approach is not changed when the legacy requirements are applicable, i.e. the bit is set to ‘0’.

- Check if the spec version number can be removed from next release.

For the spec version number, our understanding is that if the requirements are not changed from that release to the latest release, the version number can be removed. If the requirements are changed later, the spec version number should be kept.

Proposal 2: If the requirement is unchanged compared with the latest spec, the spec version number can be removed. Otherwise, the spec version number is kept.

- Check if the requirements are changed several times, how to handle the legacy bits.

In current spec, different approach is used. Some band uses separate bits, for example,

| n96 | 0 (leftmost bit) | Support of all band n96 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 | This bit may be set to 1 by a Rel-16 UE to indicate support of all network signalling labels for n96 as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 |
| --- | --- | --- | --- |
|  | 1 | Support of all band n96 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 | This bit may be set to 1 by a Rel-16 and Rel-17 UE to indicate support of all network signalling labels for n96 as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 |
| n102 | 0 (leftmost bit) | Support of all band n102 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 | This bit may be set to 1 by a Rel-16 UE to indicate support of all network signalling labels for n102 as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 |
|  | 1 | Support of all band n102 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 | This bit may be set to 1 by a Rel-16 and Rel-17 UE to indicate support of all network signalling labels for n102 as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 |

Some band uses the same bit but the bit has different meaning for different release.

| n104 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |
| --- | --- | --- | --- |

We prefer separate bit approach because it’s clearer.

Proposal 3: For the requirements which are changed several times, a separate ModifiedMPR-Behavior bit is used for a different requirement.

- Check if any good approach on how to handle the bits for different form factors, if dedicated bits should be used to differentiate the form factors, FWA and smart phone, for future new cases.

For the requirements for different formfactors, current spec uses the same bit for FWA and handheld UE. Taking n104 as the example, the different form factors use the same bit, which makes the spec not easy to be understood especially the handheld UE requirements are also changed. Separate bits for different form factors can make the spec more readable.

Proposal 4: For the requirement for a different form factor, a separate ModifiedMPR-Behavior bit is used.

There’s another issue that for there will be some agreement that cleanup is needed, which release can be changed. Our understanding is that wording change can be applied to earlier release, but for any new bits, they should be added from R20 earliest considering R19 spec is to be frozen soon.

Proposal 5: Only wording can be updated for the earlier release specs on ModifiedMPR-Behavior bit.

Proposal 6: If new bits are added, it should be added from R20.

- Conclusion

This contribution provides our updated views of ModifiedMPR-Behavior and how to handle the spec according to the discussion in last RAN4 meeting [1].

For the open issues in the WF [1], we have the following observations and proposals,

Observation 1: Not much difference is seen between bit ‘0’ and absent from UE perspective.

Proposal 1: Current approach is not changed when the legacy requirements are applicable, i.e. the bit is set to ‘0’.

Proposal 2: If the requirement is unchanged compared with the latest spec, the spec version number can be removed. Otherwise, the spec version number is kept.

Proposal 3: For the requirements which are changed several times, a separate ModifiedMPR-Behavior bit is used for a different requirement.

Proposal 4: For the requirement for a different form factor, a separate ModifiedMPR-Behavior bit is used.

For spec handling,

Proposal 5: Only wording can be updated for the earlier release specs on ModifiedMPR-Behavior bit.

Proposal 6: If new bits are added, it should be added from R20.

- References

[1] R4-2507957, “General rules for modifiedMPR behavior and the spec improvement”, Xiaomi, RAN4#115

[2] R4-2505757, “Discussion on ModifiedMPR-Behavior and spec”, Xiaomi, RAN4#115