3GPP TSG-RAN4 Meeting #106	R4-2509812

# Bengaluru, India, 25th August 2025 – 29th August 2025

Agenda Item:	5.15.1

Source: 	Xiaomi

Title: 						UE capability on UL switching time between NUL carrier and SUL carrier

Document for:	Discussion

- Introduction

In previous companies, companies found that there is inconsitent in existing specification for the swicthing time between NUL carrier and SUL carrier.  0us swithing time is mandated for SUL-NUL carrier switching,  meanwhile in Rel-18 under MC enhancement WI, non zero swicthing time was introduced according to switchingPeriodFor1T-r18 parameter within UE capability IE ULTxSwitchingBandPair-r18 or ULTxSwitchingBandPair-v1840.

CR R4-2504164 was agreed to fix this inconsistent and allow non-zero switching period for SUL and NUL carrier switching from Rel-18 onwards.

In this contribution, we discuss how to address the issue for early release UE (earlier than Rel-18) which support SUL and NUL carrier switching with non-zero period swicting time.

- Discussion

The motivation is well explained in RP-250993 (slide 4), UE may support single Tx only including PA and DC power supplier (Power tracker) which require switching time for UL carrier switching between NUL carrier and SUL carrier. 0us switching time mandate UE to implement two separate Tx chains with multiple PA and DC power suppliers.

The feature NUL and SUL carrier switching was introduced in Rel-15, to allow early release UE (earlier than Rel-18) can support this feature with non-zero period swicthing time. We propose extend UE capability IE ULTxSwitchingBandPair-r18 or ULTxSwitchingBandPair-v1840 to Rel-15.

Proposal 1: Introduce non-zero swicthing period capability for SUL + NUL carrier band combination from Rel-15.

Proposal 2: Reuse existing capbailiy IE  ULTxSwitchingBandPair-r18 or ULTxSwitchingBandPair-v1840.

A draft LS to RAN2 was also provided to inform RAN2 of using Rel-18 capability IEs for earlier release.

Reference

[1] R4-2504164, (NR_MC_enh) R18 correction of switchig time between NUL and SUL carriers, OPPO

[2] RP-250993, RAN4 R20 RF enh, OPPO

[3] R4-2509823 RAN2 on switching period capability for NUL and SUL carrier switching