3GPP TSG-RAN4 Meeting #106	R4-2509819

# Bengaluru, India, 25th August 2025 – 29th August 2025

Agenda Item:	7.1.2.2.1

Source: 	Xiaomi

Title: 	Power domain enhancements for single carrier

Document for:	Discussion

- Introduction

In Rel-19 UE RF WI, one of major objective was to study and specify power enhancement on single carrier with emission requirements relaxtion:

- Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier - Include the following scenarios: - when there is no adjacent in-band/out-of-band co-existence issue - when a UE uses a narrower channel bandwidth within a wider BS bandwidth - Include both (e)RedCap UE (only PC3) and non-RedCap UE - Limited to QSPK and 16QAM

In this contributions, views on remaning open issues were provided.

- Discussion

Sub-topic 1-1: Approaches to enable MPR reduction for both scenario 1 and scenario2 / Sub-topic 1-2: Applicable requirements

#### Issue 1-1-1: Extension values/ratios - Agreement in main session: - At least ½ and ¼ are introduced as the extension ratio to apply to both single-sided and dual-sided cases - It is FFS on the details of symmetric and asymmetric cases for dual-sided case where non-zero extension is applied for both sides. - It is FFS on details of the single side case. #### Issue 1-1-2: Guard band for extended CBW - WF - GB for extended CBW is a virtual concept to consider the boundary applying the shifted OOBE, which is not reflected in the specification and it can be calculated by extended CBW, UE CBW, NRB and extended NRB - Extended CBW is not limited to the existing UE CBW - RAN4 studies whether the extended BW emissions can be guaranteed by only measuring the existing CBW supported in the band (close to the extended BW) given the relaxed extended GB and preliminary measurements performed suggests #### Issue 1-1-3: Derivation of new inner RB region in extended CBW - WF - The following approach is considered as starting point to be further discussed in next meeting, notation should be further considered as well - For NRB based RB_start coordinate system origin, the formula of new inner region can be changed as below: - RBStart,Low_new = max(0, RBStart,Low – RBshift_low); - RBStart,High_new = min(NRB – LCRB, NRB + RBshift_high – RBStart,Low_new – LCRB); - RBStart,Low_new ≤ RBStart ≤ RBStart,High_new, and LCRB ≤ ceil((NRB+ RBshift_low + RBshift_high)/2); - RBshift_low, RBshift_high ≤ NRB/2 - Other approach is not precluded - The approach shall be generic to cover all possible extension cases, including symmetric, asymmetric and different extension ratios

- Sub-topic 1-2: Applicable requirements #### Issue 1-2-1: Derivation of the extended IBE - Agreement in main session: - Fixed IBE used for extended CBW at each side of UE CBW as the baseline, the value is derived from the outmost allocated RB of UE CBW (illustrated in a figure to be developed) - The details and the related test cases can be discussed and decided later. - If significant issue is identified, other options can be further considered. - WF Further discuss whether to apply the extended IBE in the GB of extended CBW. - Option1: Yes - Option2: No #### Issue 1-2-2: Relaxation of the extended IBE - Options - Option 1: using BPSK EVM in the IBE formula for all extended RBs - Option 2: no relaxation is needed. - WF If there is no obvious system performance impact observed, option 1 is adopted

The unclear part is where to apply out of band emission by shifting based extended CHBW and the applicable requirements in the guard-band region of extend CBW. What’s applicable requirements within the guard-band of original CHBW.

We suggest extended RB region can be applied based on Transmission BW configuration NRB and extend CHBW can be applied based on original UE CHBW. On the guard-band decision, based on NW indication on the ratio, UE can automatically get the guard-band size on each side.

Proposal 1: Extend CBW and extended RB region is decided based on original CHBW and NRB with network indication with following equation:

- Extend CHBW = R_low* CHBWUE +R_high *CHBWUE  + CHBWUE

- RB_shift_low= floor (NRB*R_low)

- Guard-band_low = R_low* CHBWUE + CHBWUE/2 -

Proposal 2: The guard-band on extended CBW can be decided by extend CHBW and extended RBs.

On applicable requirements, we prefer no requirements within guard-band of extended CHBW.

Proposal 3: No requirements applied for guard-band of extended CHBW.

Proposal 4: Using BPSK EVM in the IBE formula for all extended RBs

Sub-topic 1-3: Signaling aspects

#### Issue 1-3-1: NW signaling - Agreement in main session: - The power enhancement feature is enabled by the gNB RRC signalling according to UE capabilities, - The details of the signaling including the contents are FFS - WF - It is up to the NW to judge the gap width on both sides of the UE - NW sends info of extension ratio (at least 1/2) per side #### Issue 1-3-2: UE capability - Agreement in main session: - No separate UE capabilities for scenario 1 and scenario 2 - Capability 1: Indicates whether UE supports MPR reduction for single UL carrier with 1/2*UE CBW extension only - Both dual-sided and single sided cases are supported. - Capability 2: Indicates whether UE supports MPR reduction for single UL carrier with both ½ and 1/4*UE CBW extension - Both dual-sided symmetric and single sided cases are supported. - For dual-sided case, it is FFS if asymmetric cases and a separated capability is introduced. - Note: the other ratio number depends on discussion of issue 1-1-1

Extended CBW approach was adopted for MPR reduction in last RAN4 meeting. From deployment scenario and network scheduling perspective, extended CBW/extended RB numbers on each side of UE CHBW can have wide range with the granularity from zero to half of maximum CHBW in FR1 i.e., 50MHz CHBW.  On the other side, the concept of extended CBW was used to introduce MPR reduction by converting outer RB allocation to inner RB allocation. From RAN4 requirements perspective, we shall consider the actual impact to MPR reduction, evaluation work and test effort to decide confined extend CBW range or fixed values for RAN4 requirement introduction.

We believe single sided cases and symmetric dual-sided cases already can serve the purpose and operating scenario well enough.  If NW has asymmetric cases with different ratio, network can inform UE with small ratio on both sides as well as both sides can meet the condition or NW can inform UE with single side case pending on UE capability.

Introduce asymmetric dual-sided case bring additional UE test and implementation complexity w/o clear benefits and common scenarios.

To minimize test effort, we prefer to test only dual-sided case for conformance test.

Proposal 5: Support UE capability 1 and capability 2 with single value {1/2} only or multi-values {1/2, 1/4}

Proposal 6: Only consider dual-sided symmetric and single sided cases

- UE capability 1: {1/2, 0}, {0,1/2}, {1/2, 1/2} cases supported

- UE capability 2: {1/2, 0}, {0,1/2}, {1/2, 1/2}, {1/4, 0}, {0, 1/4}, {1/4, 1/4} cases supported

Propose 7: Not consider to support asymmetric dual-sided case.

Proposal 8: NW inform UE with the fixed ration and direction information as mentioned above cases per UE reporting capabilities.

Proposal 9: Only apply conformance test case for symmetric dual-sided cases based on UE capabilities.

3		Conclusion

In this controbution, views on remaining open issues are provided.

Proposal 1: Extend CBW and extended RB region is decided based on original CHBW and NRB with network indication with following equation:

- Extend CHBW = R_low* CHBWUE +R_high *CHBWUE  + CHBWUE

- RB_shift_low= floor (NRB*R_low)

- Guard-band_low = R_low* CHBWUE + CHBWUE/2 -

Proposal 2: The guard-band on extended CBW can be decided by extend CHBW and extended RBs.

On applicable requirements, we prefer no requirements within guard-band of extended CHBW.

Proposal 3: No requirements applied for guard-band of extended CHBW.

Proposal 4: Using BPSK EVM in the IBE formula for all extended RBs

Proposal 5: Support UE capability 1 and capability 2 with single value {1/2} only or multi-values {1/2, 1/4}

Proposal 6: Only consider dual-sided symmetric and single sided cases

- UE capability 1: {1/2, 0}, {0,1/2}, {1/2, 1/2} cases supported

- UE capability 2: {1/2, 0}, {0,1/2}, {1/2, 1/2}, {1/4, 0}, {0, 1/4}, {1/4, 1/4} cases supported

Propose 7: Not consider to support asymmetric dual-sided case.

Proposal 8: NW inform UE with the fixed ration and direction information as mentioned above cases per UE reporting capabilities.

Proposal 9: Only apply conformance test case for symmetric dual-sided cases based on UE capabilities.

Reference

[1] R4-2420527, WF on Power domain enhancement, Huawei

[2] R4-2503013	WF on power domain enhancement, Huawei

[3] R4-2505099	WF on power domain enhancements, Huawei

[4] R4-2507930	WF on NR_ENDC_RF_Ph4_part1, Huawei