3GPP TSG-RAN4 Meeting #106	R4-2509822

# Bengaluru, India, 25th August 2025 – 29th August 2025

Agenda Item:	8

Source: 	Xiaomi

Title: 		         Input on Rel-19 UE feature list

Document for:	Discussion

- Introduction

This contribution includes the RAN4 UE feature list for Rel-19 NR based on agreed feature list from last RAN4 meeting (R4-2508077) with change marks on the update.

- NR_ENDC_RF_Ph4

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46. NR_ENDC_RF_Ph4 | 46-1 | MPR enhancement for activated carrier | Indicates whether UE supports reduced MPR for single CC if single CC is activated for intra-band UL contiguous CA. |  | Yes | N/A | The FR1 UE shall support MPR requirement based on UL CA configuration even for single activated CC. | Per BC | No | FR1 only | N/A |  | Optional with capability signaling |
|  | 46-2 | FR2 MPR-Improvement Downlink Independent | Indicates whether UE supports reduced MPR by removing dependence on DL CA configuration. |  | Yes | N/A | The FR2 UE only supports MPR requirement with dependence of DL CA configuration. | Per BC | Yes | FR2 only | N/A |  | Optional with capability signaling |
|  | 46-3 | FR2 MPR Improvement Activation Dependent | Indicates whether UE supports MPR based on activation status of its configured CCs and when all activated CCs form a contiguous block in both UL and DL for intra-band contiguous CA |  | Yes | N/A | The FR2 UE shall support MPR requirement based on UL CA configuration. | Per BC | Yes | FR2 only | N/A |  | Optional with capability signaling |
|  | 46-4 | MPR reduction for single carrier with single value | Indicates whether UE supports MPR reduction for single UL carrier with 1/2*UE CHBW extension for dual-sided symmetric and single-sided case Supported cases as following ({ratio value on low side, ratio value on high side} ) based on NW indication: {1/2, 0}, {0, 1/2}, {1/2, 1/2} |  | Yes | N/A | The FR1 UE shall support MPR requirement in existing spec without enhancement. | [Per band] | No | FR1 only | N/A | This capability is applicable for single UL CC case | Optional with capability signaling |
|  | 46-5 | MPR reduction for single carrier with multiple values | Indicates whether UE supports MPR reduction for single UL carrier with 1/X*UE CHBW extension for dual-sided symmetric and single-sided case, where 1/X is a set of ratios equal and smaller than 1/2 {1/4, 1/2} Supported cases as following ({ration value on low side, ratio value on high side}) based on NW indication: {1/2, 0}, {0, 1/2}, {1/2, 1/2}, {1/4, 0}, {0,1/4}, {1/4, 1/4} |  | Yes | N/A | The FR1 UE shall support MPR requirement in existing spec without enhancement. | [Per band] | No | FR1 only | N/A | This capability is applicable for single UL CC case | Optional with capability signaling |
|  | 46-6 | Support maximum 6 MIMO layers for DL reception | Indicates whether the UE supports maximum 6 spatial multiplexing layers for DL reception for FWA. |  | Yes | N/A | FWA does not support maximum 6 MIMO layers for DL reception | Per FSPC | N/A | FR1 only | N/A |  | Optional with capability signaling |
|  | 46-7 | Support maximum 4 layers for DL reception with 6Rx | Indicates whether the UE supports maximum 4 spatial multiplexing layers for DL reception with 6Rx |  | n/a | n/a | UE does not support maximum 4 MIMO layers for DL reception with 6Rx | N/A | no | FR1 only | N/A |  | Optional without UE capability signaling |

- NR_LBCA_SW

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 55. NR_LBCA_SW | 55-1 | Switching period for switching between a LB FDD band and an SDL band | UE to indicate support of LB-LB carrier aggregation via switching between an FDD band (case 1) and SDL band (case 2): Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 switchingPeriodForFDD-SDL indicates the length of the switching time between {case1, case2}: 35us represents 35 us of switching time, 70us represents 70us of switching time,140us represents 140us of switching time, as specified in TS 38.101-1. | N/A | Yes | N/A | Operation of Carrier aggregation between FDD and SDL via switching is not supported. | Per band pair per band combination |  | Applicable to FR1 only |  |  | Optional with capability signalling |

# NR_AIML_air

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_AIML_air |  | Support known Rx beam condition for Tx beam prediction | Indicates whether UE can apply known Rx beam condition to TCI state activation procedure if the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam | N/A | Yes | No | additional RX beam sweeping delay during TCI state activation procedures | Per UE | No | YES | N/A |  | Optional with capability signaling |