3GPP TSG-RAN WG4 Meeting #116		R4-2509833

Bengaluru, India, August 25th – 29th, 2025

Agenda item:		7.20.4.1

Source:		Samsung

Title:	Discussion on RRM requirements of UE-initiated beam management in Rel-19 MIMO phase 5

Document for:		Discussion

- Introduction

In last RAN4#115 meetings, the WF was approved in [1]. In this contribution, we continue to discuss the open issues of RRM requirements for UE-initiated/event-driven beam management.

- Discussion

In RAN4#115 meeting, the latest agreement is approved in [1] as:

#### Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19? <Way forward > - Option 1: (Qualcomm, Apple, LGE) - Do not define requirements for eventInstanceCount-r19 > 1 - Option 2: (China Telecom, Nokia, Xiaomi, Samsung, CMCC, Huawei, Ericsson, ZTE, MTK) - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19

We have already agreed the measurement period of basic behavior with no time window based. It is the same as the case of time window based + eventInstanceCount-r19 = 1.

The configurable eventInstanceCount-r19, can avoid the instantaneous results to guarantee the better performance with valid counter. In addition, it can avoid like ping-pong behavior if NW use this event-triggered L1-RSRP results.

Some companies are discussed this time window are the same as TTT in existing NR therefore no need to define any requirements related to TTT.

We don’t think it is the same as TTT. TTT is a Time during which specific criteria for the event needs to be met in order to trigger a measurement report in each event. In L3 measurement, for each MO, the measurement is based on samples of SMTC. The measurement time are based on the SMTC period for both cells. During some time, UE can decide the event is met or not. In L1 measurement, the time length of the window is the configured time window length. If no definition of how to perform on each or which RS to perform the L1 measurement. The UE behavior is not clear and the performance cannot be guaranteed.

Proposal 1: RAN4 to specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19.

For how to specify the requirements when eventInstanceCount-r19 > 1, RAN4 can use the basic measurement period as basic unit, the measurement period is at least be larger than max{eventInstanceCount-r19 * measurement period (for basic), Ttime_window}. Then in the test case, one test case can be defined as eventInstanceCount-r19 = 2 or 3 and change the beam condition in different T to verify whether UE can trigger the report correctly.

Proposal 2: For the requirements, the measurement period is at least be larger than max{eventInstanceCount-r19 * measurement period (for basic), Ttime_window}

#### Issue 1-6: Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time? <Way forward > - Option 1: (Nokia, Ericsson, MTK) - No need to capture this scenario in RAN4 requirements - Option 2: (Xiaomi, Samsung) - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart. - Option 3: (Apple) - Add the applicability rule of event triggered L1 measurement: - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE. - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

For event-2/event-1, the measurement is based on current beam. The RS of current beam is coming from indicated TCI state. If UE received TCI state switching based on MAC-CE or DCI, it means the indicated TCI state has been changed. The measurement of L1 evaluation time should be changed after the MAC-CE command or DCI indicator. If the event has been triggered, the L1 evaluation time should be restart.

Proposal 3: The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart.

#### Issue 1-8: Capabilities for Support of Event Triggering and Reporting Criteria (similar Capabilities in 38.133 9.1.4)? <Way forward > - Option 1: (Samsung, Apple, Huawei, Ericsson) - For event trigger L1 reporting, do not define such capability in parallel per category like L3. - Option 2: (Nokia, MTK) - Define capabilities for support of UE-initiated/event-driven beam reporting on top of existing L3 events in section 9.1.4. For intra-frequency, numbers of event = X, where X is candidate value of [3 or 6 or 9]. #### Issue 1-9: Compatibility of capabilities with LTM <Way forward > - If such capabilities are defined in Issue 1-8, then discuss on: - Option 1: - The capability for supported number of events triggered L1 measurement reporting is defined separately for Rel-19 mobility and Rel-19 MIMO. - Option 2: - The capability for supported number of events triggered L1 measurement reporting is defined as common for Rel-19 mobility and Rel-19 MIMO.

In legacy L3 measurement in section 9.1.4, the UE shall be able to support in parallel per category up to Ecat reporting criteria according. Such as

is the total number of NR reporting criteria according to table 9.1.4.2-1, and n is the number of configured NR serving frequencies, including PCell, and SCells carrier frequencies,

Unlike L3 measurement, there is no different types of events. In L3, the different types of measurement can be measured and the serving cell and neighbour cells can measure at the same MO, the decided by software for different events whether they are met or not.

In L1 measurement, the measurement type is only based on RSRP.

For different events, they are configured from the CSI config. It will not be support to measure and report the different events at the same reporting. There is no need to define such capability for support of L1 event trigger and reporting as L3.

Proposal 4: For event trigger L1 reporting, do not define such capability in parallel per category like L3.

Proposal 5: If P4 is agreed, no need to consider such capability compatibility with LTM.

Even if option 2 in Issue 1-8 is chosen, it has been already agreed in Rel-19 mobility WI:

Agreement: - RAN4 shall develop the two features independently at current stage. Further discussion on common capability is allowed when discussing UE feature list of R19.

Proposal 6: Even if there may be capabilities in Issue 1-8. In Rel-15, these capabilities are mandatory in spec. No need to add UE features for this.

UE feature list discussion

In RAN1 LS [2], the UE feature list, the MIMO sub features are captured in [59.X]

For UEIBM, the measurement and beam reporting are specified by 59-1-1 to 59-1-6, no dedicated RAN4 sub features are needed. Even if there may be capabilities in Issue 1-8. In Rel-15, these capabilities are mandatory in spec. No need to add UE features for this.

| Features | Index | Feature group |
| --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-1-1 | UE-initiated/event-driven beam management for Event-2 based measurement and report for Mode A |
| 59. NR_MIMO_Ph5 | 59-1-2 | UE-initiated/event-driven beam management Mode B |
| 59. NR_MIMO_Ph5 | 59-1-3 | Triggering event determination via detecting ≥ M event instances for at least one new beam within a time window. |
| 59. NR_MIMO_Ph5 | 59-1-4 | UE-initiated/event-driven beam management for Event-1 based measurement and report for Mode A |
| 59. NR_MIMO_Ph5 | 59-1-5 | UE-initiated/event-driven beam management for Event-7 based measurement and report for Mode A |
| 59. NR_MIMO_Ph5 | 59-1-6 | First PUCCH and second PUSCH from different PUCCH groups |

For CJTC reporting, the basic features for CJTC-C and CJTC-F are specified by

| 59. NR_MIMO_Ph5 | 59-2-3-1 | CJTC Dd report |
| --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-2-3-2 | CJTC FO report |

For Asymmetric DL sTRP/UL mTRP, the features for two TAs which have RRM impacts are specified by:

| 59. NR_MIMO_Ph5 | 59-4-4a | Support two TAs enhancement for intra-cell beam management | Support of two TAs without the restriction of multi-DCI based multi-TRP operation for intra-cell beam management |
| --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-4-4b | Support two TAs enhancement for inter-cell beam management operation | Support of two TAs without the restriction of multi-DCI based multi-TRP operation for inter-cell beam management |
| 59. NR_MIMO_Ph5 | 59-4-4d | PDCCH ordered sent by one TRP triggers RACH procedure towards a different TRP based on CRFA for inter-cell without CORESETPoolIndex | Support of PDCCH ordered sent by one TRP triggers RACH procedure towards a different TRP based on CRFA for inter-cell |
| 59. NR_MIMO_Ph5 | 59-4-5 | Overlapping UL transmission reduction | Support of reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions when the UE is with two TA enhancement |
| NR MIMO Phase 5 | 59-4-6 | MAC-CE update of PL offset value(s) | Support of MAC-CE update of the configured PL offset value(s) |
| 59. NR_MIMO_Ph5 | 59-4-7a | Extended value range of starting bit of block in DCI format 2_3 | Support of the extended value range of starting bit of DCI format 2_3 |
| 59. NR_MIMO_Ph5 | 59-4-7b | DCI format 2_3 to indicate TPC for one of two separate SRS closed loop indexes | Support DCI format 2_3 to indicate TPC for one of two separate SRS closed loop indexes. |
| 59. NR_MIMO_Ph5 | 59-4-8 | DCI format 1_1 to indicate TPC command for SRS associated with a separate SRS CLPC adjustment state | Support of DCI format 1_1 to indicate TPC command for SRS associated with a separate SRS CLPC adjustment state |
| 59. NR_MIMO_Ph5 | 59-4-9a | DCI format 1_1 to indicate one of two separate SRS closed loop indexes under separate DL/UL TCI state mode | Support of DCI format 1_1 to indicate one or two separate SRS closed loop index(es) under separate DL/UL TCI state mode |
| 59. NR_MIMO_Ph5 | 59-4-9b | DCI format 1_1 to indicate one of two separate SRS closed loop indexes under joint TCI state mode | Support of DCI format 1_1 to indicate one or two separate SRS closed loop index(es) under joint TCI state mode |

Proposal 7: For Rel-19 MIMO Phase 5 RRM, no need to add dedicated RAN4 UE feature list for UEIBM, CJTC reporting, Asymmetric DL sTRP/UL mTRP. They have been covered in RAN1 feature list.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for UE-initiated/event-driven beam management in Rel-19 MIMO phase 5:

Proposal 1: RAN4 to specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19.

Proposal 2: For the requirements, the measurement period is at least be larger than max{eventInstanceCount-r19 * measurement period (for basic), Ttime_window}

Proposal 3: The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart.

Proposal 4: For event trigger L1 reporting, do not define such capability in parallel per category like L3.

Proposal 5: If P4 is agreed, no need to consider such capability compatibility with LTM.

Proposal 6: Even if there may be capabilities in Issue 1-8. In Rel-15, these capabilities are mandatory in spec. No need to add UE features for this.

Proposal 7: For Rel-19 MIMO Phase 5 RRM, no need to add dedicated RAN4 UE feature list for UEIBM, CJTC reporting, Asymmetric DL sTRP/UL mTRP. They have been covered in RAN1 feature list.

- Reference

[1] R4-2508271, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung

[2] R4-2509005, Updated RAN1 UE features list for Rel-19 NR after RAN1 #121

[3] R4-2508458, Draft Big CR on RRM requirements of NR_MIMO_Ph5, Samsung, MediaTek