3GPP TSG-RAN WG4 Meeting #116		R4-2509834

Bengaluru, India, August 25th – 29th, 2025

Agenda item:		7.20.4.2

Source:		Samsung

Title:	Discussion on RRM requirements of other aspects in Rel-19 MIMO phase 5

Document for:		Discussion

- Introduction

In last RAN4#115 meetings, the WF was approved in [1]. There are still open issues in RRM requirements for CJTC reporting and asymmetric DL sTRP/UL mTRP scenarios. In this contribution, we continue to discuss the RRM requirements for asymmetric DL sTRP/UL mTRP scenarios in Rel-19 MIMO phase 5.

- Discussion

CJTC Reporting

In RAN4#115 meeting, the latest agreement is approved in [1] as:

Issue 1-1: How to define measurement period requirements for delay and frequency offset reporting for CJT calibration? Agreement: - RAN4 to discuss the achievable accuracy in terms of delay and frequency offsets based on 1 sample and [3 or 5] samples in the next meeting. - RAN4 to conclude the delay requirement for delay and frequency offset reporting in the next meeting. The delay requirement for phase offset is deprioritized in Rel-19.

In last meeting, the agreements are RAN4 should specify the measurement period for achievable accuracy based on 1 sample or [3 or 5] samples.

By averaging more samples, of course there can be noise suppression benefit. By increasing the SNR condition, this benefit gain will not so obvious

According to the observation of simulation results, there is no huge gain if change the number of samples from 1 to 3.

In legacy Rel-15 L1-RSRP measurement, although there can be different sets of multiple samples in measurement periods. However, the accuracy requirement is based on 1 sample.

In last meeting, some companies proposed that RAN4 to define two sets of requirements with different number of samples and different accuracy requirements. In addition, the question is: whether the parameter of “timeRestrictionForChannelMeasurements” can be reused.

By our understanding, firstly, “timeRestrictionForChannelMeasurements” cannot be used.

In 38.214. 19.0.0, take CJTC-Dd as an example:

##### 5.2.1.4.9 CJTC-Dd reporting of delay offset For a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cjtc-Dd', the interval [δin,δin+1) in which the delay offset, Dn,offset, falls into, for CSI-RS resource set n=0,…,NTRP-1, n≠nref, is indicated by iDO=i0⋯iNTRP-1 in∈{0,1,…,MD-1} where the mapping from in to δin is given by δin=in⋅ADMD-1 and δMD=∞, and the values of AD∈{CP/2, CP}, where CP is the length of the cyclic prefix for symbols other than 0 and 7⋅2μ in the subframes associated with the NTRP configured CSI-RS resource sets for tracking, and MD∈{32, 64, 128, 256} are configured by the higher layer parameters valueOfAD and valueOfMD, respectively. The index value in=MD-1, corresponding to the interval [AD,∞), represents an out-of-range delay offset Dn,offset.

The usage of “timeRestrictionForChannelMeasurements” is in the paralleled clauses. But for CJTC-Dd and CJTC-F measurement, there is no such support.

In addition, for the proposal use two sets accuracy requirements with two sets side condition, we don’t support this solution. Side condition is only known at UE side. Even two sets of side condition can be used in RAN4 tests, it is meaningless in real implementation work for gNB and UE interactive work.

Proposal 1: Do not define multiple sets of requirements with different samples. Define the measurement period based on 1 sample. The SNR condition and accuracy can be decided in performance phase.

Issue 1-2: How to define scheduling restriction requirements for delay and frequency offset reporting for CJT calibration? Way forward: - Option 1: The scheduling restriction is specified as: - For the UE is configured with CJTC reporting, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for CQI on the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting, and on one data symbol before and one data symbol after the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting. - Option 2: No scheduling restrictions needed to be applied for CJT reporting.

Issue 1-3: How to define measurement restriction requirements for delay and frequency offset reporting for CJT calibration? Way forward: - Option 1: Reuse the legacy measurement restriction in FR1 for delay and frequency offset measurement. - Measurement restriction does apply, - If UE does not support simultaneousRxDataSSB-DiffNumerology and - SSB for L1-RSRP with different SCS as PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI and they are fully or partially overlapped on symbols. - Measurement restriction does not apply, - If UE support simultaneousRxDataSSB-DiffNumerology or - SSB for L1-RSRP with the same SCS as CSI-RS/PDSCH/PDCCH. - Option 2: If CJTC measurement collides with other L1 measurement, follow legacy principle, UE is required to measure one of but not both of different measurements when measurement resources are at the same time.

For the scheduling restriction, to achieve accurate estimation, scheduling restriction is needed.

Agreement(RAN1#116bis) For the Rel-19 aperiodic standalone CJT calibration reporting, the dynamic range and resolution parameters for delay offset reporting Dn,offset, i.e. (AD, MD), are NW-configured via higher-layer (RRC) signalling from the following candidate values: - AD ={0.5CP, 0.75CP, CP, 1.5CP, 2CP, , , } where CP and denote the length of the cyclic prefix according to the current specifications (for normal CP) within a slot and the SCS, respectively - MD ={32, 64} In addition, the inside/outside range for the 1-bit indicator dn is equal to [0, CP]. Agreement(RAN1#118) For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for delay offset reporting Dn,offset, i.e. AD, at least support the following values: {0.5CP, CP}

When the UE performed based on measurement on CSI-RS recourses, they are assumed from different TRPs with delay offset and frequency offset. If the delay offset is larger than CP, ISI will degrade the measurement performance. RAN1 introduced the 1-bit indicator to report whether is larger than the length of CP.

To achieve accurate measurement, UE is not expected to receive other DL signals for interfering. We provide it in the draft CR in [3].

Proposal 2: The scheduling restriction is specified as:

For the UE is configured with CJTC reporting, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for CQI on the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting, and on one data symbol before and one data symbol after the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting.

For measurement restriction, if CJTC measurement collides with other L1 measurement, the principle of collied measurement have the same principle in legacy. We provide it in the draft CR in [3].

Proposal 3: If CJTC measurement collides with other L1 measurement, specify the measurement restriction for FR1:

For FR1, when the CSI-RS for CJTC-Dd or CJTC-F measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for L1-RSRP measurement in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement has same SCS than CSI-RS for CJTC-Dd or CJTC-F measurement, the UE shall be able to perform CSI-RS measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement has different SCS than CSI-RS for CJTC-Dd or CJTC-F measurement, the UE shall be able to perform CSI-RS measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for CJTC-Dd or CJTC-F measurement and SSB. Longer measurement period for CSI-RS based CJTC-Dd or CJTC-F measurement is expected, and no requirements are defined.

For FR1, when the CSI-RS for CJTC-Dd or CJTC-F measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for L1-RSRP measurement without any restriction.

Asymmetric DL sTRP/UL mTRP

Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s)

Issue 2-1: For Scenario#1, whether to update RACH requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Way forward: FFS

In the case “if UE is configured with PL offset in joint/UL TCI states(s)”, the UE just maintains one single downlink reference timing. UE does not expect to receive SSB from UL TRP(s). The RACH procedure is: the UE shall transmit RACH in the specific RACH occasion which is linked to SSB in DL/UL TRP(s). It is the same as existing single TRP scenario. There is no PDCCH ordered RACH in this case. No further enhancement for RACH for 2TA enhancement.

Proposal 4: For Scenario#1, no update for RACH requirements for 2TA enhancement. They are the same as legacy single TA solution.

Issue 2-2: For Scenario#1, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Way forward: - Option 1: Gradual timing adjustment applies for each TAG and shall be met for each TAG. - Option 2: The gradual timing adjustment requirements are only applicable to TRP with both UL and DL.

In RAN1#118bis, the agreement is :

Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

It is FFS for the UE autonomous TA adjustment. However, we believe it is because this discussion is for RAN4.

In Rel-18 multi-DCI based multi-TRP operation with two TAs, the gradual timing adjustment applies for each TAG as below:

For multi-DCI based multi-TRP operation with two TAs, gradual timing adjustment specified in this clause applies for each TAG and shall be met for each TAG. The reference point for each TAG for the PUCCH/PUSCH/SRS and PRACH transmission triggered by PDCCH order is defined in clause 7.1.1.

However, in Rel-19 two TA scenario#1, there is only one downlink reference timing. When the propagation delay is changed at UE side, technically, from the scenario, the UE can move by different directions from two TRP in geographical. But there is only one downlink reference timing, the adjustments of UE uplink shall be based on the downlink reference timing. Thus, the gradual timing adjustment can only applicable to the TRP of DL/UL TRP. In RAN2 definition, this is the first TAG.

Proposal 5: The gradual timing adjustment requirements are only applicable to TRP with both UL and DL, which for the first TAG.

Issue 2-3: For Scenario#1, how to define MRTD and MTTD requirements? Way forward: FFS

In the case “if UE is configured with PL offset in joint/UL TCI states(s)”, the UE just maintains one single downlink reference timing. The MRTD and MTTD requirements can reuse the requirements in Rel-18 “multi-TRPs with two TAs for UE not capable of rxTimingDiff-r18”

Proposal 6: For Scenario#1, the MRTD and MTTD requirements can reuse the requirements in Rel-18 “multi-TRPs with two TAs for UE not capable of rxTimingDiff-r18”

- Conclusion

In this contribution, we provide our proposals for RRM requirements for for “CJTC reporting” and “asymmetric DL sTRP/UL mTRP scenarios” in Rel-19 MIMO phase 5:

Proposal 1: Do not define multiple sets of requirements with different samples. Define the measurement period based on 1 sample. The SNR condition and accuracy can be decided in performance phase.

Proposal 2: The scheduling restriction is specified as:

For the UE is configured with CJTC reporting, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for CQI on the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting, and on one data symbol before and one data symbol after the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting.

Proposal 3: If CJTC measurement collides with other L1 measurement, specify the measurement restriction for FR1:

For FR1, when the CSI-RS for CJTC-Dd or CJTC-F measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for L1-RSRP measurement in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement has same SCS than CSI-RS for CJTC-Dd or CJTC-F measurement, the UE shall be able to perform CSI-RS measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement has different SCS than CSI-RS for CJTC-Dd or CJTC-F measurement, the UE shall be able to perform CSI-RS measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for CJTC-Dd or CJTC-F measurement and SSB. Longer measurement period for CSI-RS based CJTC-Dd or CJTC-F measurement is expected, and no requirements are defined.

For FR1, when the CSI-RS for CJTC-Dd or CJTC-F measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for L1-RSRP measurement without any restriction.

Proposal 4: For Scenario#1, no update for RACH requirements for 2TA enhancement. They are the same as legacy single TA solution.

Proposal 5: The gradual timing adjustment requirements are only applicable to TRP with both UL and DL, which for the first TAG.

Proposal 6: For Scenario#1, the MRTD and MTTD requirements can reuse the requirements in Rel-18 “multi-TRPs with two TAs for UE not capable of rxTimingDiff-r18”

- Reference

[1] R4-2508272, WF on RRM requirements for NR_MIMO_Ph5_Part2, MediaTek inc

[2] R4-2508458, Draft Big CR on RRM requirements of NR_MIMO_Ph5, Samsung, MediaTek

[3] R4-2509237, Draft CR on restriction of UE for CJTC reporting