3GPP TSG-RAN WG4 Meeting #116		R4-2509836

Bengaluru, India, August 25th – 29th, 2025

Agenda item:		7.21.4.1

Source:		Samsung

Title:	Views on L1 CLI measurement requirements

Document for:		Discussion

- Introduction

In last RAN4#115 meetings, the WF was approved in [1] and draft big CR was endorsed in [2]. In this contribution, we continue to discuss the open issues of RRM requirements for L1 CLI measurement.

- Discussion

In last RAN4#115 meeting, the potential issues for further discussion are captured in WF [1] as below:

Topic 1: UE-to-UE CLI handling Sub-topic 1-1: General issues Issue 1-1-3: L1 CLI measurement in non-SBFD symbols Sub-topic 1-2: L1-SRS-RSRP Sub-topic 1-3: L1-CLI-RSSI Issue 1-3-1: Measurement period Issue 1-3-3: Measurement restriction

L1 CLI measurement in non-SBFD symbols

According to RAN1 agreements in last RAN1#121meeting:

Agreement For a CSI report associated with periodic/semi-persistent SRS-RSRP measurement resources or CLI-RSSI measurement resources, the valid symbol type for CSI derivation configured in CSI-ReportConfig is applicable for L1 UE-to-UE CLI measurement and reporting if SBFD symbols are configured for the UE - UE shall perform L1 UE-to-UE CLI measurement based on the valid symbol type configured in CSI-ReportConfig The valid symbol type does not apply for a UE if SBFD symbols are not configured for the UE - UE shall perform L1 UE-to-UE CLI measurement based on the configuration provided in CSI-ResourceConfig indicated by CSI-ReportConfig FG 60-1 is not the prerequisite of FG 60-8 and FG60-9

Proposal 1: L1-CLI measurement can perform in SBFD symbols and non-SBFD symbols. RAN4 to define the L1-CLI measurement for it on non-SBFD symbol.

Measurement Restriction for L1-SRS-RSRP

In last RAN4#115 meeting, the agreement in [1] is listed as:

#### Issue 1-2-3: Measurement restriction - Agreement - Take into account following RAN1 agreement in RAN1#121, and further discuss the measurement restriction between Method #1 and Method #2. Agreement A UE does not expect to be configured with L1-CLI-RSSI measurements resources within an UL subband and L1-SRS-RSRP measurement resources on the same symbol..

This issue can be discussed together with Issue of measurement restriction for L1-CLI-RSSI. When the measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, because the reference timing of the different measurement are different. The UE cannot measure them both at the same time. It can be avoided by UE cannot be configured for two types of measurement at the same time or measure either of them to left to UE implementation. In last meeting, we are fine to left to UE implementation. According to the agreement in last RAN1 meeting, for Method #2 and Method#3, we support to aligned with RAN1 agreements. For method #1 and method #2, the same issue is still existing. If we agree above for Method#2 and Method#3, we prefer to use the same principle.

Proposal 2: For Measurement Restriction for L1-SRS-RSRP, for Method #2 and Method#3, UE does not expect to be configured with L1-CLI-RSSI and L1-SRS-RSRP measurement resources on the same symbol. For Method#1 and Method#2, use the same principle.

Measurement period for L1-CLI-RSSI

In RAN4#114bis, RAN4 achieved the agreement as below:

- Agreements (online session on Monday) - For DUD case with method #1 and resource type#2 - In RAN4 understanding, if UE can support simultaneous PDSCH reception on the two DL subbnads for DUD, it can support simultaneous RSSI measurement on the two DL subbands based on 1 sample. - RAN4 to decide the number of samples for measurement period based on RAN1 progress on UE capability. - For method #3, the measurement period is based on 1 sample.

In latest RAN1 spec 38.214.v19.0.0:

##### 5.2.1.4.8 L1-CLI-RSSI Reporting If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to ‘notConfigured’, the UE shall derive the channel measurements for computing L1-CLI-RSSI value reported in uplink, SBFD or [flexible] slot n based on only the CLI-RSSI resources, no later than the CSI reference resource, (as defined in [4, TS 38.211]) associated with the CSI resource setting. If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to ‘Configured’, the UE shall derive the channel measurements for computing L1-CLI-RSSI reported in uplink, SBFD or [flexible] slot n based on only the most recent, no later than the CSI reference resource, occasion of CLI-RSSI resources (defined in [4, TS 38.211]) associated with the CSI resource setting. A UE configured with SBFD symbols and with a CLI-RSSI measurement resource in SBFD symbols and across two DL subbands shall derive the frequency resources for L1-CLI-RSSI measurement by excluding the frequency resources outside the RBs that are both in the active DL BWP and in the DL sub-band and shall report a single wideband L1-CLI-RSSI measurement.

In last meeting, some companies proposed to use “timeRestrictionForChannelMeasurement” to separate requirements for 1 sample and 3 samples. Although in RAN1 spec, the usage is the same as the definition in L1-RSRP, the number of samples are left to RAN4. By our understanding, the RSSI is just the power sum up from measurement resources. There is no big benefit for RSSI. Thus, we slight prefer to keep previous agreement by 1 sample.

Proposal 3: For measurement period for L1-CLI-RSSI, we support previous agreements by 1 sample.

- Conclusion

In this contribution, we provide our proposals for L1 CLI measurement:

Proposal 1: L1-CLI measurement can perform in SBFD symbols and non-SBFD symbols. RAN4 to define the L1-CLI measurement for it on non-SBFD symbol.

Proposal 2: For Measurement Restriction for L1-SRS-RSRP, for Method #2 and Method#3, UE does not expect to be configured with L1-CLI-RSSI and L1-SRS-RSRP measurement resources on the same symbol. For Method#1 and Method#2, use the same principle.

Proposal 3: For measurement period for L1-CLI-RSSI, we support previous agreements by 1 sample.

- Reference

[1] R4-2508274, WF on RRM requirements for NR_duplex_evo, Huawei

[2] R4-2508459, Draft big CR for RRM requirements of Sub-band full duplex, Huawei, Samsung