3GPP TSG-RAN WG4 Meeting #116		R4-2509837

Bengaluru, India, August 25th – 29th, 2025

Agenda item:		7.21.4.2

Source:		Samsung

Title:	Views on RRM requirements for Rel-19 SBFD operation

Document for:		Discussion

- Introduction

In last RAN4#115 meetings, the WF was approved in [1] and draft big CR was endorsed in [2]. In this contribution, we continue to discuss the open issues of RRM requirements for L1 CLI measurement.

- Discussion

In last RAN4#115 meeting, the potential issues for further discussion are captured in WF [1] as below:

### Sub-topic 2-1: RRM impacts of SBFD operation #### Issue 2-1-4: Requirements for RACH requirements Issue 2-1-5 (new): Lmax when using NR-U approach for L1 CSI-RS measurement

Lmax when using NR-U approach for L1 CSI-RS measurement

In last RAN4#115 meeting, we agreed to use NR-U approach for L1-CSI-RS measurement.

In the endorsed CR, RAN4 agreed the NR-U approach for RLM/CBD/L1-RSRP/L1-SINR measurement.

Take L1-RSRP measurement as example:

Table 9.5.4.2-1: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR1 Configuration; TL1-RSRP_Measurement_Period_CSI-RS (ms). non-DRX; max(TReport, ceil((M+L1)*P)*TCSI-RS). DRX cycle ≤ 320 ms; max(TReport, ceil(K *(M+L1)*P)*max(TDRX,TCSI-RS)). DRX cycle > 320 ms; ceil((M+L1)*P)*TDRX. NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. NOTE 3: K = 1 when TCSI-RS ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. NOTE 4: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 or measurementEnhancementCA-r17..

For the value of L1, 1> If UE does not support [SBFD capability] or [SBFD configuration] is not configured by the network 2> L1=0 1> else (if UE supports [SBFD capability] and [SBFD configuration] is configured by the network) 2> if higher layer parameter timeRestrictionForChannelMeasurement is configured 3> L1=0 2> else (if higher layer parameter timeRestrictionForChannelMeasurement is not configured) 3> if UE is configured to report L1-RSRP for SBFD symbols 4> When DRX is not configured, L1 is the number of occasions of the CSI-RS resource that are overlapping with dynamic UL transmission [or with non-SBFD symbols] during TL1-RSRP_Measurement_Period_CSI-RS 4> When DRX is configured, L1 is the number of DRX cycles in which at least one occasion of the CSI-RS resource is overlapping with dynamic UL transmission [or with non-SBFD symbols] during TL1-RSRP_Measurement_Period_CSI-RS 3> if UE is configured to report L1-RSRP for non-SBFD symbols 4> When DRX is not configured, L1 is the number of occasions of the CSI-RS resource that are overlapping with SBFD symbols during TL1-RSRP_Measurement_Period_CSI-RS, when DRX is not configured 4> When DRX is configured, L1 is the number of DRX cycles in which at least one occasion of the CSI-RS resource is overlapping with SBFD symbols during TL1-RSRP_Measurement_Period_CSI-RS

In existing spec, 38.133, the requirement for NR-U for RLM measurement:

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_SSB,CCA period becomes better than the threshold Qin_SSB,CCA within TEvaluate_in_SSB,CCA evaluation period. During the in-sync evaluation procedure, layer 1 of the UE shall not send any in-sync indication for the cell to the higher layers when Lin exceeds Lin,max, where Lin and Lin,max are defined in table 8.1A.2.2-1. Table 8.1A.2.2-1: Evaluation period TEvaluate_out_SSB,CCA and TEvaluate_in_SSB,CCA for FR1 Configuration; TEvaluate_out_SSB,CCA (ms); TEvaluate_in_SSB,CCA (ms). RLM-RS SSB Es/IotNote4 ≥-7 dB; RLM-RS SSB Es/Iot Note4 <-7 dB. no DRX; Max(200, Ceil(17*P)*TSSB); Max(200, Ceil(24*P)*TSSB); Max(100, Ceil((5+Lin)*P)*TSSB). DRX cycle≤320; Max(200, Ceil(1.5*15*P)*Max(TDRX,TSSB)); Max(200, Ceil(1.5*20*P)*Max(TDRX,TSSB)); Max(100, Ceil(1.5*(5+Lin)*P)*Max(TDRX,TSSB)). DRX cycle>320; Ceil(13*P)*TDRX; Ceil(16*P)*TDRX; Ceil((5+Lin)*P)*TDRX. NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. NOTE 2: When DRX is not configured, Lin is the number of RLM-RS SSB occasions which are not available at the UE during TEvaluate_in_SSB,CCA, where Lin ≤ Lin,max. When DRX is configured, Lin is the number of DRX cycles in which at least one RLM-RS SSB occasion is not available at the UE during TEvaluate_in_SSB,CCA, where Lin ≤ Lin,max. The UE is not required to determine the availability of SSB occasions more frequent than Once per Max(10 ms, P * TSSB) if no DRX is used, Once per Max(10 ms, Ceil(1.5 * P) * Max(TDRX, TSSB)) if DRX cycle ≤ 320 ms, Once per P * TDRX if DRX cycle > 320 ms. NOTE 3: Lin,max=7 for Max(TDRX,TSSB) ≤ 40 assuming TDRX=0 for non-DRX case, Lin,max=5 for 40<Max(TDRX,TSSB)≤320, Lin,max=3 for TDRX>320. NOTE 4: RLM-RS SSB Es/Iot is the averaged Es/Iot over the most recent previous out-of-sync evaluation period..

In existing spec, 38.133, the requirement for NR-U for BFD/CBD measurement:

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_SSB,CCA period becomes better than the threshold Qin_SSB,CCA within TEvaluate_in_SSB,CCA evaluation period. During the in-sync evaluation procedure, layer 1 of the UE shall not send any in-sync indication for the cell to the higher layers when Lin exceeds Lin,max, where Lin and Lin,max are defined in table 8.1A.2.2-1. Table 8.1A.2.2-1: Evaluation period TEvaluate_out_SSB,CCA and TEvaluate_in_SSB,CCA for FR1 Configuration; TEvaluate_out_SSB,CCA (ms); TEvaluate_in_SSB,CCA (ms). RLM-RS SSB Es/IotNote4 ≥-7 dB; RLM-RS SSB Es/Iot Note4 <-7 dB. no DRX; Max(200, Ceil(17*P)*TSSB); Max(200, Ceil(24*P)*TSSB); Max(100, Ceil((5+Lin)*P)*TSSB). DRX cycle≤320; Max(200, Ceil(1.5*15*P)*Max(TDRX,TSSB)); Max(200, Ceil(1.5*20*P)*Max(TDRX,TSSB)); Max(100, Ceil(1.5*(5+Lin)*P)*Max(TDRX,TSSB)). DRX cycle>320; Ceil(13*P)*TDRX; Ceil(16*P)*TDRX; Ceil((5+Lin)*P)*TDRX. NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. NOTE 2: When DRX is not configured, Lin is the number of RLM-RS SSB occasions which are not available at the UE during TEvaluate_in_SSB,CCA, where Lin ≤ Lin,max. When DRX is configured, Lin is the number of DRX cycles in which at least one RLM-RS SSB occasion is not available at the UE during TEvaluate_in_SSB,CCA, where Lin ≤ Lin,max. The UE is not required to determine the availability of SSB occasions more frequent than Once per Max(10 ms, P * TSSB) if no DRX is used, Once per Max(10 ms, Ceil(1.5 * P) * Max(TDRX, TSSB)) if DRX cycle ≤ 320 ms, Once per P * TDRX if DRX cycle > 320 ms. NOTE 3: Lin,max=7 for Max(TDRX,TSSB) ≤ 40 assuming TDRX=0 for non-DRX case, Lin,max=5 for 40<Max(TDRX,TSSB)≤320, Lin,max=3 for TDRX>320. NOTE 4: RLM-RS SSB Es/Iot is the averaged Es/Iot over the most recent previous out-of-sync evaluation period..

In existing spec, 38.133, the requirement for NR-U for L1-RSRP measurement:

Table 9.5A.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB_CCA for FR1 Configuration; TL1-RSRP_Measurement_Period_SSB_CCA (ms). non-DRX; max(TReport, ceil((M+L1)*P)*TSSB). DRX cycle ≤ 320 ms; max(TReport, ceil(1.5*(M+L1)*P)*max(TDRX,TSSB)). DRX cycle > 320 ms; ceil((M+L1)*P)*TDRX. Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: L1=0 if higher layer parameter timeRestrictionForChannelMeasurement is configured. Otherwise, when DRX is not configured L1 is the number of SSBs not available at the UE during TL1-RSRP_Measurement_Period_SSB_CCA, and when DRX is configured L1 is the number of DRX cycles in which at least one SSB is not available at the UE during TL1-RSRP_Measurement_Period_SSB_CCA, where L1 ≤ L1max. The UE is not required to determine the availability of SSB occasions more frequent than Once per Max(TReport, P * TSSB) if no DRX is used, Once per Max(TReport, Ceil(1.5 * P) * Max(TDRX, TSSB)) if DRX cycle ≤ 320ms, Once per P * TDRX if DRX cycle > 320ms. Note 3: L1max =7 for Max(TDRX,TSSB) ≤ 40ms assuming TDRX=0 for non-DRX, L1max =5 for 40ms < Max(TDRX, TSSB) ≤ 320ms, L1max =3 for TDRX > 320ms..

In Rel-16 NR-U discussion, Lmax were agreed and used in RLM/BFD/L1-RSRP measurement.

In NR-U system, the measurement period of RLM/BFD/L1-RSRP are extended based on the RLM-RS/BFD-RS/RS for L1-RSRP which cannot be treated as available ones. The gNB cannot transmit RS due to LBT failure. The Lmax is the maximum value of the measurement period to extension value. It related to the LBT results.

In RAN4#94 meeting, the values of Lmax = 7/5/3 are agreed in [3].

Lmax = 7/5/3 for different DRX and SMTC pattern are from the missed RS (in NR-U system, it is SSB) occasions due to LBT failure.

Now in SBFD system, the unavailable RS occasions are not due to the channel condition but due to SBFD pattern including the UL transmission or non-SBFD symbols. It is related to the gNB scheduling. If there are long time period of UL transmission or non-SBFD, there can be many RS occasion as unavailable. There is no relationship for the number of samples with DRX and SMTC pattern.

In previous meeting, RAN4 has been agreed it is different for RLM/BFD/CBD and L1-RSRP measurement of different SBFD symbol type. In L1-RSRP measurement, the unavailable RS occasions and available RS occasions are from gNB scheduling. There is no need to define Lmax in L1-RSRP measurement. If RAN4 to define it for RLM/BFD/CBD, RAN4 to define one fixed value for RLM/BFD/CBD.

Proposal 1: For L1 CSI-RS measurement in SBFD, do not define Lmax for L1-RSRP measurement. If RAN4 to define it for RLM/BFD/CBD, RAN4 to define one fixed value for RLM/BFD/CBD regardless of DRX and SMTC.

- Conclusion

In this contribution, we provide our proposals for SBFD operation:

Proposal 1: For L1 CSI-RS measurement in SBFD, do not define Lmax for L1-RSRP measurement. If RAN4 to define it for RLM/BFD/CBD, RAN4 to define one fixed value for RLM/BFD/CBD regardless of DRX and SMTC.

- Reference

[1] R4-2508274, WF on RRM requirements for NR_duplex_evo, Huawei

[2] R4-2508459, Draft big CR for RRM requirements of Sub-band full duplex, Huawei, Samsung

[3] R4-2002335, WF on NR-U RRM Requirements (Part 2), MediaTek