3GPP TSG-RAN WG4 Meeting #116		R4-2509839

Bengaluru, India, August 25th – 29th, 2025

Agenda item:		7.27.5.1

Source:		Samsung

Title:	Discussion on RRM requirements of RedCap in Rel-19 NTN phase 3

Document for:		Discussion

- Introduction

In previous meetings, we have already discussed all the RRM requirements of RedCap in NTN and latest agreements are included in [1]. There are still some open issues in [1]. In this contribution, we continue to discuss the FFS parts of RRM requirements for (e)RedCap UE with NR NTN operating in FR1-NTN bands.

- Discussion

In last RAN4#115 meeting, the potential issues for further discussion are captured in WF [1] as below:

#### Issue 1-2-2: SMTC adjustment in idle/inactive <Way Forward>: - Proposal 1: SMTC adjustment in idle/inactive is kept mandatory for (e)RedCap UEs with NTN. - Proposal 2: RAN4 will not make any modifications on the feature of SMTC adjustment in idle/inactive for RedCap UE with NTN. - Proposal 2a: Maintain the same principle used in NR NTN for RedCap devices: the requirements are transparent to whether the UE adjusts or not the SMTC window in IDLE/INACTIVE mode. - Proposal 3: For SMTC adjustment in idle/inactive, we recommend keeping it optional, yet we would support defining it as mandatory if companies have strong concern on this.

In previous meetings, RAN4 have already agreed that (e)RedCap UEs still capable to support multiple SMTCs for parallel which use parallel as the multiple SMTCs are TDMed concept. If multiple SMTCs are used, in legacy NTN with handheld UEs, the UE can adjust the offset based on the propagation delay difference. This is for the measurement for different satellites. For the options above, we support proposal 2 for no modification on legacy feature of SMTC adjustment in idle/inactive mode.

Proposal 1: For SMTC adjustment in idle/inactive mode, use the same as for handheld UEs for legacy feature of SMTC adjustment in idle/inactive mode.

#### Issue 1-3-5: The impact on NR Handover due to HD-FDD <Way Forward>: - Proposal 1: For the impact on NR handover for HD-FDD, the principle in option 1 is accepted. - Proposal 1a: Regarding Issue 3-2-1 (The impact on NR Handover due to HD-FDD), adopt Option 1 with 'availability' defined as 'no collision with UL transmission at the UE'. - Proposal 2: Adopt option 2 (no spec impact) for HO, CHO and satellite switch for HD-FDD UE.

For the impacts on NR handover due to HD-FDD, we support:

    - For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

- SSB is available at the UE once every SMTC period during Tsearch

- One SSB is available during T∆

- One SSB is available during TIU

    - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

- One SSB is available during T∆

- One SSB is available during TIU

It has already been captured in endorsed Draft Big CR with [] in [2].

Proposal 2: For the impact on NR handover for HD-FDD, the principle in option 1 is accepted. The exact update can be discussed during the CR drafting.

#### Issue 1-4-1: General principle for designing the UE capabilities for (e)RedCap UE with NTN <Way Forward>: - Option 1: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). (CATT, CMCC, ZTE, Xiaomi, E///, Samsung - compromise, Apple, QC) - FFS: The Rel-19 requirement can be release independent from Rel-17. - Option 2: RAN4 to introduce new capability to declare support over RedCap over NTN. (vivo, Samsung) - Option 3: to wait for RAN1 to confirm the working assumption (UE that supports RedCap and NTN shall support RedCap over NTN). (Nokia, QC, MTK)

From the technical view, Proposal 1 and 2 are both fine. However, the only concern of proposal 1 is the release independent issue. If use P1, whether RedCap UEs will support from Rel-17 RedCap UE in NTN from Rel-17 and Rel-18 eRedCap UEs in NTN from Rel-18? It needs to add release independent principle in the spec. If use Proposal 2, it is clear only Rel-19 UEs need to meet the requirements for (e)RedCap UE to NTN-FR1 from Rel-19. It is not so urgent for Rel-17 and Rel-18 UEs to support this feature in the commercial market. From this perspective, we support Proposal 2. In last meeting, we can compromise to option 1 only with release independent manner. Otherwise, from UE perspective, at least the UE know which release they declare to follow the requirements or not. However, from network perspective, when there are Rel-17/Rel-18 UEs and Rel-19 UEs in the same network, network cannot distinguish them. It is not feasible for NW implementation.

Proposal 3: Introduce new capability to declare support over RedCap over NTN. Can only compromise to legacy capabilities by using release independent manner. Otherwise, from network implementation perspective, it is not feasible to distinguish the UEs in the same network.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for (e)RedCap UE with NR NTN operating in FR1-NTN bands:

Proposal 1: For SMTC adjustment in idle/inactive mode, use the same as for handheld UEs for legacy feature of SMTC adjustment in idle/inactive mode.

Proposal 2: For the impact on NR handover for HD-FDD, the principle in option 1 is accepted. The exact update can be discussed during the CR drafting.

Proposal 3: Introduce new capability to declare support over RedCap over NTN. Can only compromise to legacy capabilities by using release independent manner. Otherwise, from network implementation perspective, it is not feasible to distinguish the UEs in the same network.

- Reference

[1] R4-2508287, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT

[2] R4-2508462, Draft big CR for RRM requirements of NR NTN Phase 3, CATT, Qualcomm