3GPP TSG-RAN WG4 Meeting #116		R4-2509841

Bengaluru, India, August 25th – 29th, 2025

Agenda item:		7.27.6

Source:		Samsung

Title:	Discussion on RRM performance requirements of Rel-19 NTN Phase3

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#106 meeting. The objectives of performance part of this WI are described in [1] as:

The Performance Part objectives of the work item are the following: - Specify RRM performance requirements [RAN4] - Specify UE demodulation requirements [RAN4] - Specify SAN demodulation requirements [RAN4]

In last RAN4#115meeting, the open issues of RRM performance part of Rel-19 NTN Phase 3 are captured in [2]. In this contribution, we provide our view of the RRM performance part of this WI.

- Discussion

In previous meetings, RAN4 discuss the RRM core requirements and achieved lots of agreements of Rel-19 NTN Phase 3.

In last RAN4#115 meeting, the open issues of RRM performance part are captured in [2]:

The performance requirements can be divided into two parts: Measurement accuracy and Test cases:

Measurement accuracy

Issue 2-1-1: Measurement accuracy requirement <Way Forward>: - Proposal 1 (CMCC): - For 2Rx RedCap UE, reuse the measurement accuracy requirement of NTN. - For 1Rx RedCap UE, define new measurement accuracy requirement on NTN operating bands with reusing the RSRP accuracy and side conditions from legacy 1Rx RedCap UE. - Proposal 2 (vivo): For the performance requirement in NTN for NR Phase 3, measurement accuracy requirements need to be defined for (e)Redcap UEs with FR1-NTN. - Proposal 3 (HW): RAN4 to define accuracy requirements for following measurements for RedCap in NTN. - Intra- and inter-frequency RSRP - Intra- and inter-frequency RSRQ - Intra- and inter-frequency SINR - L1-RSRP - Rx-Tx for NW verified location

For (e)RedCap in NTN, according to current agreements for core requirements, there are no differences from Rel-17 RedCap UEs and those UEs in NTN. RAN4 almost reused the principle for 1Rx and 2Rx RedCap UE from Rel-17 RedCap WI + Rel-17 NTN WI. From the accuracy requirements, if the core requirements are the same, the only difference it the channel model which is different from TN and NTN. In Rel-17 NTN WI, the accuracy requirements are reused by using the same requirements in TN system.

Proposal 1: RAN4 to reuse the same measurement accuracy requirement from RedCap UE in TN system for 1Rx and 2Rx RedCap UEs.

Test cases

In existing 38.133 spec, the test cases for NTN are specified in A.14. In this section, it includes the test cases for FR1 and FR2-NTN.

A.14 NR standalone tests for Satellite access

For Rel-17 RedCap UE, the test cases are specified in A. 16.

A.16 NR standalone tests with all NR cells in FR1 for RedCap

Since RAN4 almost reused the same core requirements for 2Rx UE in NTN. We think we should use A.14 test cases (FR1 test cases) as baseline to update them for RedCap UEs.

Proposal 2: RAN4 to use A.14 test cases (FR1 test cases) as baseline to update them for RedCap UEs.

In addition, to cover the core requirements for 1Rx (e)RedCap UEs, and HD-FDD impacts, RAN4 should add new tests for 1Rx (e)RedCap UEs and add HD-FDD duplex mode.

Proposal 3: Other than the TC in Proposal 2, RAN4 to add new tests for 1Rx (e)RedCap UEs and add HD-FDD duplex mode.

Issue 2-2-1: Test scope for downlink coverage enhancement

It is not so clear of core requirements for downlink coverage enhancement. RAN4 already agreed the default SMTC of RRC re-establishment for SSB periodicity extension. In NTN tests in A.14, RRC-establishment has already been tested if it is configured. For default value which is not configured, it is not typical. We prefer to no TC for this core requirements update.

Proposal 4: We prefer no new TC in NTN for default SMTC core requirements updates for SSB periodicity extension.

Proposal 5: For other enhancements including different SMTC periodicity in the same frequency layer, wait further progress for core requirements.

- Conclusion

In this contribution, we analyze the potential impacts on RRM performance requirements for Rel-19 NTN and provide our proposals:

Proposal 1: RAN4 to reuse the same measurement accuracy requirement from RedCap UE in TN system for 1Rx and 2Rx RedCap UEs.

Proposal 2: RAN4 to use A.14 test cases (FR1 test cases) as baseline to update them for RedCap UEs.

Proposal 3: Other than the TC in Proposal 2, RAN4 to add new tests for 1Rx (e)RedCap UEs and add HD-FDD duplex mode.

Proposal 4: We prefer no new TC in NTN for default SMTC core requirements updates for SSB periodicity extension.

Proposal 5: For other enhancements including different SMTC periodicity in the same frequency layer, wait further progress for core requirements.

- Reference

[1] RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

[2] R4-2508287, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT