3GPP TSG-RAN WG4 Meeting #116	R4-2509855

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.6.3.3.1

Source:	MediaTek Inc.

Title:	Discussion on R19 IoT-NTN HPUE TX RF requirements

Document for:	Discussion

- Introduction

In RAN#103 plenary meeting, the WID on enhanced requirements and test methodology for NR and IoT NTN was approved in [1]. One of the main objectives is to enable high power UE (HPUE) with defining RF requirements targeting at support of IoT-NTN UEs for FR1 single carrier scenario.

In this contribution, our views about HPUE TX RF requirements for NTN NB-IoT UE are provided for discussion.

- Discussion

2.1		High power UE (HPUE) for NTN

Some of the main Rel-19 WID [1] objectives for enhanced requirements and test methodology for NR and IoT NTN are shown below for reference.

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-handheld UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

2.2		Previous Meeting Agreements

In RAN4#115 meeting, the agreed WF [2] provides the following works for specifying TX requirements for IoT-NTN under HPUE operation. The partial contents are shown as follows.

Tx requirements Issue 1-1-5: A-MPR for NB-IoT based IoT-NTN PC2 - Agreement: - Wait for PC3 A-MPR baseline before concluding PC2 A-MPR for NS_04N and NS_05N (band 254) - No A-MPR is needed for [NS_03N] and NS_02N for NB-IoT PC2 in square brackets - Define [2] dB A-MPR increase for NS_24 to support NB-IoT PC2 compared to PC3 - The exact value can be revisited once MPR is agreed. Issue 1-1-8: On relaxing the upper limitation of nominal MOP or MOP tolerance for NTN HD-FDD operation. - Background (Last meeting’s WF): - Restrict the boosting condition to PC2 NB-Iot based Iot-NTN - To down-select in next meeting between: - Option 1: Increase [1] dB MOP upper tolerance - Option 2: Increase [1] dB MOP nominal power (optional with capability signalling) - WF(NEW): - Restrict the boosting condition to PC2 NB-Iot based Iot-NTN - Increase [0.8] dB MOP or add a power boosting item in Pcmax equation (optional with capability signalling)

2.3		UE TX RF requirements

Topic 1-1: A-MPR for NB-IoT based IoT-NTN PC2:

Background about B255 NS_02N in TS 36.102:

Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |

Table 6.5B.3.2-1: Category NB1 and NB2 UE spectrum emission mask

| ΔfOOB (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0 | 26 | 30 kHz |
| 100 | -5 | 30 kHz |
| 150 | -8 | 30 kHz |
| 300 | -29 | 30 kHz |
| 500-1700 | -35 | 30 kHz |

Table 6.5B.3.3.1-1: Additional requirements for "NS_02N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0.09–0.28 | -2 for PC3 -5 for PC5 | 4 kHz |
| ± 0.28–0.85 | -12 for PC3 -15 for PC5 | 4 kHz |
| ± >0.85 | -13 for PC3 and PC5 | 4 kHz |

Table 5.2-1 E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | – | 2010 MHz | 2170 MHz | – | 2200 MHz | FDD |
| 255 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |

Table 6.5B.4.4.2-1: Additional requirements for "NS_02N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 200kHz |  |  |
| 1559≤ f ≤ 1605 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -50 + 24/5 (f-1605) | 700Hz |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -40 + 24/5 (f-1605) | 1MHz |  |

Proposal 1: Regarding additional requirement of “NS_02N” for PC2, discuss updated Table 6.5B.3.3.1-1 below and further revision is not precluded if necessary.

Table 6.5B.3.3.1-1: Additional requirements for "NS_02N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0.09–0.28 | 1 for PC2 -2 for PC3 -5 for PC5 | 4 kHz |
| ± 0.28–0.85 | -9 for PC2 -12 for PC3 -15 for PC5 | 4 kHz |
| ± >0.85 | -13 for PC2, PC3 and PC5 | 4 kHz |

Proposal 2: Regarding B255 NB-IoT PC2, consider whether NS_02N AMPR is N/A, as the additional requirements for "NS_02N" indicated in Proposal 1 do not appear to be more stringent than general SEM.

Background about B256 NS_24 in TS 36.102:

Table 6.5B.3.2-1: Category NB1 and NB2 UE spectrum emission mask

| ΔfOOB (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0 | 26 | 30 kHz |
| 100 | -5 | 30 kHz |
| 150 | -8 | 30 kHz |
| 300 | -29 | 30 kHz |
| 500-1700 | -35 | 30 kHz |

Table 6.5B.4.4.3-1: Additional requirements for "NS_24"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 200kHz |  |
| Band 34 | -50 | MHz |

Table 6.2B.2-1: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 3 and 5

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 0.5 dB | 0 dB |  | ≤ 0.5 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 1 dB |  | ≤ 1 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 2 dB |  |  |  |

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_24 | 6.5B.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |

Proposal 3: Regarding NB-IoT NTN PC2 HPUE operated in L/S bands, discuss PC2 MPR Tables below.

| Modulation | π/2 BPSK, π/4 QPSK | π/2 BPSK, π/4 QPSK |
| --- | --- | --- |
| SCS | 15KHz | 3.75kHz |
| Tone positions for 1 Tone allocation | 0-11 | 0-47 |
| MPR | ≤ [0.5] dB | [0] dB |
| Note 1: For single-tone allocation, the listed modulation and SCS are supported. |  |  |

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ [1.5] dB | ≤ [0.5] dB |  | ≤ [1.5] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [2] dB |  | ≤ [2] dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [3] dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

Proposal 4: Regarding NB-IoT NTN PC2 HPUE` operated in B256, consider whether NS_24 AMPR could be ≤ 5.5dB.

Topic 1-2: On relaxing the upper limitation of nominal MOP or MOP tolerance for NTN HD-FDD operation:

Observation 1: According to WF[2] agreement, restrict the boosting condition to PC2 NB-Iot based Iot-NTN.

Observation 2: According to WF[2] agreement, increase [0.8] dB MOP or add a power boosting item in Pcmax equation (optional with capability signalling)

Proposal 5: Regarding “Add a power boosting item in Pcmax equation (optional with capability signalling)”, add the following contents to TS 36.102.

====================Start of the change 1==========================

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔFRaster	Band dependent channel raster granularity

BWChannel	Channel bandwidth

F					Frequency

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured)

FInterferer			Frequency of the interferer

FIoffset				Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

FC	Frequency of the carrier centre frequency

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

FOOB	The boundary between the E-UTRA out of band emission and spurious emission domains.

LCtone	Transmission bandwidth which represents the length of a contiguous sub-carrier allocation expressed in units of tones

NDL 	Downlink EARFCN

NOffs-DL 	Offset used for calculating downlink EARFCN

NOffs-UL 	Offset used for calculating uplink EARFCN

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_alloc	Total number of simultaneously transmitted resource blocks in Channel bandwidth or Aggregated Channel Bandwidth.

Ntone	Transmission bandwidth configuration for category NB1 and NB2, expressed in units of tones.

Ntone 3.75kHz	Transmission bandwidth configuration for category NB1 and NB2 with 3.75 kHz sub-carrier spacing, expressed in units of tones.

Ntone 15kHz 	Transmission bandwidth configuration for category NB1 and NB2 with 15 kHz sub-carrier spacing, expressed in units of tones.

NUL	Uplink EARFCN.

PCMAX	The configured maximum UE output power.

PCMAX, c	The configured maximum UE output power for serving cell c.

PEMAX 	Maximum allowed UE output power signalled by higher layers. Same as IE P-Max, defined in [6].

PEMAX, c	Maximum allowed UE output power signalled by higher layers for serving cell c. Same as IE
P-Max, defined in [6].

PInterferer	Modulated mean power of the interferer

PPowerClass	PPowerClass is the nominal UE power (i.e., no tolerance).

PPowerClass_Default	PPowerClass_Default is the default nominal UE power (i.e., no tolerance) for the band.

P-MPRc	Power Management Maximum Power Reduction for serving cell c

PUMAX	The measured configured maximum UE output power.

Puw	Power of an unwanted DL signal

Pw	Power of a wanted DL signal

ΔfOOB	Δ Frequency of Out Of Band emission

====================End of the change 1==========================

====================Start of the change 2==========================

Table 6.2B.1-1: UE Power Class

| E-UTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 | 261 | +/-2 | 23 | +/-2 | 20 | +/-2 |
| 255 | 261 | +/-2 | 23 | +/-2 | 20 | +/-2 |
| 254 |  |  | 23 | +/-2 | 20 | +/-2 |
| 253 |  |  | 23 | +/-2 | 20 | +/-2 |
| 252 |  |  | 23 | +/-2 | 20 | +/-2 |
| NOTE 1: The power limit relaxation and related requirements in NTN NB-IoT operation are specified in sub-clause 6.2B.4. |  |  |  |  |  |  |

====================End of the change 2==========================

====================Start of the change 3==========================

6.2B.4	Configured transmitted Power for category NB1 and NB2

For each slot i the category NB1 and NB2 UE is allowed to set its configured maximum output power PCMAX,c. The configured maximum output power PCMAX,c is set within the following bounds:

PCMAX_L,c ≤  PCMAX,c  ≤  PCMAX_H,c with

PCMAX_L,c = MIN { PEMAX,c ,  (PPowerClass + ΔPPowerBoost) – MAX((MPR – A-MPR), P-MPRc)}

PCMAX_H,c = MIN { PEMAX,c,  PPowerClass + ΔPPowerBoost}

Where

PEMAX,c is the value given to IE P-Max, defined in TS 36.331 [6].

PPowerClass is the maximum category NB1 and NB2 UE power specified in subclause 6.2B.1 without taking into account the associated tolerance.

The Maximum output power requirements are specified in subclause 6.2B.1

The MPR requirements are specified in subclause 6.2B.2

The A-MPR requirements are specified in subclause 6.2B.3.

When the IE powerBoostNbIotNTN-r19 is set to 1, PEMAX,c is increased by +0.8 dB for a power class 2 UE operating in FR1 bands 256, 255 and UE indicates support for UE capability powerBoosting-NbIotNTN-r19.

ΔPPowerBoost is defined as 0.8dB for power class 2, when all of the following conditions are met:

-	If the UE indicates support for UE capability powerBoosting-NbIotNTN-r19, and if IE powerBoostNbIotNTN-r19 is set to 1 and PEMAX,c, if configured, is increased by at least ΔPPowerBoost

-	If UE indicates power class 2

P-MPRc is the power management maximum power reduction for

a)	ensuring compliance with applicable electromagnetic energy absorption requirements and addressing unwanted emissions / self desense requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic energy absorption requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

The UE applies P-MPRc for serving cell c only for the above cases. For UE conducted conformance testing P-MPRc shall be 0 dB

NOTE 1:	P-MPRc was introduced in the PCMAX,f,c equation such that the UE can report to the eNB the available maximum output transmit power. This information can be used by the eNB for scheduling decisions.

NOTE 2:	P-MPRc may impact the maximum uplink performance for the selected UL transmission path.

The measurement period for PUMAX,c is at least one sub-frame (1ms) for 15 KHz channel spacing, and at least a 2ms slot (excluding the 2304Ts gap when UE is not transmitting) respectively for the 3.75 KHz channel spacing. The measured maximum output power PUMAX,c shall be within the following bounds:

PCMAX_L,c –  T(PCMAX_L,c)  ≤  PUMAX,c  ≤  PCMAX_H,c  +  T(PCMAX_H,c)

Where T(PCMAX) is defined by the tolerance table below and applies to PCMAX_L,c and PCMAX_H,c separately.

Table 6.2B.4-1: PCMAX tolerance

| PCMAX,c (dBm) | Tolerance T(PCMAX,c) (dB) |
| --- | --- |
| 23 ≤ PCMAX,c ≤ 33 | 2.0 |
| 21 ≤ PCMAX,c ≤ 23 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 2.5 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -40 ≤ PCMAX,c < 8 | 7.0 |

Table 6.2B.4-2: PCMAX tolerance for power class 5

| PCMAX,c (dBm) | Tolerance T(PCMAX,c) (dB) |
| --- | --- |
| 18 ≤ PCMAX,c ≤ 20 | 2.0 |
| 17 ≤ PCMAX,c < 18 | 2.5 |
| 16 ≤ PCMAX,c < 17 | 3.5 |
| 15 ≤ PCMAX,c < 16 | 4.0 |
| 10 ≤ PCMAX,c < 15 | 5.0 |
| 5 ≤ PCMAX,c < 10 | 6.0 |
| -40 ≤ PCMAX,c < 5 | 7.0 |

====================End of the change 3==========================

Proposal 6: Regarding “Add a power boosting item in Pcmax equation (optional with capability signalling)”, add the following table into feature list. Further update in table is not precluded if necessary.

Below UE feature list for NR_IoT_NTN_req_test_enh WI.

Table: UE features of Tx output power (limitation relaxation) for NTN NB-IoT HPUEs

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD diff. | Need of FR1/FR2 diff. | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/ Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 61-1 | Tx PC2 output power limit relaxation for NTN NB-IoT UL | Indicate the supported PC2 Tx output power limit relaxation for NTN NB-IoT UL in each FR1 band | IoT NTN | Yes | N/A | Legacy Tx output power upper limitation still apply | Per (UL) band: UE signals supported upper power limit relaxation | No need | FR1 only | Support mixture of FDD/TDD | The requirements for relaxed upper power limited are defined in Clause 6.2B.4 in TS36.102. | Optional with capability signalling |

3 Conclusion

In this paper, our views on HPUE TX RF requirements for NTN NB-IoT UE are provided as follows:

Topic 1-1: A-MPR for NB-IoT based IoT-NTN PC2:

Proposal 1: Regarding additional requirement of “NS_02N” for PC2, discuss updated Table 6.5B.3.3.1-1 below and further revision is not precluded if necessary.

Table 6.5B.3.3.1-1: Additional requirements for "NS_02N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0.09–0.28 | 1 for PC2 -2 for PC3 -5 for PC5 | 4 kHz |
| ± 0.28–0.85 | -9 for PC2 -12 for PC3 -15 for PC5 | 4 kHz |
| ± >0.85 | -13 for PC2, PC3 and PC5 | 4 kHz |

Proposal 2: Regarding B255 NB-IoT PC2, consider whether NS_02N AMPR is N/A, as the additional requirements for "NS_02N" indicated in Proposal 1 do not appear to be more stringent than general SEM.

Proposal 3: Regarding NB-IoT NTN PC2 HPUE operated in L/S bands, discuss PC2 MPR Tables below.

| Modulation | π/2 BPSK, π/4 QPSK | π/2 BPSK, π/4 QPSK |
| --- | --- | --- |
| SCS | 15KHz | 3.75kHz |
| Tone positions for 1 Tone allocation | 0-11 | 0-47 |
| MPR | ≤ [0.5] dB | [0] dB |
| Note 1: For single-tone allocation, the listed modulation and SCS are supported. |  |  |

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ [1.5] dB | ≤ [0.5] dB |  | ≤ [1.5] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [2] dB |  | ≤ [2] dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [3] dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

Proposal 4: Regarding NB-IoT NTN PC2 HPUE` operated in B256, consider whether NS_24 AMPR could be ≤ 5.5dB.

Topic 1-2: On relaxing the upper limitation of nominal MOP or MOP tolerance for NTN HD-FDD operation:

Observation 1: According to WF[2] agreement, restrict the boosting condition to PC2 NB-Iot based Iot-NTN.

Observation 2: According to WF[2] agreement, increase [0.8] dB MOP or add a power boosting item in Pcmax equation (optional with capability signalling)

Proposal 5: Regarding “Add a power boosting item in Pcmax equation (optional with capability signalling)”, add the following contents to TS 36.102.

====================Start of the change 1==========================

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔFRaster	Band dependent channel raster granularity

BWChannel	Channel bandwidth

F					Frequency

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured)

FInterferer			Frequency of the interferer

FIoffset				Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

FC	Frequency of the carrier centre frequency

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

FOOB	The boundary between the E-UTRA out of band emission and spurious emission domains.

LCtone	Transmission bandwidth which represents the length of a contiguous sub-carrier allocation expressed in units of tones

NDL 	Downlink EARFCN

NOffs-DL 	Offset used for calculating downlink EARFCN

NOffs-UL 	Offset used for calculating uplink EARFCN

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_alloc	Total number of simultaneously transmitted resource blocks in Channel bandwidth or Aggregated Channel Bandwidth.

Ntone	Transmission bandwidth configuration for category NB1 and NB2, expressed in units of tones.

Ntone 3.75kHz	Transmission bandwidth configuration for category NB1 and NB2 with 3.75 kHz sub-carrier spacing, expressed in units of tones.

Ntone 15kHz 	Transmission bandwidth configuration for category NB1 and NB2 with 15 kHz sub-carrier spacing, expressed in units of tones.

NUL	Uplink EARFCN.

PCMAX	The configured maximum UE output power.

PCMAX, c	The configured maximum UE output power for serving cell c.

PEMAX 	Maximum allowed UE output power signalled by higher layers. Same as IE P-Max, defined in [6].

PEMAX, c	Maximum allowed UE output power signalled by higher layers for serving cell c. Same as IE
P-Max, defined in [6].

PInterferer	Modulated mean power of the interferer

PPowerClass	PPowerClass is the nominal UE power (i.e., no tolerance).

PPowerClass_Default	PPowerClass_Default is the default nominal UE power (i.e., no tolerance) for the band.

P-MPRc	Power Management Maximum Power Reduction for serving cell c

PUMAX	The measured configured maximum UE output power.

Puw	Power of an unwanted DL signal

Pw	Power of a wanted DL signal

ΔfOOB	Δ Frequency of Out Of Band emission

====================End of the change 1==========================

====================Start of the change 2==========================

Table 6.2B.1-1: UE Power Class

| E-UTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 | 261 | +/-2 | 23 | +/-2 | 20 | +/-2 |
| 255 | 261 | +/-2 | 23 | +/-2 | 20 | +/-2 |
| 254 |  |  | 23 | +/-2 | 20 | +/-2 |
| 253 |  |  | 23 | +/-2 | 20 | +/-2 |
| 252 |  |  | 23 | +/-2 | 20 | +/-2 |
| NOTE 1: The power limit relaxation and related requirements in NTN NB-IoT operation are specified in sub-clause 6.2B.4. |  |  |  |  |  |  |

====================End of the change 2==========================

====================Start of the change 3==========================

6.2B.4	Configured transmitted Power for category NB1 and NB2

For each slot i the category NB1 and NB2 UE is allowed to set its configured maximum output power PCMAX,c. The configured maximum output power PCMAX,c is set within the following bounds:

PCMAX_L,c ≤  PCMAX,c  ≤  PCMAX_H,c with

PCMAX_L,c = MIN { PEMAX,c ,  (PPowerClass + ΔPPowerBoost) – MAX((MPR – A-MPR), P-MPRc)}

PCMAX_H,c = MIN { PEMAX,c,  PPowerClass + ΔPPowerBoost}

Where

PEMAX,c is the value given to IE P-Max, defined in TS 36.331 [6].

PPowerClass is the maximum category NB1 and NB2 UE power specified in subclause 6.2B.1 without taking into account the associated tolerance.

The Maximum output power requirements are specified in subclause 6.2B.1

The MPR requirements are specified in subclause 6.2B.2

The A-MPR requirements are specified in subclause 6.2B.3.

When the IE powerBoostNbIotNTN-r19 is set to 1, PEMAX,c is increased by +0.8 dB for a power class 2 UE operating in FR1 bands 256, 255 and UE indicates support for UE capability powerBoosting-NbIotNTN-r19.

ΔPPowerBoost is defined as 0.8dB for power class 2, when all of the following conditions are met:

-	If the UE indicates support for UE capability powerBoosting-NbIotNTN-r19, and if IE powerBoostNbIotNTN-r19 is set to 1 and PEMAX,c, if configured, is increased by at least ΔPPowerBoost

-	If UE indicates power class 2

P-MPRc is the power management maximum power reduction for

a)	ensuring compliance with applicable electromagnetic energy absorption requirements and addressing unwanted emissions / self desense requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic energy absorption requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

The UE applies P-MPRc for serving cell c only for the above cases. For UE conducted conformance testing P-MPRc shall be 0 dB

NOTE 1:	P-MPRc was introduced in the PCMAX,f,c equation such that the UE can report to the eNB the available maximum output transmit power. This information can be used by the eNB for scheduling decisions.

NOTE 2:	P-MPRc may impact the maximum uplink performance for the selected UL transmission path.

The measurement period for PUMAX,c is at least one sub-frame (1ms) for 15 KHz channel spacing, and at least a 2ms slot (excluding the 2304Ts gap when UE is not transmitting) respectively for the 3.75 KHz channel spacing. The measured maximum output power PUMAX,c shall be within the following bounds:

PCMAX_L,c –  T(PCMAX_L,c)  ≤  PUMAX,c  ≤  PCMAX_H,c  +  T(PCMAX_H,c)

Where T(PCMAX) is defined by the tolerance table below and applies to PCMAX_L,c and PCMAX_H,c separately.

Table 6.2B.4-1: PCMAX tolerance

| PCMAX,c (dBm) | Tolerance T(PCMAX,c) (dB) |
| --- | --- |
| 23 ≤ PCMAX,c ≤ 33 | 2.0 |
| 21 ≤ PCMAX,c ≤ 23 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 2.5 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -40 ≤ PCMAX,c < 8 | 7.0 |

Table 6.2B.4-2: PCMAX tolerance for power class 5

| PCMAX,c (dBm) | Tolerance T(PCMAX,c) (dB) |
| --- | --- |
| 18 ≤ PCMAX,c ≤ 20 | 2.0 |
| 17 ≤ PCMAX,c < 18 | 2.5 |
| 16 ≤ PCMAX,c < 17 | 3.5 |
| 15 ≤ PCMAX,c < 16 | 4.0 |
| 10 ≤ PCMAX,c < 15 | 5.0 |
| 5 ≤ PCMAX,c < 10 | 6.0 |
| -40 ≤ PCMAX,c < 5 | 7.0 |

====================End of the change 3==========================

Proposal 6: Regarding “Add a power boosting item in Pcmax equation (optional with capability signalling)”, add the following table into feature list. Further update in table is not precluded if necessary.

Below UE feature list for NR_IoT_NTN_req_test_enh WI.

Table: UE features of Tx output power (limitation relaxation) for NTN NB-IoT HPUEs

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD diff. | Need of FR1/FR2 diff. | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/ Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 61-1 | Tx PC2 output power limit relaxation for NTN NB-IoT UL | Indicate the supported PC2 Tx output power limit relaxation for NTN NB-IoT UL in each FR1 band | IoT NTN | Yes | N/A | Legacy Tx output power upper limitation still apply | Per (UL) band: UE signals supported upper power limit relaxation | No need | FR1 only | Support mixture of FDD/TDD | The requirements for relaxed upper power limited are defined in Clause 6.2B.4 in TS36.102. | Optional with capability signalling |

4 Reference

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, Huawei

- R4-2508122, “WF on IoT-NTN HPUE”, Vivo