3GPP TSG-RAN4 Meeting #116 	R4-2509882

Bengaluru, India, 25th -29th August, 2025

Source: 	Huawei, HiSilicon

Title:  	Ambient IoT general aspects

Agenda Item:	7.22.2

Document for:	Approval

- Introduction

The WI on solutions for Ambient IoT in NR was approved at RAN#106 [1] and revised at RAN#107[2]. Last meeting, a WF was agreed [3]. In this contribution, we further discuss the open issues and give our proposals on D2R CBW and R2D channel raster offset.

- Discussion

  - D2R Channel bandwidth

On D2R channel BW, the agreement from the last meeting was:

Copied from [3]

Issue 2-2-1: D2R bandwidth

Agreement in RAN4#115:

- Define D2R CBW based on all Tb, Tc, R combinations as finally approved in RAN1.

  - Assumed 10% SFO for device

  - For Reader, [90%] filter spectrum utility ([10%] guard band) is considered.

- The value of transmission BW and channel BW will be explicitly listed.

- Using following equation for BS as starting point:

D2R CBW for BS (KHz)=(2SB Transmission BW*(1/2)+2* Small frequency shift)/0.9

=(2000*(1+R)/Tb)* (1+∣SFO∣)/0.9

=(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)/0.9

  - Define [200kHz]，[3.54MHz] D2R CBW for AIoT BS. Other D2R CBWs for BS are FFS

- Using following equation for device as starting point:

D2R CBW for device (KHz)=2SB Transmission BW*(1/2)+2* Small frequency shift

=(2000*(1+R)/Tb)* (1+∣SFO∣)

=(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)

As per TS38.104, NR channel bandwidths of 5MHz or less with 15kHz SCS achieve 90% spectrum utilization (=transmission bandwidth configuration / channel bandwidth × 100%). Since A-IoT R2D uses a 15kHz SCS OFDM signal and operates in a channel bandwidth below 5MHz, the same spectrum utilization of 90% can be applied.

Proposal 1： For A-IoT BS, 90% filter spectrum utility (10% guard band) is considered for D2R CBW.

Agreement on D2R transmission bandwidth in RAN1#121 are as follows：

Copied from [4]

Agreement

For D2R transmission,

- The minimum Tb is 1.39μs.

- Support at least the following values of Tb, Tchip, and R from the table agreed in RAN1#120bis.

|  | Tchip (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tb (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 133.33 | 66.67 | 33.33 | 16.67 | 11.11 | 8.33 | 5.56 | 4.17 | 2.78 | 2.08 | 1.39 | 1.04 | 0.69 | 0.52 |
| 266.67 | R=1 | R=2 | R=4 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |  | R=256 |
| 133.33 |  | R=1 | R=2 | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |
| 66.67 |  |  | R=1 | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 |
| 33.33 |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 |
| 22.22 |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  | R=16 |  |
| 16.67 |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 |
| 11.11 |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  |
| 8.33 |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 |
| 5.56 |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  |
| 4.17 |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |
| 2.78 |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  |
| 2.08 |  |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |
| 1.39 |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |  |
| 1.04 |  |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |

Note: This does not imply the device to pre-store the table.

For reference, here's what Tb, Tc, R, Btx, D2R represent and their relationship:

Copied from [5]

Background

For D2R transmission, the following parameters and relationships are considered:

- Tb: Time duration corresponding to a bit, where the bit is that after FEC if applied.

- Tc: Chip duration, where Tb = Tc×2R.

- R: SFS factor, where the small frequency shift location is determined by ± (R/Tb = 1/2/Tc) Hz

- Btx,D2R: D2R transmission bandwidth, where Btx,D2R = 4/Tb = 2/(R*Tc).

To derive step by step the D2R channel BW, we begin by analyzing the D2R transmission bandwidth without SFO. Next, we calculate the small frequency shift location without SFO. Then, we evaluate how SFO influences both the small frequency shift location and the D2R transmission bandwidth. Finally, we derive the complete D2R channel bandwidth equation, expressed in terms of Tb, Tc, and R.

1) D2R transmission bandwidth without SFO

Assuming no SFO, the nominal D2R transmission bandwidth can be calculated as Eq. 1:

Btx,D2R_without SFO = 4 / Tb = 2 / (R × Tc)		(Eq. 1)

When the transmission bandwidth is in kHz, and Tc and Tb are in μs, the equation becomes Eq. 2:

Btx,D2R_without SFO = 4000 / Tb = 2000 / (R × Tc) 		(Eq. 2)

Table 1 Btx,D2R_without SFO with different Tb or Tc

| Nominal Bit duration:Tb(μs) | Nominal D2R chip duration Tc(μs) | Nominal Transmission BW without SFO Btx,D2R_without SFO (kHz) |
| --- | --- | --- |
| 266.67 | 133.33 | 15 |
| 133.33 | 66.67 | 30 |
| 66.67 | 33.33 | 60 |
| 33.33 | 16.67 | 120 |
| 16.67 | 8.33 | 240 |
| 8.33 | 4.17 | 480 |
| 4.17 | 2.08 | 960 |
| 1.39 | 0.69 | 2880 |

Nominal D2R Transmission BW without SFO is 15, 30, 60, 120, 240, 480, 960 and 2880 kHz.

2) Small frequency shift without SFO

The small frequency shift is defined as the offset between the 2SB carrier's center frequency and the shifted single-side carrier's center frequency.

Figure 1 Small frequency shift without SFO

The small frequency shift location is determined by ± (R/Tb = 1/2/Tc) Hz, where R is SFS factor

When small frequency shift is in kHz, and Tc and Tb are in μs, the equation becomes Eq. 3:

SFSwithout SFO=±1000 × R/Tb=1000/(2×Tc) =（R× Btx,D2R without SFO /4）		(Eq. 3)

Table 2 SFS_without SFO with different chip duration

| Nominal Tc (μs) | Nominal SFS without SFO, SFS_without SFO (kHz) |
| --- | --- |
| 133.33 | 3.75 |
| 66.67 | 7.5 |
| 33.33 | 15 |
| 16.67 | 30 |
| 8.33 | 60 |
| 4.17 | 120 |
| 2.08 | 240 |
| 1.04 | 480 |
| 0.69 | 720 |

For convenience, the table below shows how small frequency shifts related to transmission BW for different values of R, no SFO is considered:

Table 3. Small frequency shift without SFO for each (R, Btx,D2R without SFO)

| Btx,D2R without SFO / R | 15 kHz | 30 kHz | 60 kHz | 120 kHz | 240 kHz | 480 kHz | 960 kHz | 2880 kHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R = 1 | 3.75kHz | 7.5kHz | 15kHz | 30kHz | 60kHz | 120kHz | 240kHz | 720kHz |
| R = 2 | 7.5kHz | 15kHz | 30kHz | 60kHz | 120kHz | 240kHz | 480kHz |  |
| R = 4 | 15kHz | 30kHz | 60kHz | 120kHz | 240kHz | 480kHz | \ | \ |
| R = 8 | 30kHz | 60kHz | 120kHz | 240kHz | 480kHz | \ | \ | \ |
| R = 16 | 60kHz | 120kHz | 240kHz | 480kHz | \ | \ | \ | \ |
| R = 32 | 120kHz | 240kHz | 480kHz | \ | \ | \ | \ | \ |
| R = 64 | 240kHz | 480kHz | \ | \ | \ | \ | \ | \ |
| R = 128 | 480kHz | \ | \ | \ | \ | \ | \ | \ |

For example, for a 2SB 60 kHz D2R transmission bandwidth (without SFO), the possible values for R is 1, 2, 4, 8, 16 and 32.

The corresponding allowed small frequency shifts are: 15 kHz, 30 kHz, 60 kHz, 120 kHz, 240 kHz, and 480 kHz.

These configurations are illustrated in Figure 2.

Figure 2. 2SB 60kHz D2R Transmission BW Small frequency shift without SFO

For a 2SB 60 kHz D2R transmission bandwidth, the spectrum overlap when R=1 and R=2 while are adjacent for R=2 and R=4.

3) Impact of SFO

The impact of SFO can be assumed as the extension or the shortening of the chip length. Both D2R Transmission bandwidth and small frequency shift values are related with chip duration. [6] Consequently, the SFO affects D2R channel bandwidth in two ways:

- SFO Alters D2R transmission bandwidth to Btx,D2R_without SFO×(1±∣SFO∣)

- SFO scales the frequency shift value to (R /Tb) × (1 ±∣SFO∣)

The figure of the SFO impact of FDMA is demonstrated as the following Figure 3. For convenience, Figure 1 is copied to compare:

Figure 1 Small frequency shift diagram of FDMA without SFO

Figure 3. SFO impact of transmission bandwidth and small frequency shift

Table 4 and Table 5 show the calculated impact of 10% SFO to transmission BW and small frequency shift.

Table 4 SFO impact to transmission BW

| Transmission BW without SFO (kHz) | Transmission BW with SFO (kHz)= Transmission BW without SFO×(1+abs(SFO)) |
| --- | --- |
| 15 | 16.5 |
| 30 | 33 |
| 60 | 66 |
| 120 | 132 |
| 240 | 264 |
| 480 | 528 |
| 960 | 1056 |
| 2880 | 3168 |

Table 5 SFO impact to small frequency shift

| SFS without SFO (kHz) | SFS with SFO (kHz) =SFSwithout SFO×(1+abs(SFO)) |
| --- | --- |
| 3.75 | 4.1 |
| 7.5 | 8.3 |
| 15 | 16.5 |
| 30 | 33 |
| 60 | 66 |
| 120 | 132 |
| 240 | 264 |
| 480 | 528 |
| 720 | 792 |

- D2R channel bandwidth equation with SFO

Proposal 2:

Using the following equation for BS D2R CBW:

D2R CBW for BS (kHz)

= (2SB Transmission BW_without SFO× (1/2) +2× Small frequency shift_without SFO)/0.9

=(2000×(1+R)/Tb) × (1+∣SFO∣)/0.9

=(1000×(R+1)/(Tc×R)) × (1+∣SFO∣)/0.9		(Eq. 4)

The transmission bandwidth (BW) and small frequency shift are in kHz, while Tc and Tb are in μs.

The 0.9 divisor presents the 90% BS filter spectrum utility (10% guard band).

On the device side, no filter is needed since device just backscatter CW signal. From device perspective, consider small frequency shift and SFO (assumed as 10^5 ppm) impact, D2R CBW can be calculated as follows:

Proposal 3:

- Using following equation for device D2R CBW

D2R CBW for device (kHz)

=2SB Transmission BW_without SFO× (1/2) +2× Small frequency shift_without SFO

=(2000×(1+R)/Tb) × (1+∣SFO∣)

=(1000×(R+1)/ (Tc ×R)) × (1+∣SFO∣) 		(Eq. 5)

The transmission bandwidth (BW) and small frequency shift are in kHz, while Tc and Tb are in μs.

The possible CBW values for device and reader are calculated in Table 6. The 0.9 divisor presents the 90% BS filter spectrum utility (10% guard band). 10% SFO is assumed.

Table 6 D2R CBW Calculation

| A | B | C=2×A×B | D | E=(2000×(1+R)/Tb) × (1+abs(SFO))=(1000×(R+1)/(Tc×R)) × (1+abs(SFO)) | F=E/0.9 |
| --- | --- | --- | --- | --- | --- |
| SFS factor R | chip duration:Tchip(μs) | bit duration:Tb(μs) | SFO | D2R CBW(kHz) for Device | D2R CBW(kHz) for BS |
| 1 | 133.33 | 266.67 | 0.1 | 16.5 | 18.3 |
| 1 | 66.67 | 133.33 | 0.1 | 33 | 36.7 |
| 2 | 66.67 | 266.67 | 0.1 | 24.8 | 27.6 |
| 1 | 33.33 | 66.67 | 0.1 | 66 | 73.3 |
| 2 | 33.33 | 133.33 | 0.1 | 49.5 | 55 |
| 4 | 33.33 | 266.67 | 0.1 | 41.3 | 45.9 |
| 1 | 16.67 | 33.33 | 0.1 | 132 | 146.7 |
| 2 | 16.67 | 66.67 | 0.1 | 99 | 110 |
| 4 | 16.67 | 133.33 | 0.1 | 82.5 | 91.7 |
| 8 | 16.67 | 266.67 | 0.1 | 74.3 | 82.6 |
| 1 | 8.33 | 16.67 | 0.1 | 264 | 293.3 |
| 2 | 8.33 | 33.33 | 0.1 | 198 | 220 |
| 4 | 8.33 | 66.67 | 0.1 | 165 | 183.3 |
| 8 | 8.33 | 133.33 | 0.1 | 148.5 | 165 |
| 16 | 8.33 | 266.67 | 0.1 | 140.3 | 155.9 |
| 1 | 4.17 | 8.33 | 0.1 | 528 | 586.7 |
| 2 | 4.17 | 16.67 | 0.1 | 396 | 440 |
| 4 | 4.17 | 33.33 | 0.1 | 330 | 366.7 |
| 8 | 4.17 | 66.67 | 0.1 | 297 | 330 |
| 16 | 4.17 | 133.33 | 0.1 | 280.5 | 311.7 |
| 32 | 4.17 | 266.67 | 0.1 | 272.3 | 302.6 |
| 1 | 2.08 | 4.17 | 0.1 | 1056 | 1173.3 |
| 2 | 2.08 | 8.33 | 0.1 | 792 | 880 |
| 4 | 2.08 | 16.67 | 0.1 | 660 | 733.3 |
| 8 | 2.08 | 33.33 | 0.1 | 594 | 660 |
| 16 | 2.08 | 66.67 | 0.1 | 561 | 623.3 |
| 32 | 2.08 | 133.33 | 0.1 | 544.5 | 605 |
| 64 | 2.08 | 266.67 | 0.1 | 536.3 | 595.9 |
| 2 | 1.04 | 4.17 | 0.1 | 1584 | 1760 |
| 4 | 1.04 | 8.33 | 0.1 | 1320 | 1466.7 |
| 8 | 1.04 | 16.67 | 0.1 | 1188 | 1320 |
| 16 | 1.04 | 33.33 | 0.1 | 1122 | 1246.7 |
| 32 | 1.04 | 66.67 | 0.1 | 1089 | 1210 |
| 64 | 1.04 | 133.33 | 0.1 | 1072.5 | 1191.7 |
| 128 | 1.04 | 266.67 | 0.1 | 1064.3 | 1182.6 |
| 1 | 0.69 | 1.39 | 0.1 | 3168 | 3520 |

Tables 1-6 results are summarized in Table 7.

Table 7 D2R CBW with different Tc, Tb and R combinations

|  |  | Chip duration:Tc=2/SCS/(1,2,4,8,16,32,64,192) (μs) |  |  |  |  |  |  |  |  | 2SB=4/Tb（kHz） |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.69 |  |
|  |  | τ | τ2 | τ4 | τ8 | τ16 | τ32 | τ64 | τ128 | τ192 |  |
|  |  | Small Frequency Shift：SFS=1/(2×Tc)= R/Tb (kHz) |  |  |  |  |  |  |  |  |  |
|  |  | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 | 720 |  |
| Bit duration:Tb= Tc×2R(μs) |  |  |  |  |  |  |  |  |  |  |  |
| 266.67 |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 |  | 15 |
|  | Device D2R CBW (kHz) | 16.5 | 24.8 | 41.3 | 74.3 | 140.3 | 272.3 | 536.3 | 1064.3 |  |  |
|  | BS D2R CBW (kHz) | 18.3 | 27.6 | 45.9 | 82.6 | 155.9 | 302.6 | 595.9 | 1182.6 |  |  |
| 133.33 |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 |  | 30 |
|  | Device D2R CBW (kHz) |  | 33 | 49.5 | 82.5 | 148.5 | 280.5 | 544.5 | 1072.5 |  |  |
|  | BS D2R CBW (kHz) |  | 36.7 | 55 | 91.7 | 165 | 311.7 | 605 | 1191.7 |  |  |
| 66.67 |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 |  | 60 |
|  | Device D2R CBW (kHz) |  |  | 66 | 99 | 165 | 297 | 561 | 1089 |  |  |
|  | BS D2R CBW (kHz) |  |  | 73.3 | 110 | 183.3 | 330 | 623.3 | 1210 |  |  |
| 33.33 |  |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 |  | 120 |
|  | Device D2R CBW (kHz) |  |  |  | 132 | 198 | 330 | 594 | 1122 |  |  |
|  | BS D2R CBW (kHz) |  |  |  | 146.7 | 220 | 366.7 | 660 | 1246.7 |  |  |
| 16.667 |  |  |  |  |  | R=1 | R=2 | R=4 | R=8 |  | 240 |
|  | Device D2R CBW (kHz) |  |  |  |  | 264 | 396 | 660 | 1188 |  |  |
|  | BS D2R CBW (kHz) |  |  |  |  | 293.3 | 440 | 733.3 | 1320 |  |  |
| 8.333 |  |  |  |  |  |  | R=1 | R=2 | R=4 |  | 480 |
|  | Device D2R CBW (kHz) |  |  |  |  |  | 528 | 792 | 1320 |  |  |
|  | BS D2R CBW (kHz) |  |  |  |  |  | 586.7 | 880 | 1466.7 |  |  |
| 4.167 |  |  |  |  |  |  |  | R=1 | R=2 |  | 960 |
|  | Device D2R CBW (kHz) |  |  |  |  |  |  | 1056 | 1584 |  |  |
|  | BS D2R CBW (kHz) |  |  |  |  |  |  | 1173.3 | 1760 |  |  |
| 1.389 |  |  |  |  |  |  |  |  |  | R=1 | 2880 |
|  | Device D2R CBW (kHz) |  |  |  |  |  |  |  |  | 3168 |  |
|  | BS D2R CBW (kHz) |  |  |  |  |  |  |  |  | 3520 |  |

Proposal 4: Adopt Table 7's A-IoT D2R CBW values for BS and device.

There are total 36 kinds of D2R CBW for BS and device separately. This would complicate both filter design and network configuration. Fewer D2R CBW variants can streamline both BS receiver filter design and network configurations.

Since the minimum R2D channel bandwidth has been agreed as 200 kHz, we propose setting the minimum D2R channel bandwidth (CBW) to the same value (200 kHz). The maximum D2R CBW can be 3.52M for BS. Other BS D2R CBWs for testing in future release is not excluded.

Unlike NR, A-IoT devices support small frequency shifts. For D2R CBW, the configuration is flexible and dynamic, so the concept of transmission bandwidth configuration isn't needed.

Proposal 5: Transmission bandwidth configuration isn't needed for D2R.

Proposal 6: From A-IoT BS sides, define D2R CBW 200 kHz and 3.52MHz for testing. Other BS D2R CBWs for testing in future release is not excluded.

Note that Proposal 6 still allows A-IoT BS to support all D2R CBWs listed in Table 6.

  - R2D Channel raster offset

On R2D raster, the agreement from the last meeting was:

Copied from [3]

Issue 2-3-1: R2D channel raster

Agreement in RAN4#115:

-	Define 10KHz channel raster

- -	FFS on channel raster offset

In the TS38.291, A-IoT adopt the similar OFDM baseband signal generation. Figure 4 indicates Channel Raster and Transmission BW center location for A-IoT carrier.

For Odd-PRB A-IoT carriers, Channel Raster (Red Arrow) is centered on subcarrier #6; Transmission BW Center (Blue Arrow) is at the Boundary between subcarriers #5 and #6;

For Even-PRB A-IoT carriers, Channel Raster (Red Arrow) is centered on subcarrier #0; Transmission BW Center (Blue Arrow) is at the Boundary between subcarriers #11 and #0.

Figure 5 Diagram of channel raster offset

A-IoT's asymmetric channel bandwidth requires special consideration. We previously agreed to align A-IoT carrier center frequency (Channel Raster Position) with the transmission bandwidth center (Blue Arrow) similar with NB-IoT, therefore, there was an agreement to define symmetric guard band for R2D CBW.

Copied from [3]

Issue 2-1-1: R2D transmission bandwidth

Agreement in RAN4#115:

- Define symmetric guard band as 10kHz, 20kHz, 30kHz and 40kHz for 1PRB, 2PRB, 3PRB and 4PRB channel bandwidth configuration

Per TS 38.291 specifications, we need now remove the previous symmetric guardband agreement. Specify only minimum guardband requirements.

Proposal 7: No need to define R2D channel raster offset.

Proposal 8: Remove the agreement in previous meeting on symmetric guard band for R2D CBW.

Proposal 9: Use the minimum guardband requirements for R2D CBW in Table 8.

Table 8: Minimum guardband (kHz)

| R2D CBW | 200kHz | 400kHz | 600kHz | 800kHz |
| --- | --- | --- | --- | --- |
| Minimum guardband(kHz) | 2.5 | 12.5 | 22.5 | 32.5 |

Figure 6: BS PRB utilization

- Summary

In this contribution, we discuss key open issues regarding A-IoT general aspects and present the following observations and proposals:

Proposal 1： For A-IoT BS, 90% filter spectrum utility (10% guard band) is considered for D2R CBW.

Proposal 2:

Using the following equation for BS D2R CBW:

D2R CBW for BS (kHz)

= (2SB Transmission BW_without SFO× (1/2) +2× Small frequency shift_without SFO)/0.9

=(2000×(1+R)/Tb) × (1+∣SFO∣)/0.9

=(1000×(R+1)/ (Tc ×R)) × (1+∣SFO∣)/0.9		(Eq. 4)

The transmission bandwidth (BW) and small frequency shift are in kHz, while Tc and Tb are in μs.

The 0.9 divisor presents the 90% BS filter spectrum utility (10% guard band).

Proposal 3:

- Using following equation for device D2R CBW

D2R CBW for device (kHz)

=2SB Transmission BW_without SFO× (1/2) +2× Small frequency shift_without SFO

=(2000×(1+R)/Tb) × (1+∣SFO∣)

=(1000×(R+1)/ (Tc ×R)) × (1+∣SFO∣) 		(Eq. 5)

The transmission bandwidth (BW) and small frequency shift are in kHz, while Tc and Tb are in μs.

Proposal 4: Adopt Table 7's A-IoT D2R CBW values for BS and device.

Table 7 D2R CBW with different Tc,Tb, and R combinations

|  |  | Chip duration:Tc=2/SCS/(1,2,4,8,16,32,64,192) (μs) |  |  |  |  |  |  |  |  | 2SB=4/Tb（kHz） |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.69 |  |
|  |  | τ | τ2 | τ4 | τ8 | τ16 | τ32 | τ64 | τ128 | τ192 |  |
|  |  | Small Frequency Shift：SFS=1/(2×Tc)= R/Tb (kHz) |  |  |  |  |  |  |  |  |  |
|  |  | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 | 720 |  |
| Bit duration:Tb= Tc×2R(μs) |  |  |  |  |  |  |  |  |  |  |  |
| 266.67 |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 |  | 15 |
|  | Device D2R CBW (kHz) | 16.5 | 24.8 | 41.3 | 74.3 | 140.3 | 272.3 | 536.3 | 1064.3 |  |  |
|  | BS D2R CBW (kHz) | 18.3 | 27.6 | 45.9 | 82.6 | 155.9 | 302.6 | 595.9 | 1182.6 |  |  |
| 133.33 |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 |  | 30 |
|  | Device D2R CBW (kHz) |  | 33 | 49.5 | 82.5 | 148.5 | 280.5 | 544.5 | 1072.5 |  |  |
|  | BS D2R CBW (kHz) |  | 36.7 | 55 | 91.7 | 165 | 311.7 | 605 | 1191.7 |  |  |
| 66.67 |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 |  | 60 |
|  | Device D2R CBW (kHz) |  |  | 66 | 99 | 165 | 297 | 561 | 1089 |  |  |
|  | BS D2R CBW (kHz) |  |  | 73.3 | 110 | 183.3 | 330 | 623.3 | 1210 |  |  |
| 33.33 |  |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 |  | 120 |
|  | Device D2R CBW (kHz) |  |  |  | 132 | 198 | 330 | 594 | 1122 |  |  |
|  | BS D2R CBW (kHz) |  |  |  | 146.7 | 220 | 366.7 | 660 | 1246.7 |  |  |
| 16.667 |  |  |  |  |  | R=1 | R=2 | R=4 | R=8 |  | 240 |
|  | Device D2R CBW (kHz) |  |  |  |  | 264 | 396 | 660 | 1188 |  |  |
|  | BS D2R CBW (kHz) |  |  |  |  | 293.3 | 440 | 733.3 | 1320 |  |  |
| 8.333 |  |  |  |  |  |  | R=1 | R=2 | R=4 |  | 480 |
|  | Device D2R CBW (kHz) |  |  |  |  |  | 528 | 792 | 1320 |  |  |
|  | BS D2R CBW (kHz) |  |  |  |  |  | 586.7 | 880 | 1466.7 |  |  |
| 4.167 |  |  |  |  |  |  |  | R=1 | R=2 |  | 960 |
|  | Device D2R CBW (kHz) |  |  |  |  |  |  | 1056 | 1584 |  |  |
|  | BS D2R CBW (kHz) |  |  |  |  |  |  | 1173.3 | 1760 |  |  |
| 1.389 |  |  |  |  |  |  |  |  |  | R=1 | 2880 |
|  | Device D2R CBW (kHz) |  |  |  |  |  |  |  |  | 3168 |  |
|  | BS D2R CBW (kHz) |  |  |  |  |  |  |  |  | 3520 |  |

Proposal 5: Transmission bandwidth configuration isn't needed for D2R.

Proposal 6: From A-IoT BS sides, define D2R CBW 200 kHz and 3.52MHz for testing. Other BS D2R CBWs for testing in future release is not excluded.

Proposal 7: No need to define R2D channel raster offset.

Proposal 8: Remove the agreement in previous meeting on symmetric guard band for R2D CBW.

Proposal 9: Use the minimum guardband requirements for R2D CBW in Table 8.

Table 8: Minimum guardband (kHz)

| R2D CBW | 200kHz | 400kHz | 600kHz | 800kHz |
| --- | --- | --- | --- | --- |
| Minimum guardband(kHz) | 2.5 | 12.5 | 22.5 | 32.5 |

- References

[1]RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN1 Vice-chair (Huawei), RAN#106 Dec.,2024

[2]RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA Inc.，RAN#107 Mar.,2025

[3]R4-2508116, WF on A-IOT device requirements, CMCC, RAN4#115, May, 2025

[4] R1-2504894, Chair notes RAN1#121 (9.4 R19 Ambient IoT) v08 (eom) , RAN1#121, May, 2025

[5] R1-2503020, Summary #1 for coding aspects of physical channel design, RAN1#120bis, April, 2025

[6] R1-2407669, On general aspects of physical layer design for Ambient IoT, Huawei, HiSilicon, RAN1#118bis, October,2024

- Text Proposal for TS38.191

<Start of Text Proposal>

## 5.3	Channel bandwidth

### 5.3.1	R2D Channel bandwidth

### 5.3.1.1	General

The R2D channel bandwidth supports a single reader RF carrier in R2D link at the reader.

The relationship between the R2D channel bandwidth, the guardband and the transmission bandwidth configuration is shown in figure 5.3.1.1-1.

.

Figure 5.3.1.1-1: Definition of channel bandwidth and transmission bandwidth configuration for one reader channel

### 5.3.1.2	Transmission bandwidth configuration

The transmission bandwidth configuration NRB for each reader channel bandwidth and subcarrier spacing is specified in table 5.3.1.2-1.

Table 5.3.1.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 200 kHz | 400 kHz | 600 kHz | 800 kHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 15 | 1 | 2 | 3 | 4 |

NOTE:	All BS Tx and device Rx requirements are defined based on transmission bandwidth configuration specified in table 5.3.2-1.

### 5.3.1.3	Minimum guardband and transmission bandwidth configuration

The minimum guardband for each reader channel bandwidth and SCS is specified in table 5.3.3-1.

Table 5.3.3-1: Minimum guardband (kHz)

| R2D CBW | 200kHz | 400kHz | 600kHz | 800kHz |
| --- | --- | --- | --- | --- |
| Minimum guardband(kHz) | 2.5 | 12.5 | 22.5 | 32.5 |

The number of RBs configured in any reader channel bandwidth shall ensure that the minimum guardband specified in this clause is met.

Figure 5.3.1.3-1: BS PRB utilization

### 5.3.1.4	RB alignment

For each reader channel bandwidth, BS transmission bandwidth configuration must fulfil the minimum guardband requirement specified in clause 5.3.3.

### 5.3.1.5	BS channel bandwidth per operating band

The requirements in this specification apply to the combination of BS channel bandwidths, SCS and operating bands shown in table 5.3.5-1 for FR1. The transmission bandwidth configuration in table 5.3.2-1 shall be supported for each of the BS channel bandwidths within the BS capability. The BS channel bandwidths are specified for the Tx path.

Table 5.3.1.5-1: Reader channel bandwidths and SCS per operating band

| NR Band | SCS (kHz) | Reader channel bandwidth (kHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 200 | 400 | 600 | 800 |
| n8 | 15 | 200 | 400 | 600 | 800 |

### 5.3.2	D2R Channel bandwidth

For reader, D2R CBW can be calculated as follows:

D2R CBW for BS (kHz)

= (2SB Transmission BW_without SFO× (1/2) +2× Small frequency shift_without SFO)/0.9

=(2000×(1+R)/Tb) × (1+∣SFO∣)/0.9

=(1000×(R+1)/(Tc×R)) × (1+∣SFO∣)/0.9

The transmission bandwidth (BW) and small frequency shift are in kHz, while Tc and Tb are in μs.

The 0.9 divisor presents the 90% BS filter spectrum utility (10% guard band).

For device, D2R CBW can be calculated as follows:

D2R CBW for device (kHz)

=2SB Transmission BW_without SFO× (1/2) +2× Small frequency shift_without SFO

=(2000×(1+R)/Tb) × (1+∣SFO∣)

=(1000×(R+1)/ (Tc ×R)) × (1+∣SFO∣)

The transmission bandwidth (BW) and small frequency shift are in kHz, while Tc and Tb are in μs.

D2R CBW are summarized in Table 5.3.2-1.

Table 5.3.2-1 D2R CBW with different Tc, Tb and R combinations

|  |  | Chip duration:Tc=2/SCS/(1,2,4,8,16,32,64,192) (μs) |  |  |  |  |  |  |  |  | 2SB=4/Tb（kHz） |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.69 |  |
|  |  | τ | τ2 | τ4 | τ8 | τ16 | τ32 | τ64 | τ128 | τ192 |  |
|  |  | Small Frequency Shift：SFS=1/(2×Tc)= R/Tb (kHz) |  |  |  |  |  |  |  |  |  |
|  |  | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 | 720 |  |
| Bit duration:Tb= Tc×2R(μs) |  |  |  |  |  |  |  |  |  |  |  |
| 266.67 |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 |  | 15 |
|  | Device D2R CBW (kHz) | 16.5 | 24.8 | 41.3 | 74.3 | 140.3 | 272.3 | 536.3 | 1064.3 |  |  |
|  | BS D2R CBW (kHz) | 18.3 | 27.6 | 45.9 | 82.6 | 155.9 | 302.6 | 595.9 | 1182.6 |  |  |
| 133.33 |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 |  | 30 |
|  | Device D2R CBW (kHz) |  | 33 | 49.5 | 82.5 | 148.5 | 280.5 | 544.5 | 1072.5 |  |  |
|  | BS D2R CBW (kHz) |  | 36.7 | 55 | 91.7 | 165 | 311.7 | 605 | 1191.7 |  |  |
| 66.67 |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 |  | 60 |
|  | Device D2R CBW (kHz) |  |  | 66 | 99 | 165 | 297 | 561 | 1089 |  |  |
|  | BS D2R CBW (kHz) |  |  | 73.3 | 110 | 183.3 | 330 | 623.3 | 1210 |  |  |
| 33.33 |  |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 |  | 120 |
|  | Device D2R CBW (kHz) |  |  |  | 132 | 198 | 330 | 594 | 1122 |  |  |
|  | BS D2R CBW (kHz) |  |  |  | 146.7 | 220 | 366.7 | 660 | 1246.7 |  |  |
| 16.667 |  |  |  |  |  | R=1 | R=2 | R=4 | R=8 |  | 240 |
|  | Device D2R CBW (kHz) |  |  |  |  | 264 | 396 | 660 | 1188 |  |  |
|  | BS D2R CBW (kHz) |  |  |  |  | 293.3 | 440 | 733.3 | 1320 |  |  |
| 8.333 |  |  |  |  |  |  | R=1 | R=2 | R=4 |  | 480 |
|  | Device D2R CBW (kHz) |  |  |  |  |  | 528 | 792 | 1320 |  |  |
|  | BS D2R CBW (kHz) |  |  |  |  |  | 586.7 | 880 | 1466.7 |  |  |
| 4.167 |  |  |  |  |  |  |  | R=1 | R=2 |  | 960 |
|  | Device D2R CBW (kHz) |  |  |  |  |  |  | 1056 | 1584 |  |  |
|  | BS D2R CBW (kHz) |  |  |  |  |  |  | 1173.3 | 1760 |  |  |
| 1.389 |  |  |  |  |  |  |  |  |  | R=1 | 2880 |
|  | Device D2R CBW (kHz) |  |  |  |  |  |  |  |  | 3168 |  |
|  | BS D2R CBW (kHz) |  |  |  |  |  |  |  |  | 3520 |  |

## 5.4	Channel arrangement

### 5.4.1	R2D Channel raster

#### 5.4.2.1	AIoT-ARFCN and channel raster

The global frequency raster defines a set of RF reference frequencies FREF. The RF reference frequency is used in signalling to identify the position of RF channels, SS blocks and other elements. The global frequency raster is defined for all frequencies from 0 to 100 GHz. The granularity of the global frequency raster is ΔFGlobal.

RF reference frequencies are designated by an A-IoT Absolute Radio Frequency Channel Number (AIoT-ARFCN) in the range [0…3279165] on the global frequency raster. The relation between the AIoT-ARFCN and the RF reference frequency FREF in MHz is given by the following equation, where FREF-Offs and NRef-Offs are given in table 5.4.2.1-1 and NREF is the AIoT-ARFCN.

FAREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Range of frequencies (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |

The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. The RF reference frequency for an RF channel maps to a resource element on the carrier. For each operating band, a subset of frequencies from the global frequency raster are applicable for that band and forms a channel raster with a granularity ΔFRaster, which may be equal to or larger than ΔFGlobal.

R2D channel raster is 10 kHz.

#### 5.4.2.2	Channel raster to resource element mapping

The mapping between the RF reference frequency on the channel raster and the corresponding resource element is given in table 5.4.2.2-1 and can be used to identify the RF channel position. The mapping depends on the total number of RBs that are allocated in the channel and applies to both UL and DL. The mapping must apply to at least one numerology supported by the BS.

Table 5.4.2.2-1: Channel Raster to Resource Element Mapping

|  | NRBmod2=0 | NRBmod2=1 |
| --- | --- | --- |
| Resource element index | 0 | 6 |
| Physical resource block index |  |  |

NRB is the transmission bandwidth configuration specified in sub-clause 5.3.2, is the PRB index within the NRB, and  is the resource element index within this PRB.

<End of Text Proposal>