3GPP TSG-RAN4 Meeting #116 	R4-2509883

Malta, MT, 25th – 29rd Aug, 2025

Source: 	Huawei, HiSilicon

Title:  	Ambient IoT BS RF requirements

Agenda Item:	7.22.3.1

Document for:	Approval

- Introduction

The WI on solutions for Ambient IoT in NR was approved at RAN#106 [1] and revised at RAN#107[2]. Last meeting, a WF was agreed [3]. This contribution provides our consideration and proposal for the open issues for Ambient-IoT BS.

- Ambient IoT BS TX requirements

2. 1 Modulation quality

On modulation quality, the agreement from the last meeting was:

Copied from [3]:

- Agreement:

  - Consider following options

    - Option 1: Take the table below as a starting point.

| Tari | Parameter | Symbol | Mimimum | Nominal | Maximum | Units |
| --- | --- | --- | --- | --- | --- | --- |
| Tari= 103M*15 (us) M = 2, 6, 12 and 24 | Modulation Depth | (A–B)/A | 80 | 90 | 100 | % |
|  | RF Envelope Ripple | Mh = Ml | 0 |  | [0.5~0.15] (A–B) | V/m orA/m |
|  | RF Envelop Rise Time | Tr,10-90 |  |  | 0.33Tari | µs |
|  | RF Envelop Fall Time | Tf,10-90 |  |  | 0.33Tari | µs |
|  | RF Pulsewidth | PW |  | 0.5Tari | [0.65/0.525] Tari | µs |

      - Note 1: The ripple is calculated based on per-chip, comparing peak value of each chip to its own average.

      - Note 2: SIP is excluded for ripple calculation.

    - Option 2: No need to define RF envelop parameters.

Option 2 is not device-friendly. Without clear transmit signal quality requirements from the reader side, the device would lack defined design and demodulation criteria, making system performance difficult to guarantee.

Option 1 is preferable since Device 1 uses envelope detection to demodulates OOK time-domain signals similarly to RFID. Consider effect to device modulation performance, it is need to define modulation depth, RF envelope ripple, RF envelop rise/fall time and RF Pulse width.

R2D signal is an OFDM-based OOK-4 waveform with subcarrier spacing of 15kHz, and there are Mchipsymb consecutive chips within one OFDM symbol.

Symbol duration (15kHz SCS) = 103/15 μs

R2D Chip duration = (103/15)/ Mchipsymb μs, where Mchipsymb ∈ {2,6,12,24}

R2D chip duration TchipR2D values for different M are given in Table 1.

Table 1: TchipR2D values for different M

| Mchipsymb | NRBmin | Chip duration | Value(μs) |
| --- | --- | --- | --- |
| 2 | 1 | TchipR2D=1MchipsymbΔf | 33.33 |
| 6 | 1 |  | 11.11 |
| 12 | 2 |  | 5.56 |
| 24 | 3 |  | 2.78 |

Based on TS38.291[5], R2D signal includes SIP (Start indicator part), CAP (Clock acquisition part), PRDCH, the R2D postamble and padding if needed. Agreement in [5] is that SIP（start indicator part） of R-TAS（R2D timing acquisition signal） is adopted with 2 OFDM symbol duration, i.e. ON-OFF-ON-OFF with a ratio of 2:2:1:3. The R-TAS SIP consists of NSIP=8 bits denoted S0,S1,S2, S3,S4,S5,S6,S7=1, 1, 0, 0, 1, 0, 0, 0 As shown in Figure 1.

Figure 1 R2D signal composition

Figure 2 Simulation R2D transmission signal (1RB, M=6)

Figure3 R2D signal diagram

An is measured peak high level for the nth chip, in units of V/m or A/m

Bn is measured peak low level for the nth chip, in units of V/m or A/m

Anavg is the measured average high level for the nth chip during 1/3 center chip duration, in units of V/m or A/m

Bnavg is the measured average low level for the nth chip during 1/3 center chip duration, in units of V/m or A/m

Ripple (1):

Ripple_high (%) = ((An − Anavg) / (Anag-Bnavg)) × 100%

Ripple_low (%) = ((Bn − Bnavg) / (Anavg-Bnavg)) × 100%

Modulation depth (2):

For each chip, Modulation depth =(Anavg-Bnavg)/Anavg

RF Envelop Rise Time (3):

The time from 0.1 ×(Anavg-Bnavg) +Bnavg to 0.9 ×(Anavg-Bnavg)+Bnavg

RF Envelop Fall Time (4):

The time from 0.9 ×(Anavg-Bnavg) +Bnavg to 0.1 ×(Anavg-Bnavg)+Bnavg

Pulsewidth (5)

The pulse width is the time between two points on the pulse where the signal reaches 50% of (Anavg-Bnavg)+Bnavg

Proposal 1： Adopt the RF envelop parameters definitions 1~5.

Simulation results of Time domain baseband signal are as follow Table 2.

Table 2: Simulation results of A-IoT BS RF envelope parameters

| R2D Chip duration：Tc | Parameter | Symbol | Simulation values | Units |
| --- | --- | --- | --- | --- |
| Tc=103M*15(us) M∈ {2,6,12,24} | Modulation Depth | (A–B)/A | 90 | % |
|  | RF Envelope Ripple | Ripple_high Ripple_low | 10 | % |
|  | RF Envelop Rise Time | Tr,10-90 | 0.66Tc | µs |
|  | RF Envelop Fall Time | Tf,10-90 | 0.66Tc | µs |
|  | RF Pulsewidth | PW | 1.1 Tc | µs |

The overall signal quality over the air depends on both the baseband signal quality and degradation due to digital filtering and RF impairments may need to be included.Of these, the baseband signal quality has the greater impact.

Simulation results of Time domain baseband signal for Different M with 1-4 RB numbers are as follows：

Proposal 2: Consider RF impairments, the RF envelope requirements can be defined as Table 3.

Table 3: A-IoT BS RF envelope parameters

| R2D Chip duration：Tc | Parameter | Symbol | Mimimum | Nominal | Maximum | Units |
| --- | --- | --- | --- | --- | --- | --- |
| Tc=103M*15(us) M∈ {2,6,12,24} | Modulation Depth | (A–B)/A | 80 | 90 | 100 | % |
|  | RF Envelope Ripple | Ripple_high Ripple_low | 0 |  | ±15 | % |
|  | RF Envelop Rise Time | Tr,10-90 |  |  | 0.66Tc | µs |
|  | RF Envelop Fall Time | Tf,10-90 |  |  | 0.66Tc | µs |
|  | RF Pulsewidth | PW |  | 1Tc | 1.3 Tc | µs |

2. 2	Adjacent Channel Leakage Power Ratio

Copied from R4-2508101[3]

### Issue 1-5-2: ACLR

- Agreement:

  - Consider following options for ACLR requirements

|  | Option 1 |  | Option 2 |  | Option 3 |  | Option 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R2D CBW | offset | ACLR | offset | ACLR | offset | ACLR | offset | ACLR |
| 200 kHz | 100kHz | 40/50dB | 200kHz | 40/50dB | 100kHz | 30/40dB | 100kHz | 30/40dB |
| 400 kHz | 100kHz | 40/50dB | 200kHz | 40/50dB | 100kHz | 30/40dB | [100]kHz | 30/40dB |
| 600 kHz | 100kHz | 40/50dB | 200kHz | 40/50dB | 100kHz | 30/40dB | N/A | 30/40dB |
| 800 kHz | 100kHz | 40/50dB | 200kHz | 40/50dB | 100kHz | 30/40dB | N/A | 30/40dB |

For co-existence Scenario 2-1c (NR UE only outdoor) in TR38.769, simulation results show that even with A-IoT BS ACLR of 20dB, there is negligible degradation in NR DL throughput, both for average and cell-edge throughput.

While A-IoT R2D uses OFDM-based signals, reusing NB-IoT ACLR requirements isn't appropriate. The R2D baseband signal spectrum spreads significantly due to OOK modulation, and the 400/600/800kHz channel bandwidths are new configurations specific to A-IoT R2D.

Proposal 3: For A-IoT R2D carriers with 200-800kHz CBW, use the ACLR values shown below Table 4

Table 4: A-IoT BS ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (kHz) | Offset(kHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- | --- |
| 200/400/600/800 | 100 | BWChannel | A-IoT of same BW | Square (BWConfig) | 40 dB |
|  | 100 | 2 x BWChannel | A-IoT of same BW | Square (BWConfig) | 45 dB |

Figure 4 Diagram of A-IoT ACLR with 100kHz Foffset

2.3	Operating band unwanted emissions

For 200kHz R2D CBW, Operating band unwanted emissions can reuse the requirements of standalone NB-IoT Medium Range BS.

For A-IoT medium range BS (maximum output power 31 < Prated,c  38 dBm), emissions shall not exceed the maximum levels specified in Table 5.

Table 5 A-IoT medium range BS operating band unwanted emission limits, BS maximum output power 31 < Prated,c  38 dBm, R2D CBW=200kHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2, 3) | Measurement bandwidth (Note 8) |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz (Note 1) | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0.065 MHz f_offset < 0.165 MHz |  | 30 kHz |
| 0.15 MHz f < 0.6 MHz (Note 1) | 0.015MHz f_offset < 0.615MHz |  | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615MHz f_offset < 1.015MHz |  | 30 kHz |
| (Note 6) | 1.015MHz f_offset < 1.5 MHz | Prated,c - 65 dB | 30 kHz |
| 1 MHz f 2.8 MHz | 1.5 MHz f_offset < 3.3 MHz | Prated,c - 52 dB | 1 MHz |
| 2.8 MHz f 5 MHz | 3.3 MHz f_offset < 5.5 MHz | min(Prated,c - 52 dB, -15dBm) | 1 MHz |
| 5 MHz f fmax | 5.5 MHz f_offset < f_offsetmax | Prated,c - 56 dB | 1 MHz |
| NOTE 1: The limits in this table only apply for operation with a standalone NB-IoT carrier adjacent to the Base Station RF Bandwidth edge. NOTE 2: For BS supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. NOTE 3: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

For A-IoT medium range BS (maximum output power Prated,c  31 dBm), emissions shall not exceed the maximum levels specified in Table 6.

Table 6 A-IoT medium range BS operating band unwanted emission limits, BS maximum output power Prated,c  31 dBm , R2D CBW=200kHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2, 3, 4) | Measurement bandwidth (Note 8) |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz (Note 1) | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0.065 MHz f_offset < 0.165 MHz |  | 30 kHz |
| 0.15 MHz f < 0.6 MHz (Note 1) | 0.015MHz f_offset < 0.615MHz |  | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615MHz f_offset < 1.015MHz |  | 30 kHz |
| (Note 6) | 1.015MHz f_offset < 1.5 MHz | -34 dBm | 30 kHz |
| 1 MHz f 5 MHz | 1.5 MHz f_offset < 5.5 MHz | -21 dBm | 1 MHz |
| 5 MHz f fmax | 5.5 MHz f_offset < f_offsetmax | -25 dBm | 1 MHz |
| NOTE 1: The limits in this table only apply for operation with a NB-IoT carrier adjacent to the Base Station RF Bandwidth edge. NOTE 2: For BS supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. NOTE 3: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 4: In case the carrier adjacent to the RF bandwidth edge is a NB-IoT carrier, the value of X = PNB-IoTcarrier – 31, where PNB-IoTcarrier is the power level of the NB-IoT carrier adjacent to the RF bandwidth edge. In other cases, X = 0. |  |  |  |

Proposal 4: For A-IoT micro BS with 200 kHz R2D CBW, reuses the operating band unwanted emissions requirements of standalone NB-IoT Medium Range BS for A-IoT micro BS.

Proposal 5: For A-IoT micro BS with 400/600/800kHz R2D CBW, use the operating band unwanted emissions requirements in Table 7 and 8.

Table7 A-IoT medium range BS operating band unwanted emission limits, BS maximum output power 31 < Prated,c  38 dBm , for 400/600/800kHz R2D CBW

| R2D CBW | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 8) |
| --- | --- | --- | --- | --- |
| 400kHz | 0 MHz f < 0.4 MHz | 0.015 MHz f_offset < 0.415 MHz | Prated,c -40dB - 110.4 (f_offsetMHz - 0.015) dB | 30 kHz |
|  | 0.4 MHz f < 0.8 MHz | 0.415 MHz f_offset < 0.815 MHz | Prated,c - 51dB- 50.4 (f_offsetMHz-0.415) dB | 30 kHz |
|  | 0.8 MHz f < 1.6 MHz | 0.815 MHz f_offset < 1.6 MHz | Prated,c - 56dB | 30 kHz |
|  | 1.6 MHz f < fmax | 1.6 MHz f_offset < f_offsetmax | -25dBm | 100kHz |
| 600kHz | 0 MHz f < 0.6 MHz | 0.015 MHz f_offset < 0.615 MHz | Prated,c – 40dB - 130.6 (f_offsetMHz-0.015) dB | 30 kHz |
|  | 0.6 MHz f < 1.2 MHz | 0.615 MHz f_offset < 1.2 MHz | Prated,c – 53dB - 50.6 (f_offsetMHz-0.615) dB | 30 kHz |
|  | 1.2 MHz f < 1.8 MHz | 1.2 MHz f_offset < 1.8 MHz | Prated,c - 58dB | 30 kHz |
|  | 1.8 MHz f < fmax | 1.8 MHz f_offset < f_offsetmax | -25dBm | 100K |
| 800kHz | 0 MHz f < 0.8 MHz | 0.015 MHz f_offset < 0.815 MHz | Prated,c – 40dB- 140.8 (f_offsetMHz-0.015) dB | 30 kHz |
|  | 0.8 MHz f < 1.6 MHz | 0.815 MHz f_offset < 1.6 MHz | Prated,c – 54dB- 50.8 (f_offsetMHz-0.815) dB | 30 kHz |
|  | 1.6 MHz f < 2.4 MHz | 1.6 MHz f_offset < 2.4 MHz | Prated,c - 59dB | 30 kHz |
|  | 2.4 MHz f < fmax | 2.4 MHz f_offset < f_offsetmax | -25dBm | 100K |

Table 8 A-IoT medium range BS operating band unwanted emission limits, BS maximum output power Prated,c  31 dBm , for 400/600/800kHz R2D CBW

| R2D CBW | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 8) |
| --- | --- | --- | --- | --- |
| 400kHz | 0 MHz f < 0.4 MHz | 0.015 MHz f_offset < 0.415 MHz | Prated,c -30dB - 110.4 (f_offsetMHz - 0.015) dB | 30 kHz |
|  | 0.4 MHz f < 0.8 MHz | 0.415 MHz f_offset < 0.815 MHz | Prated,c - 41dB- 50.4 (f_offsetMHz-0.415) dB | 30 kHz |
|  | 0.8 MHz f < 1.6 MHz | 0.815 MHz f_offset < 1.6 MHz | -31dBm | 30 kHz |
|  | 1.6 MHz f < fmax | 1.6 MHz f_offset < f_offsetmax | -25dBm | 100kHz |
| 600kHz | 0 MHz f < 0.6 MHz | 0.015 MHz f_offset < 0.615 MHz | Prated,c – 30dB - 130.6 (f_offsetMHz-0.015) dB | 30 kHz |
|  | 0.6 MHz f < 1.2 MHz | 0.615 MHz f_offset < 1.2 MHz | Prated,c – 43dB - 50.6 (f_offsetMHz-0.615) dB | 30 kHz |
|  | 1.2 MHz f < 1.8 MHz | 1.2 MHz f_offset < 1.8 MHz | -21dBm | 30 kHz |
|  | 1.8 MHz f < fmax | 1.8 MHz f_offset < f_offsetmax | -25dBm | 100K |
| 800kHz | 0 MHz f < 0.8 MHz | 0.015 MHz f_offset < 0.815 MHz | Prated,c – 30dB- 140.8 (f_offsetMHz-0.015) dB | 30 kHz |
|  | 0.8 MHz f < 1.6 MHz | 0.815 MHz f_offset < 1.6 MHz | Prated,c – 44dB- 50.8 (f_offsetMHz-0.815) dB | 30 kHz |
|  | 1.6 MHz f < 2.4 MHz | 1.6 MHz f_offset < 2.4 MHz | -21dBm | 30 kHz |
|  | 2.4 MHz f < fmax | 2.4 MHz f_offset < f_offsetmax | -25dBm | 100K |

- Ambient IoT BS Rx requirements

  - Reference sensitivity level

The conclusion of reference sensitivity level for A-IoT BS in SI is as follows:

PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ desens target]

Copied from R4-2508101[3]

Issue 2-1-1: Reference sensitivity level

- Agreement:

  - Define REFSENS for both OOK and BPSK. Depending on the simulations, they may or may not be the same.

  - Use [X dB] as the desens target, with CW single tone input through a signal generator, CW input level to BS Ant is assumed as [TBD] dBm;

Proposal 6: R=1 is  the worst case for sensitivity，suggest to consider reference sensitivity requirements for the reader without frequency shift (R=1)

To determine the D2R modulation SNR, 15kHz D2R TBW LLS is performed under the assumption in the following table.

Table 9: D2R LLS simulation parameters Applied

| Parameters | Simulation parameter | HW option |
| --- | --- | --- |
| Carrier frequency | 900 MHz |  |
| SCS | 15 kHz |  |
| Channel model | AWGN |  |
| Number of Tx/Rx chains for Ambient IoT device | 1 |  |
| Number of BS antenna elements | 1 |  |
| Reference data rate | 1 kbps (M) | NA |
| Message size | Option 1： {20 bits, 96 bits, 400 bits} are considered for message size. Note 1: companies to report the M value and chip length used for each message size Note 2: CRC is not included for the message size Option 2： 96 bits | 96 bits |
| BLER target | 10% |  |
| Sampling frequency（SFO） | Option 1: Randomly select a value from the range of [104 ppm~105 ppm] for device 1 Option 2: Use Residual SFO based on companies report | Option1 |
| [OOK/BPSK chip rate] | 7.5 kcps for 2SB 15 kHz Transmission bandwidth |  |
| Receiver bandwidth | Option1: the same as transmission bandwidth. Option 2：18 kHz (consider SFO and BS filter) for 2SB 15 kHz transmission bandwidth Option 3: first sidelobes as receiver bandwidth Companies can report | 18kHz |
| Waveform (CW) | unmodulated single tone |  |
| Modulation | Option1：OOK Option 2：BPSK Companies can report | BPSK |
| Line code | Manchester encoding |  |
| FEC | 1/3 Convolutional Code |  |
| ADC bit width | Ideal |  |
| D2R receiver | coherent receiver |  |
| Repetition | None |  |
| Preamble configuration | Companies can report | 31 bit m-sequence |
| Midamble configuration | companies can report | No midamble |

D2R demodulation SNR simulation result is as follows Figure 5:

Figure 5: 15kHz D2R TBW demodulation SNR simulation results（BPSK）

Proposal 7: The SNR is -6dB @ 10% BLER for BPSK 15kHz 2SB D2R TBW.

2.88MHz D2R TBW LLS is performed under the assumption in the following table.

Table 10: D2R LLS simulation parameters Applied

| Parameters | Simulation options |
| --- | --- |
| Carrier frequency | 900 MHz |
| SCS | 15 kHz |
| Channel model | AWGN |
| Number of Tx/Rx chains for Ambient IoT device | 1 |
| Number of BS antenna elements | 1 |
| Message size | 96 bits |
| BLER target | 10% |
| Sampling frequency（SFO） | Randomly select a value from the range of [104 ppm~105 ppm] for device 1 |
| [OOK/BPSK chip rate] | 1440 kcps for 2SB 2880kHz Transmission bandwidth |
| Receiver bandwidth | 3.52 MHz (consider SFO and BS filter) for 2SB 2880 kHz transmission bandwidth |
| Waveform (CW) | unmodulated single tone |
| Modulation | BPSK |
| Line code | Manchester encoding |
| FEC | 1/3 Convolutional Code |
| ADC bit width | Ideal |
| D2R receiver | coherent receiver |
| Repetition | None |
| Preamble configuration | 31 bit m-sequence |
| Midamble configuration | 31 bit m-sequence |

Proposal 8: The SNR is -6dB @ 10% BLER for BPSK 2880kHz 2SB D2R TBW

Under identical conditions (AWGN channel, 10% BLER, coherent detection), BPSK typically requires 3 dB less Eb/N0 than OOK to reach the demodulation threshold. BPSK has twice-as-large Euclidean distance. OOK transmits only in the "on" state, wasting 50% of energy

Proposal 9: Use a 3 dB higher SNR than that of BPSK to calculate the OOK reference sensitivity under the same conditions.

CW effect to reference sensitivity include 3 main aspects: ideal CW single tone, CW phase noise, reciprocal mixing noise.

For ideal CW single tone:

To evaluate the desens target, maximum input power at A-IoT BS receiver antenna can be calculated as :

CW output power: 33dBm

The distance between CW and A-IoT BS: 20m

Using the InF-DH NLOS pathloss model as in TR38.901, pathloss for 20m isolation distance: 61dB

Normally the CW node and A-IoT node deploy at the same height, the antenna gain is about -5 dBi at 90°offset from the boresight for both TX antenna and RX antenna.

Maximum input power from CW to A-IoT BS receiver: 33-5-5-61=-38dBm

Analog cancellation in the RF front-end can be applied to reduce the desensitization of the receiver. Assumed analog cancellation capability is 10dB.

The input interference to the A-IoT BS LNA is -38-10=-48dBm, compared with -44dBm narrow-band bloking, A-IoT BS receiver will not be blocked.

For D1T1-B, through spatial isolation(61dB@20m), Analog cancellation （10dB）and Digital cancellation（80dB）, the overall suppression to the carrier-wave interference can reach about (61+10+80) = 151 dB. The leakage of CW after CW cancellation is 33-151=-118 dBm.

A-IoT noise floor：-174+10*lg (15)+30+10=-122dBm/15kHz

Leakage of CW is 4dB higher than noise floor, the desen =10*lg((10^(4/10)+1))=5.5dB

For ideal CW single tone, there is about 6dB desensitivity.

For CW phase noise:

The effect of CW phase noise includes:

1) CW leakage level to BS receiver

2) CW phase noise

3) small frequency shift——The worst case occurs with a 15 kHz transmission bandwidth and no frequency shift. Applying a frequency shift reduces phase noise impact—the greater the shift, the lower the impact.

Assume range of residual integral phase noise in 15kHz is 50 to 90dBc, Sensitivity deterioration is calculated as follows:

Table 11: Desen due to CW phase noise

| Maximum input power from CW to A-IoT BS receiver（dBm） | CW integral phase noise（dBc） | interference level from CW phase noise（dBm） | AIoT BS RX noise floor（dBm/15kHz) | Interference higher than noise floor （dB） | Referense sensitivity Desen （dB） |
| --- | --- | --- | --- | --- | --- |
| -38 | 50 | -88 | -122 | 34 | 34 |
| -38 | 60 | -98 | -122 | 24 | 24 |
| -38 | 70 | -108 | -122 | 14 | 14.2 |
| -38 | 80 | -118 | -122 | 4 | 5.5 |
| -38 | 90 | -128 | -122 | -6 | 1 |

The impact of CW phase noise on reference sensitivity varies significantly (1-34 dB range in our calculations) due to differences in phase noise performance and cancellation capabilities.

For reciprocal mixing of CW and BS receiver LO phase noise:

With -38dBm CW input power to A-IoT BS -38dBm, assume range of BS receiver LO integral phase noise in 15kHz is 50 to 90dBc, Sensitivity deterioration is calculated as follows:

Table 12: Desen due to reciprocal mixing of CW and BS receiver LO phase noise

| Maximum input power from CW to A-IoT BS receiver（dBm） | BS receiver LO integral phase noise（dBc） | interference level from BS receiver LO phase noise（dBm） | AIoT BS RX noise floor（dBm/15kHz) | Interference higher than noise floor （dB） | Referense sensitivity Desen （dB） |
| --- | --- | --- | --- | --- | --- |
| -38 | 50 | -88 | -122 | 34 | 34 |
| -38 | 60 | -98 | -122 | 24 | 24 |
| -38 | 70 | -108 | -122 | 14 | 14.2 |
| -38 | 80 | -118 | -122 | 4 | 5.5 |
| -38 | 90 | -128 | -122 | -6 | 1 |

The impact of reciprocal mixing of CW and BS receiver LO phase noise on reference sensitivity varies significantly (1-34 dB range in our calculations from Section 3.1) due to differences in BS receiver LO phase noise performance.

Both CW phase noise and reciprocal mixing noise affect reference sensitivity across a wide range, with the exact degradation heavily dependent on phase noise characteristics, and implementation approach.

CW phase noise affects reference sensitivity across a wide range, with the exact degradation heavily dependent on phase noise characteristics and implementation approach.

Proposal 10 Use 30 dB as the desens target, with CW single tone input through a signal generator, CW input level to BS Ant is assumed as -38 dBm;

For BPSK：

15kHz D2R channel BW：

PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ desens target]

=-174+10*log10(15k)+10+2-6+30=-96dBm

A-IoT BS reference sensitivity levels is -96dBm for 15kHz SCS for BPSK, with CW single tone input level to BS Antenna is assumed as -38 dBm;

3520kHz D2R channel BW：

PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ desens target]

=-174+10*log10(2880k)+10+2-6+30=-73dBm

Proposal 11 Use the following table as A-IoT Micro Area BS reference sensitivity levels

Table 13: A-IoT Micro Area BS reference sensitivity levels

| BS channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | BPSK | -96 |
|  |  | OOK | -93 |
| 3520 | 15 | BPSK | -73 |
|  |  | OOK | -70 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

  - Dynamic range

Assume SNR is -6dB for BPSK, for 15kHz transmission bandwidth D2R：

Pintf=-174+10*log10(BWconfig)+NF+20=-102.2dBm

Pwanted=-96dBm+20dB=-76dBm

Assume SNR is -6dB for BPSK, for 3.52MHz D2R CBW,

Pintf=-174+10*log10(BWconfig)+NF+20=-79.4dBm

Pwanted =-90dBm+20dB=-70dBm

Table 14 Dynamic range

| D2R channel bandwidth [kHz] | D2R transmission bandwidth[kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | 15 | -76 | -102.2 | AWGN |
| 3520 | 2880 | -70 | -79.4 | AWGN |

Proposal 12 Use Table 14 as the dynamic range requirement.

  - In-band selectivity and blocking

    - Adjacent Channel Selectivity

For ACS interference aggressor, the potential aggressor is outdoor NR UE.

Copied from R4-2505097 [3]

### Issue 2-3: ACS

- Agreement:

  - Evaluate ACS requirements considering no legacy NR UE indoor assumption

  - For ACS requirements, use legacy NB-IoT requirements as starting point

  - FFS on the frequency offset for ACS requirement

Figure 6 CDF of interference level from outdoor NR UE

Based on simulation, the interference level from outdoor NR UE is -84dBm for case 2-1f, which is much lower than the -52dBm ACS interference level due to penetration loss. There is no need to define ACS requirement.

If it must be defined, considering the following ACS requirements for A-IoT BS.

Table 15: Adjacent channel selectivity for A-IoT Medium Range BS

| A-IoT channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 19.5dB (Note) | -60 | ±100 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |
| 3520 | PREFSENS + 19.5dB (Note) | -60 | ±100 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |
| Note: PREFSENS depends on the sub-carrier spacing as specified in Table X |  |  |  |  |

Proposal 13: No need to define ACS requirements for A-IoT BS.If it must be defined, considering the following ACS requirements for A-IoT BS.

Table 15: Adjacent channel selectivity for A-IoT Medium Range BS

| A-IoT channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 19.5dB (Note) | -60 | ±100 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |
| 3520 | PREFSENS + 19.5dB (Note) | -60 | ±100 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |
| Note: PREFSENS depends on the sub-carrier spacing as specified in Table X |  |  |  |  |

    - narrowband blocking

Similar with ACS situation, the interference level from outdoor NR UE is quite low for case 2-1f.

If it must be defined, considering the following narrowband blocking requirements for A-IoT BS.

Table 16: Narrowband blocking requirement for A-IoT Medium Range BS

|  | A-IoT channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| Medium Range BS | 200 | PREFSENS + 12 dB (Note 1) | -60 | See Table 17 |
|  | 3520 | PREFSENS + 12 dB (Note 2) | -60 | See Table 17 |
| Note 1: PREFSENS depends on the sub-carrier spacing as specified in Table x. Note 2: PREFSENS depends on the sub-carrier spacing as specified in Table x. |  |  |  |  |

Table 17: Interfering signal for Narrowband blocking requirement for A-IoT Medium Range BS

| A-IoT channel bandwidth of the lowest/highest carrier received [kHz] | Interfering RB centre frequency offset to the lower/upper Base Station RF Bandwdith edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- |
| 200 | ±(240 +m*180), m=0, 1, 2, 3, 4, 9, 14 | 3 MHz E-UTRA signal, 1 RB* |
| 3520 | ±(240 +m*180), m=0, 1, 2, 3, 4, 9, 14 | 3 MHz E-UTRA signal, 1 RB* |
| Note*: Interfering signal consisting of one resource block is positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. |  |  |

Proposal 14: No need to define narrowband blocking requirements for A-IoT BS. If it must be defined, considering the Table 16 and 17 narrowband blocking requirements for A-IoT BS.

  - Receiver intermodulation

    - general intermodulation

For general intermodulation, it is assumed that there is no NR interference indoors, so there's no need to define it.

Proposal 15: No need to define general intermodulation for A-IoT BS since it is assumed that there is no NR interference indoors.

    - narrowband intermodulation

For NB-IoT narrowband intermodulation, there are two interfering signal, one is CW, and another is 1RB;  However, for A-IoT, there is already carrier wave signal, no need to define narrowband intermodulation.

Proposal 16: No need to define narrowband intermodulation for A-IoT BS.

- Summary

In this contribution, we give our opinions of A-IoT BS requirements, and has the following observations and proposals:

For TX requirements:

Proposal 1： Adopt the RF envelop parameters definitions 1~5.

Ripple (1):

Ripple_high (%) = ((An − Anavg) / (Anag-Bnavg)) × 100%

Ripple_low (%) = ((Bn − Bnavg) / (Anavg-Bnavg)) × 100%

Modulation depth (2):

For each chip, Modulation depth =(Anavg-Bnavg)/Anavg

RF Envelop Rise Time (3):

The time from 0.1 ×(Anavg-Bnavg) +Bnavg to 0.9 ×(Anavg-Bnavg)+Bnavg

RF Envelop Fall Time (4):

The time from 0.9 ×(Anavg-Bnavg) +Bnavg to 0.1 ×(Anavg-Bnavg)+Bnavg

Pulsewidth (5)

The pulse width is the time between two points on the pulse where the signal reaches 50% of (Anavg-Bnavg)+Bnavg

Figure3 R2D signal diagram

Proposal 2: Consider RF impairments, the RF envelope requirements can be defined as Table 2.

Table 2: A-IoT BS RF envelope parameters

| R2D Chip duration：Tc | Parameter | Symbol | Mimimum | Nominal | Maximum | Units |
| --- | --- | --- | --- | --- | --- | --- |
| Tc=103M*15(us) M∈ {2,6,12,24} | Modulation Depth | (A–B)/A | 80 | 90 | 100 | % |
|  | RF Envelope Ripple | Ripple_high Ripple_low | 0 |  | ±15 | % |
|  | RF Envelop Rise Time | Tr,10-90 |  |  | 0.66Tc | µs |
|  | RF Envelop Fall Time | Tf,10-90 |  |  | 0.66Tc | µs |
|  | RF Pulsewidth | PW |  | 1Tc | 1.3 Tc | µs |

Proposal 3: For A-IoT R2D carriers with 200-800kHz CBW, use the ACLR values shown below Table 4

Table 4: A-IoT BS ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (kHz) | Offset(kHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- | --- |
| 200/400/600/800 | 100 | BWChannel | A-IoT of same BW | Square (BWConfig) | 40 dB |
|  | 100 | 2 x BWChannel | A-IoT of same BW | Square (BWConfig) | 45 dB |

Proposal 4: For A-IoT micro BS with 200 kHz R2D CBW, reuses the operating band unwanted emissions requirements of standalone NB-IoT Medium Range BS for A-IoT micro BS.

Table 5 A-IoT medium range BS operating band unwanted emission limits, BS maximum output power 31 < Prated,c  38 dBm, R2D CBW=200kHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2, 3) | Measurement bandwidth (Note 8) |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz (Note 1) | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0.065 MHz f_offset < 0.165 MHz |  | 30 kHz |
| 0.15 MHz f < 0.6 MHz (Note 1) | 0.015MHz f_offset < 0.615MHz |  | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615MHz f_offset < 1.015MHz |  | 30 kHz |
| (Note 6) | 1.015MHz f_offset < 1.5 MHz | Prated,c - 65 dB | 30 kHz |
| 1 MHz f 2.8 MHz | 1.5 MHz f_offset < 3.3 MHz | Prated,c - 52 dB | 1 MHz |
| 2.8 MHz f 5 MHz | 3.3 MHz f_offset < 5.5 MHz | min(Prated,c - 52 dB, -15dBm) | 1 MHz |
| 5 MHz f fmax | 5.5 MHz f_offset < f_offsetmax | Prated,c - 56 dB | 1 MHz |
| NOTE 1: The limits in this table only apply for operation with a standalone NB-IoT carrier adjacent to the Base Station RF Bandwidth edge. NOTE 2: For BS supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. NOTE 3: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

For A-IoT medium range BS (maximum output power Prated,c  31 dBm), emissions shall not exceed the maximum levels specified in Table 6.

Table 6 A-IoT medium range BS operating band unwanted emission limits, BS maximum output power Prated,c  31 dBm , R2D CBW=200kHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2, 3, 4) | Measurement bandwidth (Note 8) |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz (Note 1) | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0.065 MHz f_offset < 0.165 MHz |  | 30 kHz |
| 0.15 MHz f < 0.6 MHz (Note 1) | 0.015MHz f_offset < 0.615MHz |  | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615MHz f_offset < 1.015MHz |  | 30 kHz |
| (Note 6) | 1.015MHz f_offset < 1.5 MHz | -34 dBm | 30 kHz |
| 1 MHz f 5 MHz | 1.5 MHz f_offset < 5.5 MHz | -21 dBm | 1 MHz |
| 5 MHz f fmax | 5.5 MHz f_offset < f_offsetmax | -25 dBm | 1 MHz |
| NOTE 1: The limits in this table only apply for operation with a NB-IoT carrier adjacent to the Base Station RF Bandwidth edge. NOTE 2: For BS supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. NOTE 3: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 4: In case the carrier adjacent to the RF bandwidth edge is a NB-IoT carrier, the value of X = PNB-IoTcarrier – 31, where PNB-IoTcarrier is the power level of the NB-IoT carrier adjacent to the RF bandwidth edge. In other cases, X = 0. |  |  |  |

Proposal 5: For A-IoT micro BS with 400/600/800kHz R2D CBW, use the operating band unwanted emissions requirements in Table 7 and 8.

Table7 A-IoT medium range BS operating band unwanted emission limits, BS maximum output power 31 < Prated,c  38 dBm , for 400/600/800kHz R2D CBW

| R2D CBW | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 8) |
| --- | --- | --- | --- | --- |
| 400kHz | 0 MHz f < 0.4 MHz | 0.015 MHz f_offset < 0.415 MHz | Prated,c -40dB - 110.4 (f_offsetMHz - 0.015) dB | 30 kHz |
|  | 0.4 MHz f < 0.8 MHz | 0.415 MHz f_offset < 0.815 MHz | Prated,c - 51dB- 50.4 (f_offsetMHz-0.415) dB | 30 kHz |
|  | 0.8 MHz f < 1.6 MHz | 0.815 MHz f_offset < 1.6 MHz | Prated,c - 56dB | 30 kHz |
|  | 1.6 MHz f < fmax | 1.6 MHz f_offset < f_offsetmax | -25dBm | 100kHz |
| 600kHz | 0 MHz f < 0.6 MHz | 0.015 MHz f_offset < 0.615 MHz | Prated,c – 40dB - 130.6 (f_offsetMHz-0.015) dB | 30 kHz |
|  | 0.6 MHz f < 1.2 MHz | 0.615 MHz f_offset < 1.2 MHz | Prated,c – 53dB - 50.6 (f_offsetMHz-0.615) dB | 30 kHz |
|  | 1.2 MHz f < 1.8 MHz | 1.2 MHz f_offset < 1.8 MHz | Prated,c - 58dB | 30 kHz |
|  | 1.8 MHz f < fmax | 1.8 MHz f_offset < f_offsetmax | -25dBm | 100K |
| 800kHz | 0 MHz f < 0.8 MHz | 0.015 MHz f_offset < 0.815 MHz | Prated,c – 40dB- 140.8 (f_offsetMHz-0.015) dB | 30 kHz |
|  | 0.8 MHz f < 1.6 MHz | 0.815 MHz f_offset < 1.6 MHz | Prated,c – 54dB- 50.8 (f_offsetMHz-0.815) dB | 30 kHz |
|  | 1.6 MHz f < 2.4 MHz | 1.6 MHz f_offset < 2.4 MHz | Prated,c - 59dB | 30 kHz |
|  | 2.4 MHz f < fmax | 2.4 MHz f_offset < f_offsetmax | -25dBm | 100K |

Table 8 A-IoT medium range BS operating band unwanted emission limits, BS maximum output power Prated,c  31 dBm , for 400/600/800kHz R2D CBW

| R2D CBW | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 8) |
| --- | --- | --- | --- | --- |
| 400kHz | 0 MHz f < 0.4 MHz | 0.015 MHz f_offset < 0.415 MHz | Prated,c -30dB - 110.4 (f_offsetMHz - 0.015) dB | 30 kHz |
|  | 0.4 MHz f < 0.8 MHz | 0.415 MHz f_offset < 0.815 MHz | Prated,c - 41dB- 50.4 (f_offsetMHz-0.415) dB | 30 kHz |
|  | 0.8 MHz f < 1.6 MHz | 0.815 MHz f_offset < 1.6 MHz | -31dBm | 30 kHz |
|  | 1.6 MHz f < fmax | 1.6 MHz f_offset < f_offsetmax | -25dBm | 100kHz |
| 600kHz | 0 MHz f < 0.6 MHz | 0.015 MHz f_offset < 0.615 MHz | Prated,c – 30dB - 130.6 (f_offsetMHz-0.015) dB | 30 kHz |
|  | 0.6 MHz f < 1.2 MHz | 0.615 MHz f_offset < 1.2 MHz | Prated,c – 43dB - 50.6 (f_offsetMHz-0.615) dB | 30 kHz |
|  | 1.2 MHz f < 1.8 MHz | 1.2 MHz f_offset < 1.8 MHz | -21dBm | 30 kHz |
|  | 1.8 MHz f < fmax | 1.8 MHz f_offset < f_offsetmax | -25dBm | 100K |
| 800kHz | 0 MHz f < 0.8 MHz | 0.015 MHz f_offset < 0.815 MHz | Prated,c – 30dB- 140.8 (f_offsetMHz-0.015) dB | 30 kHz |
|  | 0.8 MHz f < 1.6 MHz | 0.815 MHz f_offset < 1.6 MHz | Prated,c – 44dB- 50.8 (f_offsetMHz-0.815) dB | 30 kHz |
|  | 1.6 MHz f < 2.4 MHz | 1.6 MHz f_offset < 2.4 MHz | -21dBm | 30 kHz |
|  | 2.4 MHz f < fmax | 2.4 MHz f_offset < f_offsetmax | -25dBm | 100K |

For RX requirements:

Proposal 6: R=1 is  the worst case for sensitivity，suggest to consider reference sensitivity requirements for the reader without frequency shift (R=1)

Proposal 7: The AWGN D2R modulation SNR is -6dB @ 10% BLER for BPSK.

Proposal 8: The SNR is -6dB @ 10% BLER for BPSK 2880kHz 2SB D2R TBW

Proposal 9: Use a 3 dB higher SNR than that of BPSK to calculate the OOK reference sensitivity under the same conditions.

Proposal 10 Use 30 dB as the desens target, with CW single tone input through a signal generator, CW input level to BS Ant is assumed as -38 dBm;

Proposal 11 Use the following table as A-IoT Micro Area BS reference sensitivity levels

Table 13: A-IoT Micro Area BS reference sensitivity levels

| BS channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | BPSK | -96 |
|  |  | OOK | -93 |
| 3520 | 15 | BPSK | -73 |
|  |  | OOK | -70 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

Proposal 12 Use Table 14 as the dynamic range requirement.

Table 14 Dynamic range

| D2R channel bandwidth [kHz] | D2R transmission bandwidth[kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | 15 | -76 | -102.2 | AWGN |
| 3520 | 2880 | -70 | -79.4 | AWGN |

Proposal 13: No need to define ACS requirements for A-IoT BS.If it must be defined, considering the following ACS requirements for A-IoT BS.

Table 15: Adjacent channel selectivity for A-IoT Medium Range BS

| A-IoT channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 19.5dB (Note) | -60 | ±100 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |
| 3520 | PREFSENS + 19.5dB (Note) | -60 | ±100 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |
| Note: PREFSENS depends on the sub-carrier spacing as specified in Table X |  |  |  |  |

Proposal 14: No need to define narrowband blocking requirements for A-IoT BS. If it must be defined, considering the Table 16 and 17 narrowband blocking requirements for A-IoT BS.

Proposal 15: No need to define general intermodulation for A-IoT BS since it is assumed that there is no NR interference indoors.

Proposal 16: No need to define narrowband intermodulation for A-IoT BS.

- References

[1] RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN1 Vice-chair (Huawei), RAN#106 Dec.,2024

[2] RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA Inc.，RAN#107 Mar.,2025

[3] R4-2508101, WF on requirements for A-IoT BS and CW，Huawei ,RAN4#115, May 2025

[4] 3GPP TS 38.291, NR; Ambient IoT Physical layer

[5]R1-2504894, Session notes for 9.4 Solutions for Ambient IoT in NR, Ad-Hoc Chair (Huawei),RAN1#121,May 2025

[6]R4-2505097, WF on requirements for A-IoT BS and CW, Moderator (Huawei), RAN4#114bis, April, 2025

- Text Proposal for TS38.194

<Start of Text Proposal>

- 5	Operating bands and channel arrangement

5.1	General

The channel arrangements presented in this clause are based on the operating bands and BS channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and BS channel bandwidths may be considered in future releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which A-IoT can operate according to the present version of the specification are identified as described in table 5.1-1.

Table 5.1-1: Definition of frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |

5.2	Operating bands

A-IoT is designed to operate in the operating bands defined in table 5.2-1

Table 5.2-1: A-IoT operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Notes |
| --- | --- | --- | --- | --- |
| n8 | 880 – 915 | 925 – 960 | FDD |  |

5.3	BS channel bandwidth

5.3.1	General

The R2D channel bandwidth supports a single reader RF carrier in R2D link at the reader.

The relationship between the R2D channel bandwidth, the guardband and the transmission bandwidth configuration is shown in figure 5.3.1.1-1.

.

Figure 5.3.1.1-1: Definition of channel bandwidth and transmission bandwidth configuration for one reader channel

5.3.2	Transmission bandwidth configuration

The transmission bandwidth configuration NRB for each reader channel bandwidth and subcarrier spacing is specified in table 5.3.1.2-1.

Table 5.3.1.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 200 kHz | 400 kHz | 600 kHz | 800 kHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 15 | 1 | 2 | 3 | 4 |

NOTE:	All BS Tx and device Rx requirements are defined based on transmission bandwidth configuration specified in table 5.3.2-1.

5.3.3	Minimum guardband and transmission bandwidth configuration

The minimum guardband for each reader channel bandwidth and SCS is specified in table 5.3.3-1.

Table 5.3.3-1: Minimum guardband (kHz)

| R2D CBW | 200kHz | 400kHz | 600kHz | 800kHz |
| --- | --- | --- | --- | --- |
| Minimum guardband(kHz) | 2.5 | 12.5 | 22.5 | 32.5 |

The number of RBs configured in any reader channel bandwidth shall ensure that the minimum guardband specified in this clause is met.

Figure 5.3.1.3-1: BS PRB utilization

5.3.4	RB alignment

For each reader channel bandwidth, BS transmission bandwidth configuration must fulfil the minimum guardband requirement specified in clause 5.3.3.

5.3.5	BS channel bandwidth

The requirements in this specification apply to the combination of BS channel bandwidths, SCS and operating bands shown in table 5.3.5-1 for FR1. The transmission bandwidth configuration in table 5.3.2-1 shall be supported for each of the BS channel bandwidths within the BS capability. The BS channel bandwidths are specified for the Tx path.

Table 5.3.1.5-1: Reader channel bandwidths and SCS

| SCS (kHz) | Reader channel bandwidth (kHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 200 | 400 | 600 | 800 |
| 15 | 200 | 400 | 600 | 800 |

|  |  | Chip duration:Tc=2/SCS/(1,2,4,8,16,32,64,192) (μs) |  |  |  |  |  |  |  |  | 2SB=4/Tb（kHz） |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.69 |  |
|  |  | τ | τ2 | τ4 | τ8 | τ16 | τ32 | τ64 | τ128 | τ192 |  |
|  |  | Small Frequency Shift：SFS=1/(2×Tc)= R/Tb (kHz) |  |  |  |  |  |  |  |  |  |
|  |  | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 | 720 |  |
| Bit duration:Tb= Tc×2R(μs) |  |  |  |  |  |  |  |  |  |  |  |
| 266.67 |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 |  | 15 |
|  | Device D2R CBW (kHz) | 16.5 | 24.8 | 41.3 | 74.3 | 140.3 | 272.3 | 536.3 | 1064.3 |  |  |
|  | BS D2R CBW (kHz) | 18.3 | 27.6 | 45.9 | 82.6 | 155.9 | 302.6 | 595.9 | 1182.6 |  |  |
| 133.33 |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 |  | 30 |
|  | Device D2R CBW (kHz) |  | 33 | 49.5 | 82.5 | 148.5 | 280.5 | 544.5 | 1072.5 |  |  |
|  | BS D2R CBW (kHz) |  | 36.7 | 55 | 91.7 | 165 | 311.7 | 605 | 1191.7 |  |  |
| 66.67 |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 |  | 60 |
|  | Device D2R CBW (kHz) |  |  | 66 | 99 | 165 | 297 | 561 | 1089 |  |  |
|  | BS D2R CBW (kHz) |  |  | 73.3 | 110 | 183.3 | 330 | 623.3 | 1210 |  |  |
| 33.33 |  |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 |  | 120 |
|  | Device D2R CBW (kHz) |  |  |  | 132 | 198 | 330 | 594 | 1122 |  |  |
|  | BS D2R CBW (kHz) |  |  |  | 146.7 | 220 | 366.7 | 660 | 1246.7 |  |  |
| 16.667 |  |  |  |  |  | R=1 | R=2 | R=4 | R=8 |  | 240 |
|  | Device D2R CBW (kHz) |  |  |  |  | 264 | 396 | 660 | 1188 |  |  |
|  | BS D2R CBW (kHz) |  |  |  |  | 293.3 | 440 | 733.3 | 1320 |  |  |
| 8.333 |  |  |  |  |  |  | R=1 | R=2 | R=4 |  | 480 |
|  | Device D2R CBW (kHz) |  |  |  |  |  | 528 | 792 | 1320 |  |  |
|  | BS D2R CBW (kHz) |  |  |  |  |  | 586.7 | 880 | 1466.7 |  |  |
| 4.167 |  |  |  |  |  |  |  | R=1 | R=2 |  | 960 |
|  | Device D2R CBW (kHz) |  |  |  |  |  |  | 1056 | 1584 |  |  |
|  | BS D2R CBW (kHz) |  |  |  |  |  |  | 1173.3 | 1760 |  |  |
| 1.389 |  |  |  |  |  |  |  |  |  | R=1 | 2880 |
|  | Device D2R CBW (kHz) |  |  |  |  |  |  |  |  | 3168 |  |
|  | BS D2R CBW (kHz) |  |  |  |  |  |  |  |  | 3520 |  |

5.4	Channel arrangement

5.4.1	Channel spacing

5.4.2	Channel raster

#### 5.4.2.1	AIoT-ARFCN and channel raster

The global frequency raster defines a set of RF reference frequencies FREF. The RF reference frequency is used in signalling to identify the position of RF channels, SS blocks and other elements. The global frequency raster is defined for all frequencies from 0 to 100 GHz. The granularity of the global frequency raster is ΔFGlobal.

RF reference frequencies are designated by an A-IoT Absolute Radio Frequency Channel Number (AIoT-ARFCN) in the range [0…3279165] on the global frequency raster. The relation between the AIoT-ARFCN and the RF reference frequency FREF in MHz is given by the following equation, where FREF-Offs and NRef-Offs are given in table 5.4.2.1-1 and NREF is the AIoT-ARFCN.

FAREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Range of frequencies (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |

The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. The RF reference frequency for an RF channel maps to a resource element on the carrier. For each operating band, a subset of frequencies from the global frequency raster are applicable for that band and forms a channel raster with a granularity ΔFRaster, which may be equal to or larger than ΔFGlobal.

R2D channel raster is 10 kHz.

#### 5.4.2.2	Channel raster to resource element mapping

The mapping between the RF reference frequency on the channel raster and the corresponding resource element is given in table 5.4.2.2-1 and can be used to identify the RF channel position. The mapping depends on the total number of RBs that are allocated in the channel and applies to both UL and DL. The mapping must apply to at least one numerology supported by the BS.

Table 5.4.2.2-1: Channel Raster to Resource Element Mapping

|  | NRBmod2=0 | NRBmod2=1 |
| --- | --- | --- |
| Resource element index | 0 | 6 |
| Physical resource block index |  |  |

NRB is the transmission bandwidth configuration specified in sub-clause 5.3.2, is the PRB index within the NRB, and  is the resource element index within this PRB.

5.4.3	Synchronization raster

- 6	A-IoT BS transmitter characteristics

6.2	Base station output power

6.2.1	General

The BS conducted output power requirement is at antenna connector for BS type 1-C,.

The rated carrier output power of the BS type 1-C shall be as specified in table 6.2.1-1.

Table 6.2.1-1: BS type 1-C rated output power limits for BS classes

| BS class | Prated,c,AC |
| --- | --- |
| Medium Range BS | ≤ 38 dBm |

### 6.2.2	Minimum requirement for BS type 1-C

In normal conditions, Pmax,c,AC shall remain within +2 dB and -2 dB of the rated carrier output power Prated,c,AC, declared by the manufacturer.

In extreme conditions, Pmax,c,AC shall remain within +2.5 dB and -2.5 dB of the rated carrier output power Prated,c,AC, declared by the manufacturer.

In certain regions, the minimum requirement for normal conditions may apply also for some conditions outside the range of conditions defined as normal.

6.3	Transmit ON/OFF power

6.3.1	Transmitter OFF power

#### 6.3.1.1	General

Transmit OFF power requirements apply to FDD operation of the A-IoT micro BS.

Transmitter OFF power is defined as the mean power measured over 70/N us filtered with a square filter of bandwidth equal to the transmission bandwidth configuration of the BS (BWConfig) centred on the assigned channel frequency during the transmitter OFF period. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

#### 6.4.1.2	Minimum requirement for BS type 1-C

For BS type 1-C, the requirements for transmitter OFF power spectral density shall be less than -85 dBm/MHz per antenna connector.

6.3.2	Transmitter transient period

6.4	Transmitted signal quality

6.4.1	Frequency error

#### .6.4.1.1	General

The requirements in clause 6.4.1 apply to the transmitter ON period.

Frequency error is the measure of the difference between the actual BS transmit frequency and the assigned frequency. The same source shall be used for RF frequency and data clock generation.

For BS type 1-C this requirement shall be applied at the antenna connector supporting transmission in the operating band.

#### 6.4.1.2	Minimum requirement for BS type 1-C

For BS type 1-C, the modulated carrier frequency of each NR carrier configured by the BS shall be accurate to within the accuracy range given in table 6.4.1.2-1 observed over 1 ms.

Table 6.5.1.2-1: Frequency error minimum requirement

| BS class | Accuracy |
| --- | --- |
| Medium Range BS | ±0.1 ppm |

6.4.2	Modulation quality

Based on TS38.291[5], R2D signal includes SIP (Start indicator part), CAP (Clock acquisition part), PRDCH, the R2D postamble and padding if needed. Agreement in [5] is that SIP（start indicator part） of R-TAS（R2D timing acquisition signal） is adopted with 2 OFDM symbol duration, i.e. ON-OFF-ON-OFF with a ratio of 2:2:1:3. The R-TAS SIP consists of NSIP=8 bits denoted S0,S1,S2, S3,S4,S5,S6,S7=1, 1, 0, 0, 1, 0, 0, 0 As shown in Figure 1.

Figure 1 R2D signal composition

Figure3 R2D signal diagram

An is measured peak high level for the nth chip, in units of V/m or A/m

Bn is measured peak low level for the nth chip, in units of V/m or A/m

Anavg is the measured average high level for the nth chip during 1/3 center chip duration, in units of V/m or A/m

Bnavg is the measured average low level for the nth chip during 1/3 center chip duration, in units of V/m or A/m

Ripple (1):

Ripple_high (%) = ((An − Anavg) / (Anag-Bnavg)) × 100%

Ripple_low (%) = ((Bn − Bnavg) / (Anavg-Bnavg)) × 100%

Modulation depth (2):

For each chip, Modulation depth =(Anavg-Bnavg)/Anavg

RF Envelop Rise Time (3):

The time from 0.1 ×(Anavg-Bnavg) +Bnavg to 0.9 ×(Anavg-Bnavg)+Bnavg

RF Envelop Fall Time (4):

The time from 0.9 ×(Anavg-Bnavg) +Bnavg to 0.1 ×(Anavg-Bnavg)+Bnavg

Pulsewidth (5)

The pulse width is the time between two points on the pulse where the signal reaches 50% of (Anavg-Bnavg)+Bnavg

the RF envelope requirements can be defined as Table 3.

Table 3: A-IoT BS RF envelope parameters

| R2D Chip duration：Tc | Parameter | Symbol | Mimimum | Nominal | Maximum | Units |
| --- | --- | --- | --- | --- | --- | --- |
| Tc=103M*15(us) M∈ {2,6,12,24} | Modulation Depth | (A–B)/A | 80 | 90 | 100 | % |
|  | RF Envelope Ripple | Ripple_high Ripple_low | 0 |  | ±15 | % |
|  | RF Envelop Rise Time | Tr,10-90 |  |  | 0.66Tc | µs |
|  | RF Envelop Fall Time | Tf,10-90 |  |  | 0.66Tc | µs |
|  | RF Pulsewidth | PW |  | 1Tc | 1.3 Tc | µs |

6.5	Unwanted emissions

6.5.1	General

6.5.2	Occupied bandwidth

#### 6.5.2.1	General

The occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage /2 of the total mean transmitted power. See also Recommendation ITU-R SM.328 [3].

The value of /2 shall be taken as 0.5%.

The occupied bandwidth requirement shall apply during the transmitter ON period for a single transmitted carrier. The minimum requirement below may be applied regionally. There may also be regional requirements to declare the occupied bandwidth according to the definition in the present clause.

For BS type 1-C this requirement shall be applied at the antenna connector supporting transmission in the operating band.

#### 6.5.2.2	Minimum requirement for BS type 1-C and BS type 1-H

The occupied bandwidth for A-IoT carrier shall be less than the BS channel bandwidth. For intra-band contiguous CA, the occupied bandwidth shall be less than or equal the Aggregated BS Channel Bandwidth.

6.5.3	Adjacent Channel Leakage Power Ratio

6.6.3.1	General

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

The requirements shall apply outside the Base Station RF Bandwidth or Radio Bandwidth whatever the type of transmitter considered (single carrier or multi-carrier) and for all transmission modes foreseen by the manufacturer’s specification.

The requirement shall apply during the transmitter ON period.

6.6.3.2	Limits and Basic limits

The ACLR is defined with a square filter of bandwidth equal to the transmission bandwidth configuration of the transmitted signal (BWConfig) centred on the assigned channel frequency and a filter centred on the adjacent channel frequency according to the tables below.

For operation in paired and unpaired spectrum, the ACLR shall be higher than the value specified in table 6.6.3.2-1 in any operating band.

Table 4: A-IoT BS ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (kHz) | Offset(kHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- | --- |
| 200/400/600/800 | 100 | BWChannel | A-IoT of same BW | Square (BWConfig) | 40 dB |
|  | 100 | 2 x BWChannel | A-IoT of same BW | Square (BWConfig) | 45 dB |

Figure 4 Diagram of A-IoT ACLR with 100kHz Foffset

6.5.4	Operating band unwanted emissions

#### 6.5.4.1	General

Unless otherwise stated, the operating band unwanted emission (OBUE) limits in FR1 are defined from ΔfOBUE below the lowest frequency of each supported downlink operating band up to ΔfOBUE above the highest frequency of each supported downlink operating band. The values of ΔfOBUE are defined in table 6.6.1-1 for the NR operating bands.

The requirements shall apply whatever the type of transmitter considered and for all transmission modes foreseen by the manufacturer’s specification. In addition, for a BS operating in non-contiguous spectrum, the requirements apply inside any sub-block gap. In addition, for a BS operating in multiple bands, the requirements apply inside any Inter RF Bandwidth gap.

Basic limits are specified in the tables below, where:

-	f is the separation between the channel edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the channel edge frequency and the centre of the measuring filter.

-	f_offsetmax is the offset to the frequency ΔfOBUE outside the downlink operating band, where ΔfOBUE is defined in table 6.6.1-1.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For Medium Range BS, the requirements in clause  x shall apply (Category A and B).

#### 6.5.4.2	Basic limits

For 200kHz R2D CBW, Operating band unwanted emissions can reuse the requirements of standalone NB-IoT Medium Range BS.

For A-IoT medium range BS (maximum output power 31 < Prated,c  38 dBm), emissions shall not exceed the maximum levels specified in Table 5.

Table 5 A-IoT medium range BS operating band unwanted emission limits, BS maximum output power 31 < Prated,c  38 dBm, R2D CBW=200kHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2, 3) | Measurement bandwidth (Note 8) |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz (Note 1) | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0.065 MHz f_offset < 0.165 MHz |  | 30 kHz |
| 0.15 MHz f < 0.6 MHz (Note 1) | 0.015MHz f_offset < 0.615MHz |  | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615MHz f_offset < 1.015MHz |  | 30 kHz |
| (Note 6) | 1.015MHz f_offset < 1.5 MHz | Prated,c - 65 dB | 30 kHz |
| 1 MHz f 2.8 MHz | 1.5 MHz f_offset < 3.3 MHz | Prated,c - 52 dB | 1 MHz |
| 2.8 MHz f 5 MHz | 3.3 MHz f_offset < 5.5 MHz | min(Prated,c - 52 dB, -15dBm) | 1 MHz |
| 5 MHz f fmax | 5.5 MHz f_offset < f_offsetmax | Prated,c - 56 dB | 1 MHz |
| NOTE 1: The limits in this table only apply for operation with a standalone NB-IoT carrier adjacent to the Base Station RF Bandwidth edge. NOTE 2: For BS supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. NOTE 3: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

For A-IoT medium range BS (maximum output power Prated,c  31 dBm), emissions shall not exceed the maximum levels specified in Table 6.

Table 6 A-IoT medium range BS operating band unwanted emission limits, BS maximum output power Prated,c  31 dBm , R2D CBW=200kHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2, 3, 4) | Measurement bandwidth (Note 8) |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz (Note 1) | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0.065 MHz f_offset < 0.165 MHz |  | 30 kHz |
| 0.15 MHz f < 0.6 MHz (Note 1) | 0.015MHz f_offset < 0.615MHz |  | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615MHz f_offset < 1.015MHz |  | 30 kHz |
| (Note 6) | 1.015MHz f_offset < 1.5 MHz | -34 dBm | 30 kHz |
| 1 MHz f 5 MHz | 1.5 MHz f_offset < 5.5 MHz | -21 dBm | 1 MHz |
| 5 MHz f fmax | 5.5 MHz f_offset < f_offsetmax | -25 dBm | 1 MHz |
| NOTE 1: The limits in this table only apply for operation with a NB-IoT carrier adjacent to the Base Station RF Bandwidth edge. NOTE 2: For BS supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. NOTE 3: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 4: In case the carrier adjacent to the RF bandwidth edge is a NB-IoT carrier, the value of X = PNB-IoTcarrier – 31, where PNB-IoTcarrier is the power level of the NB-IoT carrier adjacent to the RF bandwidth edge. In other cases, X = 0. |  |  |  |

Proposal 4: For A-IoT micro BS with 200 kHz R2D CBW, reuses the operating band unwanted emissions requirements of standalone NB-IoT Medium Range BS for A-IoT micro BS.

Proposal 5: For A-IoT micro BS with 400/600/800kHz R2D CBW, use the operating band unwanted emissions requirements in Table 7 and 8.

Table7 A-IoT medium range BS operating band unwanted emission limits, BS maximum output power 31 < Prated,c  38 dBm , for 400/600/800kHz R2D CBW

| R2D CBW | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 8) |
| --- | --- | --- | --- | --- |
| 400kHz | 0 MHz f < 0.4 MHz | 0.015 MHz f_offset < 0.415 MHz | Prated,c -40dB - 110.4 (f_offsetMHz - 0.015) dB | 30 kHz |
|  | 0.4 MHz f < 0.8 MHz | 0.415 MHz f_offset < 0.815 MHz | Prated,c - 51dB- 50.4 (f_offsetMHz-0.415) dB | 30 kHz |
|  | 0.8 MHz f < 1.6 MHz | 0.815 MHz f_offset < 1.6 MHz | Prated,c - 56dB | 30 kHz |
|  | 1.6 MHz f < fmax | 1.6 MHz f_offset < f_offsetmax | -25dBm | 100kHz |
| 600kHz | 0 MHz f < 0.6 MHz | 0.015 MHz f_offset < 0.615 MHz | Prated,c – 40dB - 130.6 (f_offsetMHz-0.015) dB | 30 kHz |
|  | 0.6 MHz f < 1.2 MHz | 0.615 MHz f_offset < 1.2 MHz | Prated,c – 53dB - 50.6 (f_offsetMHz-0.615) dB | 30 kHz |
|  | 1.2 MHz f < 1.8 MHz | 1.2 MHz f_offset < 1.8 MHz | Prated,c - 58dB | 30 kHz |
|  | 1.8 MHz f < fmax | 1.8 MHz f_offset < f_offsetmax | -25dBm | 100K |
| 800kHz | 0 MHz f < 0.8 MHz | 0.015 MHz f_offset < 0.815 MHz | Prated,c – 40dB- 140.8 (f_offsetMHz-0.015) dB | 30 kHz |
|  | 0.8 MHz f < 1.6 MHz | 0.815 MHz f_offset < 1.6 MHz | Prated,c – 54dB- 50.8 (f_offsetMHz-0.815) dB | 30 kHz |
|  | 1.6 MHz f < 2.4 MHz | 1.6 MHz f_offset < 2.4 MHz | Prated,c - 59dB | 30 kHz |
|  | 2.4 MHz f < fmax | 2.4 MHz f_offset < f_offsetmax | -25dBm | 100K |

Table 8 A-IoT medium range BS operating band unwanted emission limits, BS maximum output power Prated,c  31 dBm , for 400/600/800kHz R2D CBW

| R2D CBW | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 8) |
| --- | --- | --- | --- | --- |
| 400kHz | 0 MHz f < 0.4 MHz | 0.015 MHz f_offset < 0.415 MHz | Prated,c -30dB - 110.4 (f_offsetMHz - 0.015) dB | 30 kHz |
|  | 0.4 MHz f < 0.8 MHz | 0.415 MHz f_offset < 0.815 MHz | Prated,c - 41dB- 50.4 (f_offsetMHz-0.415) dB | 30 kHz |
|  | 0.8 MHz f < 1.6 MHz | 0.815 MHz f_offset < 1.6 MHz | -31dBm | 30 kHz |
|  | 1.6 MHz f < fmax | 1.6 MHz f_offset < f_offsetmax | -25dBm | 100kHz |
| 600kHz | 0 MHz f < 0.6 MHz | 0.015 MHz f_offset < 0.615 MHz | Prated,c – 30dB - 130.6 (f_offsetMHz-0.015) dB | 30 kHz |
|  | 0.6 MHz f < 1.2 MHz | 0.615 MHz f_offset < 1.2 MHz | Prated,c – 43dB - 50.6 (f_offsetMHz-0.615) dB | 30 kHz |
|  | 1.2 MHz f < 1.8 MHz | 1.2 MHz f_offset < 1.8 MHz | -21dBm | 30 kHz |
|  | 1.8 MHz f < fmax | 1.8 MHz f_offset < f_offsetmax | -25dBm | 100K |
| 800kHz | 0 MHz f < 0.8 MHz | 0.015 MHz f_offset < 0.815 MHz | Prated,c – 30dB- 140.8 (f_offsetMHz-0.015) dB | 30 kHz |
|  | 0.8 MHz f < 1.6 MHz | 0.815 MHz f_offset < 1.6 MHz | Prated,c – 44dB- 50.8 (f_offsetMHz-0.815) dB | 30 kHz |
|  | 1.6 MHz f < 2.4 MHz | 1.6 MHz f_offset < 2.4 MHz | -21dBm | 30 kHz |
|  | 2.4 MHz f < fmax | 2.4 MHz f_offset < f_offsetmax | -25dBm | 100K |

6.5.5	Transmitter spurious emissions

6.6	Transmitter intermodulation

- 7	A-IoT BS receiver characteristics

7.1	General

7.2	Reference sensitivity level

### 7.2.1	General

The reference sensitivity power level PREFSENS is the minimum mean power received at the antenna connector for BS type 1-C at which a throughput requirement shall be met for a specified reference measurement channel.

### 7.2.2	Minimum requirements for BS type 1-C

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.X with parameters specified in table X for Medium Range BS.

Table 7.2.2-2: A-IoT Medium Range BS reference sensitivity levels

| BS channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD | [-96] |
|  |  | TBD | [-93] |
| 3520 | 15 | TBD | [-73] |
|  |  | TBD | [-70] |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

7.3	Dynamic range

### 7.3.1	General

The dynamic range is specified as a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal at the antenna connector for BS type 1-C inside the received BS channel bandwidth. In this condition, a throughput requirement shall be met for a specified reference measurement channel. The interfering signal for the dynamic range requirement is an AWGN signal.

### 7.3.2	Minimum requirement for BS type 1-C

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.X with parameters specified in table x for Medium Range BS.

Table 7.3.2-1: Medium Range BS dynamic range

| D2R channel bandwidth [kHz] | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 200 | 15 | TBD | -76 | -102.2 | AWGN |
| 3520 | 15 | TBD | -70 | -79.4 | AWGN |

<End of Text Proposal>