3GPP TSG-RAN4 Meeting #116 	R4-2509885

Bengaluru, India, 25th -29th August, 2025

Source: 	Huawei, HiSilicon

Title:  	draft TP to TS38194 on Reference sensitivity level and Dynamic range

Agenda Item:	7.26.2

Document for:	Approval

- Text Proposal to TS38.194

<Start of Change>

7.1	General

7.2	Reference sensitivity level

### 7.2.1	General

The reference sensitivity power level PREFSENS is the minimum mean power received at the antenna connector for BS type 1-C at which a throughput requirement shall be met for a specified reference measurement channel.

### 7.2.2	Minimum requirements for BS type 1-C

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.X with parameters specified in table X for Medium Range BS.

Table 7.2.2-2: A-IoT Medium Range BS reference sensitivity levels

| BS channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD | [-96] |
|  |  | TBD | [-93] |
| 3520 | 15 | TBD | [-73] |
|  |  | TBD | [-70] |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

7.3	Dynamic range

### 7.3.1	General

The dynamic range is specified as a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal at the antenna connector for BS type 1-C inside the received BS channel bandwidth. In this condition, a throughput requirement shall be met for a specified reference measurement channel. The interfering signal for the dynamic range requirement is an AWGN signal.

### 7.3.2	Minimum requirement for BS type 1-C

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.X with parameters specified in table x for Medium Range BS.

Table 7.3.2-1: Medium Range BS dynamic range

| D2R channel bandwidth [kHz] | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 200 | 15 | TBD | -76 | -102.2 | AWGN |
| 3520 | 15 | TBD | -70 | -79.4 | AWGN |

<End of Change>

- References

[1] R4-2505228, TS 38.194, v0.1.0 Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception v0.1.0,