3GPP TSG-RAN WG4 Meeting #116		R4-2509906
Bangalore, India, 25th – 29th Aug 2025

Title:	R16 discussion on Modified MPR behaviour

Source:	OPPO

Agenda item:	4.2.1

Document for:	Approval

- Introduction

Modified MPR behaviour has been discussed for a long time, and WF [1] was agreed to further discuss and see what can be optimise. This paper shares views on it.

And two sets of CRs with different kind of changes based on this discussion paper have been submitted to this meeting for further discussion.

- Discussion

  - How much change can be made for modifiedMPR-Behaviour

Now, 5G has come to the end releases, and all the 5G UE/NW products has been on the market for a long time. If there is no critical error in the spec, then for the commercial releases, it should be kept unchanged as much as possible.

Therefore, for this modifiedMPR-Behaviour, if purely optimisation we would prefer not to do it at least for the early releases.

Proposal 1:         Keep modifiedMPR-Behaviour unchanged for the commercial/under development releases if no error in the spec. And discussion should focus on Rel-19 or 6G for the optimisation.

  - Bit set to ‘0’ and absent

In RAN2 specs, the modifiedMPR-Behaviour is 8 bits string format and it is not mandatory to be present in the signalling which means this IE could be absent. And if a band has used several bits of this IE, then one bit cannot be absent alone, it has to be either 1 or 0.

Observation 1:   For the 8 bits string format, this IE can be totally absent, but cannot be used in a way that one bit is absent but other bits are present.

Proposal 2:         Avoid using wording like “bit is not set” which may cause different interpretations. Alternative could be “bit is not set to 1” or “bit is set to 0” or modifiedMPR-Behaviour IE is absent.

  - Applicable requirements when bit is not set to 1

In RAN4 spec, the detailed meaning has been defined, however, with many different kinds of usage. Below is one capture of from 38.101-1 where “shall be set to 1”, “may be set to 1” and “not set” are used currently. The ambiguity mainly comes from this bit is set to 0 or absent.

For n5 case, it defines the applicable requirements when this bit is set to 1, and no mention of which requirements will be applied if this bit is set to 0 or this IE is absent. One interpretation is that the requirements before the targeted version of specification is applied. If there is no other interpretation then it doesn’t matter, otherwise, explicitly specify which requirement is applied will be clearer.

For n30 case, it defines the applicable requirement when this IE is “not set”. As discussed above, the wording “this bit is not set” should be either set to “0” or this IE is absent.

Observation 2:   If there are only two sets of applicable requirements among releases, then no mention of the applicable requirements when the bit is not set to 1 should be fine, otherwise, should be explicitly specified.

The MPR/AMPR requirements shall be kept as stable as possible and no frequent changes in the spec. From this perspective, more than two changes (tighten or relaxation) of same requirements are not expected and shouldn’t happen. Then explicitly specify the applicable requirements when the bit is not set to 1 seems unnecessary.

Observation 3:   MPR/AMPR requirements shall be kept as stable as possible and more than two changes (tighten or relaxation) of same requirements are not expected and shouldn’t happen.

Proposal 3:         The applicable requirements when the bit is not set to 1 is not explicitly specified in the modifiedMPR-Behaviour table.

  - Set 1 for later releases

Basically, this is unnecessary considering there is only one set of requirements in these releases. However, as discussed early, in this late stage of 5G, if no error in the spec then it should keep consistent and take UE/BS implementations into account. For 6G, if this IE is to be there, then this modifiedMPR-Behaviour should not be present for later releases.

Proposal 4:         In 5G, keep the existing “set to 1” for later releases unchanged due to UE/BS may already implemented. And for 6G, this IE should not present in the later release.

  - Version numbers

It is valid point that the MPR/AMPR may change (correction) in the later releases and fix the version number will make these changes bypassed. Therefore, for the later release, this version number should be removed, instead could saying “MPR/AMPR defined in xxx of present specification”.

Proposal 5:         Version number should be removed from the modifiedMPR-Behaviour table, instead could saying “MPR/AMPR defined in xxx of present specification”, otherwise, the change (correction) in the later releases will be bypassed.

  - UE types

For some bands there are UE types mentioned in the tables. In our view, this is fine as long as it is clear.

Proposal 6:         Keep UE types in the modifiedMPR-Behaviour table as long as it is clearly specified and no new IE is defined for modified MPR purpose at this late 5G stage.

  - N41 related bits

In n41 several modifiedMPR bits have been defined, and bit 0/1 are mandatory to set to 1 since Rel-16, meanwhile the bit 2 may be set to 1. If look at the bit definition, bit 0/1 are only for MPR requirements defined in 101-3 v15.5.0, and bit 2 is for both MPR and AMPR defined in 101-3 v16.4.0. Here, bit 0/1 and bit 2 are referring to different MPR tables. If bit 0/1 are mandatory to set to 1, then 101-3 v15.5.0 MPR is applied, how could UE further set bit 2 to 1 and refer to another MPR table?

To find out the difference of MPR/AMPR among different releases, below captured the changing history of n41.

- In and before 38.101-3 v15.4.0, the MPR of EN-DC referred to LTE and NR MPR spec respectively.

- Then in 38.101-3 v15.5.0, the MPR specifically for EN-DC was defined and modifiedMPR bit 0/1 for n41 were introduced.

- In 38.101-3 v16.3.0 (CR R4-2008904), the AMPR was changed together with PC1.5 DC_(n)41AA introduced. And the n41 modifiedMPR bit 2 was introduced. Up to now, the Rel-16 modifiedMPR still “may” instead of “shall”.

  - Note: MPR is not changed in this CR, but in another CR R4-2006650 there is indeed one typo correction

- Then, in RAN4#95e the modifiedMPR table was agreed to move from 101-3 to 101-1(R15 CR R4-2009177). And in RAN4#96e, the R16 CR R4-2010343 was agreed and changed n41 modifiedMPR bit 0/1 from “may” to “shall” (in previous meeting this is no conclusion), and add bit 2 to indicate the MPR/AMPR in 101-3 v16.4.0.

From above history, it is unclear why the bit 0/1 was changed from “may” to “shall” and how to handle the MPR contradiction that it has caused with bit 2.

One solution might be to remove the MPR from bit 2, and keep it purely for AMPR.

Observation 4:   For n41, bit 0/1 is contradicting with bit 2 in the applied MPR due to “shall set to 1” for bit 0/1 but still “may set to 1” for bit 2.

Observation 5:   Comparing the MPR difference for n41 between 101-3 v15.5.0 and v16.4.0, the only difference is one typo correction of table number.

Proposal 7:         To solve the MPR contradiction between bit 0/1 and bit 2, it is proposed to remove the MPR from bit 2, and keep it purely for AMPR.

- Conclusion

This paper discussed the modifiedMPR-Behaviour and got below observations and proposals.

Proposal 1:         Keep modifiedMPR-Behaviour unchanged for the commercial/under development releases if no error in the spec. And discussion should focus on Rel-19 or 6G for the optimisation.

Observation 1:   For the 8 bits string format, this IE can be totally absent, but cannot be used in a way that one bit is absent but other bits are present.

Proposal 2:         Avoid using wording like “bit is not set” which may cause different interpretations. Alternative could be “bit is not set to 1” or “bit is set to 0” or modifiedMPR-Behaviour IE is absent.

Observation 2:   If there are only two sets of applicable requirements among releases, then no mention of the applicable requirements when the bit is not set to 1 should be fine, otherwise, should be explicitly specified.

Observation 3:   MPR/AMPR requirements shall be kept as stable as possible and more than two changes (tighten or relaxation) of same requirements are not expected and shouldn’t happen.

Proposal 3:         The applicable requirements when the bit is not set to 1 is not explicitly specified in the modifiedMPR-Behaviour table.

Proposal 4:         In 5G, keep the existing “set to 1” for later releases unchanged due to UE/BS may already implemented. And for 6G, this IE should not present in the later release.

Proposal 5:         Version number should be removed from the modifiedMPR-Behaviour table, instead could saying “MPR/AMPR defined in xxx of present specification”, otherwise, the change (correction) in the later releases will be bypassed.

Proposal 6:         Keep UE types in the modifiedMPR-Behaviour table as long as it is clearly specified and no new IE is defined for modified MPR purpose at this late 5G stage.

Observation 4:   For n41, bit 0/1 is contradicting with bit 2 in the applied MPR due to “shall set to 1” for bit 0/1 but still “may set to 1” for bit 2.

Observation 5:   Comparing the MPR difference for n41 between 101-3 v15.5.0 and v16.4.0, the only difference is one typo correction of table number.

Proposal 7:         To solve the MPR contradiction between bit 0/1 and bit 2, it is proposed to remove the MPR from bit 2, and keep it purely for AMPR.

- Reference

[1] R4-2507957 WF on Modified MPR Behavior, Xiaomi, #115 2025-05