3GPP TSG-RAN WG4 Meeting #116		R4-2509907
Bangalore, India, 25th – 29th Aug 2025

Title:	R16 discussion on SUL NUL switching period

Source:	OPPO

Agenda item:	4.2.1

Document for:	Approval

- Introduction

In current 38.101-1, the SUL band combinations have to support 0us switching time between SUL band and corresponding NUL band as seen below.

And in RAN4#115, CRs [1,2] were agreed which introduced the none zero switching period between SUL band and corresponding NUL band under the CA UL Tx switching framework as below.

This paper discussed the potential enhancement of none zero Tx switching between SUL and NUL in SUL band combination.

- Discussion

  - Why none zero Tx switching between SUL and NUL in SUL band combination is needed

In the beginning of 5G, the SUL band was discussed a lot on how this kind of band should be used. At that time, many different views on the relation between SUL and NUL and finally the it was agreed that the SUL band NUL band has to be switched transmission, and the switching period is 0us which means UE shall support separate Tx chain for SUL and NUL. When one of the bands is transmitting, the other band is stand by and preparing for transmission.

However, in reality, most of commercial 5G UEs (more than 70% UEs in China) can only support 1Tx chain at a time. The zero Tx switching requirement restricts these UEs from supporting SUL to enhance the UL coverage. Basically, this restriction is unnecessary.

Observation 1:   The 0us Tx switching period mandates UE to support two independent Tx chains which excludes most of commercial UEs to enhance UL coverage with SUL band combinations.

  - How none zero Tx switching can be supported

To support none zero Tx switching period between SUL and NUL in early releases like Rel-16. The switching period need to be defined.

In Rel-16, the UL Tx switching was introduced with uplinkTxSwitchingPeriodLocation indicates which carrier the UL switching period should be located, and with ULTxSwitchingBandPair indicates the switched band pair and also switching period length (uplinkTxSwitchingPeriod). There are also other parameters that are applicable to the UL Tx switching like uplinkTxSwitching-PowerBoosting or uplinkTxSwitching-OptionSupport, etc.

For SUL and NUL band combination, the switching between SUL and NUL is much simpler than the Rel-16 Tx switching. The switching period length could reuse the values defined in Rel-16 UL Tx switching, and the location could be either fixed to one of the carriers or reuse similar scheme as UL Tx switching. Therefore, in terms of none zero Tx switching scheme, there is nothing new comparing to Rel-16 UL Tx switching or even simpler.

However, to support the none zero Tx switching between SUL and NUL, some TEI works are needed in both RAN4 and RAN1/2. It is proposed to discuss first in RAN4 and then trigger RAN1/2 discussion by LS in TEI-16.

Observation 2:   To support SUL/NUL none zero Tx switching, the schemes defined in Rel-16 UL Tx switching can be reused or simplified. And this enhancement can be done in TEI.

Proposal 1:         Discuss the feasibility of introducing none zero Tx switching between SUL/NUL in TEI-16 of RAN4, and check RAN1/2 with LS if necessary.

Proposal 2:         If feasible, introduce none zero Tx switching period between SUL/NUL from Rel-16.

- Reference

[1] R4-2506926 (NR_MC_enh) R18 correction of switchig time between NUL and SUL carriers, OPPO, #115 2025-05

[2] R4-2506927 (NR_MC_enh) R19 correction of switchig time between NUL and SUL carriers, OPPO, #115 2025-05