3GPP TSG-RAN WG4 Meeting #116		R4-2509910
Bangalore, India, 25th – 29th Aug 2025

Title:	R19 release independent of 6Rx

Source:	OPPO

Agenda item:	7.1.2.3.2

Document for:	Approval

- Introduction

In previous meetings, 6Rx has been discussed a lot in terms of 6Layer or 4Layer + 2Div. And from which release this 6Rx can be applied is still open. This paper shares the view of these release independent issues.

- Discussion

For 6Layer, RAN4 agreed to introduce it for FWA devices only in Rel-19, and has sent LS to RAN2 to ask for optional signalling design. Basically, when new signalling is introduced, it needs to be careful to be release independent from earlier releases due to the inter-operation problems that may confront for earlier BS/UE. Therefore, in our view, 6Layer can be applied since Rel-19 at least, we are also supportive to apply it to earlier releases like Rel-18 to benefit the industry.

For 4Layer + 2Div, this can already be supported from MIMO layer point of view today. In terms of defining new signalling to differentiate UE supporting 4Layer and UE supporting 4Layer+2Div, if is introduced, then this will be in as similar situation of 6Layer.

Proposal 1:         For 6Layer, apply it at least from Rel-19, and can be supported from Rel-18.

Proposal 2:         For 6Rx with 4Layer, feature can be supported in early releases without signaling impact, and requirements can be introduced from Rel-19 spec.

- Conclusions

This paper discussed the release independence of 6Layer, 4Layer+2Div. And got below proposals.

Proposal 1:         For 6Layer, apply it at least from Rel-19, and can be supported from Rel-18.

Proposal 2:         For 6Rx with 4Layer, feature can be supported in early releases without signaling impact, and requirements can be introduced from Rel-19 spec.