3GPP TSG-RAN4 Meeting #116	R4-2509913

# Bengaluru, India, 25th - 29th August, 2025

Agenda item:		7.10.3.3

Source:				Keysight Technologies UK Ltd

Title:					On TMT and CTMT metric definitions

Document for:		Approval

# Introduction

This contribution is briefly addressing the topic of combining TP results and how to get to a single performance CTMT metric.

Measured results from a a previous measurement campaign [2] using the UMa channel model for all 4 DUT orientiations and 4 rotations for each orientations were examined for one DUT. The respective measured TPs as a function of time are shown in Figure 1. The CDF of each TP curve is shown in Figure 2; from each of these curves, the {10,50,90}%-tile of the CDF could be assessed but the test plan defines the CTMT as the CDF of the Total Measured TP (TMT) , i.e., the concatenated TP curves.

Figure 1: Measured TP as a function of time for 4 different device orientations and 4 rotations per device orientation

Figure 2: CDF curves of the measured TP from Figure 1

The concatenated TP curves from Figure 1, TMTUMa, are shown in Figure 3

Figure 3: TMT curve (concatenated TP curves from Figure 1)

The resulting CDF curve of the TMT in Figure 3 is shown in Figure 4.

Figure 4: CDF curve of TMT curve

The CDF values for each of the CDF %-iles (10%, 50%, 90%) are tabulated in Table 1. The average from the 16 different values is shown in the second last column while the last column shows the CTMTs determined from the CDF of the TMT, see Figure 3 and Figure 4. It can be observed that the average of the 16 different CDFs does not match the CTMT from the concatenated curve.

Observation 1: The CTMT values do not match the average of the 16 CDFs of the TP curves that make up the TMT.

Table 1: Comparison of CTMT with CDF values from individual TP curves

| CDF %-ile | CDF of TP Curves [Mbps] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CTMT [Mbps] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DML-Left |  |  |  | DMP |  |  |  | DMSU |  |  |  | DML-Right |  |  |  | AVG |  |
|  | 0° | 90° | 180° | 270° | 0° | 90° | 180° | 270° | 0° | 90° | 180° | 270° | 0° | 90° | 180° | 270° |  |  |
| 10 | 62.7 | 63.2 | 59.9 | 56.8 | 57.9 | 63.2 | 59.2 | 63.2 | 48.7 | 49.5 | 45.7 | 54.0 | 64.8 | 53.5 | 69.2 | 69.1 | 58.8 | 56.5 |
| 50 | 157.4 | 157.3 | 169.9 | 165.6 | 157.1 | 157.7 | 157.2 | 161.0 | 125.0 | 150.3 | 148.6 | 131.6 | 173.6 | 157.3 | 156.5 | 164.8 | 155.7 | 156.4 |
| 90 | 381.2 | 435.4 | 429.3 | 378.5 | 395.1 | 382.7 | 393.2 | 392.7 | 431.1 | 439.3 | 378.3 | 407.0 | 439.3 | 416.8 | 378.3 | 426.4 | 406.5 | 420.7 |

As discussed in previous meetings, there is a desire to reduce the number of metrics/performance requirements by combining channel models and removing/combining CDF %-iles.

As stated and argued in [3], we still believe that there is value in considering multiple CDF %-iles mainly due to the dynamic link adaptation approach considered here, e.g.,

For example, which device is better, e.g., one that varies evenly between 0 and 100 MBps performance or one that always has 50 MBps performance? The second device gives the user an acceptable guaranteed level of performance, while the first one could lead to a disappointed user at least part of the time, e.g., blaming the device and/or the network for the poor performance. One potential solution to this would be to take a few percentiles across the throughput CDF and use them as the performance metric.

Additionally, some of the measurement results in [2] have demonstrated the value/benefit of multiple %-iles as devices do perform/rank differently in those three distinct CDF %-iles, e.g., Figure 5 and Table 2 of [2]

Figure 5: Results for the five UEs in the DMP orientation with 0° rotation for UMa route. Top: TP vs time, Bottom: CDF vs TP. … The three different percentile CDF values (10%, 50%, 90%) from Figure 5 including the post-processed means and medians are tabulated in Table 2. The relative performance ranking of the non-post-processed results is shown with colour coding (red: poor, yellow: medium, green: good) and the corresponding best and worst results are shown with bolded font. Table 2: Comparison of device performance for TRS and Dynamic MIMO OTA assessment in DMP with 0° rotation UE; TRS [dBm]; TP per 3GPP (UE Noise-Limited) [Mbps]; TP per CTIA (SIR-based) [Mbps]. CDF10%; CDF50%; CDF90%; Mean; Median; CDF10%; CDF50%; CDF90%; Mean; Median. UE#1; -85.8; 49.1; 125.1; 431.2; 201.8; 125.1; 212.3; 282.7; 376.5; 290.5; 282.7. UE#2; -87.8; 34.3; 110.1; 427.5; 190.6; 110.1; 158.1; 257.9; 363.4; 259.8; 257.9. UE#3; -84.0; 93.2; 161.8; 433.8; 229.6; 161.8; 172.6; 256.1; 378.0; 268.9; 256.1. UE#4; -87.2; 85.6; 159.8; 439.5; 228.3; 159.8; 160.8; 291.9; 433.4; 295.4; 291.9. UE#5; -84.1; 62.1; 138.1; 374.8; 191.7; 138.1; 175.3; 246.4; 372.0; 264.5; 246.4.

Proposal 1: Continue to consider the three different CDF %-iles, CTMT10%, CTMT50%, and CTMT90%

In terms of combining the UMi and UMa channel model results, a similar concatenation approach as for the 16 different orientations&rotations could be applied to combine the CTMT values for the UMi and UMa results, i.e.,

TMTUMi&UMa=TMTUMi, TMTUMa

with the three separate CTMT values, i.e., the 10%, 50%, and 90%-ile CDF, defined as

-	CTMTUMi&UMa,10%=CDF10%TMTUMi&UMa

-	CTMTUMi&UMa,50%=CDF50%TMTUMi&UMa

-	CTMTUMi&UMa,90%=CDF90%(TMTUMi&UMa)

Proposal 2: Combine the UMi&UMa results by concatenating the respective TMTs, i.e., TMTUMi&UMa=TMTUMi, TMTUMa into a single data pool and determine the three different CTMTs (10%, 50%, 90%) from that single TMT.

As shown in Table 1, a regular/unweighted average of the three different CTMTs would results in the 10% and 50% CDF values to have very little effect on the average as those values are significantly smaller than the 90% value. In order to provide each CDF value similar weight for the final metric, it is proposed to use a weighted average to determine the single metric, i.e.,

CTMTUMi&UMa=w10CTMTUMi&UMa,10%+w50CTMTUMi&UMa,50%+w90CTMTUMi&UMa,90%3

with w10>w50>w90.

The absolute values of w10, w50, w90 are currently FFS.

Proposal 3: To determine a single metric from the three different CTMTs (10%, 50%, 90%), use a weighted averaging approach to give CTMTs with low TP values an almost equal weight as CTMTs with the highest TP value.

To define the weights before defining requirements with a single performance metric using a measurement campaign, it is proposed to perform a separate small campaign with a limited set of DUTs.

Proposal 4: Perform a separate measurement campaign with a limited set of DUTs to define the weights w10>w50>w90 before defining requirements using a measurement campaign

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: The CTMT values do not match the average of the 16 CDFs of the TP curves that make up the TMT.

Proposal 1: Continue to consider the three different CDF %-iles, CTMT10%, CTMT50%, and CTMT90%

Proposal 2: Combine the UMi&UMa results by concatenating the respective TMTs, i.e., TMTUMi&UMa=TMTUMi, TMTUMa into a single data pool and determine the three different CTMTs (10%, 50%, 90%) from that single TMT.

Proposal 3: To determine a single metric from the three different CTMTs (10%, 50%, 90%), use a weighted averaging approach to give CTMTs with low TP values an almost equal weight as CTMTs with the highest TP value.

Proposal 4: Perform a separate measurement campaign with a limited set of DUTs to define the weights w10>w50>w90 before defining requirements using a measurement campaign

# References

- TR 38.762, Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) dynamic test methodology for FR1 UEs

- R4-2416479, Preliminary Results from Dynamic MIMO OTA Measurement Campaign, Keysight Technologies, CAICT, 3GPP TSG-RAN4 Meeting #112bis, October 2024

- OTAMIMO_2021_010_004, Proposed Figure of Merit for Dynamic MIMO OTA Test, Keysight Technologies
This is a CTIA Certification Program Working Group Document available to CTIA Certification Program Working Group members