3GPP TSG-RAN4 Meeting #116	R4-2509917

# Bengaluru, India, 25th - 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.870 | CR | 0030 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Partial Radiated Metrics PRP/PRS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, ETS-Lindgren |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRP_TRS_MIMO_OTA_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-07-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Partial Radiated Metrics are considered for NTN OTA testing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the partial radiated metrics for TX and RX visually and mathematically |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Partial radiated metrics cannot be implemented properly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Discussion Paper in R4-2509918 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

## 5.3	Definition of the Partial Radiated Metrics (PRP/PRS)

### 5.3.1	Definition of the Partial Radiated Metrics (PRP/PRS) for AC

#### 5.3.1.1	Applicability of metrics

The PRP/PRS metrics are defined in Clauses 5.3.1.2 through 5.3.1.7, with the applicability provided in Table 5.3.1.1-1 below.

Table 5.3.1.1-1: Applicability of PRP/PRS metrics

| Clause | TX/RX | PRP/PRS definition | Applicability |
| --- | --- | --- | --- |
| 5.3.1.2 | TX | UPRP±30≅12a2cutTX,2+i=01cutTX,i | FFS |
| 5.3.1.3 | TX | UPRP±60≅12a4cutTX,4+i=03cutTX,i | FFS |
| 5.3.1.4 | TX | UHPRP≅12a6cutTX,6+i=05cutTX,i | FFS |
| 5.3.1.5 | RX | UPRS±30=21cutRX,0+b1cutRX,1 | FFS |
| 5.3.1.6 | RX | UPRS±60=21cutRX,0+cutRX,1+b2cutRX,2 | FFS |
| 5.3.1.7 | RX | UHPRS=21cutRX,0+cutRX,1+cutRX,2+b3cutRX,3 | FFS |
| where cutTX,i=1Mj=0M-1EiRPθθi,ϕj+EiRPϕθi,ϕj⋅Wθi and cutRX,i=1Mj=0M-11EISθ(θi,ϕj)+1EISϕ(θi,ϕj)⋅W(θi), Wθi are the quadrature weights of either the Clenshaw-Curtis (CC) weights or the traditional sin(q) weights, and ak=1-cosk*15°-i=0k-1W(θi)W(θk), bk=1-cosk*30°-i=0k-1W(θi)W(θk) the coefficients for the partial segments. |  |  |  |

#### 5.3.1.2	Upper Partial Radiated Power Integrated over ±30° Sector (UPRP±30)

The UPRP±30 metric is defined as the total power radiated with EIRP integrated from q=0° to q=30° in the upper hemisphere. This metric with the partial surface visualized in Figure 5.3.1.2-1 can be expressed mathematically as

UPRP±30≅12a2cutTX,2+i=01cutTX,i

with

a2=1-cos30°-i=01W(θi)W(θ2)≈0.5059,  &sin(q) quadrature0.4632,  &Clenshaw-Curtis quadrature

Figure 5.3.1.2-1: Visualization of partial surface with ±30° angular width for classical TRP grid.

#### 5.3.1.3	Upper Partial Radiated Power Integrated over ±60° Sector (UPRP±60)

The UPRP±60 metric is defined as the total power radiated with EIRP integrated from q=0° to q=60° in the upper hemisphere. This metric with the partial surface visualized in Figure 5.3.1.3-1 can be expressed mathematically as

UPRP±60≅12a4cutTX,4+i=03cutTX,i

with

a4=1-cos60°-i=03W(θi)W(θ4)≈0.5126,  &sin(q) quadrature0.4875,  &Clenshaw-Curtis quadrature

Figure 5.3.1.3-1: Visualization of partial surface with ±60° angular width for classical TRP grid.

#### 5.3.1.4	Upper Hemisphere Partial Radiated Power (UHRPP)

The UHPRP metric, which could also be referred to as UPRP±90, is defined as total radiated power in the upper hemisphere. This metric with the partial surface visualized in Figure 5.3.1.4-1 can be expressed mathematically as

UHPRP≅12a6cutTX,6+i=05cutTX,iwhere

cutTX,i=1Mj=0M-1EiRPθθi,ϕj+EiRPϕθi,ϕj⋅Wθi

with

a6=1-cos90°-i=05W(θi)W(θ6)≈0.5128,  &sin(q) quadrature0.5000,  &Clenshaw-Curtis quadrature

for both Clenshaw-Curtis or sin(q) quadratures whose weights are expressed using Wθi, defined in Clause 5.1.1.2.

Figure 5.3.1.4-1: Visualization of upper hemisphere partial surface with ±90° angular width for classical TRP grids.

#### 5.3.1.5	Upper Partial Radiated Sensitivity Integrated over ±30° Sector (UPRS±30)

The UPRS±30 metric is defined as the total sensitivity with EIS integrated from q=0° to q=30° in the upper hemisphere. This metric with the partial surface visualized in Figure 5.3.1.5-1 can be expressed mathematically as

UPRS±30=21cutRX,0+b1cutRX,1

with

b1=1-cos60°-i=00W(θi)W(θ1)≈0.5117,  &sin(q) quadrature0.4150,  &CC quadrature

Figure 5.3.1.5-1: Visualization of partial surface with ±30° angular width for classical TRS grid.

#### 5.3.1.6	Upper Partial Radiated Sensitivity Integrated over ±60° Sector (UPRS±60)

The UPRS±60 metric is defined as the total sensitivity with EIS integrated from q=0° to q=60° in the upper hemisphere. This metric with the partial surface visualized in Figure 5.3.1.6-1 can be expressed mathematically as

UPRS±60=21cutRX,0+cutRX,1+b2cutRX,2

with

b2=1-cos60°-i=01W(θi)W(θ2)≈0.5253,  &sin(q) quadrature0.4757,  &CC quadrature

Figure 5.3.1.6-1: Visualization of partial surface with ±60° angular width for classical TRS grid.

#### 5.3.1.7	Upper Hemisphere Partial Radiated Sensitivity (UHRPS)

The UHPRS metric, which could also be referred to as UPRS±90, is defined as total radiated sensitivity in the upper hemisphere. This metric with the partial surface visualized in Figure 5.3.1.7-1 can be expressed mathematically as

UHPRS=21cutRX,0+cutRX,1+cutRX,2+b3cutRX,3where

cutRX,i=1Mj=0M-11EISθ(θi,ϕj)+1EISϕ(θi,ϕj)⋅W(θi)

with

b3=1-cos90°-i=02W(θi)W(θ3)≈0.5438,  &sin(q) quadrature0.5000,  &CC quadrature

for both Clenshaw-Curtis or sin(q) quadratures whose weights are expressed using Wθi, defined in Clause 5.1.1.2.

Figure 5.3.1.7-1: Visualization of upper hemisphere partial surface with ±90° angular width for classical TRS grids.

<<< END OF CHANGES >>>