3GPP TSG-RAN WG4 Meeting #116	R4-2509922

Bangalore, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	On RAN4 Impacts for RRM measurement prediction in AIML Mobility

Agenda item:	7.19.3.1

Document for:	Discussion

Introduction

The discussions on AI/ML Mobility continued in RAN4#115 in the scope of Rel-19 SI ‎[1] and ‎‎[2]. Agreements achieved during this meeting are summarized in the WF ‎[3] and [4].

Some of the issues related to AI/ML Mobility RRM measurement prediction use case require further discussion. In this paper, we provide some additional views on issues related to RRM requirements, as listed below:

- Performance metrics

- Ground truth definition for RSRP accuracy for FR1

- Impacts from observation and prediction windows

- Inter-frequency prediction scenario

General aspects and Measurement event prediction use case are discussed in our accompanying papers [5] and [6].

Discussion

  - Performance Metrics

The following scenarios have been prioritized as high priority in RAN2:

| Scenario | Prioritization | Evaluation scenario combination | Objective | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | Measurement reduction | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | Measurement reduction | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | Mobility optimization | Intra-cell |

Relative Accuracy

In the RAN4#115 meeting, the following agreements were made on relative RSRP accuracy

Agreement:​ - For intra-frequency L3-RSRP, ​ - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), ​ - cell 1 and cell 2 are on the same frequency ​ - the reported RSRP of cell2 can be measured or predicted. ​ - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy ​ - Relative RSRP accuracy for Beam level measurements is FFS during WI phase depending upon RAN2 progress.

Agreement:​ - For inter-frequency L3-RSRP, ​ - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), ​ - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1 ​ - the reported RSRP of cell2 can be measured or predicted. ​ - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy ​

Following agreement was reached in RAN2#130:

Agreement - The scenario of intra-frequency spatial domain prediction (in cell dimension) is not considered by the UE-side model in 5G. RAN2 assumes that a network-sided model requires no specification impacts. Intra-frequency spatial domain prediction (in cell dimension) involves measuring one or more cells as input to the model to derive L3 filtered cell-level measurements for the same time instance of another cell(s). - The specification impact of intra-frequency spatial domain prediction in beam dimension will be studied by RAN2 for both UE-side and network-side model.

Agreement

- For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used.   For L1-filtered beam-level RSRP reporting can be configured by setting co-efficient to zero.   FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction.

- To support cell/beam level prediction, one enhancement is to report RRM measurement results per cell at multiple time instances in one measurement report for NW-sided model.

As shown in the above agreements, RAN2 is considering beam level measurements in addition to cell measurements for the different scenarios.

For intra-frequency L3-RSRP relative accuracy definition, RAN4 to cover comparison between two RSRP levels from different beams in the same cell.

The current definition of the relative RSRP accuracy of predicted L3-RSRP involves predicted RSRP of cell 1 and RSRP of cell 2 (whether predicted or measured) as well as ground truth values for both cells. These reported RSRP values can possibly relate to different time instants. In particular, for inter-frequency case, UE required to conduct measurements on different cell should follow measurement gap procedure as specified in TS38.133. It encompasses per-FR measurement gap and per-UE Measurement gap which are established based on network configuration. However, in the legacy definition of relative RSRP accuracy, the time difference is not considered.

Therefore, the definition of relative RSRP accuracy of predicted L3 RSRP can also not involve the time difference between cell 1 and cell 2 RSRP (whether predicted or measured). The possible concerns related to the measurement gap procedure can be further investigated in RAN4 as a testability issue.

RAN4 to not consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy. It can be further investigated as a testability issue.

Regarding measurement prediction especially in the domain of temporal L3 RSRP prediction as presented in  Figure 1, the applied filtering coefficient necessitates that the UE may or may not combine and reports both predicted and measured samples as one value. These combined samples must then be compared with reports from other cells or beams to ensure accurate performance assessment. Given this process, it is crucial that the requirements for predicted relative accuracy are clearly defined and aligned with the established definitions and requirements for legacy relative accuracy specified in [TS 38.133]. This alignment ensures consistency and reliability in performance metrics, facilitating seamless integration and comparison across different network elements and technologies. Additionally, maintaining this alignment helps in optimizing network performance and enhances the overall user experience by ensuring that the predicted values are as close to the actual measured values as possible.

Figure 1: Examples of temporal domain predictions.

It is expected Case-B temporal domain prediction with applied L3 filtering to include both combination of measured and predicted values and comply with accuracy performance requirements similar to those of the legacy relative and absolute accuracy performance defined for measurement.

For temporal domain Case-B prediction, RAN4 may consider accuracy performance requirements similar to those of the legacy relative and absolute accuracy performance defined for measurement.

To summarize, relative RSRP accuracy may depend on several factors as well considered scenarios (as agreed in RAN2) listed below:

- Scenario either frequency domain or temporal domain prediction

- Inter or intra frequency (FR1/FR2) case

- Cell / beam measurement contribution (high priority for cell level measurements and beam level measurements involvement to be confirmed with RAN2 agreements progress)

- Measured or predicted measurement contribution (for cell 2)

- Impact of time difference between reported measurements on different cells (cell 1 and cell 2)

Ground truth definition for RSRP accuracy for FR1

In the RAN4#114 meeting, the following agreements were made for FR1 Ground truth definition for RSRP accuracy:

Issue 2-1-2: Ground Truth Definition for RSRP accuracy Agreement: - The way the ground truth is extracted in the testing may differentiate between FR1 and FR2.

Issue 2-1-2: Ground Truth Definition for RSRP accuracy - Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list

Following agreement was reached in RAN4#114bis meeting:

Issue 2-1-5: Ground truth definition for RSRP Accuracy for FR1 Agreement: During the SI, RAN4 to provide analysis on potential issue and possible solution to the two Alternatives. If RAN4 cannot make decision during the SI phase, RAN4 will discuss in the WI phase. - At least capture the definition of the two Alternatives in the TR.

Following is the RAN4#114 agreement for AIML Beam management usecase:

Agreement: - Use CDL-based channel model as starting point

Alternative 1, the ground truth for FR1 can be derived by TE based on the transmitted or received power:

- This is aligned with the legacy requirement for L3 RSRP accuracy. However, the legacy test is based on AWGN channel model. The AWGN channel is a deterministic channel, where signal power will be constant for both spatial and time domain.

- Based on the RAN4#114 agreement for Beam management which indicates the fading channel i.e. CDL channel model to be considered for FR2 as the starting point, hence CDL may also be considered as starting point for FR1 and FR2 AIML Mobility use case.

- Derivation of ground truth based on the transmitted or received power is not suitable for the fading channel since it is challenging to identify the exact time period of the measured and the predicted samples and this may lead to inaccuracies of ground truth derivation.

- The L1 filtering method, filtering co-efficient and the number of samples for filtering are not known by the TE since it is UE proprietary implementation, hence it will impact the derivation of the ground truth at TE.

- The advantage with this alternative is that there will be no issue of UE cheating since the ground truth is derived at TE.

Alternative 1 where the FR1 ground truth can be derived by TE based on the transmitted or received power (legacy approach) is not suitable for AIML Mobility due to the fading channel and due to the L1 filtering which is UE implementation specific and not known to the TE.

Alternative 2, the ground truth for FR1 is derived based on the measurements reported by UE based on certain conditions:

- With this approach, there is no complexity of considering the channel variations and the parameters that are UE implementation specific (e.g.: L1 filtering related information) since these would be considered at UE while reporting the measurements.

- If the timing window is configured, some UEs may not be able to measure and predict at the same time (depending upon the UE capabilities). However, this can be handled during testing as described below:

  - For temporal domain case A, measurement is always performed (i.e. there is no measurement reduction), hence UE can always report the measured values that will serve as ground truth.

  - For temporal domain case B, few measurements are skipped in the observation window. Hence the continuous measurement values will not be available at UE to report to TE. However, there are mechanisms to overcome this issue during testing where UE can perform legacy measurements and AIML predictions parallelly and report to TE depending upon the UE capability. Another method is to perform the test with iteration 1 configured for legacy method of measurement reporting (serves as ground truth) and next iteration of test can be configured for AIML predictions under same channel conditions in both the iterations.

- S(I)NR variations in the test at different time instances may pose the challenge for this alternative. However, this can be handled in multiple ways. For example: One option is to maintain average SINR value during the test however measure at higher SINR level for more accurate ground truth and the SINR may be lower during the prediction with reference to the average SINR.

- Another important aspect is to ensure that UE cheating does not occur with this alternative. This can be handled via various test procedures. Test procedures to identify UE cheating have been described in our accompanying paper on the testability aspects [1].

Alternative 2, where the FR1 ground truth is derived based on the measurements reported by UE based on certain conditions, is more suitable for AIML Mobility since the channel variations and L1 filtering are already considered by UE while performing measurements that serve as ground truth.

Alternative 2, where the FR1 ground truth is derived based on the measurements reported by UE based on certain conditions, is the preferred option for ground truth definition for RSRP accuracy even though there are inaccuracies in the UE measurements, since it is more feasible option for the dynamic channel conditions.

Impacts from observation and prediction windows

Following agreement was reached in RAN4#114 meeting:

Issue 2-1-9: Impacts from observation and prediction windows Agreement: - RAN4 to consider the impact of UE speed, MRRT and OW and PW lengths on prediction accuracy requirements for temporal predictions. - RAN4 to also consider number of measurements performed during OW and its impacts on the prediction accuracy.

For temporal domain prediction case A, no measurement reduction is considered. Depending on the subcase, either L1 measurements (sub-cases 1 and 3) or L3 measurement (sub-case 2) are considered as input to the ML model. The estimation of the number of L1 measurement samples within the OW should account for multiple factors such as L1 sampling period and filter parameters (which are UE proprietary implementation). Thus, it is not straightforward to evaluate the number of L1 measurements performed during the OW and therefore assess its impact on the prediction accuracy. Furthermore, it is possible to know the number of L3 measurement samples in a given OW (using measurement periodicity), however the number of L3 measurements input to the AI/ML model may still be considered as UE proprietary and should not be considered by RAN4.

RAN4 to not consider number of L1/L3 measurements performed during OW and its impacts on the prediction accuracy.

For temporal domain prediction case B, the UE is not required to measure all the time and may use instead predicted measurements. The degree of measurement reduction can be selected through the Measurement reduction rate in temporal domain (MRRT) parameter.

For the testing, the MRRT can be set to [50, 60, 66, 75, 80] % and UE speed can be set to [30, 60, 90, 120] km/h, resulting in a total number of 20 combinations. The number of test configurations can further increase with different OW and PW values as well.

The number of possible test cases can increase considerably for a set of selected OW, PW and MRRT with multiple UE speed cases.

For temporal domain prediction case B, RAN4 to agree on pre-selected reference testing scenarios relating to OW, PW and MRRT as well as UE speed.

However, for a given MRRT value, different possible ways can be employed to achieve the targeted measurement reduction level. As shown in the example below, 50% MRRT can be reached with two different skipping patterns: either completely replacing consecutive measurement slots with predictions in the PW (example 1) or alternating measured and skipped slots within the OW (example 2).

Figure 2: Example of MRRT 50% scenario 2 with different skipping patterns

As discussed in RAN2, for a given MRRT, the skipping pattern applies to L1 samples and depend on the UE proprietary L1 filter implementation. For this reason, it is under discussion if the UE needs to be able to indicate the measurement skipping patterns it supports to the NW. Based on this, the NW can eventually configure the MRRT through the explicit indication of the selected skipping pattern (from the set indicated by the UE).

Therefore, for RAN4, two options can be foreseen for the testing of the measurement reduction feature:

- Option 1: The skipping pattern information is UE proprietary and is not shared with the NW. The TE will then only indicate the MRRT to the DUT without precise knowledge of what is skipped or not.

- Option 2: The skipping pattern information is UE proprietary, but the UE informs the NW about the skipping patterns it supports. In this case, the TE should select the skipping pattern related to the MRRT to be tested and indicate it to the DUT as part of the test configuration.

Note that with option 1, the UE can possibly cheat, and the test framework is not robust. In this case, the TE may only set up a requirement on the prediction accuracy and verify if the test passes or not.

If skipping pattern information is not shared by the UE, RAN4 to only consider MRRT, OW and PW to UE as test configuration and set up a requirement on prediction accuracy.

If skipping pattern information is shared by the UE, TE should select the supported skipping pattern related to the MRRT to test. The test configuration should then include skipping pattern, OW and PW.

Inter-frequency prediction scenario

One of the high-priority study items for AI/ML in NR inter-cell mobility is inter-cell inter-frequency measurement prediction. Inter-frequency measurements require configuring measurement gaps, during which the UE must temporarily stop communicating with the serving cell to measure other frequencies. This process can lead to service interruptions and increased power consumption. AI/ML techniques can help reduce the number of inter-frequency measurements a UE needs to perform, thereby minimizing the need for continuous measurement and enhancing UE energy efficiency. An additional advantage of reducing inter-frequency measurements is that it can also reduce the number of configured measurement gaps, which can improve UE throughput and reduce latency.

Figure 3: Inter-Frequency Prediction Model

By utilizing measurement data from two frequency layers, an ML model, e.g., as shown in Figure 3, can be trained to understand the correlation between radio measurements across these layers. Specifically, the model can either predict first-layer measurements based on second-layer measurements or determine if a UE connected to a macro layer can connect to a micro layer. This may involve a clustering approach, where measurements from multiple cells serve as inputs for prediction.

On inter-frequency measurement prediction, following agreement was reached during RAN4#115 meeting:

Agreement:                                                                                                                                                                     RAN4 to study whether and how correlation coefficient can be considered in simulation assumption for RAN4 requirements.

Previously, RAN4 discussed the factors that may impact RSRP prediction accuracy for inter-frequency prediction scenario. One of the factors is the Correlation Coefficient (CC) between measured and predicted frequency layers. CC between different frequency layers needs some clarification in RAN4. In the following, we first provide an overview from RAN2 before discussing RAN4 details.

RAN-2 Overview

Inter-frequency measurement prediction is always done for cell-level measurements. In RAN2, it was agreed to restrict the inter-frequency measurement prediction to collocated deployments only, where the Set A and B cells in the prediction share the same site and sector, but are on different frequencies [TR 38.744 V0.0.6 7]. This was limited to allow for non-site-specific models to be used.

In RAN2#127bis, it was agreed that companies should report with their simulation the CC. The CC is intended to show how much the L3 RSRP of the two cells in different frequencies are correlated in simulation. It can help reflect whether good prediction accuracy results from AI or from a stronger correlation. It should be noted that CC is not a parameter defined by TS 38.901 but rather a calculated value. For inter-frequency RRM measurement prediction, the two vectors can be “series of RSRP in multiple time instances of the serving cell” and “series of RSRP in the same multiple time instances of the co-sector cell”. The series of L3 RSRP values of the two cells can be collected within the same time window, but it can be up to companies how to set the time window in their simulation (e.g., whole simulation time or any time window during simulation time).

During the discussion on simulation assumptions in RAN2, it was agreed that the Pearson Correlation Coefficient (PCC), which indicates the correlation between two vectors as a scalar value from -1 to 1, would be used to measure the correlation between two measured and predicted frequency layers [TR 38.744 V0.0.6 7].

For inter-frequency prediction scenario, it was agreed in RAN2 to use PCC to demonstrate the results. Also, it was agreed to restrict to co-located deployments only.

In RAN2, the evaluation results show that the higher the CC is between two frequency layers, the higher the prediction accuracy. Besides, the prediction accuracy is comparable between higher-to-lower frequency and lower-to-higher frequency case. Higher-to-lower frequency case refers to the scenario where measurements on a lower frequency (2GHz in the simulations) were predicted based on the actual measurement results on a higher frequency (4GHz in the simulations) and vice versa for lower-to-higher frequency case.

RAN2 results demonstrate that the higher the CC is between two frequency layers, the higher the prediction accuracy. Besides, the prediction accuracy is comparable when predicting higher-to-lower and lower-to-higher frequency.

Another definition that has been used to define CC is the measure of frequency distance [R2407092]. In summary, it is demonstrated that inter-frequency prediction highly depends on the frequency distance (inversely proportion to correlation) between input/output frequencies, i.e., the higher the frequency distance the worse the prediction performance.

R2407092 defines CC as measure of frequency distance. The results show that the higher the frequency distance the worse the prediction performance.

RAN-4 Overview

In RAN4, there is no clear definition of CC. However, RAN2 has used PCC to evaluate the results. As discussed in the previous subsection, RAN2 agreed that the PCC would be used to measure the correlation between two measured and predicted frequency layers [TR 38.744 V0.0.6 7].  For the sake of simplicity, it is better to use the same definition in both RAN2 and RAN4 WGs. Besides, for both intra-frequency and inter-frequency prediction, RAN4 thinks RF and baseband errors on the measurements would impact the prediction accuracy.

For inter-frequency prediction scenario, RAN4 should consider the RAN2 definition of correlation coefficient (CC) as a starting point.

However, the current definition for CC is simple and it might get complicated when different aspects such as beam pattern, non-co-location scenario, channel model in each frequency, are taken into consideration. Therefore, it might be required to redefine the definition for CC.

Different aspects, e.g., beam pattern/configuration, channel model in each frequency, might have an impact on the currently used definition for CC. Hence, it might be necessary to redefine CC when such aspects are taken into consideration.

R4-2506507 [MediaTek] highlights that it is not possible to set a fixed CC factor and then generate the channels of measured frequency layer or predicted frequency layer fulfil the CC. R4-2506507 don’t think it is feasible to study the impact of CC unless RAN4 defines a common dataset.

R4-2506548 [Vivo] As the key element affecting RSRP prediction accuracy for inter-frequency prediction, CC need to consider it when defining metrics. Regarding EPRE difference, as well as frequency distance and frequency separation, these are coupled with the CC and no need to splitting them into more detailed factors for discussion.

In RAN4, there is no clear definition of CC. Unless RAN4 defines a common dataset, it is not feasible to study the impact of CC. Hence, it’s important to define CC for RAN4 requirements.

To start discussion on RAN4 requirements, it is recommended to first define the requirements for the agreed scenario in RAN2. For example, RAN4 requirements must be first studied for co-located scenario. Besides, it is possible that different/same frequency pairs might have different CCs for co-located and non-co-located scenarios. Hence, RAN4 might need to define requirements accordingly.

In RAN2, inter-frequency prediction focuses on the co-located scenario. Hence, RAN4 can take the same assumption to define the requirements.

In inter-frequency prediction scenario, it is possible that different/same frequency pairs might have different CCs in co-located and/or non-co-located scenario. Hence, RAN4 requirements can be defined accordingly.

Besides, in RAN2 evaluations, same beam pattern (beam configuration, grid of beams) is assumed [TR 38.744 V0.0.6 7]. Beam pattern might impact the prediction performance or the CC. It is expected that with different grid of beams, prediction performance might degrade compared to same beam pattern. In our view, different gird of beams might have different impact; hence, RAN4 requirements must be defined accordingly.

For inter-frequency prediction scenario, it is possible that different beam patterns might show different performance. Therefore, RAN4 need to take this into account when defining the requirements.

Besides, channel models could be different in different frequency layers. Hence, one effective definition for such a scenario could be the correlation between power-delay profiles (PDPs) of different frequency layers. PDP helps distinguish fading due to movement versus frequency selectivity.

To define correlation coefficient (CC) for inter-frequency predictions, RAN4 to consider the channel models in different frequency layers. In this case, one way to define the CC is the correlation between PDPs of different frequency layers.

Conclusion

In this paper we provide our further view on the RRM Measurement prediction use case and RRM requirements to be studied, we particularly addressed the following topics:

- Performance metrics

- Ground truth definition for RSRP accuracy for FR1

- Impacts from observation and prediction windows

- Inter-frequency prediction scenario

The following Observations and Proposals were made:

- Observation 1: It is expected Case-B temporal domain prediction with applied L3 filtering to include both combination of measured and predicted values and comply with accuracy performance requirements similar to those of the legacy relative and absolute accuracy performance defined for measurement.

- Observation 2: Alternative 1 where the FR1 ground truth can be derived by TE based on the transmitted or received power (legacy approach) is not suitable for AIML Mobility due to the fading channel and due to the L1 filtering which is UE implementation specific and not known to the TE.

- Observation 3: Alternative 2, where the FR1 ground truth is derived based on the measurements reported by UE based on certain conditions, is more suitable for AIML Mobility since the channel variations and L1 filtering are already considered by UE while performing measurements that serve as ground truth.

- Observation 4: The number of possible test cases can increase considerably for a set of selected OW, PW and MRRT with multiple UE speed cases.

- Observation 5: For inter-frequency prediction scenario, it was agreed in RAN2 to use PCC to demonstrate the results. Also, it was agreed to restrict to co-located deployments only.

- Observation 6: RAN2 results demonstrate that the higher the CC is between two frequency layers, the higher the prediction accuracy. Besides, the prediction accuracy is comparable when predicting higher-to-lower and lower-to-higher frequency.

- Observation 7: R2407092 defines CC as measure of frequency distance. The results show that the higher the frequency distance the worse the prediction performance.

- Observation 8: Different aspects, e.g., beam pattern/configuration, channel model in each frequency, might have an impact on the currently used definition for CC. Hence, it might be necessary to redefine CC when such aspects are taken into consideration.

- Observation 9: In RAN4, there is no clear definition of CC. Unless RAN4 defines a common dataset, it is not feasible to study the impact of CC. Hence, it’s important to define CC for RAN4 requirements.

- Observation 10: For inter-frequency prediction scenario, it is possible that different beam patterns might show different performance. Therefore, RAN4 need to take this into account when defining the requirements.

- Proposal 1: For intra-frequency L3-RSRP relative accuracy definition, RAN4 to cover comparison between two RSRP levels from different beams in the same cell.

- Proposal 2: RAN4 to not consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy. It can be further investigated as a testability issue.

- Proposal 3: For temporal domain Case-B prediction, RAN4 may consider accuracy performance requirements similar to those of the legacy relative and absolute accuracy performance defined for measurement.

- Proposal 4: Alternative 2, where the FR1 ground truth is derived based on the measurements reported by UE based on certain conditions, is the preferred option for ground truth definition for RSRP accuracy even though there are inaccuracies in the UE measurements, since it is more feasible option for the dynamic channel conditions.

- Proposal 5: RAN4 to not consider number of L1/L3 measurements performed during OW and its impacts on the prediction accuracy.

- Proposal 6: For temporal domain prediction case B, RAN4 to agree on pre-selected reference testing scenarios relating to OW, PW and MRRT as well as UE speed.

- Proposal 7: If skipping pattern information is not shared by the UE, RAN4 to only consider MRRT, OW and PW to UE as test configuration and set up a requirement on prediction accuracy.

- Proposal 8: If skipping pattern information is shared by the UE, TE should select the supported skipping pattern related to the MRRT to test. The test configuration should then include skipping pattern, OW and PW.

- Proposal 9: For inter-frequency prediction scenario, RAN4 should consider the RAN2 definition of correlation coefficient (CC) as a starting point.

- Proposal 10: In RAN2, inter-frequency prediction focuses on the co-located scenario. Hence, RAN4 can take the same assumption to define the requirements.

- Proposal 11: In inter-frequency prediction scenario, it is possible that different/same frequency pairs might have different CCs in co-located and/or non-co-located scenario. Hence, RAN4 requirements can be defined accordingly.

- Proposal 12: To define correlation coefficient (CC) for inter-frequency predictions, RAN4 to consider the channel models in different frequency layers. In this case, one way to define the CC is the correlation between PDPs of different frequency layers.

- References

- RP-242393, “Revised SID on AIML for mobility in NR”, OPPO, RAN#105 meeting, 9 – 12 September 2024, Melbourne, Australia.

- 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR; (Rel. 19)

- RF-2508268, WF on AI/ML Mobility Part 1, Nokia, RAN4#115, Malta, Malta, 19th – 23rd May, 2025.

- R4-2508269, WF on FS_NR_AI/ML_Mob_Part 2, OPPO, RAN5#115, Malta, Malta, 19th – 23rd May, 2025.

- R4-2509924, “On General aspects in AIML Mobility”, Nokia, RAN4#116 meeting, Bangalore, India, 25th - 29th August 2025.

- R4-2509923, “On RAN4 Impacts for Measurement Event Prediction in AIML Mobility”, Nokia, RAN4#116 meeting, Bangalore, India, 25th - 29th August 2025.