3GPP TSG-RAN WG4 Meeting #116	R4-2509924

Bangalore, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	On General aspects in AIML Mobility

Agenda item:	7.19.2

Document for:	Discussion

Introduction

The discussions on AI/ML Mobility continued in RAN4#115 in the scope of Rel-19 SI ‎[1]. The RAN4#116 meeting will discuss RAN4's general aspects and impact on RRM measurement prediction, measurement event prediction, and AI/ML mobility testability and interoperability. This contribution provides our views on RAN4 general aspects.

Discussion

Priorities of different scenarios

Following agreement was reached in RAN2#130:

Agreements RAN2#130: - The scenario of intra-frequency spatial domain prediction (in cell dimension) is not considered by the UE-side model in 5G. RAN2 assumes that a network-sided model requires no specification impacts. Intra-frequency spatial domain prediction (in cell dimension) involves measuring one or more cells as input to the model to derive L3 filtered cell-level measurements for the same time instance of another cell(s). - The specification impact of intra-frequency spatial domain prediction in beam dimension will be studied by RAN2 for both UE-side and network-side model.

Therefore, RAN4 should follow RAN2 and only study specification impact of inter-frequency spatial domain prediction in beam dimension for UE sided models.

RAN4 to study the specification impact of intra-frequency spatial domain prediction in beam dimension for UE sided models.

Impacts of measurement errors

In the SI phase, the potential performance gain using AIML-based mobility has been evaluated with an assumption on no measurement error based on RAN2 metrics such as a mean square error. However, in the WI phase, RAN4 should evaluate the performance of the AIML mobility with having an assumption on the measurement error to understand the impact on the prediction accuracy.

Therefore, the RAN4#113 meeting has discussed the impact of measurement errors for AI/ML Mobility evaluation and identified a number of options on measurement errors.

Issue 2-1-3: Impacts of measurement error on prediction accuracy - Candidate options: - Option 1: RAN4 should study the impact of measurement error on the prediction accuracy during SI phase - Option 2: RAN4 to not study the impact of measurement error on the prediction accuracy during SI phase - Option 3: RAN4 should wait for evaluation outcomes of RAN2 AI/ML for mobility (option3) and RAN4 AI/ML for beam management (option 4) - Option 4: RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event. Way Forward: Many companies are supporting Option 4. Companies are encouraged to bring proposals on what input RAN4 should provide to RAN2 to consider impact of error on measurements used for prediction on the accuracy of the predicted measurement/event

Also, in RAN4#114, it has been agreed that RF and baseband errors on the measurements would have an impact on the measurement prediction accuracy.

Issue 2-1-4: Impacts of measurement error on prediction accuracy Agreement: For AI based RRM measurement prediction in the SI, while RAN4 has not agreed yet on the exact error model(s) in the SI phase, RAN4 think RF and baseband errors on the measurements would impact the prediction accuracy.

In mobility scenarios, it is important that measurement errors will occur when utilizing data as input or ground truth for AI/ML models. These errors can significantly affect network performance by resulting in inaccurate predictions, which in turn may impact mobility management, handover optimization, and RRM. Therefore, it is essential for RAN4 to have a common understanding on the cause and modeling of the measurement error. Thus, the following WF was recommended in RAN4#114bis meeting:

WF: Companies are encouraged to bring proposals on how to capture factors to be considered for error models to consider the impact of measurement errors on prediction accuracy.

To define the measurement errors in the inputs of AI/ML models, there exist RF and baseband measurement errors. The models of RF and baseband measurement error can be the same or different.

Also, it is widely assumed to use a uniform or a Gaussian distribution as a model of a measurement error. The uniform or Gaussian distribution modelling can be readily applicable to RF and baseband error. Note that any other modelling can be further considered in the Work Item phase.

RAN4 to capture the following error modelling principles in the TR:

- RF and baseband measurement error modelling may be considered.

- A uniform or a Gaussian distribution can be used as a model for measurement error for RF and baseband.

- Alternative modelling approaches (such as based on link-level simulations) are not precluded and can be explored in the Work Item phase.

Conclusion

In this paper we provide our further view on the RRM Measurement prediction use case and RRM requirements to be studied in RAN4 working group for this use case.

The following Proposals were made:

- RAN4 to study the specification impact of intra-frequency spatial domain prediction in beam dimension for UE sided models.

RAN4 to capture the following error modelling principles in the TR:

- RF and baseband measurement error modelling may be considered.

- A uniform or a Gaussian distribution can be used as a model for measurement error for RF and baseband.

- Alternative modelling approaches (such as based on link-level simulations) are not precluded and can be explored in the Work Item phase.

- References

- 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR; (Rel. 19).