3GPP TSG-RAN WG4 Meeting #116	R4-2509926

# Bengaluru, India, August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR Scheduling availability during UE intra-frequency measurements based on OD-SSB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Scheduling availability for UE intra-frequency measurements without gap based on On-demand SSB is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify scheduling availability requirements of UE during intra-frequency measurements based on On-demand SSB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 9.2.5.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <<Start of Change1>>

#### 9.2.5.X	Scheduling availability of UE during intra-frequency measurements based on On-demand SSB

UE shall be capable of measuring without measurement gaps when the OD-SSB is completely contained in the active bandwidth part of the UE. When any of the conditions in the following clauses is met, there are restrictions on the scheduling availability; otherwise, there is no scheduling restriction. Note that all the configured OD-SSB symbols on the target carrier are referred to in this clause.

##### 9.2.5.X.1	Scheduling availability of UE performing measurements in TDD bands on FR1

When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on OD-SSB symbols to be measured, and on 1 data symbol before each consecutive OD-SSB symbols to be measured and 1 data symbol after each consecutive OD-SSB symbols to be measured within the Fast Measurement window duration.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on OD-SSB symbols to be measured, and on 1 data symbol before each consecutive OD-SSB symbols to be measured and 1 data symbol after each consecutive OD-SSB symbols to be measured within SMTC window duration outside the Fast Measurement window duration. If the high layer in TS 38.331 [2] signalling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRQ measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on OD-SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive OD-SSB to be measured/RSSI symbols and 1 data symbol after each consecutive OD-SSB to be measured/RSSI symbols within the Fast Measurement window duration.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on OD-SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive OD-SSB to be measured/RSSI symbols and 1 data symbol after each consecutive OD-SSB to be measured/RSSI symbols within SMTC window duration outside Fast measurement window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided, there are no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band. Otherwise, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols if nonCollocatedTypeNR-CA-r18 is provided.

When TDD inter-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols, if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair.

When inter-band FR1+FR1 NR-DC is performed, the scheduling restrictions due to a given serving cell also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols, if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair.

##### 9.2.5.X.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UE which does not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to SS-RSRP/RSRQ/SINR measurement

-	If deriveSSB_IndexFromCell is enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on OD-SSB symbols to be measured, and on 1 data symbol before each consecutive OD-SSB symbols to be measured and 1 data symbol after each consecutive OD-SSB symbols to be measured within the Fast Measurement window duration.

-	If deriveSSB_IndexFromCell is enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on OD-SSB symbols to be measured, and on 1 data symbol before each consecutive OD-SSB symbols to be measured and 1 data symbol after each consecutive OD-SSB symbols to be measured within SMTC window duration outside the Fast Measurement window duration. If the high layer signalling of smtc2 is configured (in TS 38.331 [2]), the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

-	If deriveSSB_IndexFromCell is not enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within Fast Measurement window duration.

-	If deriveSSB_IndexFromCell is not enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration outside Fast Measurement window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

If the following conditions are met:

-	The UE has been notified about system information update through paging,

-	The gap between the UE’s reception of PDCCH that UE monitors in the Type 2-PDCCH CSS set that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots

The UE shall receive the PDCCH that the UE monitors in the Type0-PDCCH CSS set, and/or the corresponding PDSCH, on SSB symbols to be measured.

When intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided, there are no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band. Otherwise, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols if nonCollocatedTypeNR-CA-r18 is provided.

##### 9.2.5.X.3	Scheduling availability of UE performing measurements on FR2-1

The following scheduling restriction applies due to SS-RSRP or SS-SINR measurement on an FR2 intra-frequency cell

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on OD-SSB symbols to be measured, and on 1 data symbol(s) before each consecutive OD-SSB symbols to be measured and 1 data symbol(s) after each consecutive OD-SSB symbols to be measured within Fast Measurement window duration.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on OD-SSB symbols to be measured, and on 1 data symbol(s) before each consecutive OD-SSB symbols to be measured and 1 data symbol(s) after each consecutive OD-SSB symbols to be measured within SMTC window duration outside Fast Measurement window duration.

The following scheduling restriction applies to SS-RSRQ measurement on an FR2 intra-frequency cell

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on OD-SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol(s) before each consecutive OD-SSB to be measured/RSSI symbols and 1 data symbol(s) after each consecutive OD-SSB to be measured/RSSI symbols within Fast Measurement window duration.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on OD-SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol(s) before each consecutive OD-SSB to be measured/RSSI symbols and 1 data symbol(s) after each consecutive OD-SSB to be measured/RSSI symbols within SMTC window duration outside Fast Measurement window duration.

where

-	If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

-	The signaling deriveSSB-IndexFromCell is always enabled.

When intra-band carrier aggregation in FR2 is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

When inter-band carrier aggregation in FR2 is performed, there are no scheduling restrictions on FR2 serving cells in the bands due to SS-RSRP, SS-RSRQ or SS-SINR measurement on an FR2 intra-frequency cell in different bands, provided that UE is capable of independent beam management on this FR2 band pair. Additionally, there is no scheduling restriction if the UE is configured with different numerology between SSB on one FR2 band and data on the other FR2 band provided the UE is configured for IBM operation for the band pair.

NOTE:	When inter-band carrier aggregation in FR2 is performed, the scheduling restrictions as defined in clause 9.2.5.X.1 due to a given serving cell also apply to another serving cell in a different FR2 band on the symbols that fully or partially overlap with the aforementioned restricted symbols, if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this FR2 band pair.

If following conditions are met:

-	The UE has been notified about system information update through paging,

-	The gap between the UE’s reception of PDCCH that UE monitors in the Type 2-PDCCH CSS set that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots

##### 9.2.5.X.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA

There are no scheduling restrictions on FR1 serving cell(s) due to measurements performed on FR2 serving cell frequency layer. However, the scheduling restrictions as defined in clause 9.2.5.X.1 due to a given serving cell in FR2 also apply to another serving cell in an FR1 band on the symbols that fully or partially overlap with the aforementioned restricted symbols, if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this FR1-FR2 band pair.

There are no scheduling restrictions on FR2 serving cell(s) due to measurements performed on FR1 serving cell frequency layer. However, the scheduling restrictions as defined in clause 9.2.5.X.1 due to a given serving cell in FR1 also apply to another serving cell in an FR2 band on the symbols that fully or partially overlap with the aforementioned restricted symbols, if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this FR1-FR2 band pair.

<<End of Change1>>