3GPP TSG-RAN WG4 Meeting #116	R4-2509931

# Bengaluru, India, August 25th – 29th, 2025

Source:	Nokia

Title:	Discussion on Testing Issues for CSI Compression Two-sided Models

Agenda item:	7.18.3

Document for:	Discussion

Introduction

In this contribution, we continue the discussion about the testing issues for the CSI compression use case. Following a WF from the RAN4#115 meeting, we report and discuss in this contribution the simulation results related to Option 3 Track 2. We also provide our views on non-simulation issues regarding what to capture/specify for the studied test decoder options and concluding on the feasibility of these options.

Discussion

Further simulation activity for Option 3 Track 2

In the RAN4#115 meeting, the following further investigations for Option 3 Track 2 were highlighted.

In order to further elaborate option 3 track 2 considering lower complexity encoders, interested companies may contribute simulations to RAN4#116. The following guidance, and also the guidance in R4-2508084 is provided and recommended for companies to follow in order to obtained aligned simulations. For option 3, track 2 consider the following investigations for lower complexity encoder structures. - Performance assuming training and testing with the Samsung frozen decoder - Performance achieved with a jointly trained low complexity encoder and decoder - Performance achieved when a frozen decoder is trained based on all of the encoder structures (i.e. different encoder complexity) - Consider transformer, CNN, MLP

Table 1 provides a summary of the additional results from Option 3 Track 2 expected for RAN4#116, following the guidance in [1]. In this table, Encoder-1 refers to the reference high-complexity CNN-based encoder. Encoder-2, Encoder-3, and Encoder-4 refer to the lower-complexity transformer-based, CNN-based, and MLP-based encoders, respectively. The architecture and parameters of these lower-complexity encoders are detailed in [1]. Based on earlier agreement from RAN4#114bis meeting, the results reported below are generated using quantized models. The dataset used for both training and testing purposes is the mixed dataset.

Table 1. Option 3 Track 2 results following the format recommended in [R4-2508084].

| Encoder | （1-to-1) Joint training | New encoder training with (Samsung’s) frozen decoder | （N-to-1) Joint training (SGCS-4 Step-1) | New encoder training with frozen decoder from Step-1 (SGCS-4 Step-2) |
| --- | --- | --- | --- | --- |
| Encoder-1 | 0.698 | 0.713 | 0.657 | 0.693 |
| Encoder-2 | 0.702 | 0.687 | 0.641 | 0.700 |
| Encoder-3 | 0.686 | 0.690 | 0.655 | 0.684 |
| Encoder-4 | 0.671 | 0.637 | 0.644 | 0.654 |

For SGCS-4 results, we used the same method proposed in [1] for calculating the training loss. SGCS-4 Step-1 refers to the joint training of the reference decoder (the agreed high-complexity CNN decoder) using all considered encoders (HC CNN, LC Transformer, LC CNN, and LC MLP), while SGCS-4 Step-2 refers to the separate training of each of these encoders with the frozen decoder derived from the SGCS-4 Step-1. During SGCS-4 Step 1, the training SGCS is the value recorded while training the shared reference decoder together with all considered encoders using 80% of the mixed dataset. During SGCS-4 Step 2, the training SGCS is measured while training the corresponding encoder using the frozen Decoder-2 (derived from Step 1 of SGCS-4) with the same 80% of the mixed dataset. For both SGCS-4 Step 1 and SGCS-4 Step 2, the testing SGCS values are recorded while testing the trained models on the remaining 20% of the mixed dataset.

As reported in Table 3 in the Annex, the training results in SGCS-4 Step-1 are the same for all encoders because we compute the training SGCS as 1- Loss, while we compute the testing results in SGCS-4 Step-1 for each encoder separately (e.g., testing SGCS for Encoder-1 = SGCS (V,V1)). Compared to Table 3, the results we report in Table 1 are testing SGCS only.

We start by comparing SGCS-2 to SGCS-1 results (column 3 in Table 1). Based on the results reported in Table 1, we can observe that SGCS-2 results with Encoder-2, Encoder-3, and Encoder-4 are lower than SGCS-1 with Encoder-1. This means that the separate training of the lower-complexity encoders with the high-complexity CNN-based frozen Samsung’s decoder may lead to a slightly reduced SGCS performance compared to the separate training with same backbone and complexity (i.e., high-complexity CNN-based encoder and frozen decoder). This can be explained by the backbone and complexity mismatches between the considered encoders and the frozen decoder.

The separate training of the lower-complexity encoders with the high-complexity CNN-based frozen decoder may lead to a reduced SGCS performance compared to the separate training of the encoder and frozen decoder with the same backbone and complexity (i.e., high-complexity CNN-based).

We then compare SGCS-0 to SGCS-3 results (column 2 in Table 1). Ideally, Encoder-1 (high-complexity CNN-based encoder) is expected to deliver the best performance due to the architectural match with the decoder. The slightly higher SGCS value (by 0.004) of Encoder-2 (low-complexity transformer-based encoder) could be attributed to random variations in the simulation parameters. It is worth noting that the lowest value among the SGCS-2 and SGCS-3 results is obtained when the lower-complexity MLP-based Encoder-4 is used.

The joint and separate training of the lower-complexity MLP-based encoder with the high-complexity CNN-based decoders give the lowest SGCS performance compared to the other joint and separate training results with the encoder backbones and complexities assumed in [R4-2508084].

Lastly, we compare SGCS-4 Step-2 results (column 5 in Table 1) to SGCS-1 (0.713). Based on the results reported in Table 1, we can derive the following observation.

Using multiple encoders with different backbones and complexities to derive the test decoder did not enhance the SGCS performance compared to using a single high-complexity CNN-based frozen decoder for the separate training.

Table 2 provides a summary of the additional results from Option 3 Track 2 expected for RAN4#116, more precisely related to “Performance achieved with a jointly trained low complexity encoder and decoder”. Following the same nomenclature used in [1] for different encoders, Decoder-1 refers to the reference high-complexity CNN-based decoder. Decoder-2, Decoder-3, and Decoder-4 refer to the lower-complexity transformer-based, CNN-based, and MLP-based decoders, respectively. The structures and parameters for the matching decoders are set in a way that mimics the encoder structures and parameters. As a result, we used the structures and parameters reported in the Annex for Decoder-2 (Figure 1 and Table 5, respectively), Decoder-3 (Figure 2 and Table 6, respectively), and Decoder-4 (Figure 3 and Table 7, respectively). Based on earlier agreement from RAN4#114bis meeting, the results reported below are generated using quantized models. The dataset used for training and testing purposes is the mixed dataset.

Table 2: Option 3 Track 2 results with jointly trained encoders and matching decoders.

| Encoder | Decoder | Testing SGCS |
| --- | --- | --- |
| Encoder-1 | Decoder-1 | 0.698 |
| Encoder-2 | Decoder-2 | 0.659 |
| Encoder-3 | Decoder-3 | 0.671 |
| Encoder-4 | Decoder-4 | 0.626 |

To better interpret the results in Table 2, we compare them with those in Table 1 column 2, where Decoder-1 (a high-complexity CNN-based decoder) is jointly trained with four different encoders of varied architectural backbones (low-complexity Transformer, low-complexity CNN, and low-complexity MLP). The SGCS values obtained from low-complexity encoder–decoder pairs with matching architectural backbones are consistently lower than those from low-complexity encoders paired with a high-complexity decoder, even when the architectural backbones are mismatched. This reinforces the observation that a higher-complexity model (in this case, the decoder) is better at learning and capturing patterns from the training data, resulting in superior SGCS performance regardless of the encoder’s architectural backbone.

A comparison between the SGCS performance of jointly trained low-complexity encoders with their matching decoders and that of a high-complexity decoder paired with an architecturally mismatched encoder confirms that decoder complexity is a key factor. A higher-complexity decoder consistently delivers superior SGCS performance, regardless of the encoder’s architectural backbone.

RAN4 to select a high-complexity decoder model for the fully standardized test decoder.

What should be captured or specified for options 3/4

- The following agreements regarding what to capture/specify for Option 3 were reached in the RAN4#114bis and RAN4#115 meetings.

RAN4#114bis Agreement - The encoder that corresponds to the standardized test decoder is stored within 3GPP - TBC whether in a TR, or in any TS, whether in RAN4 or RAN5, or whether simply kept in a 3GPP database (with no reference from TR or TS) - FFS whether several encoders would be captured (e.g. due to different complexity) RAN4#115 Agreement The training dataset used to train the test decoder in 3GPP should be captured - FFS whether channel, Eigenvectors or something else

- The following agreements regarding what to capture/specify for Option 4a were reached in the RAN4#114bis and RAN4#115 meetings.

RAN4#114bis Agreements - Dataset is specified for option 4a. - Assume reference decoder is captured. RAN4#115 Agreements - Reference encoder should be captured - FFS whether the labelled dataset is based on Eigenvector, channel estimation or something else - The study has only considered the case of a common assumption on model structure for the “own” test decoder. This corresponds to standardized model structure. - Structure refers to backbone, numbers of layers, type of layers and all description of the model, but not the parameters. - Interoperability in the case that the structure of the TE decoder is not specified has not been investigated

- The following agreements regarding what to capture/specify for Option 4b were reached in the RAN4#114bis and RAN4#115 meetings.

RAN4#114bis Agreements - Dataset is specified for option 4a. - FFS on dataset is captured for option 4b. - Assume reference decoder is captured. - FFS on reference encoder for option 4a - Test decoder structure is (i) fully specified or (ii) partially specified - NOTE: RAN4 understands that further decision on option 4a/4b and option 3 will be made in the future WI phase. RAN4#115 Agreements - Specify dataset to train frozen encoder and decoder - TE vendor shall use the dataset to train the test decoder - This is the same dataset as used to train the frozen encoder - FFS whether the datasets consist of Eigenvector, channel or something else - The study has only considered the case of a common assumption on model structure for the “own” test decoder. This corresponds to standardized model structure. - Structure refers to backbone, numbers of layers, type of layers and all description of the model, but not the parameters. - Interoperability in the case that the structure of the TE decoder is not specified has not been investigated

According to the above agreements from RAN4#114bis and RAN4#115, the specification work for options 3, 4a, and 4b is similar, since a decoder, an encoder, and a training dataset need to be captured for the three options. However, there are still some open points related to the encoder and training dataset to be specified/captured.

In the RAN4#114bis meeting, RAN4 agreed that the encoder that corresponds to the fully standardized test decoder is the reference encoder to be captured for Option 3. In the RAN4#115 meeting, RAN4 agreed that a reference encoder should be captured also for Option 4a. For Option 4b, the encoder is to be specified. In our view, a reference encoder would be required not only for test decoder verification but potentially for the definition of RAN4 requirements also. Therefore, it should be captured the RAN4 specifications.

To define performance requirements and to verify test decoder, RAN4 needs to specify a reference encoder. It should be captured in RAN4 specifications.

The reference encoder can be captured in RAN4 specifications using a link to a specified location of the model.

The second open discussion point is regarding whether several encoders would be captured due to different complexity. One potential solution here is to specify one reference encoder in terms of complexity. Based on the feasibility study simulations, especially the latest activity using different encoder/decoder complexities, the SGCS results obtained with high-complexity CNN-based model structure and low-complexity CNN-based model structure are comparable. Hence, there is no need to specify/capture multiple encoders with different complexities.

RAN4 to capture one reference encoder in terms of complexity in RAN4 specifications.

The third and last open discussion point is related to the format of the training dataset to be specified/captured. We believe that this point is closely related to the testing setup to be defined for AI/ML-based CSI compression. For instance, if the training dataset is captured using eigenvectors, we can face the situation where DUT is trained on eigenvectors while during the test the input of the encoder is presented in terms of channel estimates. To avoid such situations, we propose to define the training dataset in terms of raw channel data, from which eigenvectors can be derived later if the input data of the encoder is defined in the testing setup in terms of eigenvectors.

RAN4 to adopt the raw channel data format for the training dataset to capture in the three test decoder options.

Concluding on the feasibility of the test decoder options

The following agreement made in RAN4#115 meeting means that the 3 options are worthwhile to consider further in the WI, but it does not conclude on the feasibility of the three options.

RAN4#115 Agreement At the current moment in time, neither option 3 track 2, nor option 4a or 4b should be ruled out for a WI. Option 3 track 1 can be ruled out. Option 3 and 4 are similar: - An encoder, decoder and training dataset are always captured - The difference is that for option 3, the TE vendor is mandated to use the standardized test decoder. In option 4 they could choose to use the reference test decoder or to create their own based on the standardized information.

In addition, we still have some FFSs about the feasibility criteria under the last steps of the Option 3/Option 4a/Option 4b procedures, as stated below. The “Feasibility of test decoder verification procedure” row in Table 7.4.2.4-1 in the ongoing TP to TR 38.843 is also kept FFS for all options.

Step 5 of Option 3 procedure:

- Step 5: Conclude on overall feasibility of Option 3

  - consider the conditions under which Option 3 is feasible if found feasible

  - FFS on what the feasibility criteria

Step 7 of Options 4a/4b procedures:

- Step 7: Conclude on overall feasibility of Option 4a

  - Feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc.).

  - Dataset for feasibility evaluation to be discussed; may be common test dataset or own test dataset

Hence, we believe that the feasibility criteria, and optimally the conclusion on the feasibility of the test decoder options, should be discussed during this meeting, as it is the last meeting of the study. As a strict definition of feasibility requires more study and potentially more input from other WGs (e.g., whether an option can be implemented in real-life), we propose selecting feasibility criteria for concluding this study based on the simulations done and the agreements made so far in RAN4. For instance, an option can be feasible if it: (1) provides sufficient performance, and (2) can be standardized. A sufficient performance can be defined in terms of SGCS and in the range of ±5% from the SGCS obtained with Rel-16 eType2 codebook. For the second criterion, the agreements about what to capture/specify for each option can be used to decide whether an option can be standardized.

RAN4 to consider the following feasibility criteria for concluding on the feasibility of the test decoder options: An option can be feasible if it: (1) provides sufficient performance (e.g., ±5% from the SGCS obtained with Rel-16 eType2 codebook), and (2) can be standardized (e.g., based on the agreements about what to capture/specify for each option).

Conclusion

The following observations and proposals were made in this contribution:

Observation 1: The separate training of the lower-complexity encoders with the high-complexity CNN-based frozen decoder may lead to a reduced SGCS performance compared to the separate training of the encoder and frozen decoder with the same backbone and complexity (i.e., high-complexity CNN-based).

Observation 2: The joint and separate training of the lower-complexity MLP-based encoder with the high-complexity CNN-based decoders give the lowest SGCS performance compared to the other joint and separate training results with the encoder backbones and complexities assumed in [R4-2508084].

Observation 3: Using multiple encoders with different backbones and complexities to derive the test decoder did not enhance the SGCS performance compared to using a single high-complexity CNN-based frozen decoder for the separate training.

Observation 4: A comparison between the SGCS performance of jointly trained low-complexity encoders with their matching decoders and that of a high-complexity decoder paired with an architecturally mismatched encoder confirms that decoder complexity is a key factor. A higher-complexity decoder consistently delivers superior SGCS performance, regardless of the encoder’s architectural backbone.

Proposal 1: RAN4 to select a high-complexity decoder model for the fully standardized test decoder.

Proposal 2: To define performance requirements and to verify test decoder, RAN4 needs to specify a reference encoder. It should be captured in RAN4 specifications.

Proposal 3: The reference encoder can be captured in RAN4 specifications using a link to a specified location of the model.

Proposal 4: RAN4 to capture one reference encoder in terms of complexity in RAN4 specifications.

Proposal 5: RAN4 to adopt the raw channel data format for the training dataset to capture in the three test decoder options.

Proposal 6: RAN4 to consider the following feasibility criteria for concluding on the feasibility of the test decoder options: An option can be feasible if it: (1) provides sufficient performance (e.g., ±5% from the SGCS obtained with Rel-16 eType2 codebook), and (2) can be standardized (e.g., based on the agreements about what to capture/specify for each option).

- References

- R4-2508084, “Simulation assumptions on AI/ML CSI compression”, Huawei, Hisilicon, OPPO, CATT, Vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Mediatek, APPLE, Rohde & Schwarz, Samsung, Intel, ZTE Corporation, Sanechips, CAICT, RAN4#115 meeting, 19th – 23rd May 2025, St. Julian’s, Malta.

- Annex

Extra simulation results with number of parameters and training SGCS

Table 3: Set 1 of extra results with number of parameters and training SGCS.

| Encoder | （1-to-1）Joint training |  | New encoder training with (Samsung’s) frozen decoder |  | （N-to-1）Joint training |  | New encoder training with frozen decoder from Step-1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Encoder-1 | Number of parameters | 1930275 | Number of parameters | 1930275 | Number of parameters | 3713508 | Number of parameters | 1930275 |
|  | Training SGCS | 0.712 | Training SGCS | 0.714 | Training SGCS | 0.769 | Training SGCS | 0.712 |
|  | Testing SGCS | 0.698 | Testing SGCS | 0.713 | Testing SGCS | 0.657 | Testing SGCS | 0.693 |
| Encoder-2 | Number of parameters | 2035234 | Number of parameters | 2035234 | Number of parameters | 3713508 | Number of parameters | 2035234 |
|  | Training SGCS | 0.711 | Training SGCS | 0.687 | Training SGCS | 0.769 | Training SGCS | 0.704 |
|  | Testing SGCS | 0.702 | Testing SGCS | 0.687 | Testing SGCS | 0.641 | Testing SGCS | 0.700 |
| Encoder-3 | Number of parameters | 642739 | Number of parameters | 642739 | Number of parameters | 3713508 | Number of parameters | 642739 |
|  | Training SGCS | 0.709 | Training SGCS | 0.691 | Training SGCS | 0.769 | Training SGCS | 0.694 |
|  | Testing SGCS | 0.686 | Testing SGCS | 0.690 | Testing SGCS | 0.655 | Testing SGCS | 0.684 |
| Encoder-4 | Number of parameters | 1915490 | Number of parameters | 1915490 | Number of parameters | 3713508 | Number of parameters | 1915490 |
|  | Training SGCS | 0.691 | Training SGCS | 0.639 | Training SGCS | 0.769 | Training SGCS | 0.681 |
|  | Testing SGCS | 0.671 | Testing SGCS | 0.637 | Testing SGCS | 0.644 | Testing SGCS | 0.654 |

Table 4: Set 2 of extra results with number of parameters and training SGCS.

| Encoder | Decoder | Results |  |
| --- | --- | --- | --- |
| Encoder-1 | Decoder-1 | Number of parameters | 1930275 |
|  |  | Training SGCS | 0.712 |
|  |  | Testing SGCS | 0.698 |
| Encoder-2 | Decoder-2 | Number of parameters | 462304 |
|  |  | Training SGCS | 0.648 |
|  |  | Testing SGCS | 0.659 |
| Encoder-3 | Decoder-3 | Number of parameters | 97971 |
|  |  | Training SGCS | 0.674 |
|  |  | Testing SGCS | 0.671 |
| Encoder-4 | Decoder-4 | Number of parameters | 222816 |
|  |  | Training SGCS | 0.627 |
|  |  | Testing SGCS | 0.626 |

Lower-complexity transformer-based decoder (Decoder-2)

Figure 1: Detailed model diagram of Decoder-2.

Table 5: Transformer model parameters used for Decoder-2.

| Model type | Transformer |
| --- | --- |
| FLOPs | 3.63M |
| Model size | 1.08Mbyte |
| d_model | 64 |
| d_feedforward | 256 |
| N_head | 8 |
| d_head | 64 |
| N_TF | 4 |

Lower-complexity CNN-based decoder (Decoder-3)

Figure 2: Detailed model diagram of Decoder-3.

Table 6: CNN model parameters used for Decoder-3.

| Model type | CNN (Resnet like design) |
| --- | --- |
| FLOPs | 5.865M |
| Model size | 178Kbyte |
| Number of Resnet blocks | 2 |
| Layer after Resnet blocks | 1 layer (one Conv layer) |
| Layer before Resnet blocks | 2 layers (one FC layer, one Conv layer) |
| Layer within a resnet block | 2 Conv layer and direct combination |
| CNN kernel parameters | - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter) |
| Kernel/Channel size within a resnet block | (3,3,16) |
| Size of message (feature map) passed across layers | (13,32) |

Lower-complexity MLP-based decoder (Decoder-4)

Figure 3: Detailed model diagram of Decoder-4.

Table 7: MLP model parameters used for Decoder-4.

| Model type | MLP |
| --- | --- |
| FLOPs | 55.775K |
| Model size | 451Kbyte |
| K | 64 |
| Hidden width | 128 |