3GPP TSG-RAN WG4 Meeting #116	R4-2509932

Bangalore, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	RRM performance requirements for beam management and positioning accuracy

Agenda item:	7.17.6

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM and Positioning use cases were continued in RAN4#115 in the scope of Rel-19 WI NR_AIML_air [1] based on the objectives agreed in RAN4#107. Agreements achieved during RAN4#115 meeting are summarized in the WF [2].

In this paper, we address RRM performance requirements for AI/ML enabled beam management and positioning use cases. We discuss following main aspects:

- Performance requirements for AI/ML BM

- Testing framework for AI/ML based Beam Management

- Performance requirements for AI/ML Positioning

Discussion

Performance requirements for AI/ML BM

Measurement error impact and the ground truth

During RAN4#112 and RAN4#112bis meetings, RAN4 agreed to study the impact of measurement errors to AI/ML BM functionality on the performance of prediction of AI/ML BM. The agreements of RAN4 #113 are provided below:

Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training (set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. - impact on different prediction accuracy metrics should be taken into account - studies already performance by other groups (RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

Agreement: ​ RAN4 to perform study to assess the impact of measurement errors on prediction accuracy​ Perform LLS to evaluate the RSRP prediction accuracy and other KPIs​ - UE Model trained using ideal measurements​ Simulation assumptions in R4-2417211 can be used as starting point.

Below agreement on beam ID prediction accuracy was reached in RAN4#114bis meeting:

Agreement (RAN4 #114bis) - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values - Note: if x=0 and N=1, it means Top-K/1 (%): the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Following the updated simulation assumptions [3] from RAN4#115, the evaluation of measurement error on AI/ML BM-case1 using Beam Id prediction accuracy, referred to as KPI1, for Scenario 1 (both Set B and set A are narrow) is provided in Table 1:

| Scenario1 (N=1) | Case 1 | Case 2a |  |  | Case 2b |  |  | Case 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Alt 1 | Alt 2 | Alt 3 | Alt 1 | Alt 2 | Alt 3 | Alt 1 | Alt 2 | Alt 3 |
| K=1, X=1 | 0.84 | 0.83 | 0.74 | 0.73 | 0.81 | 0.62 | 0.61 | 0.76 | 0.61 | 0.60 |
| K=1, X=2 | 0.88 | 0.87 | 0.80 | 0.79 | 0.87 | 0.69 | 0.68 | 0.82 | 0.68 | 0.66 |
| K=1, X=3 | 0.91 | 0.90 | 0.84 | 0.83 | 0.90 | 0.75 | 0.73 | 0.86 | 0.74 | 0.72 |
| K=2, X=1 | 0.94 | 0.93 | 0.88 | 0.88 | 0.92 | 0.80 | 0.79 | 0.89 | 0.80 | 0.79 |
| K=2, X=2 | 0.95 | 0.95 | 0.91 | 0.91 | 0.95 | 0.85 | 0.84 | 0.91 | 0.84 | 0.84 |
| K=2, X=3 | 0.96 | 0.96 | 0.93 | 0.93 | 0.96 | 0.89 | 0.88 | 0.93 | 0.88 | 0.87 |
| K=3, X=1 | 0.97 | 0.96 | 0.93 | 0.93 | 0.96 | 0.87 | 0.87 | 0.94 | 0.88 | 0.87 |
| K=3, X=2 | 0.98 | 0.97 | 0.95 | 0.95 | 0.97 | 0.91 | 0.90 | 0.95 | 0.91 | 0.90 |
| K=3, X=3 | 0.98 | 0.98 | 0.96 | 0.96 | 0.98 | 0.93 | 0.93 | 0.96 | 0.93 | 0.93 |
| K=4, X=1 | 0.98 | 0.98 | 0.96 | 0.96 | 0.98 | 0.91 | 0.91 | 0.96 | 0.92 | 0.92 |
| K=4, X=2 | 0.98 | 0.98 | 0.97 | 0.97 | 0.98 | 0.94 | 0.94 | 0.97 | 0.94 | 0.94 |
| K=4, X=3 | 0.99 | 0.99 | 0.98 | 0.98 | 0.99 | 0.96 | 0.95 | 0.98 | 0.96 | 0.95 |
| K=5, X=1 | 0.99 | 0.99 | 0.97 | 0.97 | 0.98 | 0.94 | 0.93 | 0.98 | 0.95 | 0.94 |
| K=5, X=2 | 0.99 | 0.99 | 0.98 | 0.98 | 0.99 | 0.96 | 0.95 | 0.98 | 0.96 | 0.96 |
| K=5, X=3 | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 | 0.97 | 0.97 | 0.99 | 0.97 | 0.97 |

Table 1: Evaluation of KPI1 for Scenario1

Note that, here, the prediction accuracy of only the strongest beam (N=1) among the top-K predicted set is evaluated. These results are in line with the expected behaviour of KPI1 which increases for increasing values of K and X (along columns). And that RF errors dominate the impact on the accuracy values.

Based on simulation results, RAN4 to define values of  K,  N,  X and the success rate of beam id prediction for performance requirement of BM-case1 and BM-case2

Performance metrics

Following agreements on beam ID prediction accuracy for performance monitoring were reached in prior RAN1 meetings:

Agreement (RAN1 #120b) For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as: - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling) - Where K is the number of predicted beam(s) in the corresponding inference report per time instance - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, 2 - FFS: detailed rule or signalling

Agreement (RAN1 #121) For beam prediction accuracy report for monitoring, the report quantity RS-PAI is Np (0 ≤ Np≤ N) - Where Np is the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s) of monitoring resources that no later than CSI reference resource corresponding to the CSI report for monitoring - condition: - for the transmission occasion of monitoring resources, it has a linked inference report - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources mapped to one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the CSI Reporting Setting for inference - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance. - Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig the size of CSI field associated with the RS-PAI is log2(N+1)

Akin to beam ID prediction accuracy metric agreed in RAN4 for testing inference capability of the AI/ML beam Id prediction model at UE, RAN1 specified a more relaxed metric called RS-PAI to gauge the beam Id prediction accuracy for performance monitoring. Unlike the RAN4 KPI, RS-PAI (Top-K/M) is intended solely for performance monitoring. And it limits the number of best monitoring beams in its evaluation to a maximum of 2. Note, that the number of best monitoring beams is configured by the NW through the RRC parameter nrofBestBeamforMonitoring-r19 [4]

At this we present few key points, some reiterated from our past TDocs, on the description of beam ID prediction accuracy metric in the light of conformance testing from RAN4 perspective.

First, the number of top predicted beams to be reported by UE, referred as K in the metrics’ definition, is indicated by NW through the RRC parameter-nrofreportedpredictedrs-r19 [4].And, K is chosen from the possible values of 1,2,3,4 as agreed by RAN1. Setting aside RAN1’s reasoning on values for K, it is highly unlikely that NW configures a UE with K = 1 in the field due to scheduling and load-balancing constraints. Hence, the minimum meaningful value of the number predicted beams is K=2. Thus, it leads to narrowing the K to relevant values of 2, 3 and 4

K >= 2

Second, even the simple accuracy metric, RA-PAI, agreed in RAN1 for performance monitoring provides the flexibility to include up to 2 best monitoring beams (N as per RAN4) in its calculation. Clearly, the choice of N = 2 is inevitable. Additionally, Observation 1 implies that the NW will at least want to obtain the accuracy of 2 strong-enough beams. This leaves no reasoning to restrict the accuracy evaluation only for the highest RSRP beam. So, N >= 2

2 <= N <= K

- Third, for a fair evaluation the threshold for ith ranked predicted beam, bi could be set as RSRP of the corresponding ith ranked genie-aided beam bi – X. Hence, the condition to be checked for the ith ranked beam is Ci=lRSRPbi≥RSRPbi-X, where X is a fixed value for all i.

The comparison threshold for ith ranked predicted beam, bi can be considered as the RSRP of the corresponding ith ranked genie-aided beam, bi – X

- Fourth, a penalty corresponding to the loss incurred due to mismatched RSRPs of equal ranked beams from top-K prediction beam set ranked using ground truth and the true top-K beam set has to be captured in the accuracy evaluation. This is to ensure that the metric is a reflector of true performance (for example, UE throughput) observed in the field. Hence, accounting for such loss, the success rate of correct prediction can be computed as a weighted average of the underlying successful prediction of each top-N beams using the below formulation:

- Weighted Success Rate,  Sw=1Nt n=1Nti=1Nwi,n lRSRPbi,n≥RSRPbi,n-X,

- Where,

X: allowed RSRP margin

RSRPb: ground-truth RSRP of beam b

N: top-N strongest beams from top-K predicted beam set that are ranked based on ground-truth RSRP

l(C): Indicator function. It equals 1 when the condition C is true, else 0

Nt: Number of inference instances

- wi,n: weighing factors associated with ith ranked beam during the inference instant n.

wi,n=RSRPbi,nRSRPbi,ni=1KRSRPbi,nRSRPbi,n with RSRPs here in linear scale

NOTE: All the RSRPs and their ranking in the above are determined from the ground-truth values

The success rate of correct prediction can be computed as a weighted average

From the above discussion, we request RAN4 to consider the following proposals

RAN4 to discuss the adaptation of Option 3 with the following changes:

- At least N strongest beams among the top-K predicted beams must be considered in the success rate evaluation, where 2≤N≤K

  - RAN4 should consider N=2 if there is no consensus in RAN4 for N>2.

- RSRP of each of the strongest N predicted beams must be compared with the RSRP of its equal ranked genie-aided beam – X

Allow the evaluation of Beam Id prediction accuracy as a weighted success rate using the following formulation and weighing factors

Sw=1Nt n=1Nti=1Nwi,n lRSRPbi,n≥RSRPbi,n-X

wi,n=RSRPbi,nRSRPbi,ni=1KRSRPbi,nRSRPbi,n with RSRPs in linear scale

Performance requirements related test cases for AI/ML based beam management

In this meeting it is important that we discuss a) if to introduce test cases for AI/ML based beam management and b) what would be the test scope. In this paper we present our views on the test cases that need to be defined for AI/ML based beam management.

Test case to cover the measurement delay requirements

- In RAN4#115 it was agreed to reuse the parameters M, P, and N from the legacy requirements. However, in our view RAN4 should consider the legacy L1-RSRP measurements with the same values of M, N, and P for measurement delay requirements for BM-Case1. RAN4 should consider the enhancement of measurement period for L1-RSRP measurement for BM-Case2, where the parameter M needs to be redefined.

Agreement RAN4#115:

Case 1: The starting point is to use M, N, P (reference the values from an existing clause in the same version of the spec where the measurement period is defined)

Case 2: postpone discussion until RAN1 concludes

RAN4 should consider defining a test case for the measurement period for L1-RSRP measurement for BM-Case2, where the parameter M needs to be redefined.

- Test case to cover the LCM and performance monitoring related requirements

Agreement from RAN4#115 in general aspects: Activation delay: - Activation delay requirements created for the periodic, semi-persistent and aperiodic CSI reporting for beam prediction - It is FFS for CSI prediction - For Option A, RAN4 will send LS to RAN2 to clarify the related mechanism. - For semi-persistent CSI reporting - Activation delay starts at the reception of the MAC-CE/DCI - For aperiodic CSI reporting - Activation delay starts at the reception of the DCI

The above requirements for AI/ML BM, the LCM operations, e.g., activation, deactivation, and fall-back (e.g., fall-back from AI/ML beam prediction to legacy mode), have already been discussed in the core requirements of this WI. We need the test cases where the Test equipment will verify the RRM core requirements related to the LCM operations of activation, deactivation, and fallback to legacy mode.

RAN4 to consider having a Test case for verifying the core requirements for LCM operation related to activation/switching delay requirements.

RAN4 to consider having Test case for verifying the core requirements for LCM operation related to deactivation delay requirements.

- Test case for verifying the Conditions for known TCI state and TCI state switch

The following is the agreement from RAN4#115 for Rx beam knowledge and TCI state handling.

Agreement from RAN4#115: Issue 2-3: Rx beam knowledge and TCI handling Agreement: - if the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, the corresponding Rx beam is known. - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, depending on UE capability, the corresponding Rx beam may be known or unknown. - Known Rx beam means no additional Rx beam sweeping is needed - Unknown Rx beam means additional Rx beam sweeping is needed. - Rx beam is known mean TCI state is known if conditions below are also met The following conditions related to whether the TCI state is known or unknown is defined as the starting point. Companies are invited to bring analysis. - The target TCI state is known when the following is met: - The TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L-1 RSRP report of the target TCI state (clause 8.10.2 of 38.133) - Applies for case 1 - FFS for case 2 - Whether RS resource is only from set B should be clarified - FFS on whether prediction also impacts if a TCI state is known or unknown - Companies are invited to bring analysis on how prediction would impact whether a TCI state is known or unknown - FFS on the time delay if prediction is used as a condition for a known TCI state

Test mechanisms for downlink TCI state switch delay requirement in AIML BM should be considered. TCI switch delay requirements should be verified accordingly when the TCI switch command is received within 1280ms or >1280ms of UE providing the L1-RSRP report. The TCI switch command could depend on BM case1 (Predicted beam are QCL-D to known Tx beam) or not, or also longer delay requirements according to BM case2 which should be verified by the SS/TE in a test case.

RAN4 to consider having Test case for verifying the core requirements for Conditions for known TCI state and TCI state switch delay requirements.

Testing framework for AI/ML based Beam Management

The discussions on finalizing the testing framework for AI/ML based BM use case continued during the previous few RAN4 meetings and below is the summary of the agreements achieved on the topic.

From RAN4#110 meeting Issue 2-3: Test setup feasibility for FR2 Companies are invited to provide further analysis on what the test setup should enable in terms of test environment From RAN4#113 meeting 2.3.2 Agreements in main session: Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs. From RAN4#114 meeting 2.4 Testing for one sided models Issue 4-1: Baseline test setup Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs Issue 4-2: Channel model Agreement: - Use CDL-based channel model as starting point From RAN4#114-bis meeting Issue 2-6: Test system setup Companies are invited to bring further analysis on the test system setup. At least the following aspects to be considered in the analysis: - Whether the test can/should reflect performance in the field and whether field data should be usable in the test - What channel model should be used - UE prediction model input (e.g. whether any spatial information like UE Rx beam, UE beam pattern, etc is used) - Pros and cons of proposed single AoA and multi AoA setups For the multi AoA test setups, companies are invited to bring further analysis on the following aspects: - Define candidate test setups which are already feasible (e.g. based on eIFF, MPAC etc.) - Propose simplified channels suitable for the defined candidate test setup(s) - Validate the proposed simplified channels through AIML BM system level simulations, especially by comparing the performance with the non-simplified (theoretical) channels. - Assess the implementation of the simplified channels on the candidate systems, after the probe layout and the channel model have been clarified.

It is clear from the above agreements that there is a need for a solution which can test the performance of AI/ML based BM feature in such a way that the existing test systems can be reused, and the solution should be AI/ML BM performance centric. Therefore, we provide our further views on the topic in the following section and propose few candidate solutions along with their corresponding test procedure.

Test Setup: Single AoA or multiple AoAs?

- For a while, RAN4 has been discussing whether the conformance testing setup of AI/ML based BM use case should be based on single AoA or multiple AoAs. Undoubtedly, each of these test setups have their own pros and cons which need to be carefully considered before deciding the test framework. But in our view, the two most important questions which need to be answered before taking this decision are:

- Whether to emulate the CDL (or simplified CDL) channel model’s clusters using the

  - Test probe antennas available inside the testing chamber (i.e., physical emulation), or

  - Within channel emulator (i.e., through software)?

- How to generate the 32 Set A beams during the conformance test? Should UE rotation be allowed or not?

- If the single AoA based test systems are chosen, the only way to emulate the CDL channel model’s clusters is within the channel emulator and then apply the resultant signal, obtained by combining the signal contributions from individual cluster in the channel emulator, to the single available test probe antenna from which it is transmitted towards the DUT. This approach is less practical because of the following reasons:

- The effect of the real propagation environment is “simulated”.

- DUT’s Rx beam pattern is either assumed or needs to be measured.

- DUT receives the signals through “only one” direction.

- The received RSRP at the DUT also depends on the multi-path signal’s combining (and corresponding combining weights) used inside the channel emulator.

- Regarding the emulation of 32 Set A beams during the conformance test using a single AoA based test system, different Set A beams can only be emulated by applying different power offsets to the different RSs transmitted via the same (single) available test probe unless UE rotation is allowed. This is because with fixed test probe and without UE rotation, the UE can receive the signals only from “single” direction which is towards the test probe (or reflector). In case UE rotation is allowed, the number of UE rotations required will be equal to the number of Set A beams to be emulated, i.e., 32, which also increases (in fact maximizes) the test time as compared to that with multi-AoAs based test systems.

- On the other hand, with multi-AoAs based test systems, CDL channel model’s clusters can be emulated within the channel emulator or physically through the test probes inside the testing chamber or via a hybrid approach using the combination of both. This solution is more practical and closer to the real propagation environment as the DUT can receive the multi-path signals through “several” directions. Furthermore, with multiple available test probes and even with no UE rotation allowed, more beams can be emulated relative to single AoA based systems by utilizing the available test probes. Moreover, with UE rotation allowed, the number of UE rotations required to emulate the 32 Set A beams will be significantly less than that required by the single AoA based test systems.

For single AoA based test systems, CDL (or simplified CDL) channel model can “only” be emulated within the channel emulator.

Multi-AoA based test systems are capable of emulating CDL (or simplified CDL) channel model’s clusters either physically using multiple test probes or through the channel emulator or using a combination of both.

Multi-AoA based test systems can emulate more beams having different AoAs than single AoA based systems as well as require relatively a smaller number of UE rotations to emulate a given number of beams.

From the above discussion, multi-AoA based test systems are better suited for the conformance testing of AI/ML based BM use case.

RAN4 to agree on using multi-AoA based test systems for the conformance testing of AI/ML based BM use case.

- In the following sub-sections, we provide three candidate solutions along with the corresponding test procedures for the conformance testing of AI/ML based BM use case. Without loss of generality, we present the solutions considering multi-AoA based test systems but the solutions equally apply to single AoA based test systems unless explicitly stated.

      - Emulation of Set A and/or Set B beams without using UE rotation

- This solution provides a method, applicable only for the multiple AoAs based test systems, to emulate the Set A and/or Set B beams without using the UE rotation. Considering a test chamber equipped with ‘M’ test probe antennas where each test probe antenna is contributing to form a definite Tx antenna pattern and/or Tx beam(s) which is also affected by the amount of Tx power corresponding to the signals applied to each test probe antenna. A particular Tx antenna pattern (containing at least a Tx beam) can be formed by selecting and activating any of the ‘N’ test probes out of the available ‘M’ probes, where 1≤N≤M, i.e.,

- MC1 Tx antenna patterns can be formed if only 1 probe is selected/activated.

- …

- MCM Tx antenna patterns can be formed if ‘M’ probes are selected/activated.

- Hence, a test system with ‘M’ test probe antennas in the test chamber can emulate (MC1 + … +MCM) Tx antenna radiation patterns (or beams). For example, for a test system having 6 test probes, simultaneously activating at least 3 test probe antennas will result in the creation of (20+15+6+1) = 42 Tx antenna patterns which is sufficient to emulate 32 Set A beams during the conformance testing.

RAN4 to discuss and explore the feasibility of generating Set A and Set B beams during the conformance testing of the AI/ML-BM use case by selecting unique combinations of available test probes and decide the transmit power of each selected probe. Tx power of each selected probe is FFS.

This solution may need a special beam codebook for the generation of Set A and Set B beams using unique combinations of test probes during the conformance testing. To ensure that the UE can perform correct inferences using the measurement results of Set B beams generated using this method, a pre-alignment check may be needed before the exact start of the conformance test. The objective of the pre-alignment check is to ensure that the TE provided parameters (e.g., parameters corresponding to the beam codebook) and the DUT’s model/functionality/configuration’s current/preferred parameter (e.g., the parameters using which the training was performed) agree with each other. This is needed to ensure that the test does not fail because of the (AI/ML) parameter(s) mismatch between the DUT (pre)-training and conformance testing. One of the RAN4 agreements (shown below) also highlights the importance of consistent environments during training and testing.

RAN4 to discuss and decide the contents of pre-alignment check which needs to be performed between the TE and DUT before the exact start of the conformance test.

- We next provide a test procedure which utilizes this concept and will call it as “Test Procedure A”.

- Test Procedure A:

- Place the DUT inside the testing chamber in the pre-selected or standardized orientation.

- Perform the pre-alignment check between the TE and the DUT. If the pre-alignment check is successful, TE parameters and DUT training parameters are not mismatched. FFS the content of pre-alignment check. Example: parameters related to beam codebook for the testing.

- If the pre-alignment check is successful, go to Step 4 else end the test and mark it as not feasible/applicable.

- TE selects a unique combination of three or more test probes from the available test probes antennas and decides the Tx power of the corresponding signals applied to the selected test probes to form one Tx antenna pattern comprising one Set A (or Set B) beam.

- DUT measures the RSRP of the received Set A (or Set B) beam and stores it for either ground truth reporting or UE side AI/ML based inference purposes.

- TE selects another unique combination of three or more test probes and decides the Tx power of the corresponding signals applied to the selected test probes to form another Tx pattern containing a different Set A (or Set B) beam.

- DUT measures the RSRP of the received Set A (or Set B) beam and stores it for either ground truth reporting or UE side AI/ML based inference purposes.

- Repeat till all Set A (or Set B) beams are transmitted, and DUT has measured the RSRP of all transmitted beams.

- If Set A beams are transmitted first, DUT reports the ground truth RSRP of the measured Set A beams to the TE.

- TE stores the UE reported Set A measurement results for deriving the test verdict.

- TE transmits each Set B beams to the DUT by forming a unique Tx antenna pattern via selecting a unique combination of three or more test probes selected from the combinations used for transmitting Set A beams.

- DUT measures the RSRP of each Set B beam and perform the AI/ML based inference at the UE side.

- DUT reports the inference results back to the TE (e.g., reporting top-K beams along with their predicted RSRPs).

- TE derives the test verdict based on the UE reported prediction results and UE reported measurement results.

Figure 1: Illustration of AoAs/Probes

      - Emulation of Set A and/or Set B beams with UE rotation

- This solution provides a method, which is applicable for both single AoA and multiple AoAs based test systems, to emulate the Set A and/or Set B beams by using the concept of UE rotation. During the test, it is expected that the beams in Set A and/or Set B should be generated without the UE rotation as the similar scenario will be observed in the real deployments. However, based on the TE vendors’ input, only a limited number of beams with different AoAs (RX-PAS profiles) can be emulated inside the test chamber due to which multiple beams in Set A and/or Set B will arrive at the DUT from the same/similar direction(s). Hence, different possible UE rotations can be used during the test to ensure that different beams arrive the DUT from different directions. One benefit of using the UE rotation is that it enables the test system to emulate the AoA angles which are outside the angular range of test probes (from UE’s point of view) and hence the test system can generate any possible AoA beam angles.

The solution proposes the DUT to perform beam sweeping in multiple iterations and to measure the RSRP of the corresponding beams in Set A and/or Set B during the conformance test for which a new test command is to be defined and to be sent to the DUT. After the DUT receives this test command from the TE and configures itself to perform beam sweeping in multiple iterations, one of the possible UE orientations is selected and TE selects the power level (typically different) for each of the available test probes to form different beams towards the UE. The above solution is valid for emulating all Set A and/or Set B beams in an iterative manner to perform beam sweeping during the conformance testing.

Figure 2: An example of the proposed test system with UE rotation.

Given the Set A and/or Set B beams can be generated iteratively during the conformance test as proposed above, DUT can measure the L1-RSRP of each beam in Set B and provides it as input to its AI/ML model for Set A beam prediction.

RAN4 to discuss the feasibility of performing beam sweeping using UE rotation in multiple iterations during the conformance testing of AI/ML based BM use case.

- We next provide a test procedure which utilizes this concept and will call it as “Test Procedure B”.

- Test Procedure B:

- Place the DUT inside the testing chamber in the pre-selected or standardized orientation.

- TE sends the test command to the DUT to configure itself and perform beam sweeping in multiple iterations.

- DUT configures itself to perform beam sweeping in multiple iterations and sends an acknowledgement to the TE for the same.

- TE transmits one of the Set A (or Set B) beams to the DUT using the available test probes in the testing chamber. This means that for single AoA based test systems, the only available test probe will be used to transmit the required beam towards the DUT whereas for multi-AoA based test systems, either one or more available test probes will be selected based on the agreement on channel model to be used during the conformance test.

- DUT measures the RSRP of the received Set A (or Set B) beam and stores it for either ground truth reporting or UE side AI/ML based inference purposes.

- Rotate the UE to a different orientation. Allow the UE to settle down.

- Transmit next Set A (or Set B) beams to the DUT.

- DUT measures the RSRP of the received Set A (or Set B) beam and stores it for either ground truth reporting or UE side AI/ML based inference purposes.

- Repeat Steps 6-8 till all Set A (or Set B) beams are transmitted, and DUT has measured the RSRP of all transmitted beams.

- If Set A beams are transmitted first, DUT reports the ground truth RSRP of the measured Set A beams to the TE.

- TE stores the UE reported Set A measurement results for deriving the test verdict.

- TE transmits each Set B beams to the DUT using a subset of the UE orientations previously used to transmit Set A beams.

- DUT measures the RSRP of each Set B beam and performs the AI/ML based inference at the UE side.

- DUT reports the inference results back to the TE (e.g., reporting top-K beams along with their predicted RSRPs).

- TE derives the test verdict based on the UE reported prediction results and UE reported measurement results.

      - Conformance testing without transmitting Set A beams during the test

- This solution provides a method to perform the conformance testing of AI/ML BM use-case without transmitting Set A beams during the test which will result in simplified test systems and smaller testing time. The details of the proposed method are provided below.

- It is a common understanding among companies participating in RAN4 that the TE requires the ground-truth to compare the DUT’s AI/ML based predictions with the corresponding ground truth to decide whether the DUT has passed or failed the test/test iteration. As the ground truth depends on DUT’s architecture and implementation, TE first needs to transmit each Set A beam during the conformance test after which DUT will measure and report the ground truth (e.g., L1-RSRP) measurement corresponding to each transmitted Set A beam back to the TE. However, the emulation of Set A beams during the AI/ML based BM conformance test results in several issues:

- As the number of Set A beams to be emulated during the conformance test is 32, many test probes are required which are neither supported by the existing test systems nor preferred by the TE vendors due to the increased cost and complexity associated with the enhancement of existing test system.

- As the DUT itself is reporting the ground truth back to the TE, it is possible that DUT either manipulates the reported ground truth values or the reported prediction values to pass the test. This will raise concern about UE cheating during the test.

- If using DUT rotations during the conformance test for measuring the ground truth of Set A beams, multiple DUT rotations will be required resulting in a large testing time.

- So, an important question to ask is: Can ground truth be generated at the TE without transmitting Set A beams? If yes, how? In the following, we present a solution for the above problem.

The proposed solution exploits the fact that if the characteristics of the OTA test chamber (e.g., path loss from each test probe antenna to the DUT) and the UE Rx antenna pattern and/or Rx beamforming gain towards the direction of test probe antennas can be obtained either prior to the test or during the starting phase of the test, UE’s ground truth for the Set A beams can be generated by using the obtained UE Rx antenna pattern and/or Rx beamforming gain knowledge and OTA characteristics knowledge at the TE, and without actually transmitting the Set A beams during the conformance testing. The derivation provided below assumes the case where only one test probe antenna is used to form a Tx beam towards the UE but can easily be extended for multiple active test probe antenna case.

Let xi be the ith test signal (having unit energy) whose transmit power is ρi Watts, and the channel implemented by the channel emulator to transmit ρixi be a scalar hi due to single activated probe antenna. Hence, the baseband equivalent of the signal transmitted by the test probe antenna#‘i’ can be written as ρihixi which then propagates through the over the air wireless channel of the testing chamber and is received by the DUT’s antenna panel(s). The baseband equivalent of the signal received at the DUT is given by

yi=wihOTA,iρihixi+ni

where wi and hOTA,i denote the impact of UE Rx (antenna pattern and/or Rx beamforming gain)  and the properties (e.g., pathloss) of the testing chamber’s wireless channel in the direction of test probe#‘i’, and ni represents the additive noise added by the UE hardware. Hence, the RSRP of the received signal yi can be approximated as

RSRPTSi= Eyi2= EwihOTA,iρihixi+ni2≈wihOTA,i2×ρihi2xi2+σ2

where E[.] denotes the expectation operation and is performed by taking the linear average over the power distribution (in [W]) over all resource elements that carry the RSs configured by the network for RSRP measurements, and σ2 denotes the average noise power or noise variance. In the above expression for RSRPTSi, the only unknowns parameters for TE are wi and hOTA,i as TE knows the test parameters it has selected, i.e., ρi, hi, xi for transmitting the test signal#‘i’, and noise power/variance can be estimated by the DUT using the already known techniques. Therefore, DUT’s ground truth can be generated by the TE if the information wihOTA,i2 is available at the TE for all possible/relevant test directions ‘i’. One possible way of obtaining this information at the TE is to take the ratio of RSRPTSi and TPTSi where TPTSi is the transmit power of the test signal#‘i’ transmitted by the test probe antenna ‘i’ and can be written as TPTSi= ρihi2xi2. Therefore, the ratio is given by

γi≝RSRPTSiTPTSi=wihOTA,i2×ρihi2xi2+σ2ρihi2xi2= wihOTA,i2+σ2ρihi2xi2≈wihOTA,i2

as the term σ2ρihi2xi2 will be very small due to high ρi and small σ2.

If the test probe antennas have a directional gain, it can be assumed that the joint impact of test probe antenna and characteristics of the OTA testing chamber for test probe antenna#‘i’ is captured in hOTA,i. The DUT will be configured to also report the information of Rx beam (e.g., Rx beam ID) it has used to calculate the RSRPTSi which is required to generate the ground truth at the TE without transmitting Set A beams during the test.

Generation of UE side ground truth at the TE for Set A beams using the ratio γi

The TE uses the obtained wihOTA,i2 to generate the UE side ground truth as follows:

For the Set A Tx beam corresponding to the RS id#‘m’ having a transmit power of ρm, the corresponding test signal (RS) is being mapped to test probe antenna#‘j’, where j = (‘m’ modulo ‘t’), t is the max. number of test probe antennas in the test chamber to be used for the conformance test. If the channel model to be used is hj, the TE already knows the test probe information, hj and ρm. Therefore, the UE side ground truth for this Set A beam can be generated by the TE without actually transmitting this beam as:

RSRPTSm,j≈ wjhOTA,j2×ρmhj2xm2+σ2

In this way, the ground truth for all Set A beams can be generated at the TE without transmitting them.

Configuration for measuring Set B beams for consistent prediction of top-K Set A beams

As the ground truth was generated by the TE, it needs to configure the DUT for measuring Set B beams for the consistent prediction of top-K Set A beams based on the ground truth generated by the TE. For this, TE needs to indicate the DUT that when a Set B beam is transmitted by the test probe antenna#‘l’, the DUT must use the antenna pattern and/or UE Rx beamforming gain which results in wlhOTA,l2 being the overall impact of OTA testing chamber and UE hardware for the corresponding Set B beam.

RAN4 to discuss the possibility of testing AI/ML BM use-case considering (1) No transmission of Set A beams during the test, (2) exploit and reuse the characteristics of the testing environment between the test probes and UE baseband during the test.

Channel model: Use CDL-based channel model as starting point

Issue 4-2: Channel model

Agreement:

- Use CDL-based channel model as starting point

Based on the off-line discussion and Tdoc from different companies, the simplification of current CDL model is considered to be used in the OTA test. In this chapter, we will present our view on simplification of CDL model.

The current 3GPP CDL models are omni-directional channel model as shown in the picture below, which means the CDL model contain all the clusters in 360 degrees in the azimuth plane, gNB is in the center. It is measured based on using omni-directional antenna during channel measurement. However, for beam management, all the beams are generated by antenna array, and every beam only point to one direction, they can only be interactive with part of the channel, i.e., one beam can only interactive with some of the clusters in the current CDL model. This is fundamentally different from using omni-directional antenna.

Figure 3 Clusters of omni-directional channel vs. clusters of one directional channel (FR2)

Therefore, we can see a big gap between using CDL model directly in beam management even before considering simplification of CDL model. We need to check what is the directional channel based on beam patterns, and how those directional channel looks like first.

Current 3GPP CDL models are omni-directional channel model contain all the clusters in 360 degrees in the azimuth plane. But for beam management, all the beams are generated by antenna array, and every beam only point to one direction, one beam is not possible to interactive with all the clusters in the CDL model.

We need to check what is the directional channel based on beam patterns, and how many clusters those directional channels interact with in general. The clusters numbers will be greatly reduced compared with the number of clusters in current CDL models.

In order to acquire the directional channel model (CDL model), then use the spatial filter corresponding to the beam width of SSB to filter out the clusters of omni-directional CDL channel models. The spatial filter is defined in 38.901 Chapter 7.7.4 Spatial filter for generating TDL channel model to generate TDL model. We can simply modify it without removing the angular information of each cluster, so we can have CDL model based on the beam width of the SSB/CSI-RS beam.

We can use the spatial filter corresponding to the beam width of SSB to filter out the clusters of omni-directional CDL channel models (current CDL models). The spatial filter is defined in 38.901 Chapter 7.7.4 Spatial filter for generating TDL channel model to generate TDL model, so, we need to modify the procedure to keep the angular information of each cluster.

One the other hand, we also see the issue to simplify the CDL model:

- Simplify the CDL model from 24 clusters to 4 clusters (based on off-line discussion) will cause issue that, the 4 clusters CDL model can be totally a different channel scenario we do not know.

- Remove some of the clusters from 24 clusters will change the properties of the channel, so, which cluster can be removed need to be carefully considered.

On the other hand, reduce the number of clusters in the current CDL model will change the properties of the channel. Reducing the 24 clusters to 4 clusters in a CDL model means creating a different channel scenario we do not know.

So, the alternative suggestions on the table:

- Use ray tracing or other simulation to generate 4 clusters CDL model directly for directional channels based on SSB. This is a known scenario with full channel properties.

- Use spatial filter to find out how many clusters in the CDL model should be used for each of SSB, before doing simplification of CDL model.

- Use hybrid way to do simulate the CDL model + SSB + probe (Power Angular Spectrum). Emulate full CDL in a way where total number of CDL model’s clusters are divided into ‘P’ sub-groups where ‘P’ is the number of available test probes in the testing chamber, it also means the number of clusters one SSB interact with. Clusters within each subgroup are emulated in the channel emulator and the combined output of each group is applied to the corresponding test probe.

Use spatial filter to find out how many clusters in the directional model for SSB before doing simplification of CDL model.

Bear in mind that removing the clusters of a CDL model for simplification will change the channel properties. Or we can use the alternatives that use ray tracing or other simulation to simulate directional channels with 4 clusters directly.

If the number of clusters interact with each SSB/CSI-RS is less than 4 (or the number of clusters the probes of TE can simulate at the same time), we may only need to divide 24 clusters into “p” sub-groups and each sub-group match the clusters one SSB/CSI-RS interacts with.

Generalization Aspects

The recent RAN4 agreements on the generalization aspects are provided below.

Agreement from RAN4#114-bis: Relation between generalization testing and additional conditions signalling Agreement: - There shall be consistency between applicable conditions signalling and testing - No test will be defined that implies a change of network condition or associated ID during the test. Non-static scenarios Agreement: - Static scenarios are assumed by default. Non-static scenarios are introduced on a use-case specific basis if needed for testing. - FFS whether a mixed datset can be created for testing generalization, and whether such a mixed dataset would be a static or non-static scenario Agreement from RAN4#115: The high level and general guidelines for the generalization test are provided as follow. The exact decisions are to be made case by case. - Study and , if feasible, define requirements for each AI/ML functionality - Define one test per UE capability as a minimum - FFS >1 test per UE capability - Define a minimum set of test configurations (including NW sided conditions if any) as mandatory for testing of AI/ML-enabled Feature Potential areas to consider for generalization testing: - gNB array parameters - port layouts, array size, antenna virtualization - propagation conditions - Deployment scenarios - Carrier frequencies - Speeds - Indoor/outdoor - Bandwidths - SNR

The above agreements show that the default assumption is to use static scenarios during the test and non-static scenarios can be used on need basis. To test the generalization, RAN4 may define the test having multiple sub-tests with each with a different static condition, i.e., TE configures the same specified UE configuration for AI/ML BM in each of the test where the same UE configuration is tested under different scenarios (in different subtests) to verify the UE model’s capability to generalize.

In order to address the testability issue of generalization aspects of AI/ML BM use case, we propose a new metric to indicate the generalization capabilities of an AI/ML functionality, which may be referred as “generalization score”. This may be used to verify/check whether the AI/ML BM functionality generalize well or not. As an example, if there are 3 scenarios of interest, any one scenario can be the baseline setup and the remaining 2 scenarios will then be used as other set ups to test the generalization aspects. The generalization score can be calculated as a function of individual scenario verdicts, e.g., as simple average or weighted average of the individual generalization verdict of each Scenario ‘j’ (i.e., Scenario ‘1’ to Scenario ‘N), where the verdict of Scenario ‘j’ = 1 if the functionality generalizes to Scenario ‘j’ else it is equal to 0.

Overall Generalization Score= (VerdictScenario1+…+VerdictScenarioN)N×100%

RAN4 to consider designing a new metric to verify the generalization performance of the AI/ML BM functionality in different (static) scenarios.

In the tables below, we attempt to capture different parameters that may be needed to define different generalization scenarios for AI/ML enabled BM use case based on set of parameters/configurations to both DUT/TE. For identified scenarios in AI/ML enabled BM-Case1 and BM-Case2, the minimum level of performance may need to be defined. Similarly acceptable tolerance margin for other scenarios may also need to be agreed, potentially based on simulation results.

Table 1: Example for Parameters for Generalization Scenarios

| Parameters | Description |
| --- | --- |
| gNB Parameters: |  |
| - BS antenna configurations | Case1: One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ Case2: One panel: (M, N, P, Mg, Ng) = (8, 16, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| Deployment Scenario: |  |
| - Indoor / Outdoor | Case1:100% Outdoor Case2:80% Indoor, 20% Outdoor |
| - UE Speed | Case1: 3 Km/h (only for BMCase1) Case2: 30 Km/h (only for BMCase2) Case3: 60 Km/h (only for BMCase2) Case4: 120 Km/h (only for BMCase2) |

For the verification/testing of generalization related aspects in RAN4 for AI/ML BM, RAN4 should define the generalization test which consists of multiple subtests each associated with the testing of different scenarios based on parameters listed in Table 1.

Performance requirements for AI/ML Positioning

Requirements for Case 3a

      - Report mapping for ToA measurement

RAN1#121 discussed the reporting of the ToA measurement for Case 3a and concluded the following.

Conclusion For measurement report of AI/ML assisted positioning Case 3a, when timing information is reported from gNB to LMF, - From RAN1 perspective, LMF shall be able to distinguish whether the timing information is pre Rel-19 legacy timing measurement or Rel-19 Case 3a timing measurement. - It is up to RAN3 to decide how to ensure that LMF can distinguish between the two types of timing measurement.

Thus, RAN3 is in charge to support the distinction between pre Rel-19 legacy ToA measurement and Rel-19 AI/ML based ToA measurement, so that LMF is aware of it. From RAN4 point of view, the same report mapping can be used for path-based reporting, no matter whether gNB reports the legacy pre Rel-19 or the AIML based Rel-19 measurement.

- Reuse report mapping tables in TS 38.133 clause 13.1.1 for UL RTOA first path reporting and report mapping tables in TS 38.133 clause 13.1.1A for UL RTOA additional path reporting in Case 3a.

      - Report mapping for power measurement

As for the ToA measurement above, from RAN4 point of view, the same report mapping can be used for path-based reporting for the power measurement, i.e. SRS-RSRP or SRS-RSRPP, no matter whether gNB reports the legacy pre Rel-19 or the AIML based Rel-19 measurement.

- Reuse report mapping table in TS 38.133 clause 13.3.1 for UL SRS RSRP reporting and report mapping table in TS 38.133 clause 13.6.1 for UL SRS RSRPP reporting in Case 3a.

      - Other reported quantities

RAN1#121 agreed the following on mandatory/optional inference output reporting for Case 3a.

Agreement For Case 3a, the FFS in RAN1#118bis agreement is resolved by updating the agreement as follows. Agreement (RAN1#118bis) From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. - FFS: LOS/NLOS indicator. - (Optional) LOS/NLOS indicator with legacy format Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3. Note: It is up to RAN3 to decide whether the field of LOS/NLOS indicator should be removed or kept..

In our view, no RAN4 requirements for inference need to be defined for this LOS/NLOS indicator agreed by RAN1 for Case 3a.

- No RAN4 requirements for inference need to be defined for the LOS/NLOS indicator agreed by RAN1 for Case 3a.

Requirements for Case 3b

The following agreements were reached for Case 3b at RAN4#115 [3].

Issue 3-2: Report mapping for sample based timing measurement reporting Agreement: - Use the value of the legacy UL RTOA mapping for 1st sample - FFS for the additional sample reporting Issue 3-3: Requirements for case 3b Agreement: - Reuse legacy path based Rx-Tx accuracy requirements for case 3b - Requirement applies only for the 1st path

      - Measurement accuracy requirements

RAN1#121 has agreed the definitions for sample-based measurements for Case 3b, i.e. UL SRS-TDCT and UL SRS TDCP. The following is a replication of TS 38.215 V19.0.0:

### 5.2.9 UL SRS time domain channel timing (UL SRS-TDCT) Definition; UL SRS time domain channel timing (UL SRS-TDCT) is defined as Nt values of timing information of the estimated channel response in time domain, Tn, corresponding to the resource elements carrying sounding reference signals (SRS), the Nt values are derived from Nt consecutive time instances with timing granularity 2k∙Tc that start from the first detected path rounded down with timing granularity 2k∙Tc, where Nt and Nt are given by higher layers and Tc is given in Clause 4.1 of [3, TS38.211], and k is the reporting granularity factor given by higher layer parameter timingReportingGranularityFactor. The Nt values of timing information are selected such that they correspond to the time instances with the highest power values among the Nt consecutive time instances where a power value for a time instance is determined as the power of linear average of the estimated channel response at time instance Tn. For frequency range 1 and 2, if receiver diversity is in use by the gNB for power of linear average of the estimated channel response measurement: - The measured received signal power, P1, value for the selected first time instance, T1, shall not be lower than the corresponding received signal power for the selected time instance, T1, of any of the individual receiver branches and the received signal power, Pn, shall be with the same receiver branch(es) as applied for the selected first time instance, T1. The timing information are reported relative to reference time, Tref. The reference time, Tref, is equal to the UL RTOA reference time T0+tSRS, where - T0 is the nominal beginning time of SFN 0 provided by SFN Initialization Time [15, TS 38.455] - tSRS=10nf+nsf×10-3, where nf and nsf are the system frame number and the subframe number of the SRS, respectively. The reference point for UL SRS-TDCT shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector.. ### 5.2.10 UL SRS time domain channel power (UL SRS-TDCP) Definition; UL SRS time domain channel power (UL SRS-TDCP) is defined as a number of Nt values of power of the estimated channel response in time domain, Pn, where, for a given n, Pn is defined as the power of the linear average of the estimated channel response at time instance Tn, for n = 1, …, Nt, where time instance Tnis defined in Clause 5.2.9, from resource elements carrying sounding reference signals (SRS).. UL SRS-TDCP measurements may not be reported without reporting of UL SRS-TDCT measurements. The reference point for UL SRS-TDCP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-TDCP measurements: - The reported received signal power, P1, value for the selected first time instance, T1, shall not be lower than the corresponding received signal power for the selected time instance, T1, of any of the individual receiver branches and the received signal power, Pn, shall be provided for the same receiver branch(es) as applied for the selected first time instance, T1..

RAN4 already agreed to reuse legacy accuracy requirements for gNB Rx-Tx time difference for the UL SRS-TDCT measurements for the first path. It is noted that gNB Rx-Tx time difference accuracy was specified in Rel-16 based on N=4 measurement samples. For reduced number of measurement samples, introduced in Rel-17, no accuracy requirements were defined. Thus, accuracy requirements for UL SRS-TDCT apply for the same number of measurement samples N=4.

The same is valid for UL SRS-TDCP. Rel-16 measurement accuracy requirements for UL SRS RSRP were based on N=4 measurement samples.

Measurement accuracy requirements for gNB Rx-Tx time difference apply for UL SRS-TDCT first path under the condition that the reported measurement is based on N=4 samples.

- Measurement accuracy requirements for UL SRS RSRP apply for UL SRS-TDCP under the condition that the reported measurement is based on N=4 samples.

Report mapping for ToA measurement

- For report mapping for TOA measurement, i.e. UL SRS-TDCT, in Case 3b, it was agreed at RAN4#115 to reuse the UL RTOA mapping for the first sample. Based on the former RAN1#120 agreement, i.e.

Agreement For Rel-19 AI/ML based positioning, for Case 3b, “FFS: k” in RAN1#119 agreement is resolved by supporting: - k = {0...5}

Hence, RAN4 is to reuse the report mapping tables for UL RTOA in TS 38.133 clause 13.1.1 for k= 0 to 5.

RAN1#120 also agreed the number of reported samples.

Agreement For Rel-19 AI/ML based positioning, for Case 3b, “FFS: Nt´ values” in RAN1#119 agreement is resolved by supporting: - Nt´ = {8, 16, 24}

- The report mapping for the additional (Nt-1) samples was left open at RAN4#115.

First, we analyse if the reporting range for UL RTOA additional path in TS 38.133 clause 13.1.1A can be reused (hereafter only shown for k=0).

### 13.1.1A Additional Path Report Mapping for UL-RTOA The reporting range of additional path reporting for UL Relative Time of Arrival (UL-RTOA), as defined in Clause 5.2.2 of TS 38.215 [4], is defined from -8175´Tc to +8175´Tc. The reporting resolution is uniform across the reporting range and is defined as T = Tc´2k where k is selected by gNB from the set {-6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5}. Tc is defined in TS 38.211 [6]. LMF provides a recommended resolution parameter, timingReportingGranularityFactor [35]. gNB selects parameter k based on timingReportingGranularityFactor [35] and informs the LMF. The mapping of measured quantity for each reporting resolution (k) is defined in Table 13.1.1A-1 to Table 13.1.1A-12. Table 13.1.1A-1: Measurement report mapping for k=0 Reported Quantity Value, path_i; Measured Quantity Value, Dpath; Unit. path_00000; Dpath < -8175; Tc. path_00001; -8175 £ Dpath < -8174; Tc. path_00002; -8174 £ Dpath < -8173; Tc. ¼; ¼; …. path_08175; -1 £ Dpath < 0; Tc. path_08176; 0 £ Dpath < 1; Tc. …; …; …. path_ 16349; 8173 £ Dpath < 8174; Tc. path_ 16350; 8174 £ Dpath < 8175; Tc. path_ 16351; 8175 £ Dpath; Tc.

- For legacy NR positioning, it is observed that the reporting range for the additional path for UL RTOA is symmetric around the path used for the UL RTOA measurement and corresponds to a time range of [-4.158 us; 4.158 us]. This means an additional path can be reported which is either prior or after the reported main reception path used for the UL RTOA measurement (presumably with highest path related power).

- Thereagainst, sample-based reporting is based on Nt samples according to TS 38.215 (see above) and the first sample is reported as UL RTOA measurement, subsequent samples in time are reported as additional samples, hence the path difference (= sample difference) is always positive. Assuming that signalling bandwidth on NRPPa is not critical, we may reuse the single-sided range for additional path reporting for UL RTOA. For k= 0, this results in following mapping table for additional samples for sample-based reporting.

Table 13.1.1X-1: Measurement report mapping for k=0

| Reported Quantity Value, sample_i | Measured Quantity Value, Dsample | Unit |
| --- | --- | --- |
| sample_0000 | 0 £ Dsample < 1 | Tc |
| sample_0001 | 1 £ Dsample < 2 | Tc |
| sample_0002 | 2 £ Dsample < 3 | Tc |
| … | … | … |
| sample_8173 | 8173 £ Dsample < 8174 | Tc |
| sample_8174 | 8174 £ Dsample < 8175 | Tc |
| sample_8175 | 8175 £ Dsample | Tc |

- Dsample refers to the time difference relative to the previous reported sample. This means, for the second sample, Dsample refers to the delta in time to the first sample, for the third sample, Dsample refers to the delta in time to the second sample, and so on.

The mapping should be agnostic to the number of Nt  values configured by LMF, as only few bits can be saved.

For instance, for Nt  = 8 and k= 0, 7 additional samples have to be reported. This results in 7 * 13 bits = 91 bits.

For Nt  = 16 and k= 0, 15 additional samples have to be reported. This results in 15 * 13 bits = 195 bits.

For Nt  = 24 and k= 0, 23 additional samples have to be reported. This results in 23 * 13 bits = 299 bits.

If the reported range is halved, we only save 7, 15 or 23 bits, respectively, for the above cases.

- Add new report mapping tables for additional samples for UL RTOA by reusing single-sided range of existing additional path reporting for UL RTOA and keeping the reporting range agnostic to the number of Nt configured by LMF.

- Add following tables to the new clause 13.1.1X of TS 38.133 for additional sample reporting for sample-based reporting:

Table 13.1.1X-1: Measurement report mapping for k=0

| Reported Quantity Value, sample_i | Measured Quantity Value, Dsample | Unit |
| --- | --- | --- |
| sample_0000 | 0 £ Dsample < 1 | Tc |
| sample_0001 | 1 £ Dsample < 2 | Tc |
| sample_0002 | 2 £ Dsample < 3 | Tc |
| … | … | … |
| sample_8173 | 8173 £ Dsample < 8174 | Tc |
| sample_8174 | 8174 £ Dsample < 8175 | Tc |
| sample_8175 | 8175 £ Dsample | Tc |

Table 13.1.1X-2: Measurement report mapping for k=1

| Reported Quantity Value, sample_i | Measured Quantity Value, Dsample | Unit |
| --- | --- | --- |
| sample_0000 | 0 £ Dsample < 2 | Tc |
| sample_0001 | 2 £ Dsample < 4 | Tc |
| sample_0002 | 4 £ Dsample < 6 | Tc |
| … | … | … |
| sample_4086 | 8172 £ Dsample < 8174 | Tc |
| sample_4087 | 8174 £ Dsample | Tc |

Table 13.1.1X-3: Measurement report mapping for k=2

| Reported Quantity Value, sample_i | Measured Quantity Value, Dsample | Unit |
| --- | --- | --- |
| sample_0000 | 0 £ Dsample < 4 | Tc |
| sample_0001 | 4 £ Dsample < 8 | Tc |
| sample_0002 | 8 £ Dsample < 12 | Tc |
| … | … | … |
| sample_2042 | 8172 £ Dsample < 8174 | Tc |
| sample_2043 | 8174 £ Dsample | Tc |

Table 13.1.1X-4: Measurement report mapping for k=3

| Reported Quantity Value, sample_i | Measured Quantity Value, Dsample | Unit |
| --- | --- | --- |
| sample_0000 | 0 £ Dsample < 8 | Tc |
| sample_0001 | 8 £ Dsample < 16 | Tc |
| sample_0002 | 16 £ Dsample < 24 | Tc |
| … | … | … |
| sample_1019 | 8152 £ Dsample < 8160 | Tc |
| sample_1020 | 8160 £ Dsample < 8168 | Tc |
| sample_1021 | 8168 £ Dsample | Tc |

Table 13.1.1X-5: Measurement report mapping for k=4

| Reported Quantity Value, sample_i | Measured Quantity Value, Dsample | Unit |
| --- | --- | --- |
| sample_000 | 0 £ Dsample < 16 | Tc |
| sample_001 | 16 £ Dsample < 32 | Tc |
| sample_002 | 32 £ Dsample < 48 | Tc |
| … | … | … |
| sample_508 | 8128 £ Dsample < 8144 | Tc |
| sample_509 | 8144 £ Dsample < 8160 | Tc |
| sample_510 | 8160 £ Dsample | Tc |

Table 13.1.1X-6: Measurement report mapping for k=5

| Reported Quantity Value, sample_i | Measured Quantity Value, Dsample | Unit |
| --- | --- | --- |
| sample_000 | 0 £ Dsample < 32 | Tc |
| sample_001 | 32 £ Dsample < 64 | Tc |
| sample_002 | 64 £ Dsample < 96 | Tc |
| … | … | … |
| sample_253 | 8098 £ Dsample < 8128 | Tc |
| sample_254 | 8128 £ Dsample < 8160 | Tc |
| sample_255 | 8160 £ Dsample | Tc |

      - Report mapping for power measurement

For report mapping for power measurement, i.e. UL SRS-TDCP, in Case 3b, it was agreed at RAN1#121 to reuse the report mapping for UL SRS RSRPP.

Agreement For AI/ML positioning Case 3b, for gNB channel measurements reported to LMF, regarding the power information (if included), - reuse the existing measurement report mapping table for SRS-RSRPP in 38.133.

The report mapping in TS 38.133 clause 13.3.1 is shown below.

### 13.3.1 Report mapping The reporting range of UL SRS RSRP, as defined in clause 5.2.5 of 38.215 [4], is defined from -156 dBm to -31 dBm with resolution 1 dB. The mapping of measured quantity is defined in table 13.3.1-1. The range in the signalling may be larger than the guaranteed accuracy range. Table 13.3.1-1: UL SRS RSRP report mapping Reported value; Measured quantity value; Unit. SRS_RSRP_0; SRS-RSRP<-156; dBm. SRS_RSRP_1; -156≤SRS-RSRP<-155; dBm. SRS_RSRP_2; -155≤SRS-RSRP<-154; dBm. SRS_RSRP_3; -154≤SRS-RSRP<-153; dBm. SRS_RSRP_4; -153≤SRS-RSRP<-152; dBm. SRS_RSRP_5; -152≤SRS-RSRP<-151; dBm. SRS_RSRP_6; -151≤SRS-RSRP<-150; dBm. SRS_RSRP_7; -150≤SRS-RSRP<-149; dBm. SRS_RSRP_8; -149≤SRS-RSRP<-148; dBm. SRS_RSRP_9; -148≤SRS-RSRP<-147; dBm. SRS_RSRP_10; -147≤SRS-RSRP<-146; dBm. SRS_RSRP_11; -146≤SRS-RSRP<-145; dBm. SRS_RSRP_12; -145≤SRS-RSRP<-144; dBm. SRS_RSRP_13; -144≤SRS-RSRP<-143; dBm. SRS_RSRP_14; -143≤SRS-RSRP<-142; dBm. SRS_RSRP_15; -142≤SRS-RSRP<-141; dBm. SRS_RSRP_16; -141≤SRS-RSRP<-140; dBm. SRS_RSRP_17; -140≤SRS-RSRP<-139; dBm. SRS_RSRP_18; -139≤SRS-RSRP<-138; dBm. …; …; …. SRS_RSRP_111; -46≤SRS-RSRP<-45; dBm. SRS_RSRP_112; -45≤SRS-RSRP<-44; dBm. SRS_RSRP_113; -44≤SRS-RSRP<-43; dBm. SRS_RSRP_114; -43≤SRS-RSRP<-42; dBm. SRS_RSRP_115; -42≤SRS-RSRP<-41; dBm. SRS_RSRP_116; -41≤SRS-RSRP<-40; dBm. SRS_RSRP_117; -40≤SRS-RSRP<-39; dBm. SRS_RSRP_118; -39≤SRS-RSRP<-38; dBm. SRS_RSRP_119; -38≤SRS-RSRP<-37; dBm. SRS_RSRP_120; -37≤SRS-RSRP<-36; dBm. SRS_RSRP_121; -36≤SRS-RSRP<-35; dBm. SRS_RSRP_122; -35≤SRS-RSRP<-34; dBm. SRS_RSRP_123; -34≤SRS-RSRP<-33; dBm. SRS_RSRP_124; -33≤SRS-RSRP<-32; dBm. SRS_RSRP_125; -32≤SRS-RSRP<-31; dBm. SRS_RSRP_126; -31≤SRS-RSRP; dBm.

This yields to the following proposal.

- Reuse the UL SRS RSRPP report mapping table in TS 38.133 clause 13.3.1 for the Nt reported samples for UL SRS TDCP for sample-based reporting.

      - Requirements on ground truth accuracy in data collection

In RAN1#116-bis meeting, training data collection of AI/ML based positioning has been discussed to identify the collect ed data samples as follows.

Agreement(RAN1#116bis) For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Agreement(RAN1#120) - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications. RAN1 send an LS to RAN3 and RAN2 to inform the above. Agreement(RAN1#120bis) For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes timing information, support the following for providing label and quality indicator of label, - The corresponding label is provided by LMF to gNB in the format of timing information. “Timing Measurement Quality” in 38.455 can serve as quality indicator of the label. Note: It is assumed that user data privacy of non-PRU UE is preserved. Conclusion For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes the LOS/NLOS indicator, - The corresponding label is provided from LMF to the gNB by using the existing IE LOS-NLOS-Indicator-r17. There is no need of a separate field for quality indicator of LOS/NLOS indicator.

Also, an agreement is made in RAN1#120 on the label and quality indicator of label.

Agreement For AI/ML based positioning Case 1, from RAN1 perspective, when the label data of location is generated by LMF and transferred from LMF to UE, label and quality indicator of label can be provided by reusing existing IEs. · From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The location estimate uncertainty and confidence (if included with the geographic shapes) can serve as quality indicator of the label.

In Part B, data collection with ground-truth dataset is essential for training. In particular, the training data needs to include both the ground-truth UE location and measurements associated with the UE’s location. To enhance the accuracy of AIML positioning, it is important to have accurate ground-truth information for positioning labelling. When collecting ground-truth information from the UE side, a high accuracy positioning information needs to be acquired by the UE. If UE is unable to acquire a high accuracy ground-truth information, the network/testing equipment may assist UE to acquire a high accuracy ground-truth positioning label. For instance, UE may obtain positioning labels from multiple sources e.g., non-NR and/or NR RAT-dependent positioning methods. Therefore, the degree of accuracy on ground-truth positioning label can be defined as a quality indicator. As captured in RAN1’s agreement, if the existing IE is used with a geographic shape in TS 23.032, the uncertainty/confidence on location estimate can be used as a quality indicator of the label. In this case, the legacy positioning does not specify a requirement on the quality indicator. Also, the ground-truth accuracy can be ensured by the definition on labels. Hence, a requirement on training data collection is not required in Case 3a/3b.

RAN4 to not specify a requirement on training data collection of Part B in Case 3a/3b.

Conclusion

In this paper we share our views on RRM performance requirements for AI/ML enabled beam management and positioning use cases. Specifically, we cover following aspects:

- Performance requirements for AI/ML BM

- Testing framework for AI/ML based Beam Management

- Performance requirements for AI/ML Positioning

The following Proposals were made for AI/ML BM use case:

- Based on simulation results, RAN4 to define values of  K,  N,  X and the success rate of beam id prediction for performance requirement of BM-case1 and BM-case2

RAN4 to discuss the adaptation of Option 3 with the following changes:

- At least N strongest beams among the top-K predicted beams must be considered in the success rate evaluation, where 2≤N≤K

  - RAN4 should consider N=2 if there is no consensus in RAN4 for N>2.

- RSRP of each of the strongest N predicted beams must be compared with the RSRP of its equal ranked genie-aided beam – X

Allow the evaluation of Beam Id prediction accuracy as a weighted success rate using the following formulation and weighing factors

Sw=1Nt n=1Nti=1Nwi,n lRSRPbi,n≥RSRPbi,n-X

wi,n=RSRPbi,nRSRPbi,ni=1KRSRPbi,nRSRPbi,n with RSRPs in linear scale

RAN4 should consider defining a test case for the measurement period for L1-RSRP measurement for BM-Case2, where the parameter M needs to be redefined.

RAN4 to consider having a Test case for verifying the core requirements for LCM operation related to activation/switching delay requirements.

RAN4 to consider having Test case for verifying the core requirements for LCM operation related to deactivation delay requirements.

RAN4 to consider having Test case for verifying the core requirements for Conditions for known TCI state and TCI state switch delay requirements.

RAN4 to agree on using multi-AoA based test systems for the conformance testing of AI/ML based BM use case.

RAN4 to discuss and explore the feasibility of generating Set A and Set B beams during the conformance testing of the AI/ML-BM use case by selecting unique combinations of available test probes and decide the transmit power of each selected probe. Tx power of each selected probe is FFS.

RAN4 to discuss and decide the contents of pre-alignment check which needs to be performed between the TE and DUT before the exact start of the conformance test.

RAN4 to discuss the feasibility of performing beam sweeping using UE rotation in multiple iterations during the conformance testing of AI/ML based BM use case.

RAN4 to discuss the possibility of testing AI/ML BM use-case considering (1) No transmission of Set A beams during the test, (2) exploit and reuse the characteristics of the testing environment between the test probes and UE baseband during the test.

We need to check what is the directional channel based on beam patterns, and how many clusters those directional channels interact with in general. The clusters numbers will be greatly reduced compared with the number of clusters in current CDL models.

We can use the spatial filter corresponding to the beam width of SSB to filter out the clusters of omni-directional CDL channel models (current CDL models). The spatial filter is defined in 38.901 Chapter 7.7.4 Spatial filter for generating TDL channel model to generate TDL model, so, we need to modify the procedure to keep the angular information of each cluster.

Use spatial filter to find out how many clusters in the directional model for SSB before doing simplification of CDL model.

Bear in mind that removing the clusters of a CDL model for simplification will change the channel properties. Or we can use the alternatives that use ray tracing or other simulation to simulate directional channels with 4 clusters directly.

If the number of clusters interact with each SSB/CSI-RS is less than 4 (or the number of clusters the probes of TE can simulate at the same time), we may only need to divide 24 clusters into “p” sub-groups and each sub-group match the clusters one SSB/CSI-RS interacts with.

RAN4 to consider designing a new metric to verify the generalization performance of the AI/ML BM functionality in different (static) scenarios.

For the verification/testing of generalization related aspects in RAN4 for AI/ML BM, RAN4 should define the generalization test which consists of multiple subtests each associated with the testing of different scenarios based on parameters listed in Table 1.

The following Proposals were made for AI/ML Positioning use case:

- Reuse report mapping tables in TS 38.133 clause 13.1.1 for UL RTOA first path reporting and report mapping tables in TS 38.133 clause 13.1.1A for UL RTOA additional path reporting in Case 3a.

- Reuse report mapping table in TS 38.133 clause 13.3.1 for UL SRS RSRP reporting and report mapping table in TS 38.133 clause 13.6.1 for UL SRS RSRPP reporting in Case 3a.

- No RAN4 requirements for inference need to be defined for the LOS/NLOS indicator agreed by RAN1 for Case 3a.

- Measurement accuracy requirements for gNB Rx-Tx time difference apply for UL SRS-TDCT first path under the condition that the reported measurement is based on N=4 samples.

- Measurement accuracy requirements for UL SRS RSRP apply for UL SRS-TDCP under the condition that the reported measurement is based on N=4 samples.

- Add new report mapping tables for additional samples for UL RTOA by reusing single-sided range of existing additional path reporting for UL RTOA and keeping the reporting range agnostic to the number of Nt configured by LMF.

- Add following tables to the new clause 13.1.1X of TS 38.133 for additional sample reporting for sample-based reporting:

Table 13.1.1X-1: Measurement report mapping for k=0

| Reported Quantity Value, sample_i | Measured Quantity Value, Dsample | Unit |
| --- | --- | --- |
| sample_0000 | 0 £ Dsample < 1 | Tc |
| sample_0001 | 1 £ Dsample < 2 | Tc |
| sample_0002 | 2 £ Dsample < 3 | Tc |
| … | … | … |
| sample_8173 | 8173 £ Dsample < 8174 | Tc |
| sample_8174 | 8174 £ Dsample < 8175 | Tc |
| sample_8175 | 8175 £ Dsample | Tc |

Table 13.1.1X-2: Measurement report mapping for k=1

| Reported Quantity Value, sample_i | Measured Quantity Value, Dsample | Unit |
| --- | --- | --- |
| sample_0000 | 0 £ Dsample < 2 | Tc |
| sample_0001 | 2 £ Dsample < 4 | Tc |
| sample_0002 | 4 £ Dsample < 6 | Tc |
| … | … | … |
| sample_4086 | 8172 £ Dsample < 8174 | Tc |
| sample_4087 | 8174 £ Dsample | Tc |

Table 13.1.1X-3: Measurement report mapping for k=2

| Reported Quantity Value, sample_i | Measured Quantity Value, Dsample | Unit |
| --- | --- | --- |
| sample_0000 | 0 £ Dsample < 4 | Tc |
| sample_0001 | 4 £ Dsample < 8 | Tc |
| sample_0002 | 8 £ Dsample < 12 | Tc |
| … | … | … |
| sample_2042 | 8172 £ Dsample < 8174 | Tc |
| sample_2043 | 8174 £ Dsample | Tc |

Table 13.1.1X-4: Measurement report mapping for k=3

| Reported Quantity Value, sample_i | Measured Quantity Value, Dsample | Unit |
| --- | --- | --- |
| sample_0000 | 0 £ Dsample < 8 | Tc |
| sample_0001 | 8 £ Dsample < 16 | Tc |
| sample_0002 | 16 £ Dsample < 24 | Tc |
| … | … | … |
| sample_1019 | 8152 £ Dsample < 8160 | Tc |
| sample_1020 | 8160 £ Dsample < 8168 | Tc |
| sample_1021 | 8168 £ Dsample | Tc |

Table 13.1.1X-5: Measurement report mapping for k=4

| Reported Quantity Value, sample_i | Measured Quantity Value, Dsample | Unit |
| --- | --- | --- |
| sample_000 | 0 £ Dsample < 16 | Tc |
| sample_001 | 16 £ Dsample < 32 | Tc |
| sample_002 | 32 £ Dsample < 48 | Tc |
| … | … | … |
| sample_508 | 8128 £ Dsample < 8144 | Tc |
| sample_509 | 8144 £ Dsample < 8160 | Tc |
| sample_510 | 8160 £ Dsample | Tc |

Table 13.1.1X-6: Measurement report mapping for k=5

| Reported Quantity Value, sample_i | Measured Quantity Value, Dsample | Unit |
| --- | --- | --- |
| sample_000 | 0 £ Dsample < 32 | Tc |
| sample_001 | 32 £ Dsample < 64 | Tc |
| sample_002 | 64 £ Dsample < 96 | Tc |
| … | … | … |
| sample_253 | 8098 £ Dsample < 8128 | Tc |
| sample_254 | 8128 £ Dsample < 8160 | Tc |
| sample_255 | 8160 £ Dsample | Tc |

- Reuse the UL SRS RSRPP report mapping table in TS 38.133 clause 13.3.1 for the Nt reported samples for UL SRS TDCP for sample-based reporting.

RAN4 to not specify a requirement on training data collection of Part B in Case 3a/3b.

- References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm Incorporated, RAN#107 meeting, 12 – 14 March 2025, Incheon, S. Korea.

- R4-2508080, WF on requirements for AIML air interface part 1, Qualcomm, RAN4#115, Malta, Malta, 19th ‒ 23rd May, 2025.​

- R4-2508081, Updated simulation assumptions for AIML BM, vivo et al., RAN4#115, Malta, Malta, 19th ‒ 23rd May, 2025.​

- R1-2503243, Consolidated Rel-19 higher layers parameters list Post RAN1#121.xlsx, RAN1#121 meeting, 19th – 23rd May 2025, Malta.