3GPP TSG-RAN WG4 Meeting #116		               R4-2509935

Bengaluru, India, August 25th – 29th, 2025

Source:	Sony

Title:	RF requirements of ambient IoT device 1

Agenda item:	7.22.3.2

Document for:	Approval

# Background

The RF requirements of Ambient IoT (AIoT) device type 1 were discussed in RAN4#115 [1], and further agreements have been made in [2]. In this contribution, we share our further views on the remaining issues related to the RF requirements shall be specified for device type 1.

# System parameters

## D2R bandwidth

For D2R bandwidth, the following agreements have been made in RAN4#115[1] :

Agreement in RAN4#115:

- Define D2R CBW based on all Tb, Tc, R combinations as finally approved in RAN1.

  - Assumed 10% SFO for device

  - For reader, [90%] filter spectrum utility ([10%] guard band) is considered.

- The value of transmission BW and channel BW will be explicitly listed.

- Using following equation for BS as starting point:

D2R CBW for BS (KHz)=(2SB Transmission BW*(1/2)+2* Small frequency shift)/0.9

=(2000*(1+R)/Tb)* (1+∣SFO∣)/0.9

=(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)/0.9

  - Define [200kHz]，[3.54MHz] D2R CBW for AIoT BS. Other D2R CBWs for BS are FFS

- Using following equation for device as starting point:

D2R CBW for device (KHz)=2SB Transmission BW*(1/2)+2* Small frequency shift

=(2000*(1+R)/Tb)* (1+∣SFO∣)

=(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)

- The following are for information:

- Figure 1-1 Small frequency shift diagram of FDMA without SFO - Figure 1-2. SFO impact of transmission bandwidth and small frequency shift of FDMA - As the figures above indicate, SFO not only affects the D2R transmission bandwidth but also scales the small frequency shift value . - The 2SB transmission BW and small frequency shift _both measured in kHz——with and without SFO are as follows: - Note : BW and Small frequency shift are in kHz;Tc and Tb are in μs 2SB Transmission BW (kHz); Small frequency shift (kHz). Without SFO; With SFO; Without SFO; With SFO. Btx,D2R_ without SFO= 4*1000/Tb=2*1000/(R*Tc); Btx,D2R_ with SFO =Btx,D2R_ without SFO×(1±∣SFO∣); SFSwithout SFO=±1000*R/Tb= ±1000/(2*Tc); SFSwith SFO =SFSwithout SFO×(1±∣SFO∣).

Moreover, the following values of Tb and Tchip was agreed in RAN1#121[3]:

|  | Tchip (μs) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tb (μs) |  |  |  |  |  |  |  |  |  |
|  | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.69 |
| 266.67 | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 |  |
| 133.33 |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 |  |
| 66.67 |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 |  |
| 33.33 |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 |  |
| 16.67 |  |  |  |  | R=1 | R=2 | R=4 | R=8 |  |
| 8.33 |  |  |  |  |  | R=1 | R=2 | R=4 |  |
| 4.17 |  |  |  |  |  |  | R=1 | R=2 |  |
| 1.39 |  |  |  |  |  |  |  |  | R=1 |

In our view, it is necessary to consider the impact of SFO, which not only changes the bandwidth of D2R transmission BW but also affects its offset to the CW frequency. The impact of SFO is illustrated in Fig. 1.

Figure 1. The illustration of the impact of SFO on D2R transmission BW.

Considering that the impact due to a positive SFO is more prominent, which not only widens the spectrum of the D2R transmission BW but also pushes it further away from the CW frequency point, we consider + 10% SFO when calculating the value of corresponding BW, based on (1)-(3)

B tx,D2R=(4*1000/Tb)*(1+∣SFO∣)                                                     (1)

SFS = (1000*R/Tb) * (1+∣SFO∣)                                                  (2)

BCBW,D2R  = 0.5* Btx,D2R +2* SFS                                                  (3)

With the agreed Tb and Tchip from RAN1#121, the requirement D2R transmission BW, SFS, and channel BW can be calculated as in Table 1.

Table 1. The value of the D2R channel bandwidth with respect to transmission bandwidth and SFS.

|  | Small frequency shift (kHz) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transmission BW (kHz) | 4 | 8 | 17 | 33 | 66 | 132 | 264 | 529 | 797 |
| 16 | 17 | 25 | 41 | 74 | 140 | 272 | 537 | 1066 |  |
| 33 |  | 33 | 50 | 82 | 149 | 280 | 545 | 1074 |  |
| 66 |  |  | 66 | 99 | 165 | 297 | 562 | 1091 |  |
| 132 |  |  |  | 126 | 195 | 324 | 589 | 1352 |  |
| 264 |  |  |  |  | 264 | 396 | 727 | 1190 |  |
| 528 |  |  |  |  |  | 504 | 769 | 1298 |  |
| 1055 |  |  |  |  |  |  | 1056 | 1585 |  |
| 3165 |  |  |  |  |  |  |  |  | 3165 |

It can be observed that there are multiple D2R channel BW values corresponding to each transmission BW and each SFS. It is unclear to us if it is really needed to define all of them for Rel-19 AIoT system. It is proposed RAN4 further discuss if all the D2R channel BW value should be defined.

Observation 1: there are multiple value of D2R channel BW with similar values.

Proposal 1: RAN4 should decide if all the D2R channel BW value corresponding to transmission bandwidth and SFS shall be defined.

# RF requirement

A key question regarding Tx and Rx requirements for AIoT device type 1 is whether the core requirement should be defined only for the peak direction or for under multiple angles. The key issue here is to strike a balance between the necessity of a broad range of core requirements and the burden on verification.

There is no question that the relative angle between AIoT devices, the reader, and the CW node can vary significantly in real life. Therefore, from a core requirement perspective, it is necessary to define a minimum requirement that can be applied to a certain range of solid angles to ensure the device performs well under various deployment scenarios in real-world conditions. As an example, RAN4 may consider defining a minimum REFSENS requirement or minimum backscattering loss within a partial sphere with a 45-degree solid angle range above the device, as illustrated in Fig. 2.

Meanwhile, considering the time and cost for the verification test, RAN4 may consider testing the core requirements only with a few selected solid angles. The detailed core requirements value and solid angle range will be discussed in the following sections.

Figure. 2. Example on the applicable partial sphere (solid angle range) where the core requirement shall be applied.

## Reception requirements.

### REFSENS

For REFSENS requirements, the following agreements have been made in RAN4#114bis

- Using [-34dBm] as starting point to define REFSENS requirements, based on the peak gain direction of device antenna.

- Consider to define orientation requirements with following options:

  - Option 1: min sensitivity at peak antenna gain direction and EIS spherical coverage requirements, XdBm at Xth percentage

  - Option 2: spatial averaging EIS

  - Option 3: EIS in the peak direction

- Testing metric: 10% miss detection rate, FFS on whether to consider false alarm

As discussed above, in our view, it is more proper to define a sensitivity requirement that can be applicable for a certain range of solid angle to ensure the AIoT device 1 can have good performance in real life. Meanwhile, due to the nature of the antenna performance, the antenna gain would inevitably drop when the solid angle is offset from the boresight of the antenna.

Figure 3. Simulated antenna gain of a meander dipole antenna

A simulated antenna gain of a meander line type of dipole antenna is shown in Fig. 3, which can represent well a typical RFID tag implementation. It can be seen that there is a 3dB gain drop with respect to about a 45° off the boresight direction. Therefore, to balance the core requirement value and the applicable range of the solid angle, we proposed to consider a -37 dBm REFSENS requirement within a 45-degree offset solid angle range with respect to the bore sight direction of the antenna. The solid angle range is illustrated in Fig. 2.

Proposal 2: It is proposed to define a minimum -37 dBm REFSENS requirement, which is applicable within a 45° solid angle range partial sphere with respect to the bore sight direction (or UE declared direction) of the AIoT device 1.

For the performance metric, in our understanding, the AIoT devices can perform a contention-based procedure in response to the paging message from the reader. With a high probability of FAR, it can create more collisions and cause a higher failure rate during a contention-based access procedure, and the time required to complete the inventory for a reader will be increased consequently. Therefore, it is also proposed to consider verify the FAR performance for the AIoT device test. The corresponding test can be considered under the Demod test rather than REFSENS, but such UE behaviour must be tested and verified.

Proposal 3: RAN4 needs to verify the FAR performance of the device either as part of the REFSENS test or under the Demod test.

## Transmission part (D2R)

### The transmitted power

For backscattering loss requirements, the following agreements have been made in RAN4#115

- Performance metric: EIRP

  - FFS on how to determine device’s peak antenna gain direction

  - FFS on whether to measure multiple directions

- Take [10] dB backscatter loss for OOK as starting point, based on the assumption that the incoming CW signal and the backscatter signal are aligned with the peak gain direction of the device antenna.

  - FFS for BPSK

- The power of the backscattered signal shall include only the 1st lower sideband and the 1st upper sideband, excluding the carrier itself.

The backscattering loss has been defined as the metric to evaluate the transmit power performance of AIoT in the D2R link, which is equal to the difference between the input CW Tx power at the device antenna and the backscattering power at the device antenna.

Physically, the relation between the impinging power and backscattering power is proportional to the radar cross section (RCS) σ of the object, which is defined as:

PsPin ∝σ=limr→∞4πr2SsSi                                                                      (4)

where Ss and Si are the power density of the impinging power and scattering power.

The bistatic RCS σ of an RFID tag antenna can be described by (4) according to [4]

σ Ωi,Ωs= λ24π GΩiGΩsΓ-A 2                                                    (5)

where Ωi and Ωs are the incident angle and scatter angle. G and  Γ stand for the antenna gain and reflection coefficient on the AIoT device load. A stands for residual scattering due to the antenna structure, which is independent of load conditions. We also assume the insertion loss and mismatch loss between the AIoT antenna and load impedance is contained in the G. From (3) and (4), it can be observed that the backscattering loss is angle-dependent, and a simulated RCS with a different incident angle is shown in Fig. 4 as an example.

Fig. 3. The RCS simulation of an AIoT device with different CW incident angles.

However, (4) only provides the RCS of the AIoT device when the load impedance is fixed. With a modulated signal, the AIoT device's load impedance varies in the test, and the power on the first sideband (SB) is proportional to the difference between the modulation state rather than the absolute RCS. Therefore, in [5], a backscattering efficiency μm based on the power level on the first SB are derived:

PsPin = GTΩincGRΩscatterλ2(4π)3r12r22μm

Where μm Ωinc,Ωscatter= λ22π3 GΩincGΩscatterΓ1-Γ2 2                                     (5)

where GT and GR are the gain of CW and reader antenna, and r1 and  r2 are the distance from the CW antenna to the AIoT device and the distance from the AIoT device to the reader, respectively. Such a backscattering efficiency is essentially the RCS on the first SB.

However, please note that this backscattering efficiency cannot be used to derive the backscattering loss directly as it quantifies a link level loss rather than the loss between incoming power level and outgoing power level on the device. To quantify the backscattering loss, a reference AIoT device (e.g., a lossless AIoT device with 0 dBi realized antenna gain) may need to be defined, and further study is required to determine if this backscattering efficiency can be used directly to derive the backscattering loss.

Nevertheless, from (5), it can be observed that the backscatter loss on the AIoT device depends on the following parameters if we assume no polarization mismatch happens during the link.

- the antenna gain on the incident angle

- the antenna gain on the scattered angle

- the difference between the reflection coefficient of the two states, e.g., the modulation depth.

Observation 2: Assuming no polarization mismatch, the backscatter loss on the first SB of the AIoT device depends on the antenna gain on the incident angle, the antenna gain on the scattered angle, and the modulation depth.

In [6], a measurement of backscattering loss, which equals the backscattering power minus the incident CW power level, was carried out with three different tags with OOK. In those measurements, carried out under a reverberation chamber, the nature of the measurement is evaluated based on TRP. In other words, the results can be interpreted as indicating antenna directivity of 0 dBi at both the incident and backscattered angles.

Therefore, the value presented in [6] can be taken as a starting point, and the results further scaled based on the antenna gain/directivity values according to (5). As an example, in [6], one of the measured tag shows a backscattering loss with 12 dB for 0 dBi antenna gain. Then, on the peak direction, assuming 2 dBi directivity for both incident and backscattering angles, the backscattering loss would be around 8 dB. For an offset angle of 45° to the boresight, the directivity can be dropped down to -1 dBi, which leads to a backscattering loss in the range of about 14 dB.

To align with our REFSENS proposal, we propose defining the core requirement for backscattering loss to be applied within a 45° solid angle range around the AIoT device's boresight direction. Based on the analysis above, we propose a minimum requirement of 14 dB backscattering loss within this range.

Proposal 4: It is proposed to define the maximum backscattering loss as 14 dB, which is applicable within a 45-degree partial sphere range with respect to the bore sight direction (or UE declared direction) of the AIoT device 1, whenboth angle to the reader and the CW are within this partial sphere for OOK.

In addition, it can also be observed from [6] that the backscattering loss can depend on the input power level. Therefore, it is proposed that the backscattering core requirement can be verified under an input power level that is based on the vendor declaration. Meanwhile, RAN4 can further discuss whether a range of input power levels should be defined to limit the allowed declared range.

Proposal 5: It is proposed that the input power level can be vendor-declared, and RAN4 can further discuss whether a range of input power levels should be defined to limit the allowed declared range.

For BPSK, based on the theoretical value, it should have 6 dB less backscattering loss than OOK. Considering the practical phase accuracy in modulation, it is proposed that the backscattering loss of BPSK can be defined 5 dB less than that of OOK.

Proposal 6: It is proposed that the backscattering loss of BPSK should be 5 dB less than that of OOK.

Please note that the above discussion ignores the polarization mismatch between the CW and AIoT devices. As an additional aspect, the test setup may need to ensure the CW can be transmitted in both polarizations, and it is possible to define the requirement based on the average backscattering loss between the two polarizations. In this case, a 3dB polarization loss needs to be taken into account. We also have an accompanying paper under the OTA agenda [7] on this issue.

Proposal 7: RAN4 needs to take into account a 3 dB polarization mismatch loss if the backscattering loss is measured as the average value between two orthogonal polarized CW waves.

# Conclusion

In this paper, we provide the RF requirements for AIoT device 1, where the following observations and proposals have been made:

Observation 1: there are multiple value of D2R channel BW with similar values.

Observation 2: Assuming no polarization mismatch, the backscatter loss on the first SB of the AIoT device depends on the antenna gain on the incident angle, the antenna gain on the scattered angle, and the modulation depth.

Proposal 1: RAN4 should decide if all the D2R channel BW value corresponding to transmission bandwidth and SFS shall be defined.

Proposal 2: It is proposed to define a minimum -37 dBm REFSENS requirement, which is applicable within a 45° solid angle range partial sphere with respect to the bore sight direction (or UE declared direction) of the AIoT device 1.

Proposal 3: RAN4 needs to verify the FAR performance of the device either as part of the REFSENS test or under the Demod test.

Proposal 4: It is proposed to define the maximum backscattering loss as 14 dB, which is applicable within a 45-degree partial sphere range with respect to the bore sight direction (or UE declared direction) of the AIoT device 1, whenboth angle to the reader and the CW are within this partial sphere for OOK.

Proposal 5: It is proposed that the input power level can be vendor-declared, and RAN4 can further discuss whether a range of input power levels should be defined to limit the allowed declared range.

Proposal 6: It is proposed that the backscattering loss of BPSK should be 5 dB less than that of OOK.

Proposal 7: RAN4 needs to take into account a 3 dB polarization mismatch loss if the backscattering loss is measured as the average value between two orthogonal polarized CW waves.

# References

- R4-2507573	Topic summary for [115][134] A-IoT_device	Moderator (CMCC)

- R4-2508116	WF on A-IoT_device	CMCC

- Session notes for 9.4 Solutions for Ambient IoT in NR	 Ad-Hoc Chair (Huawei)

- C. -C. Yen, A. E. Gutierrez, D. Veeramani and D. van der Weide, "Radar Cross-Section Analysis of Backscattering RFID Tags," in IEEE Antennas and Wireless Propagation Letters

- C. Degen and P. Bosselmann, "On Complex Radar Cross Section and Backscatter Modulation Efficiency in RFID Systems," 2020 14th European Conference on Antennas and Propagation (EuCAP).

- UHF RFID tags backscattered power measurement in reverberation chamber, L. Konélamine.kone, R. Kassi, and N. Rolland

- R4-2507129	Consideration on the OTA test of ambient IoT device 1	Sony