3GPP TSG-RAN WG4 Meeting #116	R4-2509942

Bengaluru 25th – 29th Aug 2025

Source: 	Nokia

Title: 					n26 7 MHz A-MPR proposals

Agenda Item: 			7.4.3

Document for:	Approval

# 1	Introduction

We make A-MPR proposals for 7 MHz CBW on operating band n26 based on our observations and simulation results.

# 2	Simulation assumptions and scenario

## 2.1	Simulation parameters

- Operating band n26 (UL 814 MHz – 849 MHz)

- Carrier positions according to the WF of the last meeting [1]:

  - NS_12 considering 7 MHz deployed within 814-824 MHz

  - NS_13 considering 7 MHz deployed in 817-824 MHz (Fc = 820.5 MHz).

  - NS_14 considering 7 MHz deployed within 824-849 MHz

  - NS_15 considering 7 MHz deployed within 814-845 MHz or within 841.5-849 MHz.

- 7 MHz CBW with NRB = 35 as defined in [1]

- Power classes PC2 and PC3

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc

- Only additional spurious emission masks from NS_12, NS_13, NS_14 and NS_15 are simulated. Other gating factors (ACLR, SEM, etc.) are excluded because they are handled by MPR.

## 2.2	NS_12 mask

Table 6.5.3.3.17-1: Additional requirements "NS_12"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3 MHz, 5 MHz, 10 MHz |  |
| 806 ≤ f ≤ 813.5 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for NR carriers with lower channel edge at or above 814 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

## 2.3	NS_13 mask

Table 6.5.3.3.18-1: Additional requirements "NS_13"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3 MHz, 5 MHz |  |
| 806 ≤ f ≤ 816 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for NR carriers with lower channel edge at or above 817 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

## 2.4	NS_14 mask

Table 6.5.3.3.19-1: Additional requirements "NS_14"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 10 MHz, 15 MHz, 20MHz |  |
| 806 ≤ f ≤ 816 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for NR carriers with lower channel edge at or above 824 MHz NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

## 2.5	NS_15 mask

Table 6.5.3.3.20-1: Additional requirements "NS_15"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3 MHz, 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |
| 851 ≤ f ≤ 859 | -53 | 6.25 kHz |
| NOTE 1: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

# 3	A-MPR proposals

The tables in specification and WF [2] that contain the A-MPR values contain also the texts “Outer/Inner”, “Outer” or “Edge/Inner” that refer to the region definitions of MPR. We think that the texts are not needed when the A-MPR regions are defined through the equations and inequality relations in the A-MPR tables. It can also be confusing when the text conflicts with region definition; for example, when it is “Outer/Inner” and the region also contains edge allocations.

Proposal 1: RAN4 shall consider removing the texts “Outer/Inner”, “Outer” and “Edge/Inner” from the NS_12, NS_13, NS_14 and NS_15 A-MPR tables.

## 3.1	PC2 proposals

We have kept the general shape of the regions from the WF [2] but made some adjustments to the exact borders according to our simulation results. The changed regions are highlighted with yellow color. Empty cells in the tables of A-MPR values denote that MPR is sufficient according to our results.

Proposal 2: RAN4 shall consider the A-MPR in tables 1 and 2 when specifying A-MPR for n26 PC2 NS_12.

Table 1: A-MPR regions for NS_12 PC2

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc < 820.5 | ≤1.8 | ≤0.72 | A3 |
|  |  | ≤1.8 | >0.72 | A1 |
|  |  | >1.8 | >2.7 | A2 |

Table 2: A-MPR values for NS_12 PC2

| Modulation/Waveform |  | A1 | A2 | A3 |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | 6 | 2 | 2 |
|  | QPSK | 5 | 3.5 | 2.5 |
|  | 16 QAM | 6 | 3 | 3 |
|  | 64 QAM | 6 | 3 | 3.5 |
|  | 256 QAM | 6.5 |  |  |
| CP-OFDM | QPSK | 6.5 | 4 | 4 |
|  | 16 QAM | 6.5 | 4 | 4 |
|  | 64 QAM | 6.5 | 4 | 4 |
|  | 256 QAM |  |  |  |

Proposal 3: RAN4 shall consider the A-MPR in tables 3 and 4 when specifying A-MPR for n26 PC2 NS_13.

Table 3: A-MPR regions for NS_13 PC2

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz |  | ≤1.8 | <1.44 | A1 |
|  | 820.5 ≤ Fc | ≤1.8 | ≥1.44 | A2 |
|  |  | >1.8 | ≥3.6 | A3 |

Table 4: A-MPR regions for NS_13 PC2

| Modulation/Waveform |  | A1 | A2 | A3 |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | 2 | 6 | 1.5 |
|  | QPSK | 2.5 | 4.5 | 3 |
|  | 16 QAM | 3 | 4.5 | 2.5 |
|  | 64 QAM | 3 | 4.5 | 2.5 |
|  | 256 QAM |  |  |  |
| CP-OFDM | QPSK | 4 | 5.5 | 3.5 |
|  | 16 QAM | 4 | 5.5 | 3.5 |
|  | 64 QAM | 4 | 5.5 |  |
|  | 256 QAM |  |  |  |

Proposal 4: RAN4 shall consider the A-MPR in tables 5 and 6 when specifying A-MPR for n26 PC2 NS_15.

Table 5: A-MPR regions for NS_15 PC2

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 841.5 < Fc ≤ 845.5 | ≥4.68 | >0 | A1 |
|  |  | <4.68, ≥2.52 | ≥2.16 | A2 |
|  |  | <4.68, ≥2.52 | <2.16, ≥1.44 | A3 |
|  |  | <1.62 | ≤0.18 | A4 |
|  | 837 < Fc ≤ 841.5 | ≥0 | ≥3.6 | A5 |
|  |  | ≥5.94 | <2.7 | MPR is enough |

Table 6: A-MPR regions for NS_15 PC2

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | 11 | 5.5 |  | 4.5 | 3 |
|  | QPSK | 10 | 6.5 | 0.5 | 2 | 4 |
|  | 16 QAM | 10.5 | 6.5 |  | 4 | 4.5 |
|  | 64 QAM | 10.5 | 6.5 |  | 4.5 | 4.5 |
|  | 256 QAM | 10.5 | 6.5 |  |  | 5 |
| CP-OFDM | QPSK | 11.5 | 7.5 | 2.5 | 8.5 | 6 |
|  | 16 QAM | 11.5 | 7.5 | 2.5 | 8.5 | 6 |
|  | 64 QAM | 11.5 | 7.5 |  | 8.5 | 6 |
|  | 256 QAM | 11.5 | 7.5 |  | 8.5 |  |

## 3.2	PC3 proposals

We have kept the exact regions according to the WF [2] but provided A-MPR values based on our simulation results. Empty cells in the tables of A-MPR values denote that MPR is sufficient according to our results.

Proposal 5: RAN4 shall consider the A-MPR in tables 7 and 8 when specifying A-MPR for n26 PC3 NS_12.

Table 7: A-MPR regions for NS_12 PC3

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc ≤ 820.5 | ≤1.8 | ≤0.72 | A3 |
|  |  | ≤1.8 | >0.72 | A4 |
|  |  | >1.8 | >1.44 | A5 |

Table 8: A-MPR values for NS_12 PC3

| Modulation/Waveform |  | A3 | A4 | A5 |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | 2 | 3 | 1.5 |
|  | QPSK | 2 | 4 | 2 |
|  | 16 QAM | 2.5 | 4 | 2.5 |
|  | 64 QAM |  | 4 |  |
|  | 256 QAM |  |  |  |
| CP-OFDM | QPSK | 3.5 | 5 |  |
|  | 16 QAM | 3.5 | 5 |  |
|  | 64 QAM |  | 5 |  |
|  | 256 QAM |  |  |  |

Proposal 6: RAN4 shall consider the A-MPR in tables 9 and 10 when specifying A-MPR for n26 PC3 NS_13.

Table 9: A-MPR regions for NS_13 PC3

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz |  | ≤1.8 | <1.44 | A5 |
|  | 820.5 = Fc | ≤1.8 | ≥1.44 | A6 |
|  |  | >1.8 | ≥1.8 | A7 |

Table 10: A-MPR regions for NS_13 PC3

| Modulation/Waveform |  | A5 | A6 | A7 |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | 2 | 2.5 | 1 |
|  | QPSK | 2 | 3.5 | 1.5 |
|  | 16 QAM | 2.5 | 3.5 |  |
|  | 64 QAM |  | 3.5 |  |
|  | 256 QAM |  |  |  |
| CP-OFDM | QPSK | 3.5 | 4.5 |  |
|  | 16 QAM | 3.5 | 4.5 |  |
|  | 64 QAM |  | 4.5 |  |
|  | 256 QAM |  |  |  |

Proposal 7: RAN4 shall consider the A-MPR in tables 11 and 12 when specifying A-MPR for n26 PC3 NS_15.

Table 11: A-MPR regions for NS_15 PC3

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7 MHz | 841.5 < Fc ≤ 845.5 | ≥ 4.5 | > 0 | A7 |
|  |  | < 4.5, ≥ 2.16 | ≥ 1.8 | A8 |
|  |  | ≤ 1.44 | ≤ 0.36 | A9 |
|  | 838 < Fc ≤ 841.5 | > 0 | > 2.7 | A10 |
|  |  | ≥ 5.94 | ≤ 2.7 | MPR is enough |

Table 12: A-MPR regions for NS_15 PC3

| Modulation/Waveform |  | A7 | A8 | A9 | A10 |
| --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | 10.5 | 5 | 1 | 1 |
|  | QPSK | 9 | 6 |  | 2 |
|  | 16 QAM | 9 | 5.5 |  | 3.5 |
|  | 64 QAM | 9 | 5.5 |  | 3.5 |
|  | 256 QAM | 9 | 5.5 |  |  |
| CP-OFDM | QPSK | 9.5 | 6 |  | 4.5 |
|  | 16 QAM | 9.5 | 6 |  | 4.5 |
|  | 64 QAM | 9.5 | 6 |  | 4.5 |
|  | 256 QAM | 9.5 | 6 |  |  |

# 4	Conclusions

We have made the following proposals based on our observations and simulation results.

Proposal 1: RAN4 shall consider removing the texts “Outer/Inner”, “Outer” and “Edge/Inner” from the NS_12, NS_13, NS_14 and NS_15 A-MPR tables.

Proposal 2: RAN4 shall consider the A-MPR in tables 1 and 2 when specifying A-MPR for n26 PC2 NS_12.

Proposal 3: RAN4 shall consider the A-MPR in tables 3 and 4 when specifying A-MPR for n26 PC2 NS_13.

Proposal 4: RAN4 shall consider the A-MPR in tables 5 and 6 when specifying A-MPR for n26 PC2 NS_15.

Proposal 5: RAN4 shall consider the A-MPR in tables 7 and 8 when specifying A-MPR for n26 PC3 NS_12.

Proposal 6: RAN4 shall consider the A-MPR in tables 9 and 10 when specifying A-MPR for n26 PC3 NS_13.

Proposal 7: RAN4 shall consider the A-MPR in tables 11 and 12 when specifying A-MPR for n26 PC3 NS_15.

# 5	References

[1] R4-2502870, WF on requirements for 7MHz channel bandwidth, Ericsson

[2] R4-2507954, WF on 7 MHz channel BW, Ericsson