3GPP TSG-RAN WG4 Meeting #116	R4-2509945

Bengaluru, India, August 25th – 29th, 2025

Source:	Nokia

Title:	Discussion on gap association

Agenda item:	4.4.2

Document for:	Discussion

Introduction

In this contribution we come back to a discussion at RAN4#114 and RAN4#115 related to the WI NR_MG_enh2 [1] on the term “gap association”.

First, we analyze the usage of the term in TS 38.133 and then provide a proposal to introduce a definition for the term ‘gap association’ for Rel-17 concurrent measurement gap types, which also provides a basis for Rel-18 concurrent measurement gap types, considered in Case 1 and Case 2.

Discussion

The discussion on gap association was triggered at RAN4 #114 when Case 2 requirements for Rel-18 were discussed.

In particular, CR [2] was agreed including the term “gap association”. It was noticed that a definition for the term “gap association” is missing in TS 38.133. A draft CR was then provided to RAN4#114 [3] for information, which introduces the definition of the terms “gap association” and “gap association rule” for Rel-17 measurement gap types.

When analyzing the usage of the term in TS 38.133, the following is observed.

- Clause 9 in Rel-17 of TS 38.133 includes the term “association” only twice, namely in clauses 9.1.5.2 and 9.1.5.2.2.

- Clause 9 in Rel-18 of TS 38.133 has 6 occurrences of the term “association” and 4 occurrences of the term “gap association”, namely in clauses 9.1.5.3 (1), 9.1.12.2 (2) related to concurrent gaps for Case 1 and 9.1.13.2 (1) related to concurrent gaps for Case 2.

- For Rel-17, there are only two occurrences of the term “association” in clause 9 of TS 38.133, whilst for Rel-18 there are 6 occurrences of the term “association” and 4 occurrences of the term “gap association” in clause 9 of TS 38.133.

Thus, definitions of the terms “gap association” and “gap association rule” were proposed to be added at RAN4#115 [4][5] from Rel-17 for concurrent gaps, Pre-MG and NCSG gap types in clause 9.1.1.

During the RAN4#115 discussion, some companies raised following concerns:

- Gap association is not applicable to Rel-17 Pre-MG and Rel-17 NCSG, only to Rel-17 concurrent MGs.

- There is no need to define the term ‘gap association rule’.

The CR [5] was then postponed. Therefore, the change proposal here is limited to Rel-17 and Rel-18 concurrent gaps and no term ‘gap association rule’ is defined.

Add the following definition for ‘gap association’ to TS 38.133, clause 9.1.1 from Rel-17 to improve readability of the spec:

- Gap association: the association between a configured measurement object and a configured concurrent measurement gap.

The accompanied CR [6] for Rel-17 (and Cat-A CRs for Rel-18 and Rel-19) adds this definition to TS 38.133, clause 9.1.1.

Conclusion

In this contribution we analyzed the usage of the term ‘gap association’ in TS 38.133 and then provide a proposal to introduce a definition for the term ‘gap association’ for Rel-17 concurrent measurement gap types, which also provides a basis for Rel-18 concurrent measurement gap types, considered in Case 1 and Case 2.

As part of this discussion, following observation and following proposal are made:

- For Rel-17, there are only two occurrences of the term “association” in clause 9 of TS 38.133, whilst for Rel-18 there are 6 occurrences of the term “association” and 4 occurrences of the term “gap association” in clause 9 of TS 38.133.

- Add the following definition for ‘gap association’ to TS 38.133, clause 9.1.1 from Rel-17 to improve readability of the spec:

- Gap association: the association between a configured measurement object and a configured concurrent measurement gap.

The accompanied CR [6] for Rel-17 and correspondent Cat-F CR for Rel-18 and Cat-A CR for Rel-19 add this definition to TS 38.133, clause 9.1.1.

- References

- RP-232440, “Revised WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”, Intel, MediaTek

- R4-2502670, “(NR_MG_enh2-Core) CR on 38.133 MG enh2 on NCSGs“, Ericsson, Nokia, CMCC, OPPO, RAN4#114

- R4-2502686, “Draft CR38.133 Addition of definition for gap association”, Nokia, RAN4#114

- R4-2507348, “Discussion on gap association”, Nokia, RAN4#115

- R4-2508369, “(NR_MG_enh-Core) CR38.133 Gap association for concurrent MGs, Pre-MG and NCSG”, Nokia, RAN4#115

- R4-2509946, “(NR_MG_enh-Core) CR38.133 Gap association for concurrent MGs (Rel-17)”, Nokia, RAN4#116