3GPP TSG-RAN-WG4 Meeting #116 	R4-2509975

# Bangalore, India, 25th – 29th August, 2025

# Title:	TP for TR38.746 for adding HPUE for DL CA_n25-n41 with UL CA_n41C and UL CA_n25A-n41C

# Source:	T-Mobile USA, Skyworks Solutions, Inc.

# Agenda Item:	6.8.3

# Document for:	Approval

- Introduction

This contribution provides a text proposal for TR38.746[1] adding the following HPUE NRCA combinations with PC1.5 UL CA_n41C and PC2 and PC1.5 UL CA_n25A-n41C.

- DL CA_n25A-n4C

- Reference

[1] 3GPP TR38.746: “High power UE (power class 1.5 or 2) for NR inter-band Carrier Aggregation (CA) / Dual Connectivity (DC) with/without Supplementary Uplink (SUL) with high power on TDD band(s); (Release 19)”: V0.2.0

[2] 3GPP TS38.101-1: “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 19)”: V19.0.0

# Text Proposal

## 5.X	CA_n25-n41

### 5.X.1	Configurations

<Editor’s note: the CA configurations and bandwidth combinations sets should be kept same as defined in 38.101-1 Table 5.5A.3.1-1x unless additional clarification>

Table 5.X.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41C |  | n258 n418,9 CA_n25A-n41A8,13,14 CA_n41C8,9 |  | n25 |  | 5, 10, 15, 20 | 0 |  |
|  |  |  |  | n41 |  | CA_n41C_BCS0 |  |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 | 1 |  |
|  |  |  |  | n41 |  | CA_n41C_BCS1 |  |  |
|  |  | n258 n418,9 CA_n25A-n41A8,13,14 CA_n41C8,9 CA_n25A-n41C8,13,14 |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |  |
|  |  |  |  | n41 |  | CA_n41C_BCS 4 and 5 |  |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination … NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. NOTE 14 Minimum requirements for Power Class 1.5 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |  |  |  |

### 5.X.2	Maximum output power

Table 5.X.2-1 UE Power Class 1.5 for uplink intra-band CA

| Uplink CA configuration | Power class 1.5 cases for CA_nX-nY | CA power class | Carrier X power class |
| --- | --- | --- | --- |
| CA_n41C | Case a | 29dBm | 29dBm |

Table 5.X.2-2 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25A-n41C | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

Table 5.X.2-3 UE Power Class 1.5 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 1.5 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25A-n41C | Case a | 29dBm | 26dBm | 26dBm |
|  | Case b | 29dBm | 23dBm | 29dBm |

### 5.X.3	REFSENS requirements

#### The PC1.5 IMD

#### 5.X.3.1	UL CA_n41C Power Class 1.5 case a

#### The PC1.5 MSD for UL CA_n41C into DL n25 is the same as for PC3 and PC2.

Table 5.X.3-1 UE Power Class 1.5 MSD

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n41 | n25 | N/A | 5 | N/A | 1992.5 | 8.5 | FDD | IMD7 |
|  | n41 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBstart=221) | 2640 |  |  |  |

#### 5.X.3.2	UL CA_n25A-n41C Power Class 2 case a, b, d

For CA_n25A-n41C triple beat the key is that the FDD band increases by 3dB for inter-band PC2 but the CA_n41C is subject to MPR/A-MPR to meet IMD and only increases by 2dB for PC2 vs PC3. Therefore, IMD power increase is 2*3+1*2=8dB for PC2. Thus,

•	PC2 CA_n25A-n41C triple beat 15.3+8=23.3dB

Table 5.X.3-1 UE Power Class 1.5 MSD

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n41 | n25 | 1860 | 5 | 25 | 1940 | 23.3 | FDD | IMD3 |
|  | n41 | 2501 | 10 | 1 (RBstart = 25) | 2501 | N/A | TDD | N/A |
|  |  | 2556 | 100 | 1 (RBstart = 208) | 2556 |  |  |  |

#### 5.X.3.2	UL CA_n25A-n41C Power Class 1.5 case a, b

For CA_n25A-n41C triple beat the key is that the FDD band increases by 6dB with inter-band PC1.5 but the CA_n41C is subject to MPR/A-MPR to meet IMD and only increases by 3dB for PC1.5 vs PC3. Therefore, IMD power increase is 2*6+1*3=15dB for PC1.5. Thus,

•	PC1.5 CA_n25A-n41C triple beat 15.3+15=30.3dB

Table 5.X.3-1 UE Power Class 1.5 MSD

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n41 | n25 | 1860 | 5 | 25 | 1940 | 30.3 | FDD | IMD3 |
|  | n41 | 2501 | 10 | 1 (RBstart = 25) | 2501 | N/A | TDD | N/A |
|  |  | 2556 | 100 | 1 (RBstart = 208) | 2556 |  |  |  |

### 5.X.4	∆TIB and ∆RIB values

#### There is no change compared to the values for PC3 CA.