3GPP TSG-RAN-WG4 Meeting #116 	R4-2509976

# Bangalore, India, 25th – 29th August, 2025

# Title:	TP for TR38.746 for adding HPUE for DL CA_n25-n41-n66 with UL CA_n41-n66

# Source:	T-Mobile USA

# Agenda Item:	6.8.3

# Document for:	Approval

# Introduction

This contribution provides a text proposal for TR38.746[1] adding the following HPUE NRCA combinations with 3Tx PC2 and PC1.5 UL CA_n41-n66.

- CA_n25A-n41A-n66A

# Reference

[1] 3GPP TR38.746: “High power UE (power class 1.5 or 2) for NR inter-band Carrier Aggregation (CA) / Dual Connectivity (DC) with/without Supplementary Uplink (SUL) with high power on TDD band(s); (Release 19)”: V0.2.0

[2] 3GPP TS38.101-1: “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 19)”: V19.0.0

# Text Proposal

## 6.X	CA_n25-n41-n66

### 6.X.1	Configurations

Table 6.X.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A-n66A | n257 n417,9 n667 CA_n25A-n41A7 CA_n25A-n66A7 CA_n41A-n66A7,13,14 |  |  |  |  | n25 |  |  |  | 5, 10, 15, 20 | 0 |
|  |  |  |  |  |  | n41 |  |  |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  |  |  |  | n66 |  |  |  | 5, 10, 15, 20, 40 |  |
|  |  |  |  |  | n25 |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  |  |  |  | n41 |  |  |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  |  |  | n66 |  |  |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  |  | n25 |  |  |  | n25 channel bandwidths in Table 5.3.5-1 |  |  | 4 and 5 |
|  |  |  |  | n41 |  |  |  | n41 channel bandwidths in Table 5.3.5-1 |  |  |  |
|  |  |  |  | n66 |  |  |  | n66 channel bandwidths in Table 5.3.5-1 |  |  |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. NOTE 14 Minimum requirements for Power Class 1.5 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |  |  |  |

### 6.X.2	Maximum output power

<Editor’s note: In table 6.X.2-1, the power class 2 cases supported by the uplink CA should be kept as the same numbering and others that not supported should be removed. >

Table 6.X.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration |  | Power class 2 cases for CA_nX-nY |  | CA power class |  | Carrier X power class |  | Carrier Y power class |  | Status |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41-n66 |  | Case a |  | 26dBm |  | 23dBm |  | 23dBm |  | Completed |  |
|  |  | Case b |  | 26dBm |  | 23dBm |  | 26dBm |  |  |  |
|  |  | Case c |  | 26dBm |  | 26dBm |  | 23dBm |  | New |  |
|  |  | Case d |  | 26dBm |  | 26dBm |  | 26dBm |  | New |  |

Table 6.X.2-2 UE Power Class 1.5 for uplink inter-band CA (Three bands)

| Uplink CA configuration |  | Power class 1.5 cases for CA_n41-n66 |  | CA power class |  | Carrier X power class |  | Carrier Y power class |  | Status |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41-n66 |  | Case a |  | 29dBm |  | 26dBm |  | 26dBm |  | New |  |
|  |  | Case b |  | 29dBm |  | 23dBm |  | 29dBm |  |  |  |
|  |  | Case c |  | 29dBm |  | 29dBm |  | 23dBm |  | New |  |
|  |  | Case d |  | 29dBm |  | 26dBm |  | 29dBm |  |  |  |
|  |  | Case e |  | 29dBm |  | 29dBm |  | 26dBm |  |  |  |

### 6.X.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.X.3.1	Power class 2 case c, d

There is an IMD4 from UL CA_n41A-n66A into the n25 DL. The PC3 MSD is 11.0 dB. The PC2 MSD is 22.7dB in Table 7.3A.5-2a. With the LUT approach it would be 23 dB.

#### 6.X.3.2	Power class 1.5 case c, d

There is an IMD4 from UL CA_n41A-n66A into the n25 DL. The PC3 MSD is 11.0 dB. With the LUT approach the MSD is 35 dB.

### 6.X.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.