3GPP TSG-RAN WG4 Meeting # 116	R4-2510012

# Bengaluru, India, August 25th – 29th, 2025

Source:	LG Electronics

Title:	Discussion on low band carrier aggregation via switching

Agenda item:	7.3.3

Document for:	Discussion

Introduction

In 3GPP RAN4 #115, WF[1] was approved. In this paper, we provide our views on low NR band aggregation via switching.

Issue 1: Agreements related to the time mask requirement - A text proposal to TR38.768 in R4-2507932 captures the relevant agreements - How to specify the DL time mask: - The UE DL behaviour following the switch operation can be discussed in the RRM session - Considering that only one RAN4 meeting is left before the conclusion of the work item, companies are encouraged to exchange views via the RAN4 email reflector in advance of RAN4 #116 to converge on the technical content of the CR to 38.101-1 - RAN4 to further discuss whether the time mask should show the switching period/gap location and whether the diagram similar to Figure 5.2.2-1 and 5.2.3-1 in TR38.768 should be considered. Issue 2: Agreements related to UE capabilities - A new feature group is included in the Rel-19 feature list for NR_LBCA_Sw - Related to concurrent Rx operation, the following aspect is included in the feature group: - [The con-current reception on FDD band and SDL band is not supported. The con-current operation between FDD UL and SDL is not supported.] - Only switchedDL option can be considered for low-low CA downlink carrier switching in this WID. - Support 70us as a candidate value of switching gap for 15kHz SCS in addition to 35us and 140us

Discussion

LS related switching gap

RAN4 discussed switching gap in the RAN4 last meeting. The conclusion is as follow [1].

FDD to SDL Figure 5.2.2-1: FDD to SDL switch with the switching gap on the switched-from carrier. The UL slot will always end before the DL slot, neglecting any timing errors in case TA = 0 µs. The worst case in terms of switching is that in which TTA = 0 µs, the DL and UL timing are the same (again neglecting the errors that are of the order of 1 µs) and the SDL starts RTD µs before the Pcell DL slot timing. The switching gap with respect to the end of the last FDD DL symbol must therefore be at least actual switching gap length > Tswitch + MRTD + FDD UL trailing end transient period + SDL leading transient period. where the MRTD = [CP length/TAE] assuming that the FDD and SDL carriers are colocated. SDL to FDD Figure 5.2.3-1: SDL to FDD switch with the switch occurring before the start of the UL slot. The time mask could be devised such that the switching period occurs before the first symbol of the UL slot of the switched-to FDD carrier, this to avoid dropping symbols on the UL that starts TTA before the first symbol of the FDD DL slot. Notwithstanding, the switching gap on the SDL carrier must be at least Actual switching gap length > Tswitch + TTA + MRTD + SDL trailing transient period + FDD UL leading transient period with account of the SDL received at maximum RTD of a [CPlength/TAE]. The worst case is when the SDL is traling the Pcell DL as shown in the figure.

Swithcing gap was included with transient period and MRTD. RAN1 also discussed switching gap. But it is defined as follow “The UE expects that the switching gap configured by the NW for SDL carrier to FDD carrier switching covers at least the switching period as in RAN4 LS (R1-2501702) and UL TA.”. so RAN4 should send to RAN1 and RAN2 regarding with updated switching gap correspond to our companion Tdoc[2].

And also, regarding [3], this is higher layers parameter list in Rel-19. The value range for swithcing gap duration is still FFS. RAN4 need to inform parameter value consist of switching gap for deciding switching gap duration.

Proposal 1: LS regarding updated switching gap should be sent to RAN1 and RAN2.

CR for MRTD value

The MRTD value from the last RRM session was concluded in RAN4 #115.

Issue 1-1-2: MRTD assumptions for R19 LB CA via switching Agreement - The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us. Whether or not above RTD can be used as side condition for SCell activation will be discussed in another issue

According to above conclusion, it is necceary to change MRTD value in TR38.768.

Proposal 2: it is necceary to change MRTD value in TR38.768 corresponding to our companion Tdoc[4] .

- 2. Conclusion

In this contribution, we want to share some views on low NR band aggregation via switching and proposals are made as following:

Proposal 1: LS regarding updated switching gap should be sent to RAN1 and RAN2.

Proposal 2: it is necceary to change MRTD value in TR38.768 corresponding to our companion Tdoc[4] .

- 3. Reference

[1] TR38.768, “Low NR Band Carrier Aggregation via switching”

[2] R4-2510057,“LS for parameter update request with switching gap”

[3] R1-2503243, “Consolidated Rel-19 higher layers parameters list Post RAN1#121”

[4] R4-2510058, “draft CR for updating MRTD”

-