3GPP TSG-RAN WG4 Meeting #116	   	R4-2510013

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	5.5.2

Source: 	CMCC

Title:			  Discussion on Sharing factor between L1 measurement and RRM measurement for R18 ATG maintenance

Document for:	Discussion

- Introductions

During the R19 ATG discussion, an issue of ‘effective non-overlapping duration of Sharing factor between L1 measurement and RRM measurement’ came into our sight [1]. The progress is as below:

Issue 1-3-2: Sharing factor between L1 measurement and RRM measurement - effective non-overlapping duration

Proposals

- Proposal 1: For the ‘SSB-ToMeasure and SS-RSSI-Measurement are configured’, use MRTD as the reference when calculate the margin for non-overlapping duration

  - Proposal 1-1:

    - Consider impact on Klayer1_measurement due to 33usec MRTD for Case 4 antenna type, e.g.,

      - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbol before each consecutive SSB symbols and the RSSI symbols, and 2 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

  - Proposal 1-2:

    - For UE capable of [antennaArrayType-r18] and [intra-band or inter-band carrier aggregation is configured],

      - Klayer1_measurement=1,

        - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or;

        - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency in different band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbols before each consecutive SSB symbols and the RSSI symbols, and 2 symbols after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured and the UE is capable of [common Rx beam between PCC band and SCC band], where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or;

- Proposal 2:

  - Firstly align the understanding about the reference timing of SSB symbols and the RSSI symbols in the definition of Klayer1_measurement.

    - If the serving cell timing is used as the reference timing of SSB symbols and the RSSI symbols, then extend the margin considering the maximum propagation delay difference between serving cell and neighbour cells.

    - If the neighbour cell timing is used as the reference timing of SSB symbols and the RSSI symbols, then reuse the legacy 1 symbol margin

- Proposal 3:

  - reuse the legacy specification, i.e.

    - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

After our analysis, we identify that it is a Rel-18 maintenance issue. In this contribution, we will give our analysis and solution for this issue.

- Discussion

Sharing factor between L1 measurement and RRM measurement - effective non-overlapping duration

When define the ATG Klayer1_measurement in Rel-18 spec, RAN4 take the FR2 Klayer1_measurement as the reference, Below is an example of ATG Klayer1_measurement for intra-frequency measurement without gap case :

For UE capable of antennaArrayType-r18, Klayer1_measurement=1, - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement; Klayer1_measurement=1.5, otherwise.

In legacy FR2, Klayer1_measurement =1 when the L1-RSs fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured. This was introduced from R15, and only FR2-1 SCS up to 240kHz was considered at that time.

In our understanding, the location of SSB symbols and the RSSI symbols of intra-frequency neighbour cells are determined by serving cell timing and the configured SSB-ToMeasure and SS-RSSI-Measurement. Thus, 1 symbol margin is considered including the maximum propagation delay difference（3µs for 1km cell coverage）and sync margin (3µs for FDD, 1.5µs for TDD) between serving cell and neighbour cells.

Observation 1: The serving cell timing is used as the reference timing of SSB symbols and the RSSI symbols indicated by SSB-ToMeasure and SS-RSSI-Measurement.

The same logic shall be followed for FR1 ATG, that the margin shall consider the maximum propagation delay difference and sync margin between serving cell and neighbour cells. For the ATG scenario, the maximum propagation delay difference is about 1ms if we consider 300km ISD.

As a consequence, if we still want to introduce this ‘L1-RSs fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the effective L3-RSs symbols’ case, the margin shall be extend to at least 1ms, and the modification shall be done from R18, due to this is not the CA specific feature.

From our perspective, with total 2ms margin, this case will not bring too much gain for measurement delay reduction. We prefer to remove this ‘L1-RSs fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the effective L3-RSs symbols’ case from R18.

Proposal 1:Remove this ‘L1-RSs fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the effective L3-RSs symbols’ case from R18.

- Conclusion

In this contribution, we discuss the maintenance issues about the definition of the sharing factor between L1 and L3 measurements. The observation and proposals are:

Observation 1: The serving cell timing is used as the reference timing of SSB symbols and the RSSI symbols indicated by SSB-ToMeasure and SS-RSSI-Measurement.

Proposal 1:Remove this ‘L1-RSs fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the effective L3-RSs symbols’ case from R18.

- Reference

- R4-2508263, WF on RRM requirements for NR_ATG_enh, CMCC