3GPP TSG-RAN WG4 Meeting # 116		R4-2510018

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.8.6

Source: 	CMCC

Title: 			 Discussion on test cases for R19 ATG

Document for:	Discussion

- Introduction

In last meeting, the performance part was discussed and the test method was agreed. The WF has been approved in [1].

In this contribution, we further provide our views on the test scopes and other test setups for ATG.

- Discussion

  - ATG CA test cases

Issue 2-1-1: Test applicability rule

- Proposal 1:

  - UE is only required to be tested in one of the supported test configurations

  - UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration

- Proposal 2:

  - Test configuration for NR PCell and test configuration for NR SCell shall be chosen independently

- To make RRM test case more generic, test configuration for NR PCell and test configuration for NR SCell shall be chosen independently. To reduce the test effort, UE is only required to be tested in one of the supported test configurations, and only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration.

Proposal 1: Test configuration for NR PCell and test configuration for NR SCell shall be chosen independently.

Proposal 2: Introduce following test applicability rule:

- UE is only required to be tested in one of the supported test configurations

- UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration

Issue 2-1-2: Test setups - TDD pattern for SCell

- Proposal 1: Reuse the legacy TDD pattern

  - Proposal 1-1:

    - For intra-band CA, TDD pattern for SCell need to be the same with PCell.

    - For inter-band CA, the pattern used in R18 ATG test cases can also be reused.

  - Proposal 1-2:

    - For TDD case, the TDD pattern for SCell could be same as PCell

- Proposal 2: RAN4 to discuss whether or not to define a new TDD pattern for ATG

- Proposal 3: Define all ‘D’ slot TDD pattern for ATG

- As analyzed by CATT last meeting, when TDD+TDD CA configured, aggregated TDD cell need to have the same TDD configuration to ensure cross carrier scheduling. When FDD+TDD CA configured, although either the legacy TDD pattern or the new TDD pattern with all DL slots can be configured by network, we still prefer to use legacy TDD pattern with both DL and UL slots, which can increase the system capacity. For example, CA n3_n39, n39 configure legacy TDD pattern with both DL and UL slots. For UE1, n39 is SCC, but for UE2, n39 can be PCC. Such configuration give more flexibility to the network.

- To summarize, for all the cases, use legacy TDD pattern for SCell.

Proposal 3: Use legacy TDD pattern for SCell.

Issue 2-1-4: Test scope

FFS based on following summary:

- In RAN4#115, following test cases supported by no less than 5 companies

  - Interruptions during measurements on deactivated NR SCC in FR1

  - Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot

  - Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot

  - SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification)

  - SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle (together with interruption requirement verification)

  - SCell Activation and deactivation of unknown SCell in FR1 in non-DRX (together with interruption requirement verification)

  - Direct SCell activation at SCell addition of known SCell in FR1 (together with interruption requirement verification)

  - Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification)

  - Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle (together with interruption requirement verification)

  - SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification)

- In RAN4#115, no supporting company for following test cases

  - Direct SCell Activation at RRC Resume

  - non-simultaneous Timer based BWP switch delay on multiple CCs

- In RAN4#115, 1-4 supporting companies for following test cases

  - Direct SCell activation at handover with known SCell in FR1

  - TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX

  - Inter-band SSB-less SCell Activation based on A-TRS

  - Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode for ATG

  - Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode

  - Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs

  - Simultaneous RRC-based Active BWP Switch on multiple CCs

  - SCell dormancy switch

  - Event triggered reporting on SCC with deactivated SCell test under non-DRX

  - Test case for Pre-configured measurement gap requirement

  - Intra-frequency measurement(with scheduling restriction verified as well)

  - Inter-frequency measurement(with scheduling restriction verified as well)

  - L1-RSRP measurement

  - CSI-RS based intra and inter frequency Measurements for event-triggered reporting tests without gap under non-DRX

  - The interruption of SCell addition/release

- we support to introduce the following test cases with the highest priority, since it is foreseeable that this will be commonly used and fundamental features in ATG CA network:

- SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification)

- SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle (together with interruption requirement verification)

- SCell Activation and deactivation of unknown SCell in FR1 in non-DRX (together with interruption requirement verification)

- Direct SCell activation at SCell addition of known SCell in FR1 (together with interruption requirement verification)

- Direct SCell activation at handover with known SCell in FR1

- Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode for ATG

- Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification and neighbour cell measurement skipping verification)

- Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640ms SCell measurement cycle (together with interruption requirement verification)

- We also support to introduce the following test cases with medium priority, since the legacy test cases can be taken as reference, minor specification work is expected, and the features are also useful in ATG network

- Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot

- Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot

- Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification)

- Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle (together with interruption requirement verification)

- SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification)

- TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX

- Inter-band SSB-less SCell Activation based on A-TRS

- For the following other test cases, although we think it may not common used in ATG network, but we are also ok to introduce the test cases to guarantee the test coverage.

- Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs

- Simultaneous RRC-based Active BWP Switch on multiple CCs

- SCell dormancy switch

- Proposal 4: Define following test cases:

- High priority:

- SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification)

- SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle (together with interruption requirement verification)

- SCell Activation and deactivation of unknown SCell in FR1 in non-DRX (together with interruption requirement verification)

- Direct SCell activation at SCell addition of known SCell in FR1 (together with interruption requirement verification)

- Direct SCell activation at handover with known SCell in FR1

- Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode for ATG

- Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification and neighbour cell measurement skipping verification)

- Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640ms SCell measurement cycle (together with interruption requirement verification)

- Medium priority:

- Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot

- Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot

- Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification)

- Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle (together with interruption requirement verification)

- SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification)

- TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX

- Inter-band SSB-less SCell Activation based on A-TRS

- Low priority:

- Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs

- Simultaneous RRC-based Active BWP Switch on multiple CCs

- SCell dormancy switch

Issue 2-1-5: Detail test configuration and test procedure

Proposals

- Proposal 1:

  - If the ‘deactivated SCell measurement with measCycleSCell larger than 640ms’ test case will be introduced eventually, then verify the UE interruption performance on deactivated SCC measurement together in ‘deactivated SCell measurement’ test. Otherwise, introduce a separate interruption test case

  - For the test ‘Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs’, one PCell and one SCell shall be configured, and BWP switching happens on PCell and SCell simultaneously.

- Proposal 2:

  - Inter-band SSB-less SCell Activation test cases should be restricted to ATG UE explicitly supporting [Case 4 capability]

Then we provide some detail test configuration and test procedures.

Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640ms SCell measurement cycle & Interruptions during measurements on deactivated NR SCC in FR1

If the ‘Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640ms SCell measurement cycle’ test case will be introduced eventually, interruption performance (Interruptions during measurements on deactivated NR SCC in FR1) can be verified together in the ‘deactivated SCell measurement’ test case. Otherwise, a separate interruption test shall be introduced.

Proposal 5: If the ‘Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640ms SCell measurement cycle’ test case will be introduced eventually, then verify the UE interruption performance on deactivated SCC measurement together in ‘deactivated SCell measurement’ test. Otherwise, introduce a separate interruption test case.

Direct SCell Activation at Handover

In legacy, the scenario ‘handover without SCell change’ is tested. However, we think it is not the common scenario in ATG, therefore, we propose to update the test scenario as:

At the start of time duration T1, the UE is in connected mode with PCell (Cell 1) and UE is reporting CQI for PCell. The UE is configured to measure RF channel 2 in measurement gaps. During T1, the UE detects and measures Cell 2 on RF channel 2, and sends a measurement report containing Cell 2 to the test equipment. After having received a measurement report containing Cell 2, the test equipment deconfigures the measurement gaps and thereafter sends a RRC connection reconfiguration message to the UE. The time between reception of the last measurement report carrying SCell and transmission of the RRC connection reconfiguration message directly activating SCell is kept short enough to allow the SCell to remain known to the UE.

Time period T2 starts when UE receives a handover command to PCell (Cell 3) that also activates SCell 1 (Cell 2). This is done using an RRCReconfiguration message with parameter sCellState set to activated for the SCell 1 (Cell 2). The message is sent from the test equipment to the UE and is received in a subframe # denoted n at the UE antenna connector. The UE shall accomplish the activation of the SCell no later than subframe (n + Ndirect).

Time period T3 starts at (n + Ndirect), at which point UE shall be reporting a valid CQI for both PCell and SCell 1.

Proposal 6: Update the test scenario for Direct SCell Activation at Handover test to:

- At the start of time duration T1, the UE is in connected mode with PCell (Cell 1) and UE is reporting CQI for PCell. The UE is configured to measure RF channel 2 in measurement gaps. During T1, the UE detects and measures Cell 2 on RF channel 2, and sends a measurement report containing Cell 2 to the test equipment. After having received a measurement report containing Cell 2, the test equipment deconfigures the measurement gaps and thereafter sends a RRC connection reconfiguration message to the UE. The time between reception of the last measurement report carrying SCell and transmission of the RRC connection reconfiguration message directly activating SCell is kept short enough to allow the SCell to remain known to the UE.

- Time period T2 starts when UE receives a handover command to PCell (Cell 3) that also activates SCell 1 (Cell 2). This is done using an RRCReconfiguration message with parameter sCellState set to activated for the SCell 1 (Cell 2). The message is sent from the test equipment to the UE and is received in a subframe # denoted n at the UE antenna connector. The UE shall accomplish the activation of the SCell no later than subframe (n + Ndirect).

- Time period T3 starts at (n + Ndirect), at which point UE shall be reporting a valid CQI for both PCell and SCell 1.

Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs

For the test ‘Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs’, in legacy, one PCell and two SCells was configured, and BWP switching happens on two SCells. Such test design can not be reused for Rel-19 ATG with one SCell working assumption. Therefore, one PCell and one SCell shall be configured, and BWP switching happens on PCell and SCell.

Proposal 7: For the test ‘Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs’, one PCell and one SCell shall be configured, and BWP switching happens on PCell and SCell simultaneously.

- Conclusion

Based on the discussion above, the following observation and proposals are concluded.

Proposal 1: Test configuration for NR PCell and test configuration for NR SCell shall be chosen independently.

Proposal 2: Introduce following test applicability rule:

- UE is only required to be tested in one of the supported test configurations

- UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration

Proposal 3: Use legacy TDD pattern for SCell.

- Proposal 4: Define following test cases:

- High priority:

- SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification)

- SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle (together with interruption requirement verification)

- SCell Activation and deactivation of unknown SCell in FR1 in non-DRX (together with interruption requirement verification)

- Direct SCell activation at SCell addition of known SCell in FR1 (together with interruption requirement verification)

- Direct SCell activation at handover with known SCell in FR1

- Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode for ATG

- Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification and neighbour cell measurement skipping verification)

- Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640ms SCell measurement cycle (together with interruption requirement verification)

- Medium priority:

- Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot

- Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot

- Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification)

- Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle (together with interruption requirement verification)

- SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification)

- TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX

- Inter-band SSB-less SCell Activation based on A-TRS

- Low priority:

- Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs

- Simultaneous RRC-based Active BWP Switch on multiple CCs

- SCell dormancy switch

Proposal 5: If the ‘Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640ms SCell measurement cycle’ test case will be introduced eventually, then verify the UE interruption performance on deactivated SCC measurement together in ‘deactivated SCell measurement’ test. Otherwise, introduce a separate interruption test case.

Proposal 6: Update the test scenario for Direct SCell Activation at Handover test to:

- At the start of time duration T1, the UE is in connected mode with PCell (Cell 1) and UE is reporting CQI for PCell. The UE is configured to measure RF channel 2 in measurement gaps. During T1, the UE detects and measures Cell 2 on RF channel 2, and sends a measurement report containing Cell 2 to the test equipment. After having received a measurement report containing Cell 2, the test equipment deconfigures the measurement gaps and thereafter sends a RRC connection reconfiguration message to the UE. The time between reception of the last measurement report carrying SCell and transmission of the RRC connection reconfiguration message directly activating SCell is kept short enough to allow the SCell to remain known to the UE.

- Time period T2 starts when UE receives a handover command to PCell (Cell 3) that also activates SCell 1 (Cell 2). This is done using an RRCReconfiguration message with parameter sCellState set to activated for the SCell 1 (Cell 2). The message is sent from the test equipment to the UE and is received in a subframe # denoted n at the UE antenna connector. The UE shall accomplish the activation of the SCell no later than subframe (n + Ndirect).

- Time period T3 starts at (n + Ndirect), at which point UE shall be reporting a valid CQI for both PCell and SCell 1.

Proposal 7: For the test ‘Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs’, one PCell and one SCell shall be configured, and BWP switching happens on PCell and SCell simultaneously.

- Reference

- R4-2508263, WF on RRM requirements for NR_ATG_enh, CMCC