3GPP TSG-RAN WG4 Meeting #116      		R4-2510019

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.8.7

Source: 	CMCC

Title: 			 Discussion on demodulation requirements for Rel-19 ATG

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed test scope and test setups about DL CA, DL MIMO and UL MIMO feature. The WF has been approved in [1]. In this contribution, we further give our views on test setups based on our simulation results in [2].

- ATG UE DL CA demodulation requirement

Issue 1-1-2-1: TDD pattern

- Option 1: Use the legacy TDD pattern as the starting point

- Option 2: Define new TDD pattern 30D4S6U for R-19 enhanced ATG DL CA requirement

Since there are still some potential maintenance issues about the new TDD pattern we introduced in Rel-18, We propose to use legacy TDD pattern.

Proposal 1: Use the legacy TDD pattern.

Issue 1-1-2-2: Rank

- Option 1: For CA requirement, use Rank 2 as the starting point

- Option 2: Only consider Rank 1 for DL CA demodulation requirements

Although UE antenna type is transparent due to only conducted test for ATG, considering R19 DL-MIMO Rank2 requirements will only be applied to UE capable of dual-polarization antenna, in order to make CA requirements applicable for all ATG UEs, we are fine to compromise to Rank 1.

Proposal 2: For CA requirement, use Rank 1.

Issue 1-1-2-3: MCS

- Option 1: Consider 64QAM MCS22 for ATG downlink CA requirements

- Other Options are not precluded

In RAN4#114bis, RAN4 firstly agreed that 16QAM MCS 13, 64QAM MCS 22, 256QAM MCS 24 will be used for initial simulation.

Based on our simulation results, for rank 1, 256QAM MCS24 is within the testable range for both 2T2R (19 dB) and 2T4R (15.5dB), since only conducted test will be performed.

Proposal 3: Consider 256QAM MCS24 for ATG downlink CA requirements.

- Issue 1-1-3: CA configuration selection rules

- Option 1: Define the CA configuration selection rules as

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |

- Issue 1-1-4: Test applicability rules

- Option 1: Define the test rules as:

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| FDD 15 kHz + FDD 15 kHz | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| TDD 30 kHz + TDD 30 kHz | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| FDD 15 kHz + TDD 30 kHz | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

- Option 2: Define the test rules as:

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| Test x in Clause 5.2A.2.x and 5.2A.3.x | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| Test x in Clause 5.2A.2.x and 5.2A.3.x | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| Test x in Clause 5.2A.2.x and 5.2A.3.x | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

In high level, we think the legacy Rel-16 CA configuration selection rule and test applicability rule can be reused for Rel-19 ATG CA, but some refinement need to be done with the consideration of ATG specific feature, including:

- Only intra-band contiguous CA and inter-band CA are included in WID

- For FDD 15kHz +TDD 30kHz case, (e.g., Example band combination: CA_n3A-n39A), FDD CC is configured as PCC

- Only FDD 15kHz and TDD 30kHz are supported for Rel-18 single carrier requirement

- In last meeting, RAN4 agreed only TDD 30 kHz + TDD 30 kHz and  FDD 15 kHz + TDD 30 kHz will be specified for ATG network.

- Rank 1 will be used when define the requirements

After the refinement, we propose following CA configuration selection rule and test applicability rule for Rel-19 ATG CA:

Proposal 4: Define the test rules as:

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| Test x in Clause 5.2A.2.x and 5.2A.3.x （Note: for TDD 30 kHz + TDD 30 kHz） | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| Test x in Clause 5.2A.2.x and 5.2A.3.x (Note: for FDD 15 kHz + TDD 30 kHz) | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

Proposal 5: Define the CA configuration selection rules for the UE with dual-polarization antenna as:

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |

- DL MIMO related UE demodulation requirement

Issue 2-1-2: TDD pattern

- Option 1: Use legacy TDD pattern 7DS2U as the starting point.

Since there are still some potential maintenance issues about the new TDD pattern we introduced in Rel-18, We propose to use legacy TDD pattern for TDD tests.

Proposal 6: Use legacy TDD pattern.

Issue 2-1-3: MCS

- Option 1: consider MCS 13 and MCS 22

- Option 2: Consider 64QAM

- Other Options are not precluded

According to our simulation results, for the SNR of the 70% TP, all 16QAM MCS 13, 64QAM MCS 22 and 256QAM MCS 24 are within the testable range, since only conducted test will be performed. We propose to consider them all for DL-MIMO requirements.

Proposal 7: Use 16QAM Table 1 MCS13, 64QAM Table 1 MCS 22 and 256QAM Table 2 MCS 24 to define the DL-MIMO requirements.

- UL MIMO related BS Demodulation requirement

Issue 3-1-4: Antenna configuration for incremental test

- Option 1: 2T2R

- Option 2: 2T2R, 2T4R and 2T8R with X-pol low correlation as the starting point

- Option 3: 2T2R with low correlation

As stated in WID, it is feasible for UE with dual-polarization antenna to support UL MIMO. Therefore, 2T2R, 2T4R, and 2T8R with XP low correlation can be taken as the starting point.

Proposal 8: For incremental test case, take 2T2R, 2T4R and 2T8R with X-pol low correlation as the starting point.

Issue 3-1-10: MCS for incremental test

Agreement:

- 16QAM Table 1 MCS12, 64QAM Table 1 MCS 20 and [256QAM Table 2 MCS 20] for initial simulation

- Only one MCS will be used

Based on our simulation results, for the SNR of the 70% TP, all 16QAM , 64QAM and 256QAM MCS 20 are within the testable range, since only conducted test will be performed.

However, in last meeting, we agreed that only one MCS will be used. Considering the pusch-256QAM is an optional UE capability, we propose to use 64QAM Table 1 MCS 20, higher modulation is more likely to be scheduled to meet the high throughput demand in ATG network.

Proposal 9: Use 64QAM Table 1 MCS 20 for UL-MIMO requirements

- Conclusion

Based on the discussion above, the following proposals are concluded.

DL CA

Proposal 1: Use the legacy TDD pattern.

Proposal 2: For CA requirement, use Rank 1.

Proposal 3: Consider 256QAM MCS24 for ATG downlink CA requirements.

Proposal 4: Define the test rules as:

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| Test x in Clause 5.2A.2.x and 5.2A.3.x （Note: for TDD 30 kHz + TDD 30 kHz） | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| Test x in Clause 5.2A.2.x and 5.2A.3.x (Note: for FDD 15 kHz + TDD 30 kHz) | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

Proposal 5: Define the CA configuration selection rules for the UE with dual-polarization antenna as:

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 1. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |

DL MIMO

Proposal 6: Use legacy TDD pattern.

Proposal 7: Use 16QAM Table 1 MCS13, 64QAM Table 1 MCS 22 and 256QAM Table 2 MCS 24 to define the DL-MIMO requirements.

UL MIMO

Proposal 8: For incremental test case, take 2T2R, 2T4R and 2T8R with X-pol low correlation as the starting point.

Proposal 9: Use 64QAM Table 1 MCS 20 for UL-MIMO requirements

- Reference

- R4-2508724, Way Forward for [115][328] NR_ATG_enh_demod, CMCC

- R4-2510020, Simulation results for Rel-19 ATG, CMCC