3GPP TSG-RAN WG4 Meeting #116    	    	 R4-2510021

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.13.3

Source: 	CMCC

Title: 			 Discussion on interference suppressing performance for 8Rx UE

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed some general and test applicability issues for 8Rx UE, the WF has been approved in [1].

In this contribution, we further discuss the remaining open issue about receiver definition.

- Discussion

Definition of MMSE-IRC receiver capability for 8Rx UE

Options:

- Option 1: 
Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following:
- Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Option 2: 
Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following:
- Baseline 8Rx Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Option 3: 
Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following:
- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Option 4:
Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following:
- Baseline 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Option 5:
Re-use existing definition in Rel-18.

Among the options, we believe that the controversial part is whether to declare ‘MMSE-IRC’ in the receiver definition.

In Rel-18 8Rx WI, the requirement was defined with the receiver assumption of ‘MMSE-IRC’, but the receiver type was not explicitly mentioned in the spec, which means for the UE capable of more advanced receiver with even better performance can also declare the supporting of ‘Baseline SU-MIMO 8Rx Receiver’ and ‘Simplified SU-MIMO 8Rx Receiver’, and pass the related test cases.

If companies want to reuse the same logic for ICI and IUI tests, Option 3 may not the good choice which may cause some inconsistent issue. For example, for a R-ML receiver UE which declare the supporting of ‘Baseline 8Rx Receiver’, it may pass the basic 8Rx test cases, but fail to pass the 8Rx ICI and IUI test cases. Therefore, we think a new receiver capability shall be defined for 8Rx ICI and IUI test cases without explicit receiver type.

If companies want to limit the receiver type only for ICI and IUI tests, then a new receiver capability shall be defined for 8Rx ICI and IUI test cases with explicit receiver type.

- Observation 1: Option 3 may cause some inconsistent issue. For example, for a R-ML receiver UE which declare the supporting of ‘Baseline 8Rx Receiver’, it may pass the basic 8Rx test cases, but fail to pass the 8Rx ICI and IUI test cases.

We are also fine with Option 1 or Option 2, which give the receiver type explicitly in the definition. The drawback is that the test applicability range will be reduced to only ‘MMSE-IRC’ capable UE.

- Observation 2: The drawback of Option 1 and Option 2 is that the test applicability range of both R18 and R19 tests will be reduced to only ‘MMSE-IRC’ capable UE.

With the analyze above, we think we should firstly get the consensus of whether the advanced receiver need to be covered by test cases in R18 and R19 separately. Then find the way forward.

- Conclusion

In this contribution, we provide our opinion about test applicability rule and release independent. The proposals are:

- Observation 1: Option 3 may cause some inconsistent issue. For example, for a R-ML receiver UE which declare the supporting of ‘Baseline 8Rx Receiver’, it may pass the basic 8Rx test cases, but fail to pass the 8Rx ICI and IUI test cases.

- Observation 2: The drawback of Option 1 and Option 2 is that the test applicability range of both R18 and R19 tests will be reduced to only ‘MMSE-IRC’ capable UE.

- Reference

[1] R4-2508620, Way forward for [115][323] NR_demod_Ph5_Part2_UE, Moderator (Nokia)