3GPP TSG-RAN WG4 Meeting#116     		R4-2510024

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.13.4

Source: 	CMCC

Title: 			 Simulation results for BS interference suppressing with MMSE-IRC

Document for:	Discussion

- Introduction

Based on the simulation assumption approved in last meeting[1], and the new MMSE-IRC algorithm that using both all REs on DMRS symbol to derive the interference covariance matrix, we give our simulation results.

- Simulation results

PUSCH requirements with interference (CP-OFDM/FDD, 5MHz)

|  |  | SNR for HomoNet | SNR for HetNet | SNR for HomoNet (new algorithm) | SNR for HetNet (new algorithm) |
| --- | --- | --- | --- | --- | --- |
| 1T2R | MCS 16 | 11.32 | 12.13 | 12.62 | 14.4 |
| 1T4R | MCS 16 | 7.47 | 8.13 | 7.02 | 7.93 |
| 1T8R | MCS 16 | 4.11 | 4.27 | 2.79 | 3.36 |

PUSCH requirements with interference (CP-OFDM/FDD, 20MHz)

|  |  | SNR for HomoNet | SNR for HetNet | SNR for HomoNet (new algorithm) | SNR for HetNet (new algorithm) |
| --- | --- | --- | --- | --- | --- |
| 1T2R | MCS 16 | 11.04 | 12.07 | 13.5 | 12.81 |
| 1T4R | MCS 16 | 7.39 | 8.21 | 7.32 | 7.91 |
| 1T8R | MCS 16 | 4.09 | 4.32 | 3.68 | 3.9 |

PUSCH requirements with interference (CP-OFDM/TDD, 10MHz)

|  |  | SNR for HomoNet | SNR for HetNet | SNR for HomoNet (new algorithm) | SNR for HetNet (new algorithm) |
| --- | --- | --- | --- | --- | --- |
| 1T2R | MCS 16 | 11.78 | 13.22 | 13.2 | 13.87 |
| 1T4R | MCS 16 | 8.14 | 9.15 | 7.44 | 7.9 |
| 1T8R | MCS 16 | 4.7 | 5.24 | 3.27 | 3.3 |

PUSCH requirements with interference (CP-OFDM/TDD, 100MHz)

|  |  | SNR for HomoNet | SNR for HetNet | SNR for HomoNet (new algorithm) | SNR for HetNet (new algorithm) |
| --- | --- | --- | --- | --- | --- |
| 1T2R | MCS 16 | 11.66 | 12.87 | 13.2 | 13.42 |
| 1T4R | MCS 16 | 8.18 | 9.21 | 8.12 | 8.4 |
| 1T8R | MCS 16 | 4.7 | 5.27 | 3.49 | 3.48 |

- Simulation assumption

PUSCH requirements with interference

Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD, FDD |
| TDD pattern |  | 7D1S2U S=6D:4G:4U |
| SCS(kHz)/Bandwidth (MHz) |  | TDD: 30kHz/10MHz, 100MHz FDD: 15kHz/5MHz, 20MHz |
| Transmit precoding |  | Disabled |
| Antenna configuration |  | 1x2 (1 interferes) 1x4 (2 interferes) 1x8 (2 interferes) |
| Propagation condition |  | For serving cell: TDLA30-10 HetNet and TDLC300-100 for HomNet For Interference cell: TDLC300-100 |
| MCS |  | 2R: MCS 16 4R: MCS 16 8R: MCS 16 |
| PUSCH configuration | Mapping type | Type A/B |
|  | Starting symbol (S) | 0 |
|  | Length (L) | 14 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 1 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | DM-RS sequence generation | nSCID =0 for all cells Different NID0 for serving and interference cells NID0=0, 1 and 2 for cell #0 (serving), cell #1 and cell#2 (interfering) respectively |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
| Maximum HARQ transmissions |  | 4 |
| Redundancy version coding sequence |  | {0,2,3,1} |
| Test metric |  | SNR@70% max TP |

| Physical cell ID | Physical cell ID of 0 for the serving cell, and cell ID 1 for interferer 1, cell ID 2 for interferer 2 |
| --- | --- |
| INR values for Homogeneous deployment | INRs 5.35 dB in case of 1 interference cells INRs 7.28, -2.52 dB in case of 2 interference cells |
| INR values for Heterogeneous deployment | INRs 9.82 dB in case of 1 interference cells INRs 12.38, -0.97 dB in case of 2 interference cells |
| Modulation order of interfering PUSCH | 16QAM randomly modulated symbols |
| Interference cell modeling | Interfering cell has same channel model as for serving cell and it is independently generated for each cell. |

- Reference

[1] R4-2508618, Way Forward for [115][322] NR_demod_Ph5_Part1_General_BS, China Telecom