3GPP TSG-RAN WG4 Meeting #116		R4-2510028

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.23.3.2

Source: 	CMCC

Title: 			  Discussion on RRM requirement of signal adaptation for NES enhancement

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed SSB signal adaptation related issues, agreements and open issues have been approved in the WF [1]. In this paper, we further discuss open issues, and identify RAN4 impacts according to the progress of other working group.

- Discussion

Issue 1-3-4: Colliding with MG

Proposals

- Option 1: If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

For current L3 measurement, when SMTC is overlapping with MG, the measurement in MG has higher priority, and this is reflected in the scaling factor Kp. We think this principle shall also be followed for the adapted SSB and related additional SMTC.

- Proposal 1: When the selected additional SMTC is overlapping with MG, the legacy measurement principle and scaling factor Kp shall be reused.

Issue 1-4-2: SCell activation requirement for SSB adaptation – applicable SCell activation requirements

Proposals

- Option 1: R18 Scell activation with L3 reporting

- Option 2: multiple SCells activation scenario with SSB adaptation

Agreement from RAN4#114bis:

- Normal SCell activation

- Direct SCell activation

- PUCCH SCell activation

- FFS Rel-18 SCell activation with L3 reporting

- FFS Multiple SCells activation

Agreement from RAN4#114:

- For SCell activation:

- Define the SCell activation requirements based on the SSB periodicity after SSB adaptation.

  - For the case SSB adaptation command comes together with SCell activation command.

Agreement from RAN4#115:

- For the conditions for SCell activation with SSB adaptation, SSB adaptation command comes no later than SCell activation command.

In RAN4#114bis meeting, RAN4 agreed to define Direct SCell activation requirements based on the SSB periodicity after SSB adaptation, for the case that SSB adaptation command comes no later than SCell activation command as revised in RAN4#115.

However, for Direct SCell activation, before SCell activation command, there is no SCell, so no SSB adaptation can be applied for this Cell. Furthermore, the SCell activation requirement can not be defined by SSB periodicity after SSB adaptation.

- Proposal 2: The SSB adaptation based SCell activation requirements can not be applied for ‘Direct SCell activation’.

- Conclusion

Based on the discussion above, the following proposals are concluded.

- Proposal 1: When the selected additional SMTC is overlapping with MG, the legacy measurement principle and scaling factor Kp shall be reused.

- Proposal 2: The SSB adaptation based SCell activation requirements can not be applied for ‘Direct SCell activation’.

- Reference

- R4-2508441, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei