3GPP TSG-RAN WG4 Meeting #116		R4-2510030

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.24.5.2

Source: 	CMCC

Title: 			 Discussion on RRM impact of LP-WUR

Document for:	Discussion

- Introduction

In last meeting, RAN4 further discussed some general issues and detail LP-WUR requirements and simulation assumption related issues. The WF has been approved in [1]. In this contribution, we further analyze the RRM impact of LP-WUR, and provide our proposals.

- Discussion

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

Continue discussion

Agreement at RAN4 114:

- At legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation.

Recommendations at RAN4#114bis:

Further discuss whether LR evaluation requirements apply when LR is “ON” at legacy state.

- In legacy state, the LR threshold can be configured as entry condition by network, and if configured, UE must evaluate the LR based criterion. UE can entry the LP-WUS monitoring state or RRM offloading/relaxation state only of both MR threshold and LR threshold are fulfilled. Based on this, it is better to have LR evaluation requirements when LR is ‘ON’.

Proposal 1: The LR evaluation requirements will be applied when LR is ‘ON’ at legacy state.

Issue 1-1-11: RRM requirements for FR2

Continue discussion

- So far, all the discussion focus on FR1, and no company run simulation for FR2. Considering core part of the WI near the end, it seems infeasible to define FR2 RRM requirements.

Proposal 2: Don’t introduce FR2 RRM requirements for LP-WUS WI.

Issue 1-2-1-1: Detail on LR accuracy and side conditions requirements

Continue discussion

Agreement from RAN4#114bis:

For FR1:

    - [3.5] dB can be used for core requirements for LP-RSRQ accuracy and [5.5 or 6] dB can be used for core requirements for LP-RSRP accuracy, under the side conditions Ês/Iot [x, x = -3] dB

    - [3.5] dB can be used for core requirements of SSB based RSRQ accuracy and [5.5 or 6] dB can be used for core requirements for SSB based RSRP accuracy, under the side conditions Ês/Iot [y, y = -3] dB

    - Note: [2 or 2.5] dB RF impairment margin is assumed

In RAN4#114-bis, the target accuracy for LR measurement is agreed with the RF margin assumed as [2 or 2.5] dB. For MR, 1dB-2.5dB are used as RF margin when define the RRM and Demod requirement. For LR, we are ok to use 2.5dB considering the lower RF capability of LR.

Proposal 3: Use 2.5dB for LR RF impairment margin.

Issue 1-2-2-2: Upper bound on SSB-based LP-WUR measurement periodicity

Continue discussion

Agreement from RAN4#113:

If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements

- based on LO periodicity

  - FFS whether to define an upper bound

  - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement.

When LP-SS is configured, apply the same agreement as for when LP-SS is not configured

- The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity.

Agreement from RAN4#114bis:

Regardless of  whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

In past meetings, RAN4 has agreed to define SSB based LR measurement requirement by using LO periodicity. On top of this, whether an upper bound is needed keeps FFS.

Although upper bound could help with state transition in time due to shorter evaluation delay, we are fine not to introduce this upper bound with following two considerations. First, upper bound will decrease the UE power saving gain. Second, the LO periodicity can be configured by network.

Proposal 4: No upper bound is needed on top of LO periodicity.

Issue 1-5-2: eDRX related

Contribution driven

As agreed before, for LP-WUS UE entry RRM relaxation mode(case 3), the relaxation factor will be 16 regardless of DRX cycle configuration.

However, we think whether 16 times relaxation can also be applied for eDRX scenario shall be further discussed. For eDRX, the detection/evaluation delay has already been very long with longer cycle, e.g., 5.12 or 10.24s, if 16 times relaxation will be applied on top of current requirement, then for eDRX cycle 10.24s, the evaluation requirement will reach 327.68s, the cell detection requirement will reach 3768.32s, which about 1h. UE may not perform neighbour cell measurement very well with such long evaluation/detection delay. Therefore, we prefer not to define requirement for the combination of eDRX feature and LP-WUS feature.

Proposal 5: Don’t define requirements for the combination of eDRX and LP-WUS feature.

- Conclusion

Based on the discussion above, the following observation and proposals are concluded.

Proposal 1: The LR evaluation requirements will be applied when LR is ‘ON’ at legacy state.

Proposal 2: Don’t introduce FR2 RRM requirements for LP-WUS WI.

Proposal 3: Use 2.5dB for LR RF impairment margin.

Proposal 4: No upper bound is needed on top of LO periodicity.

Proposal 5: Don’t define requirements for the combination of eDRX and LP-WUS feature.

- Reference

- R4-2508280, WF on RRM requirements for NR_LPWUS, Moderator (vivo)