3GPP TSG-RAN WG4 Meeting # 116		R4-2510037

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.27.6

Source: 	CMCC

Title: 			 Discussion on test cases for Rel-19 NTN Phase3

Document for:	Discussion

- Introduction

In last meeting, the discussion of performance part was started, and the WF has been agreed at[1]. In this contribution, we will provide our views on the test case list for R19 NTN, including Redcap with NTN and DL CE feature.

- Discussion

Issue 2-1-2: Test configuration and parameters for (e)RedCap UE with FR1-NTN

<Way Forward>:

- Proposal 1 (CATT):

  - Both FDD and HD-FDD duplex mode should be considered in test configuration for (e)RedCap UE with FR1-NTN.

  - 15 kHz SCS and 10MHz bandwidth can be configured in test cases.

  - Both GSO and NGSO are configured in test cases for RedCap UE with FR1-NTN.

  - Proposal 1a (ZTE): RAN4 shall consider the following general test configurations as follows:

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 4 | NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

- Proposal 2 (HW): Adapt the test case for reduced BW, e.g. test configuration, some RMC and Io values

- Proposal 3 (CMCC): Take the NTN test setups as the reference, and use the ‘refer to’ method when drafting the spec to avoid duplicated contents.

- Proposal 4 (ZTE): RAN4 only clarify the procedure and parameters which can be reused from NTN, and then define new parameters applicable to RedCap UEs.

- For the test configurations, we think most of the NTN test setups can be taken as the reference, except both FDD and HD-FDD shall be tested for (e)RedCap UE.

Proposal 1: Take the NTN test setups as the reference, except both FDD and HD-FDD shall be involved in test configuration.

Issue 2-1-3: How and where to capture the test cases for 1Rx and 2Rx (e)RedCap UE with NTN

<Way Forward>:

- Proposal 1(ZTE): The existing NTN test cases can be used as a baseline for RedCap UEs in NTN scenario.

  - Proposal 1a (HW): Re-use existing test cases for NTN in A.14 for RedCap in NTN

    - Adapt the test case for reduced BW, e.g. test configuration, some RMC and Io values

- Proposal 2 (ZTE): For RedCap UEs in NTN scenario, some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

- About where, since we have a new suffix for RedCap over NTN core requirements, we prefer to set a new Clause for perf requirement correspondingly.

- About how, the ‘refer to’ method shall be used in order to avoid huge duplicated contents in the spec.

Proposal 2: Prefer to set a new Clause for perf requirement, and use the ‘refer to’ method when drafting the spec to avoid duplicated contents.

Issue 2-1-4-1: Whether to define separate test cases for 1Rx and 2Rx (e)RedCap UE with NTN

<Way Forward>:

- Proposal 1 (CATT, CMCC): RAN4 to define the separate test cases for 1Rx and 2Rx (e)RedCap UE with NTN.

- Proposal 2 (HW): Further discuss whether to define separate test cases for 1Rx and 2Rx RedCap UEs

- Proposal 3 (ZTE): Discuss case by case.

- In legacy test cases for RedCap UE, separate test cases are defined for 1Rx and 2Rx (e)RedCap UE, due to some test setups and test requirements may different, but a lot of duplication exists in the spec. However, if the ‘refer to NTN clause’ method will be used, then most of duplication can be avoided. Therefore, we prefer to use same logic as legacy, that define the separate test cases for 1Rx and 2Rx.

Proposal 3: Define the separate test cases for 1Rx and 2Rx (e)RedCap UE with NTN.

Issue 2-1-4-2: General test scope

- Since new requirements have been defined for RedCap UE in NTN network, the corresponding test cases shall also be involved. Based on current progress of the core requirement, we identified that at least following test cases shall be defined for RedCap UE over NTN.

- Intra-frequency Cell Re-selection for both 1Rx and 2Rx RedCap UE

- Intra-frequency cell re-selection

- Intra-frequency cell re-selection with enhanced measurement

- Time-based intra-frequency cell re-selection

- Location-based intra-frequency cell re-selection

- Intra-frequency cell re-selection with UE fulfilling relax measurement criteria

- Inter-frequency Cell Re-selection for both 1Rx and 2Rx RedCap UE

- Inter-frequency cell re-selection

- Inter-frequency cell re-selection with enhanced measurement

- Time-based inter-frequency cell re-selection

- Location-based inter-frequency cell re-selection

- Inter-frequency cell re-selection with UE fulfilling relax measurement criteria

- Inter-frequency cell re-selection from NR NTN to NR TN

- Inter-RAT cell re-selection from NR NTN to E-UTRAN TN both 1Rx and 2Rx RedCap UE

- Intra-frequency Handover for both 1Rx and 2Rx RedCap UE

- Intra-frequency Handover to known target cell

- Intra-frequency Handover to unknown target cell

- Time-based Intra-frequency Conditional Handover

- Location-based Intra-frequency Conditional Handover

- Time-based Intra-frequency conditional Handover without L3 measurement criteria

- Location-based Intra-frequency conditional Handover without L3 measurement criteria

- Inter-frequency Handover for both 1Rx and 2Rx RedCap UE

- Inter-frequency Handover to known target cell

- Inter-frequency Handover to unknown target cell

- Time-based Inter-frequency Conditional Handover

- Location-based Inter-frequency Conditional Handover

- Time-based Inter-frequency conditional Handover without L3 measurement criteria

- Location-based Inter-frequency conditional Handover without L3 measurement criteria

- Satellite switching with re-synchronization for both 1Rx and 2Rx RedCap UE

- RACH-based Hard Satellite switching

- RACH-less based Hard Satellite switching

- RACH-based Soft Satellite switching

- RACH-less based Soft Satellite switching

- Intra-frequency RRC Re-establishment for both 1Rx and 2Rx RedCap UE

- Inter-frequency RRC Re-establishment for both 1Rx and 2Rx RedCap UE

- 4-step RA type contention based random access test for both 1Rx and 2Rx RedCap UE

- 4-step RA type non-contention based random access test for both 1Rx and 2Rx RedCap UE

- Redirection from NR in FR1 to NR in FR1 for both 1Rx and 2Rx RedCap UE

- UE transmit timing for both 1Rx and 2Rx RedCap UE

- FR1 timing advance adjustment accuracy for both 1Rx and 2Rx RedCap UE

- Radio Link Monitoring Out-of-sync Test for both 1Rx and 2Rx RedCap UE

- SSB-based RLM RS in non-DRX mode

- SSB-based RLM RS in DRX mode

- CSI-RS-based RLM in non-DRX mode

- CSI-RS-based RLM in DRX mode

- Radio Link Monitoring In-sync Test for both 1Rx and 2Rx RedCap UE

- SSB-based RLM RS in non-DRX mode

- SSB-based RLM RS in DRX mode

- CSI-RS-based RLM in non-DRX mode

- CSI-RS-based RLM in DRX mode

- Beam Failure Detection and Link Recovery Test for both 1Rx and 2Rx RedCap UE

- SSB-based BFD and LR in non-DRX mode

- SSB-based BFD and LR in DRX mode

- CSI-RS-based BFD and LR in non-DRX mode

- CSI-RS-based BFD and LR in DRX mode

- Active BWP switch for both 1Rx and 2Rx RedCap UE

- DCI-based and Timer-based Active BWP Switch

- RRC-based Active BWP Switch

- UE specific CBW change on PCell in FR1 in non-DRX for both 1Rx and 2Rx RedCap UE

- Intra-frequency measurement for both 1Rx and 2Rx RedCap UE

- event triggered reporting tests without gap under non-DRX

- event triggered reporting tests without gap under DRX

- event triggered reporting tests without gap under non-DRX with SSB index reading

- event triggered reporting tests with single measurement gap under non-DRX

- event triggered reporting tests with FNO concurrent gaps under DRX for satellite access

- event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access

- Inter-frequency measurement for both 1Rx and 2Rx RedCap UE

- event triggered reporting tests for FR1 without SSB time index detection with single gap under non-DRX

- event triggered reporting tests for FR1 without SSB time index detection with single gap under DRX

- event triggered reporting tests for FR1 with SSB time index detection with single gap under non-DRX

- event triggered reporting tests for FR1 without SSB time index detection with FNO concurrent gaps under non-DRX

- event triggered reporting tests for FR1 without SSB time index detection with PPO concurrent gaps under non-DRX

- Event triggered reporting test without gap under non-DRX

- Event triggered reporting test without gap under DRX

- L1-RSRP measurement for beam reporting for both 1Rx and 2Rx RedCap UE

- SSB based L1-RSRP measurement for satellite access under non-DRX

- SSB based L1-RSRP measurement for satellite access under DRX

- CSI-RS based L1-RSRP measurement for satellite access under non-DRX

- CSI-RS based L1-RSRP measurement for satellite access under DRX

- SS-RSRP/SS-RSRQ/SS-SINR Measurement Performance for both 1Rx and 2Rx RedCap UE

- intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell

- inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell

- L1-RSRP measurement Performance for beam reporting for both 1Rx and 2Rx RedCap UE

- SSB based L1-RSRP measurement

- CSI-RS based L1-RSRP measurement on resource set with repetition off

Proposal 4: Define the test cases for RedCap UE over NTN as listed above.

Issue 2-2-1: Test scope for downlink coverage enhancement

<Way Forward>:

- Proposal 1 (ZTE): For downlink coverage enhancement, some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

- Proposal 2 (Ericsson, CATT, HW, ZTE): RAN4 to consider adding a new test case for RRC Re-establishment for the UE capable of coverage enhancement.

  - Proposal 2a (HW): Define test case for RRC Re-establishment with 160ms SSB periodicity and without serving cell based on existing test case in A.6.3.2.1.3.

- Proposal 3 (CATT, HW): Other test cases need to wait for the progress in core part.

  - Proposal 3a (HW): Wait for conclusion on potential SMTC enhancement before discussing whether to define new test cases for cell reselection and L3 measurement.

- Proposal 4 (ZTE): For RRC connection release with redirection to NR, handover, satellite switch with re-sync and cell re-selection, no new test cases shall be defined.

Based on the progress of the core part so far, RAN4 define new RRC Re-establishment core requirements for Serving cell SSB Ês/Iot (dB)<-8 case, this shall be tested by adding one new test case. Due to this new requirement is for DL-CE purpose, the SSB periodicity shall be set as 160ms.

Proposal 5: Define RRC Re-establishment test case for Serving cell SSB Ês/Iot (dB)<-8 and SSB periodicity 160ms case.

For SMTC enhancement related, we should first wait the core requirements.

Proposal 6: For SMTC enhancement related test cases, wait the core requirement progress first.

Issue 2-2-2: Test configuration for downlink coverage enhancement

<Way Forward>:

- Proposal 1: RAN4 need to discuss whether to introduced new SSB patterns for DL enhancement NTN UE.

So far, there is no 160ms periodicity SSB pattern defined in spec, which should be introduced at least for new RRC Re-establishment test case.

Proposal 7: Introduce new 160ms periodicity SSB pattern for DL CE.

- Conclusion

Based on the discussion above, the following observation and proposals are concluded.

Proposal 1: Take the NTN test setups as the reference, except both FDD and HD-FDD shall be involved in test configuration.

Proposal 2: Prefer to set a new Clause for perf requirement, and use the ‘refer to’ method when drafting the spec to avoid duplicated contents.

Proposal 3: Define the separate test cases for 1Rx and 2Rx (e)RedCap UE with NTN.

Proposal 4: Define the test cases for RedCap UE over NTN as listed above.

Proposal 5: Define RRC Re-establishment test case for Serving cell SSB Ês/Iot (dB)<-8 and SSB periodicity 160ms case.

Proposal 6: For SMTC enhancement related test cases, wait the core requirement progress first.

Proposal 7: Introduce new 160ms periodicity SSB pattern for DL CE.

- Reference

- R4-2508287, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)