# 3GPP TSG-RAN4 Meeting #116	R4-2510057

Bengaluru, India, August 25th – 29th, 2025

Title:	LS for parameter update request with switching gap

Release:	Release 19

Response to:

Work Item:	NR_LBCA_Sw-Core

Source:	RAN WG4

To:	RAN WG1 and RAN WG2

Cc:

Contact Person:

Name:	JungGeun Chi

E-mail Address:	junggeun.chi@lge.com

Attachments:

# 1. Overall Description:

RAN4 discussed ‘switching gap’ in RAN4#115. The conclusion is as follow:

FDD to SDL switch Actual switching gap length > Tswitch + MRTD + FDD UL trailing end transient period + SDL leading transient period. -Switching period : 35 or 70 or 140 𝜇s according to UE capability -Transient period: 10us -The MRTD = 3us assuming that the FDD and SDL carriers are colocated. SDL to FDD switch Actual switching gap length > Tswitch + TTA + MRTD + SDL trailing transient period + FDD UL leading transient period -Switching period : 35 or 70 or 140 𝜇s according to UE capability -Transient period: 10us -The MRTD = 3us assuming that the FDD and SDL carriers are colocated.

# 2	Actions

To RAN1 and RAN2

ACTION: 	RAN4 respectfully request RAN1 and RAN2 to take the above RAN4 agreement into account when defining value range of switching gap duration.

# 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #116bis 	13 October - 17 October 2025

TSG-RAN WG4 Meeting #117 	17 November - 21 November 2025