| 3GPP TR 38.719-03-01 V0.7.0 (2025-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Rel-19 NR Inter-band CA/DC configurations including inter band CA for 3 different bands DL with x different bands UL (x=1,2) (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	17

1	Scope	19

2	References	19

3	Definitions of terms, symbols and abbreviations	19

3.1	Terms	19

3.2	Symbols	19

3.3	Abbreviations	20

4	Background	20

4.1	Introduction	20

4.2	TR Maintenance	20

5	Three bands within FR1 Carrier Aggregation: Specific Band Combination Part	20

5.0	CA_nX-nY-nZ	20

5.0.1	Common for 1 band UL and 2 bands UL CA	20

5.0.1.1	Operating bands for CA	20

5.0.1.2	Channel bandwidths per operating band for CA	21

5.0.1.3	∆TIB,c and ∆RIB,c values	21

5.0.2	Specific for 2 bands UL CA	22

5.0.2.1	UE co-existence studies	22

5.0.2.1.1	Co-existence studies for 2UL band with 1CC per band	22

5.0.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	22

5.0.2.2	REFSENS requirements	23

5.1	CA_n28-n41-n79	23

5.1.1	Common for 1 band UL and 2 bands UL CA	23

5.1.1.1	Operating bands for CA	23

5.1.1.2	Channel bandwidths per operating band for CA	24

5.1.1.3	∆TIB,c and ∆RIB,c values	24

5.1.2	Specific for 2 bands UL CA	24

5.1.2.1	UE co-existence studies	24

5.1.2.1.1	Co-existence studies for 2UL band with 1CC per band	24

5.1.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	24

5.1.2.2	REFSENS requirements	25

5.2	CA_n8-n20-n75	25

5.2.1	Common for 1 band UL and 2 bands UL CA	25

5.2.1.1	Operating bands for CA	25

5.2.1.2	Channel bandwidths per operating band for CA	26

5.2.1.3	∆TIB,c and ∆RIB,c values	26

5.2.2	Specific for 2 bands UL CA	26

5.2.2.1	UE co-existence studies	26

5.2.2.1.1	Co-existence studies for 2UL band with 1CC per band	26

5.2.2.2	REFSENS requirements	27

5.3	CA_n8-n28-n75	27

5.3.1	Common for 1 band UL and 2 bands UL CA	27

5.3.1.1	Operating bands for CA	27

5.3.1.2	Channel bandwidths per operating band for CA	28

5.3.1.3	∆TIB,c and ∆RIB,c values	28

5.3.2	Specific for 2 bands UL CA	28

5.3.2.1	UE co-existence studies	28

5.3.2.1.1	Co-existence studies for 2UL band with 1CC per band	28

5.3.2.2	REFSENS requirements	29

5.4	CA_n20-n28-n75	29

5.4.1	Common for 1 band UL and 2 bands UL CA	29

5.4.1.1	Operating bands for CA	29

5.4.1.2	Channel bandwidths per operating band for CA	30

5.4.1.3	∆TIB,c and ∆RIB,c values	30

5.4.2	Specific for 2 bands UL CA	30

5.4.2.1	UE co-existence studies	30

5.4.2.1.1	Co-existence studies for 2UL band with 1CC per band	30

5.4.2.2	REFSENS requirements	31

5.5	CA_n3-n7-n75	31

5.5.1	Common for 1 band UL and 2 bands UL CA	31

5.5.1.1	Operating bands for CA	31

5.5.1.2	Channel bandwidths per operating band for CA	32

5.5.1.3	∆TIB,c and ∆RIB,c values	32

5.5.2	Specific for 2 bands UL CA	32

5.5.2.1	UE co-existence studies	32

5.5.2.1.1	Co-existence studies for 2UL band with 1CC per band	32

5.5.2.2	REFSENS requirements	33

5.6	CA_n3-n75-n78	34

5.6.1	Common for 1 band UL and 2 bands UL CA	34

5.6.1.1	Operating bands for CA	34

5.6.1.2	Channel bandwidths per operating band for CA	34

5.6.1.3	∆TIB,c and ∆RIB,c values	34

5.6.2	Specific for 2 bands UL CA	34

5.6.2.1	UE co-existence studies	34

5.6.2.1.1	Co-existence studies for 2UL band with 1CC per band	35

5.6.2.2	REFSENS requirements	35

5.7	CA_n7-n75-n78	36

5.7.1	Common for 1 band UL and 2 bands UL CA	36

5.7.1.1	Operating bands for CA	36

5.7.1.2	Channel bandwidths per operating band for CA	36

5.7.1.3	∆TIB,c and ∆RIB,c values	37

5.7.2	Specific for 2 bands UL CA	37

5.7.2.1	UE co-existence studies	37

5.7.2.1.1	Co-existence studies for 2UL band with 1CC per band	37

5.7.2.2	REFSENS requirements	37

5.8	CA_n1-n7-n75	38

5.8.1	Common for 1 band UL and 2 bands UL CA	38

5.8.1.1	Operating bands for CA	38

5.8.1.2	Channel bandwidths per operating band for CA	39

5.8.1.3	∆TIB,c and ∆RIB,c values	39

5.8.2	Specific for 2 bands UL CA	39

5.8.2.1	UE co-existence studies	39

5.8.2.1.1	Co-existence studies for 2UL band with 1CC per band	39

5.8.2.2	REFSENS requirements	40

5.9	CA_n1-n3-n7	41

5.9.1	Common for 1 band UL and 2 bands UL CA	41

5.9.1.1	Operating bands for CA	41

5.9.1.2	Channel bandwidths per operating band for CA	41

5.9.1.3	∆TIB,c and ∆RIB,c values	41

5.9.2	Specific for 2 bands UL CA	41

5.9.2.1	UE co-existence studies	41

5.9.2.1.1	Co-existence studies for 2UL band with 1CC per band	42

5.9.2.2	REFSENS requirements	44

5.10	CA_n41-n66-n71	44

5.10.1	Common for 1 band UL and 2 bands UL CA	44

5.10.1.1	Operating bands for CA.	44

5.10.1.2	Channel bandwidths per operating band for CA	44

5.10.2	Specific for 2 bands UL CA	44

5.10.2.1	UE co-existence studies	44

5.10.2.1.1	Co-existence studies for 2UL band with 1CC per band	44

5.10.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	44

5.10.2.2	REFSENS requirements	45

5.11	CA_n1-n41-n78	45

5.11.1	Common for 1 band UL and 2 bands UL CA	45

5.11.1.1	Operating bands for CA	45

5.11.1.2	Channel bandwidths per operating band for CA	46

5.11.1.3	∆TIB,c and ∆RIB,c values	46

5.11.2	Specific for 2 bands UL CA	46

5.11.2.1	UE co-existence studies	46

5.11.2.2	REFSENS requirements	48

5.12	CA_n1-n5-n8	49

5.12.1	Common for 1 band UL and 2 bands UL CA	49

5.12.1.1	Operating bands for CA	49

5.12.1.2	Channel bandwidths per operating band for CA	49

5.12.1.3	∆TIB,c and ∆RIB,c values	49

5.13	CA_n3-n5-n8	50

5.13.1	Common for 1 band UL and 2 bands UL CA	50

5.13.1.1	Operating bands for CA	50

5.13.1.2	Channel bandwidths per operating band for CA	50

5.13.1.3	∆TIB,c and ∆RIB,c values	50

5.14	CA_n25-n29-n77	51

5.14.1	Common for 1 band UL and 2 bands UL CA	51

5.14.1.1	Operating bands for CA	51

5.14.1.2	Channel bandwidths per operating band for CA	51

5.14.1.3	∆TIB,c and ∆RIB,c values	51

5.14.2	Specific for 2 bands UL CA	52

5.14.2.1	UE co-existence studies	52

5.14.2.1.1	Co-existence studies for 2UL band with 1CC per band	52

5.14.2.2	REFSENS requirements	52

5.15	CA_n1-n75-n78	53

5.15.1	Common for 1 band UL and 2 bands UL CA	53

5.15.1.1	Operating bands for CA	53

5.15.1.2	Channel bandwidths per operating band for CA	53

5.15.1.3	∆TIB,c and ∆RIB,c values	53

5.15.2	Specific for 2 bands UL CA	53

5.15.2.1	UE co-existence studies	53

5.15.2.1.1	Co-existence studies for 2UL band with 1CC per band	53

5.15.2.2	REFSENS requirements	54

5.16	CA_n1-n28-n40	54

5.16.1	Common for 1 band UL and 2 bands UL CA	54

5.16.1.1	Operating bands for CA	54

5.16.1.2	Channel bandwidths per operating band for CA	55

5.16.1.3	∆TIB,c and ∆RIB,c values	55

5.16.2	Specific for 2 bands UL CA	55

5.16.2.1	UE co-existence studies	55

5.16.2.1.1	Co-existence studies for 2UL band with 1CC per band	55

5.16.2.2	REFSENS requirements	57

5.17	CA_n1-n20-n71	58

5.17.1	Common for 1 band UL and 2 bands UL CA	58

5.17.1.1	Operating bands for CA	58

5.17.1.2	Channel bandwidths per operating band for CA	58

5.17.1.3	∆TIB,c and ∆RIB,c values	58

5.17.2	Specific for 2 bands UL CA	59

5.17.2.1	UE co-existence studies	59

5.17.2.1.1	Co-existence studies for 2UL band with 1CC per band	59

5.17.2.2	REFSENS requirements	61

5.18	CA_n1-n20-n78	62

5.18.1	Common for 1 band UL and 2 bands UL CA	62

5.18.1.1	Operating bands for CA	62

5.18.1.2	Channel bandwidths per operating band for CA	62

5.18.1.3	∆TIB,c and ∆RIB,c values	62

5.18.2	Specific for 2 bands UL CA	62

5.18.2.1	UE co-existence studies	62

5.18.2.1.1	Co-existence studies for 2UL band with 1CC per band	62

5.18.2.2	REFSENS requirements	64

5.19	CA_n20-n41-n71	65

5.19.1	Common for 1 band UL and 2 bands UL CA	65

5.19.1.1	Operating bands for CA	65

5.19.1.2	Channel bandwidths per operating band for CA	65

5.19.1.3	∆TIB,c and ∆RIB,c values	65

5.19.2	Specific for 2 bands UL CA	66

5.19.2.1	UE co-existence studies	66

5.19.2.1.1	Co-existence studies for 2UL band with 1CC per band	66

5.19.2.2	REFSENS requirements	68

5.20	CA_n20-n41-n77	69

5.20.1	Common for 1 band UL and 2 bands UL CA	69

5.20.1.1	Operating bands for CA	69

5.20.1.2	Channel bandwidths per operating band for CA	69

5.20.1.3	∆TIB,c and ∆RIB,c values	69

5.20.2	Specific for 2 bands UL CA	70

5.20.2.1	UE co-existence studies	70

5.20.2.1.1	Co-existence studies for 2UL band with 1CC per band	70

5.20.2.2	REFSENS requirements	72

5.21	CA_n20-n41-n78	73

5.21.1	Common for 1 band UL and 2 bands UL CA	73

5.21.1.1	Operating bands for CA	73

5.21.1.2	Channel bandwidths per operating band for CA	73

5.21.1.3	∆TIB,c and ∆RIB,c values	73

5.21.2	Specific for 2 bands UL CA	74

5.21.2.1	UE co-existence studies	74

5.21.2.1.1	Co-existence studies for 2UL band with 1CC per band	74

5.21.2.2	REFSENS requirements	76

5.22	CA_n20-n71-n78	76

5.22.1	Common for 1 band UL and 2 bands UL CA	76

5.22.1.1	Operating bands for CA	76

5.22.1.2	Channel bandwidths per operating band for CA	77

5.22.1.3	∆TIB,c and ∆RIB,c values	77

5.22.2	Specific for 2 bands UL CA	77

5.22.2.1	UE co-existence studies	77

5.22.2.1.1	Co-existence studies for 2UL band with 1CC per band	77

5.22.2.2	REFSENS requirements	80

5.23	CA_n3-n20-n71	80

5.23.1	Common for 1 band UL and 2 bands UL CA	80

5.23.1.1	Operating bands for CA	80

5.23.1.2	Channel bandwidths per operating band for CA	81

5.23.1.3	∆TIB,c and ∆RIB,c values	81

5.23.2	Specific for 2 bands UL CA	81

5.23.2.1	UE co-existence studies	81

5.23.2.1.1	Co-existence studies for 2UL band with 1CC per band	81

5.23.2.2	REFSENS requirements	83

5.24	CA_n3-n20-n77	84

5.24.1	Common for 1 band UL and 2 bands UL CA	84

5.24.1.1	Operating bands for CA	84

5.24.1.2	Channel bandwidths per operating band for CA	84

5.24.1.3	∆TIB,c and ∆RIB,c values	84

5.24.2	Specific for 2 bands UL CA	85

5.24.2.1	UE co-existence studies	85

5.24.2.1.1	Co-existence studies for 2UL band with 1CC per band	85

5.24.2.2	REFSENS requirements	87

5.25	CA_n1-n3-n71	88

5.25.1	Common for 1 band UL and 2 bands UL CA	88

5.25.1.1	Operating bands for CA	88

5.25.1.2	Channel bandwidths per operating band for CA	88

5.25.1.3	∆TIB,c and ∆RIB,c values	88

5.25.2	Specific for 2 bands UL CA	89

5.25.2.1	UE co-existence studies	89

5.25.2.1.1	Co-existence studies for 2UL band with 1CC per band	89

5.25.2.2	REFSENS requirements	91

5.26	CA_n1-n3-n8	91

5.26.1	Common for 1 band UL and 2 bands UL CA	91

5.26.1.1	Operating bands for CA	91

5.26.1.2	Channel bandwidths per operating band for CA	92

5.26.1.3	∆TIB,c and ∆RIB,c values	92

5.26.2	Specific for 2 bands UL CA	92

5.26.2.1	UE co-existence studies	92

5.26.2.1.1	Co-existence studies for 2UL band with 1CC per band	92

5.26.2.2	REFSENS requirements	94

5.27	CA_n1-n41-n71	95

5.27.1	Common for 1 band UL and 2 bands UL CA	95

5.27.1.1	Operating bands for CA	95

5.27.1.2	Channel bandwidths per operating band for CA	95

5.27.1.3	∆TIB,c and ∆RIB,c values	95

5.27.2	Specific for 2 bands UL CA	96

5.27.2.1	UE co-existence studies	96

5.27.2.1.1	Co-existence studies for 2UL band with 1CC per band	96

5.27.2.2	REFSENS requirements	98

5.28	CA_n1-n8-n41	99

5.28.1	Common for 1 band UL and 2 bands UL CA	99

5.28.1.1	Operating bands for CA	99

5.28.1.2	Channel bandwidths per operating band for CA	99

5.28.1.3	∆TIB,c and ∆RIB,c values	99

5.28.2	Specific for 2 bands UL CA	100

5.28.2.1	UE co-existence studies	100

5.28.2.1.1	Co-existence studies for 2UL band with 1CC per band	100

5.28.2.2	REFSENS requirements	102

5.29	CA_n3-n41-n71	102

5.29.1	Common for 1 band UL and 2 bands UL CA	102

5.29.1.1	Operating bands for CA	102

5.29.1.2	Channel bandwidths per operating band for CA	103

5.29.1.3	∆TIB,c and ∆RIB,c values	103

5.29.2	Specific for 2 bands UL CA	103

5.29.2.1	UE co-existence studies	103

5.29.2.1.1	Co-existence studies for 2UL band with 1CC per band	103

5.29.2.2	REFSENS requirements	106

5.30	CA_n1-n7-n20	106

5.30.1	Common for 1 band UL and 2 bands UL CA	106

5.30.1.1	Operating bands for CA	106

5.30.1.2	Channel bandwidths per operating band for CA	106

5.30.1.3	∆TIB,c and ∆RIB,c values	106

5.30.2	Specific for 2 bands UL CA	107

5.30.2.1	UE co-existence studies	107

5.30.2.1.1	Co-existence studies for 2UL band with 1CC per band	107

5.30.2.2	REFSENS requirements	109

5.31	CA_n1-n3-n75	110

5.31.1	Common for 1 band UL and 2 bands UL CA	110

5.31.1.1	Operating bands for CA	110

5.31.1.2	Channel bandwidths per operating band for CA	110

5.31.1.3	∆TIB,c and ∆RIB,c values	110

5.31.2	Specific for 2 bands UL CA	110

5.31.2.1	UE co-existence studies	110

5.31.2.1.1	Co-existence studies for 2UL band with 1CC per band	110

5.31.2.2	REFSENS requirements	111

5.32	CA_n1-n7-n78	112

5.32.1	Common for 1 band UL and 2 bands UL CA	112

5.32.1.1	Operating bands for CA	112

5.32.1.2	Channel bandwidths per operating band for CA	112

5.32.1.3	∆TIB,c and ∆RIB,c values	112

5.32.2	Specific for 2 bands UL CA	112

5.32.2.1	UE co-existence studies	112

5.32.2.1.1	Co-existence studies for 2UL band with 1CC per band	112

5.32.2.2	REFSENS requirements	114

5.33	CA_n7-n25-n29	115

5.33.1	Common for 1 band UL and 2 bands UL CA	115

5.33.1.1	Operating bands for CA	115

5.33.1.2	Channel bandwidths per operating band for CA	115

5.33.1.3	∆TIB,c and ∆RIB,c values	115

5.33.2	Specific for 2 bands UL CA	116

5.33.2.1	UE co-existence studies	116

5.33.2.1.1	Co-existence studies for 2UL band with 1CC per band	116

5.33.2.2	REFSENS requirements	116

5.34	CA_n7-n29-n66	117

5.34.1	Common for 1 band UL and 2 bands UL CA	117

5.34.1.1	Operating bands for CA	117

5.34.1.2	Channel bandwidths per operating band for CA	117

5.34.1.3	∆TIB,c and ∆RIB,c values	117

5.34.2	Specific for 2 bands UL CA	117

5.34.2.1	UE co-existence studies	117

5.34.2.1.1	Co-existence studies for 2UL band with 1CC per band	117

5.34.2.2	REFSENS requirements	118

5.35	CA_n7-n29-n77	118

5.35.1	Common for 1 band UL and 2 bands UL CA	118

5.35.1.1	Operating bands for CA	118

5.35.1.2	Channel bandwidths per operating band for CA	119

5.35.1.3	∆TIB,c and ∆RIB,c values	119

5.35.2	Specific for 2 bands UL CA	119

5.35.2.1	UE co-existence studies	119

5.35.2.1.1	Co-existence studies for 2UL band with 1CC per band	119

5.35.2.2	REFSENS requirements	120

5.36	 CA_n3-n40-n79	121

5.36.1	Common for 1 band UL and 2 bands UL CA	121

5.36.1.1	 Operating bands for CA	121

5.36.1.2	Channel bandwidths per operating band for CA	121

5.36.1.3	∆TIB,c and ∆RIB,c values	121

5.36.2	Specific for 2 bands UL CA	121

5.36.2.1	UE co-existence studies	121

5.36.2.2	REFSENS requirements	122

5.37	CA_n3-n7-n77	122

5.37.1	Common for 1 band UL and 2 bands UL CA	122

5.37.1.1	Operating bands for CA	122

5.37.1.2	Channel bandwidths per operating band for CA	123

5.37.1.3	∆TIB,c and ∆RIB,c values	123

5.37.2	Specific for 2 bands UL CA	123

5.37.2.1	UE co-existence studies	123

5.37.2.1.1	Co-existence studies for 2UL band with 1CC per band	123

5.37.2.2	REFSENS requirements	126

5.38	CA_n1-n20-n41	126

5.38.1	Common for 1 band UL and 2 bands UL CA	126

5.38.1.1	Operating bands for CA	126

5.38.1.2	Channel bandwidths per operating band for CA	127

5.38.1.3	∆TIB,c and ∆RIB,c values	127

5.38.2	Specific for 2 bands UL CA	127

5.38.2.1	UE co-existence studies	127

5.38.2.1.1	Co-existence studies for 2UL band with 1CC per band	127

5.38.2.2	REFSENS requirements	129

5.39	CA_n1-n20-n77	130

5.39.1	Common for 1 band UL and 2 bands UL CA	130

5.39.1.1	Operating bands for CA	130

5.39.1.2	Channel bandwidths per operating band for CA	130

5.39.1.3	∆TIB,c and ∆RIB,c values	130

5.39.2	Specific for 2 bands UL CA	131

5.39.2.1	UE co-existence studies	131

5.39.2.1.1	Co-existence studies for 2UL band with 1CC per band	131

5.39.2.2	REFSENS requirements	133

5.40	CA_n1-n71-n77	134

5.40.1	Common for 1 band UL and 2 bands UL CA	134

5.40.1.1	Operating bands for CA	134

5.40.1.2	Channel bandwidths per operating band for CA	134

5.40.1.3	∆TIB,c and ∆RIB,c values	134

5.40.2	Specific for 2 bands UL CA	135

5.40.2.1	UE co-existence studies	135

5.40.2.1.1	Co-existence studies for 2UL band with 1CC per band	135

5.40.2.2	REFSENS requirements	137

5.41	CA_n3-n20-n41	138

5.41.1	Common for 1 band UL and 2 bands UL CA	138

5.41.1.1	Operating bands for CA	138

5.41.1.2	Channel bandwidths per operating band for CA	138

5.41.1.3	∆TIB,c and ∆RIB,c values	138

5.41.2	Specific for 2 bands UL CA	139

5.41.2.1	UE co-existence studies	139

5.41.2.1.1	Co-existence studies for 2UL band with 1CC per band	139

5.41.2.2	REFSENS requirements	141

5.42	CA_n3-n71-n77	141

5.42.1	Common for 1 band UL and 2 bands UL CA	141

5.42.1.1	Operating bands for CA	141

5.42.1.2	Channel bandwidths per operating band for CA	142

5.42.1.3	∆TIB,c and ∆RIB,c values	142

5.42.2	Specific for 2 bands UL CA	142

5.42.2.1	UE co-existence studies	142

5.42.2.1.1	Co-existence studies for 2UL band with 1CC per band	142

5.42.2.2	REFSENS requirements	145

5.43	CA_n1-n41-n78	145

5.43.1	Common for 1 band UL and 2 bands UL CA	145

5.43.1.1	Operating bands for CA	145

5.43.1.2	Channel bandwidths per operating band for CA	146

5.43.1.3	∆TIB,c and ∆RIB,c values	146

5.43.2	Specific for 2 bands UL CA	146

5.43.2.1	UE co-existence studies	146

5.43.2.1.1	Co-existence studies for 2UL band with 1CC per band	146

5.43.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	146

5.43.2.2	REFSENS requirements	147

5.44	CA_n1-n71-n78	147

5.44.1	Common for 1 band UL and 2 bands UL CA	147

5.44.1.1	Operating bands for CA	147

5.44.1.2	Channel bandwidths per operating band for CA	148

5.44.1.3	∆TIB,c and ∆RIB,c values	148

5.44.2	Specific for 2 bands UL CA	148

5.44.2.1	UE co-existence studies	148

5.44.2.1.1	Co-existence studies for 2UL band with 1CC per band	148

5.44.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	151

5.44.2.2	REFSENS requirements	152

5.45	CA_n1-n8-n78	152

5.45.1	Common for 1 band UL and 2 bands UL CA	152

5.45.1.1	Operating bands for CA	152

5.45.1.2	Channel bandwidths per operating band for CA	153

5.45.2	Specific for 2 bands UL CA	153

5.45.2.1	UE co-existence studies	153

5.45.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	153

5.45.2.2	REFSENS requirements	154

5.46	CA_n3-n41-n78	154

5.46.1	Common for 1 band UL and 2 bands UL CA	154

5.46.1.1	Operating bands for CA	154

5.46.1.2	Channel bandwidths per operating band for CA	154

5.46.1.3	∆TIB,c and ∆RIB,c values	154

5.46.2	Specific for 2 bands UL CA	155

5.46.2.1	UE co-existence studies	155

5.46.2.1.1	Co-existence studies for 2UL band with 1CC per band	155

5.46.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	155

5.46.2.2	REFSENS requirements	156

5.47	CA_n3-n71-n78	156

5.47.1	Common for 1 band UL and 2 bands UL CA	156

5.47.1.1	Operating bands for CA	156

5.47.1.2	Channel bandwidths per operating band for CA	156

5.47.1.3	∆TIB,c and ∆RIB,c values	156

5.47.2	Specific for 2 bands UL CA	157

5.47.2.1	UE co-existence studies	157

5.47.2.1.1	Co-existence studies for 2UL band with 1CC per band	157

5.47.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	159

5.47.2.2	REFSENS requirements	160

5.48	CA_n3-n8-n78	160

5.48.1	Common for 1 band UL and 2 bands UL CA	160

5.48.1.1	Operating bands for CA	160

5.48.1.2	Channel bandwidths per operating band for CA	161

5.48.2	Specific for 2 bands UL CA	161

5.48.2.1	UE co-existence studies	161

5.48.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	161

5.48.2.2	REFSENS requirements	162

5.49	CA_n41-n71-n78	162

5.49.1	Common for 1 band UL and 2 bands UL CA	162

5.49.1.1	Operating bands for CA	162

5.49.1.2	Channel bandwidths per operating band for CA	162

5.49.1.3	∆TIB,c and ∆RIB,c values	162

5.49.2	Specific for 2 bands UL CA	163

5.49.2.1	UE co-existence studies	163

5.49.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	163

5.49.2.2	REFSENS requirements	163

5.50	CA_n8-n41-n78	164

5.50.1	Common for 1 band UL and 2 bands UL CA	164

5.50.1.1	Operating bands for CA	164

5.50.1.2	Channel bandwidths per operating band for CA	164

5.50.1.3	∆TIB,c and ∆RIB,c values	164

5.50.2	Specific for 2 bands UL CA	165

5.50.2.1	UE co-existence studies	165

5.50.2.1.1	Co-existence studies for 2UL band with 1CC per band	165

5.50.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	167

5.50.2.2	REFSENS requirements	168

5.51	CA_n1-n3-n78	169

5.51.1	Common for 1 band UL and 2 bands UL CA	169

5.51.1.1	Operating bands for CA	169

5.51.1.2	Channel bandwidths per operating band for CA	169

5.51.1.3	∆TIB,c and ∆RIB,c values	169

5.51.2	Specific for 2 bands UL CA	169

5.51.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	169

5.51.2.2	REFSENS requirements	170

5.52	CA_n1-n40-n41	171

5.52.1	Common for 1 band UL and 2 bands UL CA	171

5.52.1.1	Operating bands for CA	171

5.52.1.2	Channel bandwidths per operating band for CA	171

5.52.1.3	∆TIB,c and ∆RIB,c values	171

5.52.2	Specific for 2 bands UL CA	172

5.52.2.1	UE co-existence studies	172

5.52.2.1.1	Co-existence studies for 2UL band with 1CC per band	172

5.52.2.2	REFSENS requirements	174

5.53	CA_n3-n77-n79	174

5.53.1	  Common for 1 band UL and 2 bands UL CA	174

5.53.1.1	Operating bands for CA	174

5.53.1.2	Channel bandwidths per operating band for CA	174

5.53.1.3	∆TIB,c and ∆RIB,c values	175

5.53.2	Specific for 2 bands UL CA	175

5.53.2.1	UE co-existence studies	175

5.53.2.1.1	Co-existence studies for 2UL band with 1CC per band	175

5.54	CA_n28-n77-n79	175

5.54.1	  Common for 1 band UL and 2 bands UL CA	175

5.54.1.1	Operating bands for CA	175

5.54.1.2	Channel bandwidths per operating band for CA	175

5.54.1.3	∆TIB,c and ∆RIB,c values	175

5.54.2	Specific for 2 bands UL CA	176

5.54.2.1	UE co-existence studies	176

5.54.2.1.1	Co-existence studies for 2UL band with 1CC per band	176

5.54.2.2	REFSENS requirements	176

5.55	CA_n40-n78-n79	176

5.55.1	Common for 1 band UL and 2 bands UL CA	176

5.55.1.1	Operating bands for CA	176

5.55.1.2	Channel bandwidths per operating band for CA	176

5.55.1.3	∆TIB,c and ∆RIB,c values	176

5.55.2	Specific for 2 bands UL CA	177

5.55.2.1	UE co-existence studies	177

5.55.2.1.1	Co-existence studies for 2UL band with 1CC per band	177

5.55.2.2	REFSENS requirements	179

5.56	CA_n1-n40-n79	180

5.56.1	Common for 1 band UL and 2 bands UL CA	180

5.56.1.1	Operating bands for CA	180

5.56.1.2	Channel bandwidths per operating band for CA	180

5.56.1.3	∆TIB,c and ∆RIB,c values	180

5.56.2	Specific for 2 bands UL CA	181

5.56.2.1	UE co-existence studies	181

5.56.2.1.1	Co-existence studies for 2UL band with 1CC per band	181

5.56.2.2	REFSENS requirements	183

5.57	CA_n3-n40-n78	183

5.57.1	Common for 1 band UL and 2 bands UL CA	183

5.57.1.1	Operating bands for CA	183

5.57.1.2	Channel bandwidths per operating band for CA	183

5.57.1.3	∆TIB,c and ∆RIB,c values	183

5.57.2	Specific for 2 bands UL CA	184

5.57.2.1	UE co-existence studies	184

5.57.2.1.1	Co-existence studies for 2UL band with 1CC per band	184

5.57.2.2	REFSENS requirements	186

5.58	CA_n7-n40-n79	186

5.58.1	Common for 1 band UL and 2 bands UL CA	186

5.58.1.1	Operating bands for CA	186

5.58.1.2	Channel bandwidths per operating band for CA	186

5.58.1.3	∆TIB,c and ∆RIB,c values	186

5.58.2	Specific for 2 bands UL CA	187

5.58.2.1	UE co-existence studies	187

5.58.2.1.1	Co-existence studies for 2UL band with 1CC per band	187

5.58.2.2	REFSENS requirements	189

5.59	CA_n7-n78-n79	189

5.59.1	Common for 1 band UL and 2 bands UL CA	189

5.59.1.1	Operating bands for CA	189

5.59.1.2	Channel bandwidths per operating band for CA	189

5.59.1.3	∆TIB,c and ∆RIB,c values	190

5.59.2	Specific for 2 bands UL CA	190

5.59.2.1	UE co-existence studies	190

5.59.2.1.1	Co-existence studies for 2UL band with 1CC per band	190

5.59.2.2	REFSENS requirements	192

5.60	CA_n1-n3-n77	193

5.60.1	Common for 1 band UL and 2 bands UL CA	193

5.60.1.1	Operating bands for CA	193

5.60.1.2	Channel bandwidths per operating band for CA	193

5.60.1.3	∆TIB,c and ∆RIB,c values	193

5.60.2	Specific for 2 bands UL CA	194

5.60.2.1	UE co-existence studies	194

5.60.2.1.1	Co-existence studies for 2UL band with 1CC per band	194

5.60.2.2	REFSENS requirements	194

5.61	CA_n3-n40-n41	194

5.61.1	Common for 1 band UL and 2 bands UL CA	194

5.61.1.1	 Operating bands for CA	194

5.61.1.2	Channel bandwidths per operating band for CA	194

5.61.1.3	∆TIB,c and ∆RIB,c values	194

5.61.2	Specific for 2 bands UL CA	195

5.61.2.1	UE co-existence studies	195

5.61.2.2	REFSENS requirements	195

5.62	 CA_n3-n40-n79	196

5.62.1	Common for 1 band UL and 2 bands UL CA	196

5.62.1.1	 Operating bands for CA	196

5.62.1.2	Channel bandwidths per operating band for CA	196

5.62.1.3	∆TIB,c and ∆RIB,c values	196

5.62.2	Specific for 2 bands UL CA	196

5.62.2.1	UE co-existence studies	196

5.62.2.2	REFSENS requirements	197

5.63	CA_n2-n66-n77	197

5.63.1	Common for 1 band UL and 2 bands UL CA	197

5.63.1.1	Operating bands for CA	197

5.63.1.2	Channel bandwidths per operating band for CA	197

5.63.2	Specific for 2 bands UL CA	198

5.63.2.1	UE co-existence studies	198

5.63.2.1.1	Co-existence studies for 2UL band with 1CC per band	198

5.63.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	198

5.63.2.2	REFSENS requirements	199

5.64	CA_n2-n48-n77	199

5.64.1	Common for 1 band UL and 2 bands UL CA	199

5.64.1.1	Operating bands for CA	199

5.64.1.2	Channel bandwidths per operating band for CA	201

5.64.2	Specific for 2 bands UL CA	203

5.64.2.1	UE co-existence studies	203

5.64.2.1.1	Co-existence studies for 2UL band with 1CC per band	203

5.64.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	203

5.64.2.2	REFSENS requirements	204

5.65	CA_n5-n66-n77	204

5.65.1	Common for 1 band UL and 2 bands UL CA	204

5.65.1.1	Operating bands for CA	204

5.65.1.2	Channel bandwidths per operating band for CA	205

5.65.2	Specific for 2 bands UL CA	206

5.65.2.1	UE co-existence studies	206

5.65.2.1.1	Co-existence studies for 2UL band with 1CC per band	206

5.65.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	206

5.65.2.2	REFSENS requirements	206

5.66	CA_n48-n66-n77	207

5.66.1	Common for 1 band UL and 2 bands UL CA	207

5.66.1.1	Operating bands for CA	207

5.66.1.2	Channel bandwidths per operating band for CA	207

5.66.2	Specific for 2 bands UL CA	209

5.66.2.1	UE co-existence studies	209

5.66.2.1.1	Co-existence studies for 2UL band with 1CC per band	209

5.66.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	209

5.66.2.2	REFSENS requirements	210

5.67	CA_n3-n78-n79	210

5.67.1	Common for 1 band UL and 2 bands UL CA	210

5.67.1.1	Operating bands for CA	210

5.67.1.2	Channel bandwidths per operating band for CA	211

5.67.1.3	∆TIB,c and ∆RIB,c values	211

5.67.2	Specific for 2 bands UL CA	211

5.67.2.1	UE co-existence studies	211

5.67.2.1.1	Co-existence studies for 2UL band with 1CC per band	211

5.67.2.2	REFSENS requirements	213

5.68	CA_n28-n40-n71	214

5.68.1	Common for 1 band UL and 2 bands UL CA	214

5.68.1.1	Operating bands for CA	214

5.68.1.2	Channel bandwidths per operating band for CA	214

5.68.1.3	∆TIB,c and ∆RIB,c values	214

5.68.2	Specific for 2 bands UL CA	214

5.68.2.1	UE co-existence studies	214

5.68.2.1.1	Co-existence studies for 2UL band with 1CC per band	214

5.68.2.2	REFSENS requirements	216

5.69	CA_n28-n71-n77	216

5.69.1	Common for 1 band UL and 2 bands UL CA	216

5.69.1.1	Operating bands for CA	216

5.69.1.2	Channel bandwidths per operating band for CA	217

5.69.1.3	∆TIB,c and ∆RIB,c values	217

5.69.2	Specific for 2 bands UL CA	217

5.69.2.1.1	Co-existence studies for 2UL band with 1CC per band	217

5.70	CA_n40-n71-n77	219

5.70.1	Common for 1 band UL and 2 bands UL CA	219

5.70.1.1	Operating bands for CA	219

5.70.1.2	Channel bandwidths per operating band for CA	220

5.70.1.3	∆TIB,c and ∆RIB,c values	220

5.70.2	Specific for 2 bands UL CA	220

5.70.2.1	UE co-existence studies	220

5.70.2.1.1	Co-existence studies for 2UL band with 1CC per band	220

5.70.2.2	REFSENS requirements	223

5.71	CA_n8-n78-n79	223

5.71.1	Common for 1 band UL and 2 bands UL CA	223

5.71.1.1	Operating bands for CA	223

5.71.1.2	Channel bandwidths per operating band for CA	223

5.71.1.3	∆TIB,c and ∆RIB,c values	223

5.71.2	Specific for 2 bands UL CA	224

5.71.2.1	UE co-existence studies	224

5.71.2.1.1	Co-existence studies for 2UL band with 1CC per band	224

5.71.2.2	REFSENS requirements	226

5.72	CA_n2-n5-n77	226

5.72.1	Common for 1 band UL and 2 bands UL CA	226

5.72.1.1	Operating bands for CA	226

5.72.1.2	Channel bandwidths per operating band for CA	226

5.72.2	Specific for 2 bands UL CA	227

5.72.2.1	UE co-existence studies	227

5.72.2.1.1	Co-existence studies for 2UL band with 1CC per band	227

5.72.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	227

5.72.2.2	REFSENS requirements	228

5.73	CA_n5-n48-n77	228

5.73.1	Common for 1 band UL and 2 bands UL CA	228

5.73.1.1	Operating bands for CA	228

5.73.1.2	Channel bandwidths per operating band for CA	229

5.73.2	Specific for 2 bands UL CA	231

5.73.2.1	UE co-existence studies	231

5.73.2.1.1	Co-existence studies for 2UL band with 1CC per band	231

5.73.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)	232

5.73.2.2	REFSENS requirements	232

5.74	CA_n28-n74-n77	232

5.74.1	Common for 1 band UL and 2 bands UL CA	232

5.74.1.1	Operating bands for CA	232

5.74.1.2	Channel bandwidths per operating band for CA	233

5.74.1.3	∆TIB,c and ∆RIB,c values	233

5.74.2	Specific for 2 bands UL CA	233

5.74.2.1	UE co-existence studies	233

5.74.2.1.1	Co-existence studies for 2UL band with 1CC per band	233

5.74.2.2	REFSENS requirements	233

5.75	CA_n28-n41-n74	234

5.75.1	Common for 1 band UL and 2 bands UL CA	234

5.75.1.1	Operating bands for CA	234

5.75.1.2	Channel bandwidths per operating band for CA	234

5.75.1.3	∆TIB,c and ∆RIB,c values	234

5.75.2	Specific for 2 bands UL CA	235

5.75.2.1	UE co-existence studies	235

5.75.2.1.1	Co-existence studies for 2UL band with 1CC per band	235

5.75.2.2	REFSENS requirements	235

5.76	CA_n41-n74-n77	235

5.76.1	Common for 1 band UL and 2 bands UL CA	235

5.76.1.1	Operating bands for CA	235

5.76.1.2	Channel bandwidths per operating band for CA	236

5.76.1.3	∆TIB,c and ∆RIB,c values	236

5.76.2	Specific for 2 bands UL CA	236

5.76.2.1	UE co-existence studies	236

5.76.2.1.1	Co-existence studies for 2UL band with 1CC per band	236

5.76.2.2	REFSENS requirements	236

5.77	CA_n8-n40-n78	237

5.77.1	Common for 1 band UL and 2 bands UL CA	237

5.77.1.1	Operating bands for CA	237

5.77.1.2	Channel bandwidths per operating band for CA	237

5.77.1.3	∆TIB,c and ∆RIB,c values	238

5.77.2	Specific for 2 bands UL CA	238

5.77.2.1	UE co-existence studies	238

5.77.2.1.1	Co-existence studies for 2UL band with 1CC per band	238

5.77.2.2	REFSENS requirements	239

5.78	CA_n28-n40-n79	240

5.78.1	Common for 1 band UL and 2 bands UL CA	240

5.78.1.1	Operating bands for CA	240

5.78.1.2	Channel bandwidths per operating band for CA	240

5.78.1.3	∆TIB,c and ∆RIB,c values	241

5.78.2	Specific for 2 bands UL CA	241

5.78.2.1	UE co-existence studies	241

5.78.2.1.1	Co-existence studies for 2UL band with 1CC per band	241

5.78.2.2	REFSENS requirements	242

5.79	CA_n28-n78-n79	243

5.79.1	Common for 1 band UL and 2 bands UL CA	243

5.79.1.1	Operating bands for CA	243

5.79.1.2	Channel bandwidths per operating band for CA	243

5.79.1.3	∆TIB,c and ∆RIB,c values	243

5.79.2	Specific for 2 bands UL CA	243

5.79.2.1	UE co-existence studies	243

5.79.2.1.1	Co-existence studies for 2UL band with 1CC per band	243

5.79.2.2	REFSENS requirements	245

6	Dual Connectivity: Specific Band Combination Part	246

6.0	DC_nX-nY-nZ	246

6.0.1	Configurations for DC_nX-nY-nZ	246

Annex <A> (normative): Band group definition	247

Annex <B> (normative): Valid UL configurations	248

Annex <C> (informative): Change history	249

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for power class 3 NR inter-band CA and DC configurations including inter-band CA for 3 different bands DL with x different bands UL (x=1,2) under Rel-19 time frame. The purpose is to gather the relevant background information and studies in order to address NR inter-band CA and DC for 3 bands DL with up to 2 bands UL for the Rel-19 band combinations.

This TR contains the RF requirements of band specific combination part. The actual requirements are added to the corresponding technical specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[4]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3:  Range 1 and Range 2 Interworking operation with other radios".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, inter-band NR-DC operation and due to support for SUL operations, for serving cell c

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BS	Base Station

CA	Carrier Aggregation

DC	Dual Connectivity

DL	DownLink

FDD	Frequency Division Duplex

IMD	Inter-modulation

MSD	Maximum Sensitivity Deduction

SCS	Subcarrier spacing

TDD	Time Division Duplex

UE	User Equipment

UL	UpLink

4	Background

4.1	Introduction

The present document is a technical report for NR inter-band CA and DC configurations including inter-band CA for 3 different bands DL with x different bands UL (x=1,2) under Rel-19 time frame. The document covers the RF requirements for each band combination specific issues (i.e. one sub-clause defined per band combination) including:

-	Common issues for both 1 band UL and 2 bands UL NR CA, including ∆TIB and ∆RIB requirements.

-	2 bands UL NR CA specific issues, including MSD caused by IMD issue, etc.

It shall be noted that no new issue for inter-band NR DC combination, and the 2 bands UL NR CA specific issues shall be re-used.

4.2	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

Editor's note: It is not recommended to bring TP to TR for the following cases:

-	NR CA configurations with additional BCS other than BCS0 (such as BCS1) if there is no additional technical issue.

-	High order DL NR CA configurations, such as DL NR CA configuration CA_nXA-nYA-nZC

5	Three bands within FR1 Carrier Aggregation: Specific Band Combination Part

5.0	CA_nX-nY-nZ

5.0.1	Common for 1 band UL and 2 bands UL CA

5.0.1.1	Operating bands for CA

Editor's note: For band definition, relevant rows can be copied directly from Table 5.2-1 in 38.101-1 to Table 5.x.1.1-1 below.

Table 5.0.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| nX | xxxx MHz – xxxx MHz | xxxx MHz – xxxx MHz | XXX |
| nY | xxxx MHz – xxxx MHz | xxxx MHz – xxxx MHz | XXX |
| nZ | xxxx MHz – xxxx MHz | xxxx MHz – xxxx MHz | XXX |

5.0.1.2	Channel bandwidths per operating band for CA

Table 5.0.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_nXA-nYA-nZA | CA_nXA-nYA | nX | Channel BWs | X |
|  | CA_nXA-nZA | nY | Channel BWs |  |
|  | CA_nYA-nZA | nZ | Channel BWs |  |
| CA_nXA/B/C/(2A)-nYA/B/C/(2A)- nZA/B/C/(2A) | CA_nXA/B/C-nYA CA_nXA/B/C-nZA CA_nYA/B/C-nZA | nX | Channel BW or CA BCS | X |
|  |  | nY | Channel BW or CA BCS |  |
|  |  | nZ | Channel BW or CA BCS |  |

Editor's note: The valid UL configurations shall refer to the Annex B.

5.0.1.3	∆TIB,c and ∆RIB,c values

Editor’s note: For the table of ∆TIB,c and ∆RIB,c values, please use the same table format as in the latest TS 38.101-1.

For CA_nX-nY-nZ, the TIB,c and RIB,c values are given in the tables below.

Table 5.0.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_nx-ny-nz |  |  |  |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.0.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_nx-ny-nz |  |  |  |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.0.2	Specific for 2 bands UL CA

5.0.2.1	UE co-existence studies

Editor’s Note: The tables in this section are provided to identify potential issues to be analyzed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

5.0.2.1.1	Co-existence studies for 2UL band with 1CC per band

Editor’s Note: Since the IMD tables have already been calculated for the different two band fallbacks, the tables below may skip the IMD calculations and refer to the relevant two band TP tables. Nonetheless, the IMD issues should be stated, and the related TPs referenced. Only one example table is provided in this template but there may be up to three cases for the victim DL band and as many tables may be needed.

Table 5.0.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_nXA-nYA-nZA with UL CA_nXA-nYA.

Table 5.0.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | – |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | – |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

5.0.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Editor’s Note: Since the triple beat tables have already been calculated for the different two band fallbacks, the tables below may skip the IMD calculations and refer to the relevant two band TP tables. Nonetheless, the IMD issues should be stated, and the related TPs referenced. Only one example table is provided in this template but there may be up to three cases for the victim DL band and as many tables may be needed.

Table 5.0.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band nZ interference analysis for CA_nXA/B/C-nYA/B/C with nX/YB/C transmitting with a XXX MHz maximum instantaneous bandwidth.

Table 5.0.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (nX, nY) | – |  | – |  |
| fDL (nZ) | – |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | – |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | – |  | – |  |
| Analysis | Editor’s Note: The analysis conclusion for the two UL band with intra-band UL CA triple beat shall be included here, even if no issues are identified. |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.0.2.2	REFSENS requirements

Editor's note 1: Text can be added if IMD due to 2 bands UL with 2 UL carriers or triple beat due to 2 bands UL with 3 UL carriers issues are identified in Table 5.0.2.1.1-1 and Table 5.0.2.1.2-1, respectively.

Editor's note 2: Proponent are encouraged to provide the detailed technical analysis of the MSD requirements. For example: RF architecture, RF components parameter, and etc. In the case where the proponent is missing some elements to calculate the REFSENS exception cases, the TP can be submitted to the “not for block approval” AI for expert support.

5.1	CA_n28-n41-n79

5.1.1	Common for 1 band UL and 2 bands UL CA

5.1.1.1	Operating bands for CA

Table 5.1.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n28 | 703 MHz – 748 MHz | 703 MHz – 748MHz | FDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

5.1.1.2	Channel bandwidths per operating band for CA

Table 5.1.1.2-1: Supported bandwidths per CA band combination

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n41A-n79C | CA_n79C CA_n28A-n41A CA_n28A-n79A CA_n28A-n79C CA_n41A-n79A CA_n41A-n79C | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | CA_n79C_BCS 4 and 5 |  |
| CA_n28A-n41C-n79A | CA_n41C CA_n28A-n41A CA_n28A-n79A CA_n41A-n79A | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n79 | n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n41C-n79C | CA_n41C CA_n79C CA_n28A-n41A CA_n28A-n79A CA_n28A-n79C CA_n41A-n79A | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n79 | CA_n79C_BCS 4 and 5 |  |

5.1.1.3	∆TIB,c and ∆RIB,c values

∆TIB,c and ∆RIB,c values have already been defined in the current spec.

5.1.2	Specific for 2 bands UL CA

5.1.2.1	UE co-existence studies

The co-existence studies of fallback configurations of 2UL band with one CC per band can be applied.

5.1.2.1.1	Co-existence studies for 2UL band with 1CC per band

For CA_n28A-n41A/C-n79A/C, the two UL bands with one CC per band IMD interference analysis have already been analysed in the fallback.

5.1.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.1.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n41 interference analysis for CA_n28A-n79C with n79C transmitting with a 200MHz maximum instantaneous bandwidth.

Table 5.1.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n79C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL(n28, n79) | 703 -- 748 |  | 4400 -- 5000 |  |
| fDL(n41) | 2496 -- 2690 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -- 200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+ max2CCBW |
| IMD3 (MHz) | 503 -- 703 |  | 748 -- 948 |  |
| Analysis | There are no triple beat products caused by UL CA_n28A-n79C falls into band n41. |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.1.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n28 interference analysis for CA_n41A-n79C with n79C transmitting with a 200MHz maximum instantaneous bandwidth.

Table 5.1.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n79C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL(n41, n79) | 2496 -- 2690 |  | 4400 -- 5000 |  |
| fDL(n28) | 758 -- 803 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -- 200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+ max2CCBW |
| IMD3 (MHz) | 2296 -- 2496 |  | 2690 -- 2890 |  |
| Analysis | There are no triple beat products caused by UL CA_n41A-n79C falls into band n28. |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.1.2.2	REFSENS requirements

There is no need to define additional REFSEN requirements (i.e. MSD) for CA_n28A-n41A/C-n79C due to there are no additional triple beat co-existence issue.

5.2	CA_n8-n20-n75

5.2.1	Common for 1 band UL and 2 bands UL CA

5.2.1.1	Operating bands for CA

Table 5.2.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL19 |

5.2.1.2	Channel bandwidths per operating band for CA

Table 5.2.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n8A-n20A-n75A | CA_n8A-n20A | n8 | n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |

Editor's note: The valid UL configurations shall refer to the Annex B.

5.2.1.3	∆TIB,c and ∆RIB,c values

For CA_n8-n20-n75, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.2.2	Specific for 2 bands UL CA

5.2.2.1	UE co-existence studies

Editor’s Note: The tables in this section are provided to identify potential issues to be analyzed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

5.2.2.1.1	Co-existence studies for 2UL band with 1CC per band

Editor’s Note: Since the IMD tables have already been calculated for the different two band fallbacks, the tables below may skip the IMD calculations and refer to the relevant two band TP tables. Nonetheless, the IMD issues should be stated, and the related TPs referenced. Only one example table is provided in this template but there may be up to three cases for the victim DL band and as many tables may be needed.

Table 5.2.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n8A-n20A-n75A with UL CA_n8A-n20A.

Table 5.2.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 18–83 |  | 1712–1777 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 898–998 |  | 749–844 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2592–2692 |  | 2544–2639 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1778–1913 |  | 1581–1706 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 36–166 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3472–3607 |  | 3376–3501 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 3424–3554 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2413–2568 |  | 2658–2828 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 666–826 |  | 916–1081 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4208–4363 |  | 4352–4522 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4256–4416 |  | 4304–4469 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, no IMD produced by UL CA_n8A-n20A will fall into the n75 Rx band.

5.2.2.2	REFSENS requirements

Based on the co-existence studies there is no need to define MSD values.

5.3	CA_n8-n28-n75

5.3.1	Common for 1 band UL and 2 bands UL CA

5.3.1.1	Operating bands for CA

Table 5.3.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL19 |

5.3.1.2	Channel bandwidths per operating band for CA

Table 5.3.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n8A-n28A-n75A | CA_n8A-n28A | n8 | n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |

Editor's note: The valid UL configurations shall refer to the Annex B.

5.3.1.3	∆TIB,c and ∆RIB,c values

For CA_n8-n28-n75, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.3.2	Specific for 2 bands UL CA

5.3.2.1	UE co-existence studies

Editor’s Note: The tables in this section are provided to identify potential issues to be analyzed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

5.3.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.3.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n8A-n28A-n75A with UL CA_n8A-n28A.

Table 5.3.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 132–212 |  | 1583–1663 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1012–1127 |  | 491–616 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2463–2578 |  | 2286–2411 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1892–2042 |  | 1194–1364 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 264–424 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3343–3493 |  | 2989–3159 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 3166–3326 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1897–2112 |  | 2772–2957 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 279–484 |  | 1144–1339 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3692–3907 |  | 4008–4223 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3869–4074 |  | 4046–4241 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, there is no IMD produced by UL CA_n8A-n28A will fall into the n75 DL spectrum.

5.3.2.2	REFSENS requirements

Based on the co-existence studies there is no need to define MSD values.

5.4	CA_n20-n28-n75

5.4.1	Common for 1 band UL and 2 bands UL CA

5.4.1.1	Operating bands for CA

Table 5.4.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL19 |

5.4.1.2	Channel bandwidths per operating band for CA

Table 5.4.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n20A-n28A-n75A | CA_n20A-n28A | n20 | n20 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |

Editor's note: The valid UL configurations shall refer to the Annex B.

5.4.1.3	∆TIB,c and ∆RIB,c values

Editor’s note: For the table of ∆TIB,c and ∆RIB,c values, please use the same table format as in the latest TS 38.101-1.

For CA_n8-n28-n75, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.4.2	Specific for 2 bands UL CA

5.4.2.1	UE co-existence studies

Editor’s Note: The tables in this section are provided to identify potential issues to be analyzed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

5.4.2.1.1	Co-existence studies for 2UL band with 1CC per band

Editor’s Note: Since the IMD tables have already been calculated for the different two band fallbacks, the tables below may skip the IMD calculations and refer to the relevant two band TP tables. Nonetheless, the IMD issues should be stated, and the related TPs referenced. Only one example table is provided in this template but there may be up to three cases for the victim DL band and as many tables may be needed.

Table 5.4.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n28A-n75A with UL CA_n20A-n28A.

Table 5.4.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 84–159 |  | 1535–1610 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 916–1021 |  | 544–664 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2367–2472 |  | 2238–2358 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1748–1883 |  | 1247–1412 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 168–318 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3199–3334 |  | 2941–3106 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 3070–3220 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1950–2160 |  | 2580–2745 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 385–580 |  | 1000–1180 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3644–3854 |  | 4031–4196 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3773–3968 |  | 3902–4082 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, there is no IMD produced by UL CA_n20A-n28A will fall inte the n75 DL spectrum.

5.4.2.2	REFSENS requirements

Based on the co-existence studies there is no need to define MSD values.

5.5	CA_n3-n7-n75

5.5.1	Common for 1 band UL and 2 bands UL CA

5.5.1.1	Operating bands for CA

Table 5.5.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL19 |

5.5.1.2	Channel bandwidths per operating band for CA

Table 5.5.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| CA_n3A-n7A-n75A | CA_n3A-n7A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |

Editor's note: The valid UL configurations shall refer to the Annex B.

5.5.1.3	∆TIB,c and ∆RIB,c values

Editor’s note: For the table of ∆TIB,c and ∆RIB,c values, please use the same table format as in the latest TS 38.101-1.

For CA_n8-n28-n75, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.5.2	Specific for 2 bands UL CA

5.5.2.1	UE co-existence studies

Editor’s Note: The tables in this section are provided to identify potential issues to be analyzed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

5.5.2.1.1	Co-existence studies for 2UL band with 1CC per band

Editor’s Note: Since the IMD tables have already been calculated for the different two band fallbacks, the tables below may skip the IMD calculations and refer to the relevant two band TP tables. Nonetheless, the IMD issues should be stated, and the related TPs referenced. Only one example table is provided in this template but there may be up to three cases for the victim DL band and as many tables may be needed.

Table 5.5.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n7A-n75A with UL CA_n3A-n7A.

Table 5.5.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715–860 |  | 4210–4355 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850–1070 |  | 3215–3430 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920–6140 |  | 6710–6925 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2560–2855 |  | 5715–6000 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1430–1720 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7630–7925 |  | 9210–9495 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8420–8710 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8215–8570 |  | 4270–4640 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3930–4290 |  | 10–355 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710–12065 |  | 9340–9710 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920–11280 |  | 10130–10495 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, 4th order IMD generated by UL CA_n3A-n7A may fall into own Rx of Band n75

5.5.2.2	REFSENS requirements

Based on the co-existence studies there is no need to define MSD values.

We analyze the amount of IMD using the following component linearity assumptions:

Table 5.5.2.2-1 General linearity parameters

| Component | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| --- | --- | --- | --- | --- |
| Antenna switch | 112 | 68 | 56 | 53 |
| Diplexer | 115 | 86 | 55 | 53 |
| Triplexer | 113 | 82 | 55 | 53 |
| PA forward mixing | 28 | 32 | 30 | 28 |
| PA reverse mixing | 40 | 30 | 30 | 30 |
| LNA | 5 | -6 | -6 | -10 |

Table 5.5.2.2-2 Attenuation and isolation

| Parameters | Values (dB) |
| --- | --- |
| Antenna isolation | 10 |
| PCB isolation PA-PA | 60 |
| Diplexer isolation | 15 |
| Triplexer isolation | 15 |
| Tx duplexer rejection at Rx Band | 50 |

Based on these parameters we get the following MSD.

Table 5.5.2.2-3: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n7-n75 | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n7 | 2505 | 5 | 25 | 2625 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1450 | 10.4 | SDL | IMD4 |

5.6	CA_n3-n75-n78

5.6.1	Common for 1 band UL and 2 bands UL CA

5.6.1.1	Operating bands for CA

Table 5.6.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL19 |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |

5.6.1.2	Channel bandwidths per operating band for CA

Table 5.6.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n75A-n78A | CA_n3A-n78A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |

Editor's note: The valid UL configurations shall refer to the Annex B.

5.6.1.3	∆TIB,c and ∆RIB,c values

Editor’s note: For the table of ∆TIB,c and ∆RIB,c values, please use the same table format as in the latest TS 38.101-1.

For CA_n8-n28-n75, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.6.2	Specific for 2 bands UL CA

5.6.2.1	UE co-existence studies

Editor’s Note: The tables in this section are provided to identify potential issues to be analyzed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

5.6.2.1.1	Co-existence studies for 2UL band with 1CC per band

Editor’s Note: Since the IMD tables have already been calculated for the different two band fallbacks, the tables below may skip the IMD calculations and refer to the relevant two band TP tables. Nonetheless, the IMD issues should be stated, and the related TPs referenced. Only one example table is provided in this template but there may be up to three cases for the victim DL band and as many tables may be needed.

Table 5.6.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n75A-n78A with UL CA_n3A-n78A.

Table 5.6.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515–2090 |  | 5010–5585 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 270–380 |  | 4815–5890 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720–7370 |  | 8310–9385 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330–2055 |  | 8115–9690 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3030–4180 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430–9155 |  | 11610–13185 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10020–11170 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11415–13490 |  | 3040–3840 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6330–7980 |  | 1245–2470 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910–16985 |  | 10140–10940 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320–14970 |  | 11730–12955 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, 4th and 5th order IMD generated by UL CA_n3A-n78A may fall into own Rx of Band n75

5.6.2.2	REFSENS requirements

Based on the co-existence studies there is no need to define MSD values.

We analyze the amount of IMD using the following component linearity assumptions:

Table 5.6.2.2-1 General linearity parameters

| Component | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| --- | --- | --- | --- | --- |
| Antenna switch | 112 | 68 | 56 | 53 |
| Diplexer | 115 | 86 | 55 | 53 |
| Triplexer | 113 | 82 | 55 | 53 |
| PA forward mixing | 28 | 32 | 30 | 28 |
| PA reverse mixing | 40 | 30 | 30 | 30 |
| LNA | 5 | -6 | -6 | -10 |

Table 5.6.2.2-2 Attenuation and isolation

| Parameters | Values (dB) |
| --- | --- |
| Antenna isolation | 10 |
| PCB isolation PA-PA | 60 |
| Diplexer isolation | 15 |
| Triplexer isolation | 15 |
| Tx duplexer rejection at Rx Band | 50 |

Based on these parameters we get the following MSD.

Table 5.6.2.2-3: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n75-n78 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1450 | 10.8 | SDL | IMD4 |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1490 | 1.0 | SDL | IMD5 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |

5.7	CA_n7-n75-n78

5.7.1	Common for 1 band UL and 2 bands UL CA

5.7.1.1	Operating bands for CA

Table 5.7.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL19 |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |

5.7.1.2	Channel bandwidths per operating band for CA

Table 5.7.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n7A-n75A-n78A | CA_n7A-n78A | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |

Editor's note: The valid UL configurations shall refer to the Annex B.

5.7.1.3	∆TIB,c and ∆RIB,c values

Editor’s note: For the table of ∆TIB,c and ∆RIB,c values, please use the same table format as in the latest TS 38.101-1.

For CA_n7-n75-n78, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.7.2	Specific for 2 bands UL CA

5.7.2.1	UE co-existence studies

Editor’s Note: The tables in this section are provided to identify potential issues to be analyzed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

5.7.2.1.1	Co-existence studies for 2UL band with 1CC per band

Editor’s Note: Since the IMD tables have already been calculated for the different two band fallbacks, the tables below may skip the IMD calculations and refer to the relevant two band TP tables. Nonetheless, the IMD issues should be stated, and the related TPs referenced. Only one example table is provided in this template but there may be up to three cases for the victim DL band and as many tables may be needed.

Table 5.7.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n7A-n75A-n78A with UL CA_n7A-n78A.

Table 5.7.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730–1300 |  | 5800–6370 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1200–1840 |  | 4030–5100 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300–8940 |  | 9100–10170 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3700–4410 |  | 7330–8900 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1460–2600 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800–11510 |  | 12400–13970 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11600–12740 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10630–12700 |  | 6200–6980 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4760–6400 |  | 100–1110 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700–17770 |  | 13300–14080 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900–16540 |  | 14100–15310 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, 3rd and 4th order IMD generated by UL CA_n7A-n78A may fall into own Rx of Band n75

5.7.2.2	REFSENS requirements

Based on the co-existence studies there is no need to define MSD values.

We analyze the amount of IMD using the following component linearity assumptions:

Table 5.7.2.2-1 General linearity parameters

| Component | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| --- | --- | --- | --- | --- |
| Antenna switch | 112 | 68 | 56 | 53 |
| Diplexer | 115 | 86 | 55 | 53 |
| Triplexer | 113 | 82 | 55 | 53 |
| PA forward mixing | 28 | 32 | 30 | 28 |
| PA reverse mixing | 40 | 30 | 30 | 30 |
| LNA | 5 | -6 | -6 | -10 |

Table 5.7.2.2-2 Attenuation and isolation

| Parameters | Values (dB) |
| --- | --- |
| Antenna isolation | 10 |
| PCB isolation PA-PA | 60 |
| Diplexer isolation | 15 |
| Triplexer isolation | 15 |
| Tx duplexer rejection at Rx Band | 50 |

Based on these parameters we get the following MSD.

Table 5.7.2.2-3: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n75-n78 | n7 | 2510 | 5 | 25 | 2630 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1470 | 16 | SDL | IMD3 |
|  | n78 | 3550 | 10 | 50 | 3550 | N/A | TDD | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1500 | 10.5 | SDL | IMD4 |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |

5.8	CA_n1-n7-n75

5.8.1	Common for 1 band UL and 2 bands UL CA

5.8.1.1	Operating bands for CA

Table 5.8.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL19 |

5.8.1.2	Channel bandwidths per operating band for CA

Table 5.8.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n7A-n75A | CA_n1A-n7A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |

Editor's note: The valid UL configurations shall refer to the Annex B.

5.8.1.3	∆TIB,c and ∆RIB,c values

Editor’s note: For the table of ∆TIB,c and ∆RIB,c values, please use the same table format as in the latest TS 38.101-1.

For CA_n1-n7-n75, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.8.2	Specific for 2 bands UL CA

5.8.2.1	UE co-existence studies

Editor’s Note: The tables in this section are provided to identify potential issues to be analyzed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

5.8.2.1.1	Co-existence studies for 2UL band with 1CC per band

Editor’s Note: Since the IMD tables have already been calculated for the different two band fallbacks, the tables below may skip the IMD calculations and refer to the relevant two band TP tables. Nonetheless, the IMD issues should be stated, and the related TPs referenced. Only one example table is provided in this template but there may be up to three cases for the victim DL band and as many tables may be needed.

Table 5.8.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n7A-n75A with UL CA_n1A-n7A.

Table 5.8.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520–650 |  | 4420–4550 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270–1460 |  | 3020–3220 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340–6530 |  | 6920–7120 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3190–3440 |  | 5520–5790 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1040–1300 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8260–8510 |  | 9420–9690 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8840–9100 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8020–8360 |  | 5110–5420 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3540–3870 |  | 620–940 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920–12260 |  | 10180–10490 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340–11670 |  | 10760–11080 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, 3rd order IMD generated by UL CA_n1A-n7A may fall into own Rx of Band n75

5.8.2.2	REFSENS requirements

Based on the co-existence studies there is no need to define MSD values.

We analyze the amount of IMD using the following component linearity assumptions:

Table 5.8.2.2-1 General linearity parameters

| Component | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| --- | --- | --- | --- | --- |
| Antenna switch | 112 | 68 | 56 | 53 |
| Diplexer | 115 | 86 | 55 | 53 |
| Triplexer | 113 | 82 | 55 | 53 |
| PA forward mixing | 28 | 32 | 30 | 28 |
| PA reverse mixing | 40 | 30 | 30 | 30 |
| LNA | 5 | -6 | -6 | -10 |

Table 5.8.2.2-2 Attenuation and isolation

| Parameters | Values (dB) |
| --- | --- |
| Antenna isolation | 10 |
| PCB isolation PA-PA | 60 |
| Diplexer isolation | 15 |
| Triplexer isolation | 15 |
| Tx duplexer rejection at Rx Band | 50 |

Based on these parameters we get the following MSD.

Table 5.8.2.2-3: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n7-n75 | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n7 | 2510 | 5 | 25 | 2630 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1450 | 13.2 | SDL | IMD3 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.9	CA_n1-n3-n7

5.9.1	Common for 1 band UL and 2 bands UL CA

5.9.1.1	Operating bands for CA

Table 5.9.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |

5.9.1.2	Channel bandwidths per operating band for CA

Table 5.9.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n3A-n7A | CA_n1A-n3A CA_n1A-n7A CA_n3A-n7A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |

Editor's note: The valid UL configurations shall refer to the Annex B.

5.9.1.3	∆TIB,c and ∆RIB,c values

Editor’s note: For the table of ∆TIB,c and ∆RIB,c values, please use the same table format as in the latest TS 38.101-1.

For CA_n1A-n3A-n7A, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.9.2	Specific for 2 bands UL CA

5.9.2.1	UE co-existence studies

Editor’s Note: The tables in this section are provided to identify potential issues to be analyzed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

5.9.2.1.1	Co-existence studies for 2UL band with 1CC per band

Editor’s Note: Since the IMD tables have already been calculated for the different two band fallbacks, the tables below may skip the IMD calculations and refer to the relevant two band TP tables. Nonetheless, the IMD issues should be stated, and the related TPs referenced. Only one example table is provided in this template but there may be up to three cases for the victim DL band and as many tables may be needed.

Table 5.9.2.1.1-1, 5.9.2.1.1-2 and 5.9.2.1.1-3 provide the two UL bands with one CC per band IMD interference analysis for CA_n1A-n3A-n7A with UL CA_n1A-n3A, CA_n1A-n7A and CA_n3A-n7A.

Table 5.9.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730–1300 |  | 5800–6370 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1200–1840 |  | 4030–5100 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300–8940 |  | 9100–10170 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3700–4410 |  | 7330–8900 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1460–2600 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800–11510 |  | 12400–13970 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11600–12740 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10630–12700 |  | 6200–6980 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4760–6400 |  | 100–1110 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700–17770 |  | 13300–14080 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900–16540 |  | 14100–15310 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, no IMD produced by UL CA_n1A-n3A will fall into the n7 Rx band.

Table 5.9.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520–650 |  | 4420–4550 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270–1460 |  | 3020–3220 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340–6530 |  | 6920–7120 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3190–3440 |  | 5520–5790 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1040–1300 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8260–8510 |  | 9420–9690 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8840–9100 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8020–8360 |  | 5110–5420 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3540–3870 |  | 620–940 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920–12260 |  | 10490–10180 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340–11670 |  | 10760–11080 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, no IMD produced by UL CA_n1A-n7A will fall into the n3 Rx band.

Table 5.9.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320–1880 |  | 5220–5780 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 40–660 |  | 4620–5680 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140–7760 |  | 8520–9580 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960–2640 |  | 7920–9480 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2640–3760 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060–9740 |  | 11820–13380 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10440–11560 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11220–13280 |  | 3880–4620 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5940–7560 |  | 660–1840 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120–17180 |  | 10980–11720 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740–15360 |  | 12360–13540 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above tables, no IMD produced by UL CA_n3A-n7A will fall into the n1 Rx band.

5.9.2.2	REFSENS requirements

Based on the co-existence studies, there is no need to define MSD values.

5.10	CA_n41-n66-n71

5.10.1	Common for 1 band UL and 2 bands UL CA

5.10.1.1	Operating bands for CA.

Table 5.10.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz –2690 MHz | TDD |
| n66 | 1710 MHz – 1780 MHz | 2100 MHz – 2200 MHz | FDD |
| n71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |

5.10.1.2	Channel bandwidths per operating band for CA

Table 5.10.1.2-1: Supported bandwidths per CA band combination of band n25+n41+n77

| NR CA configuration |  | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n41C-n66A-n71A | n417,9 CA_n41A-n66A7 CA_n41C-n66A CA_n41A-n71A7 CA_n41C-n71A CA_n41C7 CA_n66A-n71A |  | n41 | CA_n41C_BCS0 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  |  | n71 | 5, 10, 15, 20 |  |
|  |  |  | n41 | CA_n41C_BCS1 | 1 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n71 | 5, 10, 15, 20 |  |
|  |  |  | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |

5.10.2	Specific for 2 bands UL CA

5.10.2.1	UE co-existence studies

5.10.2.1.1	Co-existence studies for 2UL band with 1CC per band

Co-existence study has been performed in Rel-18.

5.10.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.10.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n71.

Table 5.10.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n41C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n66, n41) | 1710 MHz – 1780 MHz |  | 2496 MHz– 2690 MHz |  |
| fDL (n71) | 617 MHz – 652 MHz |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 – 190 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1520 MHz – 1710 MHz |  | 1780 MHz – 1970 MHz |  |
| Analysis | No issue |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.10.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n66.

Table 5.10.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n41C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n71, n41) | 663 MHz – 698 MHz |  | 2496 MHz– 2690 MHz |  |
| fDL (n66) | 2110 MHz – 2200 MHz |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 – 190 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 473 MHz – 663 MHz |  | 698 MHz – 888 MHz |  |
| Analysis | No issue |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.10.2.2	REFSENS requirements

Based on the triple beat analysis of the added ULCA, 1st order triple beat IMD is not falling into bands n66 and n71, so there is no need to define MSD values.

5.11	CA_n1-n41-n78

5.11.1	Common for 1 band UL and 2 bands UL CA

5.11.1.1	Operating bands for CA

When adding CA_n1-n41-n78 to Table 5.2A.2.2-1 in 38.101-1, it must be marked with Note 3 as CA_n1-n41-n783.

Table 5.11.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

This combination supports inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability.

5.11.1.2	Channel bandwidths per operating band for CA

Table 5.11.1.2-1: Supported bandwidths per CA band combination

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n41A-n78A | CA_n1A-n41A CA_n1A-n78A CA_n41A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 100 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |

5.11.1.3	∆TIB,c and ∆RIB,c values

Requirements for CA_n1-n41-n78, for the TIB,c and RIB,c values are given in the tables below following CA_n1-n41-n77.

Table 5.11.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n41-n78 | 0.5 | 0.5 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.11.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n41-n78 | 0.2 | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.11.2	Specific for 2 bands UL CA

5.11.2.1	UE co-existence studies

Table 5.11.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n41A-n78A with UL CA_n1A-n4A.

Table 5.11.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 516 - 770 |  | 4416 - 4670 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1150 - 1464 |  | 3012 - 3460 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6336 - 6650 |  | 6912 - 7360 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3070 - 3444 |  | 5508 - 6150 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1540 - 1032 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8256 - 8630 |  | 9408 - 10050 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 9220 - 8952 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8840 - 8004 |  | 5424 - 4990 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4230 - 3528 |  | 948 - 380 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11904 - 12740 |  | 10176 - 10610 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11328 - 12030 |  | 10752 - 11320 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.11.2.1.1-1, 3rd, 4th and 5th order IMD from band n1 and Band n41 may also fall into Rx frequencies of band n78.

Table 5.11.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320 - 1880 |  | 5220 - 5780 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 40 - 660 |  | 4620 - 5680 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 - 7760 |  | 8520 - 9580 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960 - 2640 |  | 7920 - 9480 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3760 - 2640 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 - 9740 |  | 11820 - 13380 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11440 - 10560 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13280 - 11220 |  | 4620 - 3880 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4230 - 3528 |  | 948 - 380 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7560 - 5940 |  | 660 - 1840 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 - 15360 |  | 12360 - 13540 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.11.2.1.1-2, 4th order IMD from band n1 and Band n78 may also fall into Rx frequencies of band n41.

Table 5.11.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 610 - 1304 |  | 5796 - 6490 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1192 - 2080 |  | 3910 - 5104 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8292 - 9180 |  | 9096 - 10290 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3688 - 4770 |  | 7210 - 8904 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2608 - 1220 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10788 - 11870 |  | 12396 - 14090 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 12592 - 11980 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12704 - 10510 |  | 7460 - 6184 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6408 - 4520 |  | 1470 - 112 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15696 - 17890 |  | 13284 - 14560 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14892 - 16780 |  | 14088 - 15670 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.11.2.1.1-3, 4th order IMD from band n41 and Band n78 may also fall into Rx frequencies of band n1.

From the tables it is found that:

-	Band n78 may be subject to IMD3, IMD4 and IMD5.

-	Band n41 may be subject to IMD4.

-	Band n1 may be subject to IMD4.

5.11.2.2	REFSENS requirements

MSD values have been taken from CA_n1A-n41A-n77A.

Table 5.11.2.2-1: MSD due to IMD issue

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n41-n78 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3330 | 19.6 | TDD | IMD31,2 |
|  | n1 | 1975 | 5 | 10 | 2165 | N/A | FDD | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2515 | 11.5 | TDD | IMD4 |
|  | n1 | N/A | 5 | N/A | 2140 | 9.3 | FDD | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n41 | 2640 | 10 | 50 | 2640 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.12	CA_n1-n5-n8

5.12.1	Common for 1 band UL and 2 bands UL CA

5.12.1.1	Operating bands for CA

Table 5.12.1.1-1: CA band combination constituent bands definition

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n5-n8 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |

5.12.1.2	Channel bandwidths per operating band for CA

Table 5.12.1.2-1: Supported bandwidths per CA band combination of band n1-n5-n8

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n5A-n8A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n8 | 5, 10, 15, 20 |  |

5.12.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n5-n8, since there is only single carrier UL the following TIB,c  values are proposed:

Table 5.12.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n1-n5-n8 | 0.3 | 0.5 | 0.5 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

The RIB,c values are proposed to be defined following CA_n5-n8 as in the following table

Table 5.12.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n1-n5-n8 | - | 0.4 | 0.4 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.13	CA_n3-n5-n8

5.13.1	Common for 1 band UL and 2 bands UL CA

5.13.1.1	Operating bands for CA

Table 5.13.1.1-1: CA band combination constituent bands definition

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n5-n8 | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |

5.13.1.2	Channel bandwidths per operating band for CA

Table 5.13.1.2-1: Supported bandwidths per CA band combination of band n3-n5-n8

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n5A-n8A | - | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n8 | 5, 10, 15, 20 |  |

5.13.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n5-n8, since there is only single carrier UL the following TIB,c  values are proposed:

Table 5.13.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n3-n5-n8 | 0.3 | 0.5 | 0.5 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

The RIB,c values are proposed to be defined following CA_n5-n8 as in the following table.

Table 5.13.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n3-n5-n8 | - | 0.4 | 0.4 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.14	CA_n25-n29-n77

5.14.1	Common for 1 band UL and 2 bands UL CA

5.14.1.1	Operating bands for CA

Table 5.14.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n25 | 1850 MHz – 1915 MHz | 1930 MHz – 1995 MHz | FDD |
| n29 | N/A | 717 MHz – 728 MHz | SDL |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

5.14.1.2	Channel bandwidths per operating band for CA

Table 5.14.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n25A-n29A-n77A | CA_n25A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n29 | n29 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n29A-n77(2A) | CA_n25A-n77A CA_n77(2A) | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n29 | n29 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n25A-n29A-n77(3A) | CA_n25A-n77A CA_n77(2A) | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n29 | n29 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(3A)_BCS4 and 5 |  |

5.14.1.3	∆TIB,c and ∆RIB,c values

For CA_n25-n29-n77 the TIB,c and RIB,c values are given in the tables below.

Table 5.14.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n25-n29-n77 | 0.6 | N/A | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.14.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n25-n29-n77 | 0.2 | 0.2 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.14.2	Specific for 2 bands UL CA

5.14.2.1	UE co-existence studies

5.14.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.14.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n25A-n29A-n77A with UL CA_n25A-n77A.

Table 5.14.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1385–2350 |  | 5150–6115 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 500–530 |  | 4685–6550 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7000–8030 |  | 8450–10315 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1350–2445 |  | 7985–10750 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2770–4700 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8850–9945 |  | 11750–14515 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10300–12230 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11285–14950 |  | 3200–4360 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6070–8900 |  | 855–2850 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15050–18715 |  | 10700–11860 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13600–16430 |  | 12150–14145 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, there is no IMD generated by UL CA_n25A-n77A fall into own Rx of Band n29.

In addition, there is no additional MSD due to UL CA_n77(2A) needs to specified for three bands DL configuration.

5.14.2.2	REFSENS requirements

Based on the co-existence studies, there is no need to define MSD values.

5.15	CA_n1-n75-n78

5.15.1	Common for 1 band UL and 2 bands UL CA

5.15.1.1	Operating bands for CA

Table 5.15.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920 MHz –1980 MHz | 2110 MHz –2170 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL19 |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |

5.15.1.2	Channel bandwidths per operating band for CA

Table 5.15.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n75A-n78A | CA_n1A-n78A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |

5.15.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n75-n78, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.15.2	Specific for 2 bands UL CA

5.15.2.1	UE co-existence studies

Editor’s Note: The tables in this section are provided to identify potential issues to be analyzed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

5.15.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.15.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n75A-n78A with UL CA_n1A-n78A.

Table 5.15.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320–1880 |  | 5220–5780 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 40–660 |  | 4620–5680 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140–7760 |  | 8520–9580 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960–2640 |  | 7920–9480 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2640–3760 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060–9740 |  | 11820–13380 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10440–11560 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11220–13280 |  | 3880–4620 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5940–7560 |  | 660–1840 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120–17180 |  | 10980–11720 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740–15360 |  | 12360–13540 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 2nd and 5th order IMD generated by UL CA_n1A-n78A may fall into own Rx of Band n75

5.15.2.2	REFSENS requirements

IMD5 generated by UL CA_n1A-n78A fall into own Rx of Band n75.However, since the MSD value is 0 dB, there is no need to define MSD of IMD5. Referring to DC_1A_n75A-n78A, the MSD requirements can be specified as below:

Table 5.15.2.2-3: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n75-n78 | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1470 | 30.4 | SDL | IMD2 |
|  | n78 | 3400 | 10 | 52 | 3400 | N/A | TDD | N/A |

5.16	CA_n1-n28-n40

5.16.1	Common for 1 band UL and 2 bands UL CA

5.16.1.1	Operating bands for CA

Table 5.16.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n28 | 703-748 | 758-803 | FDD |
| n40 | 2300-2400 | 2300-2400 | TDD |

5.16.1.2	Channel bandwidths per operating band for CA

Table 5.16.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n28A-n40A | CA_n1A-n28A CA_n1A-n40A CA_n28A-n40A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

5.16.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n28-n40, the TIB,c and RIB,c values are already captured in the specification.

5.16.2	Specific for 2 bands UL CA

5.16.2.1	UE co-existence studies

5.16.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.16.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n28A-n40A with UL CA_n1A-n28A.

Table 5.16.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1277 - 1172 |  | 2623 - 2728 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3092 - 3257 |  | 574 - 424 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4543 - 4708 |  | 3326 - 3476 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5012 - 5237 |  | 129 - 324 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2344 - 2554 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6463 - 6688 |  | 4029 - 4224 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5246 - 5456 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1072 - 832 |  | 7217 - 6932 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1596 - 1851 |  | 4534 - 4264 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4732 - 4972 |  | 8383 - 8668 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5949 - 6204 |  | 7166 - 7436 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.16.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n28A-n40A with UL CA_n1A-n40A.

Table 5.16.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 320 - 480 |  | 4220 - 4380 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 - 1660 |  | 2620 - 2880 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6140 - 6360 |  | 6520 - 6780 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3360 - 3640 |  | 4920 - 5280 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 960 - 640 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8060 - 8340 |  | 8820 - 9180 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8440 - 8760 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7680 - 7220 |  | 5620 - 5280 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3360 - 2940 |  | 1340 - 960 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11120 - 11580 |  | 9980 - 10320 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10740 - 11160 |  | 10360 - 10740 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.16.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n28A-n40A with UL CA_n28A-n40A.

Table 5.16.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1552 - 1697 |  | 3003 - 3148 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 994 - 804 |  | 3852 - 4097 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3706 - 3896 |  | 5303 - 5548 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 291 - 56 |  | 6152 - 6497 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3394 - 3104 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4409 - 4644 |  | 7603 - 7948 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6006 - 6296 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8897 - 8452 |  | 692 - 412 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5794 - 5404 |  | 2356 - 2691 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9903 - 10348 |  | 5112 - 5392 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8306 - 8696 |  | 6709 - 7044 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.16.2.1.1-1 there is IMD4 from n1 and n28 into RX band n40.

Based on the analysis in Table 5.16.2.1.1-2 there is IMD4 from n1 and n40 into RX band n28.

5.16.2.2	REFSENS requirements

The MSD value has been taken from CA_n1-n5-n40.

Table 5.16.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n28-n40 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 780 | 8 | FDD | IMD4 |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | TDD | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2390 | 9.0 | TDD | IMD4 |

5.17	CA_n1-n20-n71

5.17.1	Common for 1 band UL and 2 bands UL CA

5.17.1.1	Operating bands for CA

Table 5.17.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n20 | 832-862 | 791-821 | FDD |
| n71 | 663-698 | 617-652 | FDD |

5.17.1.2	Channel bandwidths per operating band for CA

Table 5.17.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n20A-n71A | CA_n1A-n20A CA_n1A-n71A CA_n20A-n71A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n71 | 5, 10, 15, 20 |  |

5.17.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n20-n71, the TIB,c and RIB,c values are re-used from CA_n1-n5-n105 as given in the tables below.

Table 5.17.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n20-n71 | - | 0.2 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.17.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n20-n71 | 0.3 | 0.6 | 0.6 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.17.2	Specific for 2 bands UL CA

5.17.2.1	UE co-existence studies

5.17.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.17.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n71A with UL CA_n1A-n20A.

Table 5.17.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1148 - 1058 |  | 2752 - 2842 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2978 - 3128 |  | 316 - 196 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4672 - 4822 |  | 3584 - 3704 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4898 - 5108 |  | 516 - 666 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2116 - 2296 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6592 - 6802 |  | 4416 - 4566 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5504 - 5684 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1528 - 1348 |  | 7088 - 6818 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1254 - 1464 |  | 4276 - 4036 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5248 - 5428 |  | 8512 - 8782 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6336 - 6546 |  | 7424 - 7664 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.17.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n71A with UL CA_n1A-n71A.

Table 5.17.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1317 - 1222 |  | 2583 - 2678 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3142 - 3297 |  | 654 - 524 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4503 - 4658 |  | 3246 - 3376 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5062 - 5277 |  | 9 - 174 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2444 - 2634 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6423 - 6638 |  | 3909 - 4074 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5166 - 5356 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 872 - 672 |  | 7257 - 6982 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1746 - 1971 |  | 4614 - 4364 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4572 - 4772 |  | 8343 - 8618 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5829 - 6054 |  | 7086 - 7336 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.17.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n71A with UL CA_n20A-n71A.

Table 5.17.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 199 - 134 |  | 1495 - 1560 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 966 - 1061 |  | 464 - 564 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2327 - 2422 |  | 2158 - 2258 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1798 - 1923 |  | 1127 - 1262 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 268 - 398 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3159 - 3284 |  | 2821 - 2956 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 2990 - 3120 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1960 - 1790 |  | 2785 - 2630 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 430 - 265 |  | 1260 - 1100 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3484 - 3654 |  | 3991 - 4146 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3653 - 3818 |  | 3822 - 3982 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.17.2.1.1-1 there is IMD4 from n1 and n20 into RX band n71.

Based on the analysis in Table 5.17.2.1.1-2 there is IMD5 from n1 and n71 into RX band n20.

Based on the analysis in Table 5.17.2.1.1-3 there is IMD3 from n20 and n71 into RX band n1.

5.17.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n1-n5-n105.

Table 5.17.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n20-n71 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n20 | 859 | 5 | 25 | 818 | N/A | FDD | N/A |
|  | n71 | N/A | 5 | N/A | 627 | 9.4 | FDD | IMD4 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 4.6 | FDD | IMD5 |
|  | n71 | 689 | 5 | 25 | 643 | N/A | FDD | N/A |
|  | n1 | N/A | 5 | N/A | 2167 | 16 | FDD | IMD3 |
|  | n20 | 835 | 5 | 25 | 794 | N/A | FDD | N/A |
|  | n71 | 666 | 5 | 25 | 620 | N/A | FDD | N/A |

5.18	CA_n1-n20-n78

5.18.1	Common for 1 band UL and 2 bands UL CA

5.18.1.1	Operating bands for CA

Table 5.18.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n20 | 832-862 | 791-821 | FDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.18.1.2	Channel bandwidths per operating band for CA

Table 5.18.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n20A-n78A | CA_n1A-n20A CA_n1A-n78A CA_n20A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

5.18.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n20-n78, the TIB,c and RIB,c values are already captured in the specification.

5.18.2	Specific for 2 bands UL CA

5.18.2.1	UE co-existence studies

5.18.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.18.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n78A with UL CA_n1A-n20A.

Table 5.18.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1148 - 1058 |  | 2752 - 2842 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2978 - 3128 |  | 316 - 196 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4672 - 4822 |  | 3584 - 3704 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4898 - 5108 |  | 516 - 666 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2116 - 2296 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6592 - 6802 |  | 4416 - 4566 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5504 - 5684 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1528 - 1348 |  | 7088 - 6818 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1254 - 1464 |  | 4276 - 4036 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5248 - 5428 |  | 8512 - 8782 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6336 - 6546 |  | 7424 - 7664 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.18.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n78A with UL CA_n1A-n78A.

Table 5.18.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320 - 1880 |  | 5220 - 5780 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 40 - 660 |  | 4620 - 5680 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 - 7760 |  | 8520 - 9580 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960 - 2640 |  | 7920 - 9480 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3760 - 2640 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 - 9740 |  | 11820 - 13380 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10440 - 11560 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13280 - 11220 |  | 4620 - 3880 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7560 - 5940 |  | 660 - 1840 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 - 17180 |  | 10980 - 11720 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 - 15360 |  | 12360 - 13540 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.18.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n78A with UL CA_n20A-n78A.

Table 5.18.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2438 - 2968 |  | 4132 - 4662 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2136 - 1576 |  | 5738 - 6768 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4964 - 5524 |  | 7432 - 8462 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1304 - 714 |  | 9038 - 10568 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 5936 - 4876 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 - 6386 |  | 10732 - 12262 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8264 - 9324 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14368 - 12338 |  | 148 - 472 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9736 - 8176 |  | 4014 - 5104 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 - 16062 |  | 6628 - 7248 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 - 13124 |  | 9096 - 10186 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.18.2.1.1-1 there is IMD3 from n1 and n20 into RX band n78.

Based on the analysis in Table 5.18.2.1.1-2 there is IMD5 from n1 and n78 into RX band n20.

Based on the analysis in Table 5.18.2.1.1-3 there is IMD3 from n20 and n78 into RX band n1.

5.18.2.2	REFSENS requirements

The MSD value has been taken from CA_n1-n26-n78.

Table 5.18.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n20-n78 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3644 | 15.7 | TDD | IMD3 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 3.1 | FDD | IMD5 |
|  | n78 | 3328 | 10 | 50 | 3328 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2120 | 18.1 | FDD | IMD3 |
|  | n20 | 835 | 5 | 25 | 794 | N/A | FDD | N/A |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |

5.19	CA_n20-n41-n71

5.19.1	Common for 1 band UL and 2 bands UL CA

5.19.1.1	Operating bands for CA

Table 5.19.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n20 | 832-862 | 791-821 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |
| n71 | 663-698 | 617-652 | FDD |

5.19.1.2	Channel bandwidths per operating band for CA

Table 5.19.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n20A-n41A-n71A | CA_n20A-n41A CA_n20A-n71A CA_n41A-n71A | n20 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |

5.19.1.3	∆TIB,c and ∆RIB,c values

For CA_n20-n41-n71, the TIB,c values are taken from CA_n5-n7-n105 as given in the tables below.

Table 5.19.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n20-n41-n71 | 0.5 | 0.3 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

RIB,c values are taken from DC_5-7_n71 as given in the tables below

Table 5.19.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n20-n41-n71 | - | - | 0.2 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.19.2	Specific for 2 bands UL CA

5.19.2.1	UE co-existence studies

5.19.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.19.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n41A-n71A with UL CA_n20A-n41A.

Table 5.19.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1634 - 1858 |  | 3328 - 3552 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1026 - 772 |  | 4130 - 4548 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4160 - 4414 |  | 5824 - 6242 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 194 - 90 |  | 6626 - 7238 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3716 - 3268 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4992 - 5276 |  | 8320 - 8932 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6656 - 7104 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9928 - 9122 |  | 952 - 638 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6406 - 5764 |  | 2406 - 2884 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10816 - 11622 |  | 5824 - 6138 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9152 - 9794 |  | 7488 - 7966 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.19.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n41A-n71A with UL CA_n20A-n71A.

Table 5.19.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 199 - 134 |  | 1495 - 1560 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 966 - 1061 |  | 464 - 564 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2327 - 2422 |  | 2158 - 2258 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1798 - 1923 |  | 1127 - 1262 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 268 - 398 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3159 - 3284 |  | 2821 - 2956 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 2990 - 3120 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1960 - 1790 |  | 2785 - 2630 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 430 - 265 |  | 1260 - 1100 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3484 - 3654 |  | 3991 - 4146 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3653 - 3818 |  | 3822 - 3982 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.19.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n41A-n71A with UL CA_n41A-n71A.

Table 5.19.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2027 - 1798 |  | 3159 - 3388 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4294 - 4717 |  | 1364 - 1100 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5655 - 6078 |  | 3822 - 4086 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6790 - 7407 |  | 701 - 402 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3596 - 4054 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8151 - 8768 |  | 4485 - 4784 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6318 - 6776 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 296 - 38 |  | 10097 - 9286 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2898 - 3391 |  | 6744 - 6092 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5148 - 5482 |  | 10647 - 11458 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6981 - 7474 |  | 8814 - 9466 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.19.2.1.1-1 there is IMD5 from n20 and n41 into RX band n71.

Based on the analysis in Table 5.19.2.1.1-2 there is IMD5 from n20 and n71 into RX band n41.

5.19.2.2	REFSENS requirements

The MSD requirements are based on CA_n18-n28-n41.

Table 5.19.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n20-n41-n71 | n20 | 834 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n41 | 2686 | 10 | 25 | 2686 | N/A | TDD | N/A |
|  | n71 | N/A | 5 | N/A | 650 | 5.9 | FDD | IMD5 |
|  | n20 | 835 | 5 | 25 | 794 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2660 | 6.5 | TDD | IMD5 |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |

5.20	CA_n20-n41-n77

5.20.1	Common for 1 band UL and 2 bands UL CA

5.20.1.1	Operating bands for CA

Table 5.20.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n20 | 832-862 | 791-821 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |
| n77 | 3300-4200 | 3300-4200 | TDD |

5.20.1.2	Channel bandwidths per operating band for CA

Table 5.20.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n20A-n41A-n77A | CA_n20A-n41A CA_n20A-n77A CA_n41A-n77A | n20 | 5,10,15,20 | 0 |
|  |  | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n20A-n41A-n77(2A) | CA_n20A-n41A CA_n20A-n77A CA_n41A-n77A | n20 | 5,10,15,20 | 0 |
|  |  | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |

5.20.1.3	∆TIB,c and ∆RIB,c values

For CA_n20-n41-n77, the TIB,c and RIB,c values are re-used from CA_n5-n41-n77 as given in the tables below.

Table 5.20.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n20-n41-n77 | 0.6 | 0.3 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.20.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n20-n41-n77 | 0.2 | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.20.2	Specific for 2 bands UL CA

5.20.2.1	UE co-existence studies

5.20.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.20.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n41A-n77A with UL CA_n20A-n41A.

Table 5.20.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1634 - 1858 |  | 3328 - 3552 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1026 - 772 |  | 4130 - 4548 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4160 - 4414 |  | 5824 - 6242 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 194 - 90 |  | 6626 - 7238 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3716 - 3268 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4992 - 5276 |  | 8320 - 8932 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6656 - 7104 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9928 - 9122 |  | 952 - 638 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6406 - 5764 |  | 2406 - 2884 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10816 - 11622 |  | 5824 - 6138 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9152 - 9794 |  | 7488 - 7966 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.20.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n41A-n77A with UL CA_n20A-n77A.

Table 5.20.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2438 - 3368 |  | 4132 - 5062 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2536 - 1576 |  | 5738 - 7568 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4964 - 5924 |  | 7432 - 9262 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1704 - 714 |  | 9038 - 11768 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 6736 - 4876 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 - 6786 |  | 10732 - 13462 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8264 - 10124 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15968 - 12338 |  | 148 - 872 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10936 - 8176 |  | 4014 - 5904 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 - 17662 |  | 6628 - 7648 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 - 14324 |  | 9096 - 10986 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.20.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n41A-n77A with UL CA_n41A-n77A.

Table 5.20.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 610 - 1704 |  | 5796 - 6890 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 792 - 2080 |  | 3910 - 5904 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8292 - 9580 |  | 9096 - 11090 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3288 - 4770 |  | 7210 - 10104 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3408 - 1220 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10788 - 12270 |  | 12396 - 15290 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11592 - 13780 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14304 - 10510 |  | 7460 - 5784 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7608 - 4520 |  | 1470 - 912 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15696 - 19490 |  | 13284 - 14960 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14892 - 17980 |  | 14088 - 16470 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.20.2.1.1-1 there are IMD2, IMD3 and IMD4 from n20 and n41 into RX band n77.

Based on the analysis in Table 5.20.2.1.1-2 there are IMD2 and IMD3 from n20 and n77 into RX band n41.

Based on the analysis in Table 5.20.2.1.1-3 there are IMD2 and IMD3 from n41 and n77 into RX band n20.

5.20.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n5-n41-n77.

Table 5.20.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n20-n41-n77 | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | 2593 | 5 | 25 | 2593 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3440 | 29.7 | TDD | IMD22 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | 2506 | 5 | 25 | 2506 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 4165 | 16.1 | TDD | IMD3 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | N/A | 5 | N/A | 2593 | 30.1 | TDD | IMD2 |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n20 | 837 | 5 | 25 | 796 | N/A | FDD | N/A |
|  | n41 | N/A | 5 | N/A | 2516 | 13.2 | TDD | IMD3 |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 30.2 | FDD | IMD2 |
|  | n41 | 2594 | 5 | 25 | 2594 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n20 | N/A | 5 | N/A | 817 | 15.2 | FDD | IMD3 |
|  | n41 | 2501 | 5 | 25 | 2501 | N/A | TDD | N/A |
|  | n77 | 4185 | 10 | 50 | 4185 | N/A | TDD | N/A |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.21	CA_n20-n41-n78

5.21.1	Common for 1 band UL and 2 bands UL CA

5.21.1.1	Operating bands for CA

Table 5.21.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n20 | 832-862 | 791-821 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.21.1.2	Channel bandwidths per operating band for CA

Table 5.21.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n20A-n41A-n78A | CA_n20A-n41A CA_n20A-n78A CA_n41A-n78A | n20 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

5.21.1.3	∆TIB,c and ∆RIB,c values

For CA_n20-n41-n78, the TIB,c and RIB,c values are re-used from DC_20_n41-n78 as given in the tables below.

Table 5.21.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n20-n41-n78 | 0.5 | 0.3 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.21.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n20-n41-n78 | - | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.21.2	Specific for 2 bands UL CA

5.21.2.1	UE co-existence studies

5.21.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.21.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n41A-n78A with UL CA_n20A-n41A.

Table 5.21.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1634 - 1858 |  | 3328 - 3552 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1026 - 772 |  | 4130 - 4548 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4160 - 4414 |  | 5824 - 6242 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 194 - 90 |  | 6626 - 7238 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3716 - 3268 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4992 - 5276 |  | 8320 - 8932 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6656 - 7104 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9928 - 9122 |  | 952 - 638 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6406 - 5764 |  | 2406 - 2884 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10816 - 11622 |  | 5824 - 6138 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9152 - 9794 |  | 7488 - 7966 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.21.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n41A-n78A with UL CA_n20A-n78A.

Table 5.21.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2438 - 2968 |  | 4132 - 4662 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2136 - 1576 |  | 5738 - 6768 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4964 - 5524 |  | 7432 - 8462 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1304 - 714 |  | 9038 - 10568 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 5936 - 4876 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 - 6386 |  | 10732 - 12262 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8264 - 9324 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14368 - 12338 |  | 148 - 472 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9736 - 8176 |  | 4014 - 5104 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 - 16062 |  | 6628 - 7248 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 - 13124 |  | 9096 - 10186 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.21.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n41A-n78A with UL CA_n41A-n78A.

Table 5.21.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 610 - 1304 |  | 5796 - 6490 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1192 - 2080 |  | 3910 - 5104 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8292 - 9180 |  | 9096 - 10290 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3688 - 4770 |  | 7210 - 8904 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2608 - 1220 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10788 - 11870 |  | 12396 - 14090 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11592 - 12980 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12704 - 10510 |  | 7460 - 6184 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6408 - 4520 |  | 1470 - 112 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15696 - 17890 |  | 13284 - 14560 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14892 - 16780 |  | 14088 - 15670 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.21.2.1.1-2 there are IMD2 and IMD4 from n20 and n41 into RX band n78.

Based on the analysis in Table 5.21.2.1.1-2 there is IMD2 from n20 and n78 into RX band n41.

Based on the analysis in Table 5.21.2.1.1-3 there are IMD2 and IMD5 from n41 and n78 into RX band n20.

5.21.2.2	REFSENS requirements

The MSD requirements for IMD2 components are re-used from DC_20A-n41A-n78A.

The MSD requirements for IMD4 on Rx n78 and IMD5 on Rx n20 are reuse from CA_n5-n7-n78.

Table 5.21.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n20-n41-n78 | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | 2593 | 10 | 50 | 2593 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3440 | 28.8 | TDD | IMD2 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | 2601 | 10 | 50 | 2601 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3508 | 9.7 | TDD | IMD4 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2593 | 29.8 | TDD | IMD2 |
|  | n78 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 30.8 | FDD | IMD2 |
|  | n41 | 2593 | 10 | 50 | 2593 | N/A | TDD | N/A |
|  | n78 | 3399 | 10 | 50 | 3399 | N/A | TDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 3.3 | FDD | IMD5 |
|  | n41 | 2635 | 10 | 50 | 2635 | N/A | TDD | N/A |
|  | n78 | 3549.5 | 10 | 50 | 3549.5 | N/A | TDD | N/A |

5.22	CA_n20-n71-n78

5.22.1	Common for 1 band UL and 2 bands UL CA

5.22.1.1	Operating bands for CA

Table 5.22.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n20 | 832-862 | 791-821 | FDD |
| n71 | 663-698 | 617-652 | FDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.22.1.2	Channel bandwidths per operating band for CA

Table 5.22.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n20A-n71A-n78A | CA_n20A-n71A CA_n20A-n78A CA_n71A-n78A | n20 | 5, 10, 15, 20 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

5.22.1.3	∆TIB,c and ∆RIB,c values

For CA_n20-n71-n78, the TIB,c and RIB,c values are re-used from CA_n5-n78-n105 as given in the tables below.

Table 5.22.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n20-n71-n78 | 0.7 | 0.7 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.22.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n20-n71-n78 | 0.2 | 0.2 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.22.2	Specific for 2 bands UL CA

5.22.2.1	UE co-existence studies

5.22.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.22.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n71A-n78A with UL CA_n20A-n71A.

Table 5.22.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 199 - 134 |  | 1495 - 1560 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 966 - 1061 |  | 464 - 564 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2327 - 2422 |  | 2158 - 2258 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1798 - 1923 |  | 1127 - 1262 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 268 - 398 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3159 - 3284 |  | 2821 - 2956 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 2990 - 3120 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1960 - 1790 |  | 2785 - 2630 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 430 - 265 |  | 1260 - 1100 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3484 - 3654 |  | 3991 - 4146 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3653 - 3818 |  | 3822 - 3982 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.22.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n71A-n78A with UL CA_n20A-n78A.

Table 5.22.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2438 - 2968 |  | 4132 - 4662 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2136 - 1576 |  | 5738 - 6768 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4964 - 5524 |  | 7432 - 8462 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1304 - 714 |  | 9038 - 10568 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 5936 - 4876 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 - 6386 |  | 10732 - 12262 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8264 - 9324 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14368 - 12338 |  | 148 - 472 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9736 - 8176 |  | 4014 - 5104 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 - 16062 |  | 6628 - 7248 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 - 13124 |  | 9096 - 10186 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.22.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n20A-n71A-n78A with UL CA_n71A-n78A.

Table 5.22.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2602 - 3137 |  | 3963 - 4498 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2474 - 1904 |  | 5902 - 6937 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4626 - 5196 |  | 7263 - 8298 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1811 - 1206 |  | 9202 - 10737 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 6274 - 5204 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5289 - 5894 |  | 10563 - 12098 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7926 - 8996 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14537 - 12502 |  | 508 - 1148 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10074 - 8504 |  | 4506 - 5611 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13863 - 15898 |  | 5952 - 6592 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11226 - 12796 |  | 8589 - 9694 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.22.2.1.1-1 there is IMD5 from 20 and n71 into RX band n78.

Based on the analysis in Table 5.22.2.1.1-3 there is IMD5 from n71 and n78 into RX band n20.

5.22.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n5-n78-n105.

Table 5.22.2.2-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n20-n71-n78 | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3567 | 4 | TDD | IMD5 |
|  | n20 | N/A | 5 | N/A | 808 | 3.8 | FDD | IMD5 |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n78 | 3528 | 10 | 50 | 3528 | N/A | TDD | N/A |

5.23	CA_n3-n20-n71

5.23.1	Common for 1 band UL and 2 bands UL CA

5.23.1.1	Operating bands for CA

Table 5.23.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n20 | 832-862 | 791-821 | FDD |
| n71 | 663-698 | 617-652 | FDD |

5.23.1.2	Channel bandwidths per operating band for CA

Table 5.23.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n20A-n71A | CA_n3A-n20A CA_n3A-n71A CA_n20A-n71A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n71 | 5, 10, 15, 20 |  |

5.23.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n20-n71, the TIB,c and RIB,c values are re-used from DC_3_n5-n105 as given in the tables below.

Table 5.23.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n20-n71 | 0.3 | 0.3 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.23.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n20-n71 | - | - | 0.3 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.23.2	Specific for 2 bands UL CA

5.23.2.1	UE co-existence studies

5.23.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.23.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n20A-n71A with UL CA_n3A-n20A.

Table 5.23.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 953 - 848 |  | 2542 - 2647 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2558 - 2738 |  | 121 - 14 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4252 - 4432 |  | 3374 - 3509 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4268 - 4523 |  | 711 - 876 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1696 - 1906 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5962 - 6217 |  | 4206 - 4371 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5084 - 5294 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1738 - 1543 |  | 6308 - 5978 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 834 - 1074 |  | 3691 - 3406 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5038 - 5233 |  | 7672 - 8002 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5916 - 6156 |  | 6794 - 7079 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.23.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n20A-n71A with UL CA_n3A-n71A.

Table 5.23.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1122 - 1012 |  | 2373 - 2483 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2722 - 2907 |  | 459 - 314 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4083 - 4268 |  | 3036 - 3181 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4432 - 4692 |  | 204 - 384 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2024 - 2244 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5793 - 6053 |  | 3699 - 3879 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 4746 - 4966 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1082 - 867 |  | 6477 - 6142 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1326 - 1581 |  | 4029 - 3734 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4362 - 4577 |  | 7503 - 7838 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5409 - 5664 |  | 6456 - 6751 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.23.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n20A-n71A with UL CA_n20A-n71A.

Table 5.23.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 199 - 134 |  | 1495 - 1560 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 966 - 1061 |  | 464 - 564 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2327 - 2422 |  | 2158 - 2258 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1798 - 1923 |  | 1127 - 1262 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 268 - 398 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3159 - 3284 |  | 2821 - 2956 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 2990 - 3120 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1960 - 1790 |  | 2785 - 2630 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 430 - 265 |  | 1260 - 1100 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3484 - 3654 |  | 3991 - 4146 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3653 - 3818 |  | 3822 - 3982 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.23.2.1.1-3 there are IMD4 and IMD5 from n20 and n71 into RX band n3.

5.23.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n3-n20-n28 for IMD4 component and DC_2A-5A_n12A for IMD5 component.

Table 5.23.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n20-n71 | n3 | N/A | 5 | N/A | 1843 | 9.4 | FDD | IMD4 |
|  | n20 | 841 | 5 | 25 | 800 | N/A | FDD | N/A |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1841 | 5.9 | FDD | IMD5 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n71 | 672 | 5 | 25 | 626 | N/A | FDD | N/A |

5.24	CA_n3-n20-n77

5.24.1	Common for 1 band UL and 2 bands UL CA

5.24.1.1	Operating bands for CA

Table 5.24.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n20 | 832-862 | 791-821 | FDD |
| n77 | 3300-4200 | 3300-4200 | TDD |

5.24.1.2	Channel bandwidths per operating band for CA

Table 5.24.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n20A-n77A | CA_n3A-n20A CA_n3A-n77A CA_n20A-n77A | n3 | 5,10,15,20,25,30,35,40,45,50 | 0 |
|  |  | n20 | 5,10,15,20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n20A-n77(2A) | CA_n3A-n20A CA_n3A-n77A CA_n20A-n77A | n3 | 5,10,15,20,25,30,35,40,45,50 | 0 |
|  |  | n20 | 5,10,15,20 |  |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |

5.24.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n20-n77, the TIB,c and RIB,c values are re-used from CA_n3-n20-n78 as given in the tables below.

Table 5.24.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n20-n77 | 0.5 | 0.3 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.24.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n20-n77 | 0.2 | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.24.2	Specific for 2 bands UL CA

5.24.2.1	UE co-existence studies

5.24.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.24.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n20A-n77A with UL CA_n3A-n20A.

Table 5.24.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 953 - 848 |  | 2542 - 2647 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2558 - 2738 |  | 121 - 14 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4252 - 4432 |  | 3374 - 3509 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4268 - 4523 |  | 711 - 876 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1696 - 1906 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5962 - 6217 |  | 4206 - 4371 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5084 - 5294 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1738 - 1543 |  | 6308 - 5978 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 834 - 1074 |  | 3691 - 3406 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5038 - 5233 |  | 7672 - 8002 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5916 - 6156 |  | 6794 - 7079 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.24.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n20A-n77A with UL CA_n3A-n77A.

Table 5.24.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515 - 2490 |  | 5010 - 5985 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 780 - 270 |  | 4815 - 6690 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 - 7770 |  | 8310 - 10185 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 930 - 2055 |  | 8115 - 10890 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4980 - 3030 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 - 9555 |  | 11610 - 14385 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10020 - 11970 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15090 - 11415 |  | 3840 - 2640 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9180 - 6330 |  | 1245 - 3270 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 - 18585 |  | 10140 - 11340 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 - 16170 |  | 11730 - 13755 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.24.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n20A-n77A with UL CA_n20A-n77A.

Table 5.24.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2438 - 3368 |  | 4132 - 5062 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2536 - 1576 |  | 5738 - 7568 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4964 - 5924 |  | 7432 - 9262 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1704 - 714 |  | 9038 - 11768 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 6736 - 4876 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 - 6786 |  | 10732 - 13462 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8264 - 10124 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15968 - 12338 |  | 148 - 872 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10936 - 8176 |  | 4014 - 5904 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 - 17662 |  | 6628 - 7648 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 - 14324 |  | 9096 - 10986 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.24.2.1.1-1 there are IMD3 and IMD5 from n3 and n20 into RX band n77.

Based on the analysis in Table 5.24.2.1.1-3 there is IMD3 from n20 and n77 into RX band n3.

5.24.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n3-n18-n77.

Table 5.24.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n20-n77 | n3 | 1747 | 5 | 25 | 1842 | N/A | FDD | N/A |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3441 | 16.3 | TDD | IMD31 |
|  | n3 | N/A | 5 | N/A | 1842 | 15.7 | FDD | IMD3 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n77 | 3536 | 10 | 50 | 3536 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.25	CA_n1-n3-n71

5.25.1	Common for 1 band UL and 2 bands UL CA

5.25.1.1	Operating bands for CA

Table 5.25.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n71 | 663-698 | 617-652 | FDD |

5.25.1.2	Channel bandwidths per operating band for CA

Table 5.25.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n3A-n71A | CA_n1A-n3A CA_n1A-n71A CA_n3A-n71A | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n3 | 5,10,15,20,25,30,35,40,45,50 |  |
|  |  | n71 | 5,10,15,20 |  |
| CA_n1A-n3(2A)-n71A | CA_n1A-n3A CA_n1A-n71A CA_n3A-n71A | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n3 | CA_n3(2A)_BCS 4 and 5 |  |
|  |  | n71 | 5,10,15,20 |  |

5.25.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n3-n71, the TIB,c and RIB,c values are re-used from CA_n1-n3-n105 and are given in the tables below.

Table 5.25.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n3-n71 | 0.3 | 0.3 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.25.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n3-n71 | 0.3 | 0.3 | 0.3 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.25.2	Specific for 2 bands UL CA

5.25.2.1	UE co-existence studies

5.25.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.25.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n3A-n71A with UL CA_n1A-n3A.

Table 5.25.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 - 135 |  | 3630 - 3765 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 - 2250 |  | 1440 - 1650 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 - 5745 |  | 5340 - 5550 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3975 - 4230 |  | 3150 - 3435 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 270 - 540 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7470 - 7725 |  | 7050 - 7335 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7260 - 7530 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 - 4860 |  | 6210 - 5895 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 - 1170 |  | 2520 - 2190 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 - 9120 |  | 9390 - 9705 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 - 9315 |  | 9180 - 9510 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.25.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n3A-n71A with UL CA_n1A-n71A.

Table 5.25.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1317 - 1222 |  | 2583 - 2678 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3142 - 3297 |  | 654 - 524 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4503 - 4658 |  | 3246 - 3376 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5062 - 5277 |  | 9 - 174 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2444 - 2634 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6423 - 6638 |  | 3909 - 4074 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5166 - 5356 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 872 - 672 |  | 7257 - 6982 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1746 - 1971 |  | 4614 - 4364 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4572 - 4772 |  | 8343 - 8618 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5829 - 6054 |  | 7086 - 7336 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.25.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n3A-n71A with UL CA_n3A-n71A.

Table 5.25.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1122 - 1012 |  | 2373 - 2483 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2722 - 2907 |  | 459 - 314 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4083 - 4268 |  | 3036 - 3181 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4432 - 4692 |  | 204 - 384 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2024 - 2244 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5793 - 6053 |  | 3699 - 3879 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 4746 - 4966 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1082 - 867 |  | 6477 - 6142 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1326 - 1581 |  | 4029 - 3734 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4362 - 4577 |  | 7503 - 7838 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5409 - 5664 |  | 6456 - 6751 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.25.2.1.1-2 there is IMD5 from n1 and n71 into RX band n3.

Based on the analysis in Table 5.25.2.1.1-3 there is IMD4 from n3 and n71 into RX band n1.

5.25.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n1-n3-n105.

Table 5.25.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n71 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1855 | 4 | FDD | IMD5 |
|  | n71 | 695 | 5 | 25 | 649 | N/A | FDD | N/A |
|  | n1 | N/A | 5 | N/A | 2160 | 5 | FDD | IMD4 |
|  | n3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | n71 | 695 | 5 | 25 | 649 | N/A | FDD | N/A |

5.26	CA_n1-n3-n8

5.26.1	Common for 1 band UL and 2 bands UL CA

5.26.1.1	Operating bands for CA

Table 5.26.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n8 | 880-915 | 925-960 | FDD |

5.26.1.2	Channel bandwidths per operating band for CA

Table 5.26.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n3A-n8A | CA_n1A-n3A CA_n1A-n8A CA_n3A-n8A | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n3 | 5,10,15,20,25,30,35,40,45,50 |  |
|  |  | n8 | 5,10,15,20 |  |
| CA_n1A-n3(2A)-n8A | CA_n1A-n3A CA_n1A-n8A CA_n3A-n8A | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n3 | CA_n3(2A) BCS 4 & 5 |  |
|  |  | n8 | 5,10,15,20 |  |

5.26.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n3-n8, the TIB,c and RIB,c values have already been specified.

5.26.2	Specific for 2 bands UL CA

5.26.2.1	UE co-existence studies

5.26.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.26.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n3A-n8A with UL CA_n1A-n3A.

Table 5.26.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 - 135 |  | 3630 - 3765 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 - 2250 |  | 1440 - 1650 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 - 5745 |  | 5340 - 5550 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3975 - 4230 |  | 3150 - 3435 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 270 - 540 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7470 - 7725 |  | 7050 - 7335 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7260 - 7530 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 - 4860 |  | 6210 - 5895 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 - 1170 |  | 2520 - 2190 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 - 9120 |  | 9390 - 9705 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 - 9315 |  | 9180 - 9510 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.26.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n3A-n8A with UL CA_n1A-n8A.

Table 5.26.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 - 1005 |  | 2800 - 2895 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 - 3080 |  | 220 - 90 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 - 4875 |  | 3680 - 3810 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 - 5060 |  | 660 - 825 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2010 - 2200 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 - 6855 |  | 4560 - 4725 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5600 - 5790 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 - 1540 |  | 7040 - 6765 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 - 1320 |  | 4180 - 3930 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 - 5640 |  | 8560 - 8835 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 - 6705 |  | 7520 - 7770 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.26.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n3A-n8A with UL CA_n3A-n8A.

Table 5.26.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 905 - 795 |  | 2590 - 2700 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2505 - 2690 |  | 25 - 120 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4300 - 4485 |  | 3470 - 3615 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4215 - 4475 |  | 855 - 1035 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1590 - 1810 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6010 - 6270 |  | 4350 - 4530 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5180 - 5400 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1950 - 1735 |  | 6260 - 5925 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 675 - 930 |  | 3595 - 3300 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5230 - 5445 |  | 7720 - 8055 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6060 - 6315 |  | 6890 - 7185 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

The analysis of the intermodulation products show that the added uplink CA combinations create no intermodulation products inside the third receiver band.

5.26.2.2	REFSENS requirements

No new requirements needed.

5.27	CA_n1-n41-n71

5.27.1	Common for 1 band UL and 2 bands UL CA

5.27.1.1	Operating bands for CA

Table 5.27.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |
| n71 | 663-698 | 617-652 | FDD |

5.27.1.2	Channel bandwidths per operating band for CA

Table 5.27.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n41A-n71A | CA_n1A-n41A CA_n1A-n71A CA_n41A-n71A | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n41 | 5,10,15,20,25,30,35,40,45,50,60,70,80,90,100 |  |
|  |  | n71 | 5,10,15,20 |  |

5.27.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n41-n71, the TIB,c and RIB,c values are re-used from CA_n1-n40-n105 and are given in the tables below.

Table 5.27.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n41-n71 | 0.5 | 0.5 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.27.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n41-n71 | - | - | 0.3 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.27.2	Specific for 2 bands UL CA

5.27.2.1	UE co-existence studies

5.27.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.27.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n41A-n71A with UL CA_n1A-n41A.

Table 5.27.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 516 - 770 |  | 4416 - 4670 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1150 - 1464 |  | 3012 - 3460 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6336 - 6650 |  | 6912 - 7360 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3070 - 3444 |  | 5508 - 6150 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1540 - 1032 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8256 - 8630 |  | 9408 - 10050 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8832 - 9340 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8840 - 8004 |  | 5424 - 4990 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4230 - 3528 |  | 948 - 380 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11904 - 12740 |  | 10176 - 10610 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11328 - 12030 |  | 10752 - 11320 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.27.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n41A-n71A with UL CA_n1A-n71A.

Table 5.27.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1317 - 1222 |  | 2583 - 2678 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3142 - 3297 |  | 654 - 524 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4503 - 4658 |  | 3246 - 3376 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5062 - 5277 |  | 9 - 174 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2444 - 2634 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6423 - 6638 |  | 3909 - 4074 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5166 - 5356 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 872 - 672 |  | 7257 - 6982 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1746 - 1971 |  | 4614 - 4364 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4572 - 4772 |  | 8343 - 8618 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5829 - 6054 |  | 7086 - 7336 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.27.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n41A-n71A with UL CA_n41A-n71A.

Table 5.27.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2027 - 1798 |  | 3159 - 3388 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4294 - 4717 |  | 1364 - 1100 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5655 - 6078 |  | 3822 - 4086 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6790 - 7407 |  | 701 - 402 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3596 - 4054 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8151 - 8768 |  | 4485 - 4784 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6318 - 6776 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 296 - 38 |  | 10097 - 9286 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2898 - 3391 |  | 6744 - 6092 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5148 - 5482 |  | 10647 - 11458 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6981 - 7474 |  | 8814 - 9466 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.27.2.1.1-1 there is IMD2 and IMD5 from n1 and n41 into RX band n71.

Based on the analysis in Table 5.27.2.1.1-2 there is IMD2 and IMD4 from n1 and n71 into RX band n41.

5.27.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n1-n7-n105.

Table 5.27.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n41-n71 | n1 | 1935 | 5 | 25 | 2125 | N/A | FDD | N/A |
|  | n41 | 2565 | 10 | 50 | 2565 | N/A | TDD | N/A |
|  | n71 | N/A | 5 | N/A | 630 | 28.7 | FDD | IMD2 |
|  | n1 | 1925 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n41 | 2565 | 10 | 50 | 2565 | N/A | FDD | N/A |
|  | n71 | N/A | 5 | N/A | 645 | 1 | FDD | IMD5 |
|  | n1 | 1968.5 | 5 | 25 | 2158.5 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2634.5 | 27 | FDD | IMD22 |
|  | n71 | 666 | 5 | 25 | 620 | N/A | FDD | N/A |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.28	CA_n1-n8-n41

5.28.1	Common for 1 band UL and 2 bands UL CA

5.28.1.1	Operating bands for CA

Table 5.28.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n8 | 880-915 | 925-960 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |

5.28.1.2	Channel bandwidths per operating band for CA

Table 5.28.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n8A-n41A | CA_n1A-n8A CA_n1A-n41A CA_n8A-n41A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 100 |  |

5.28.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n8-n41, the TIB,c and RIB,c values are given in the tables below following CA_n1-n8-n40.

Table 5.28.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n8-n41 | 0.3 | 0.3 | 0.5 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.28.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n8-n41 | - | 0.2 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.28.2	Specific for 2 bands UL CA

5.28.2.1	UE co-existence studies

5.28.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.28.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n8A-n41A with UL CA_n1A-n8A.

Table 5.28.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 - 1005 |  | 2800 - 2895 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 - 3080 |  | 220 - 90 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 - 4875 |  | 3680 - 3810 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 - 5060 |  | 660 - 825 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2010 - 2200 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 - 6855 |  | 4560 - 4725 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5600 - 5790 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 - 1540 |  | 7040 - 6765 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 - 1320 |  | 4180 - 3930 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 - 5640 |  | 8560 - 8835 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 - 6705 |  | 7520 - 7770 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.28.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n8A-n41A with UL CA_n1A-n41A.

Table 5.28.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 516 - 770 |  | 4416 - 4670 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1150 - 1464 |  | 3012 - 3460 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6336 - 6650 |  | 6912 - 7360 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3070 - 3444 |  | 5508 - 6150 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1540 - 1032 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8256 - 8630 |  | 9408 - 10050 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8832 - 9340 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8840 - 8004 |  | 5424 - 4990 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4230 - 3528 |  | 948 - 380 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11904 - 12740 |  | 10176 - 10610 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11328 - 12030 |  | 10752 - 11320 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.28.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n8A-n41A with UL CA_n8A-n41A.

Table 5.28.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1581 - 1810 |  | 3376 - 3605 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 930 - 666 |  | 4077 - 4500 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4256 - 4520 |  | 5872 - 6295 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 50 - 249 |  | 6573 - 7190 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3620 - 3162 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5136 - 5435 |  | 8368 - 8985 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6752 - 7210 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9880 - 9069 |  | 1164 - 830 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6310 - 5658 |  | 2247 - 2740 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10864 - 11675 |  | 6016 - 6350 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9248 - 9900 |  | 7632 - 8125 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

From the tables it is found that:

-  Band n8 may be subject to IMD5 from n1+n41.

5.28.2.2	REFSENS requirements

The MSD value has been taken from CA_n1A-n7A-n8A.

Table 5.28.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n8-n41 | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | n8 | N/A | 5 | N/A | 927.5 | 1.0 | FDD | IMD5 |
|  | n41 | 2502.5 | 10 | 50 | 2502.5 | N/A | TDD | N/A |

5.29	CA_n3-n41-n71

5.29.1	Common for 1 band UL and 2 bands UL CA

5.29.1.1	Operating bands for CA

Table 5.29.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |
| n71 | 663-698 | 617-652 | FDD |

5.29.1.2	Channel bandwidths per operating band for CA

Table 5.29.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n41A-n71A | CA_n3A-n41A CA_n3A-n71A CA_n41A-n71A | n3 | 5,10,15,20,25,30,35,40,45,50 | 0 |
|  |  | n41 | 5,10,15,20,25,30,35,40,45,50,60,70,80,90,100 |  |
|  |  | n71 | 5,10,15,20 |  |

5.29.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n41-n71, the TIB,c and RIB,c values are re-used from CA_n3-n7-n105 and aregiven in the tables below.

Table 5.29.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n41-n71 | 0.5 | 0.5 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.29.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n41-n71 | - | - | 0.3 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.29.2	Specific for 2 bands UL CA

5.29.2.1	UE co-existence studies

5.29.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.29.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n41A-n71A with UL CA_n3A-n41A.

Table 5.29.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 711 - 980 |  | 4206 - 4475 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 - 1074 |  | 3207 - 3670 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 - 6260 |  | 6702 - 7165 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2440 - 2859 |  | 5703 - 6360 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1960 - 1422 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7626 - 8045 |  | 9198 - 9855 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8412 - 8950 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9050 - 8199 |  | 4644 - 4150 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4650 - 3918 |  | 363 - 250 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 - 12545 |  | 9336 - 9830 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 - 11640 |  | 10122 - 10735 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.29.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n41A-n71A with UL CA_n3A-n71A.

Table 5.29.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1122 - 1012 |  | 2373 - 2483 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2722 - 2907 |  | 459 - 314 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4083 - 4268 |  | 3036 - 3181 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4432 - 4692 |  | 204 - 384 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2024 - 2244 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5793 - 6053 |  | 3699 - 3879 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 4746 - 4966 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1082 - 867 |  | 6477 - 6142 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1326 - 1581 |  | 4029 - 3734 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4362 - 4577 |  | 7503 - 7838 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5409 - 5664 |  | 6456 - 6751 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.29.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n41A-n71A with UL CA_n41A-n71A.

Table 5.29.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2027 - 1798 |  | 3159 - 3388 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4294 - 4717 |  | 1364 - 1100 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5655 - 6078 |  | 3822 - 4086 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6790 - 7407 |  | 701 - 402 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3596 - 4054 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8151 - 8768 |  | 4485 - 4784 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6318 - 6776 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 296 - 38 |  | 10097 - 9286 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2898 - 3391 |  | 6744 - 6092 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5148 - 5482 |  | 10647 - 11458 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6981 - 7474 |  | 8814 - 9466 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.29.2.1.1-1 there is IMD2 from n41 and n71 into RX band n3.

5.29.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n3-n7-n105.

Table 5.29.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n41-n71 | n3 | N/A | 5 | N/A | 1875 | 16.5 | FDD | IMD2 |
|  | n41 | 2550 | 10 | 50 | 2550 | N/A | FDD | N/A |
|  | n71 | 675 | 5 | 25 | 629 | N/A | FDD | N/A |

5.30	CA_n1-n7-n20

5.30.1	Common for 1 band UL and 2 bands UL CA

5.30.1.1	Operating bands for CA

Table 5.30.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz –2170 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |

5.30.1.2	Channel bandwidths per operating band for CA

Table 5.30.1.2-1: Supported bandwidths per CA band combination of band n25+n41+n77

| NR CA configuration |  | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n1A-n7A-n20A | CA_n1A-n7A CA_n1A-n20A CA_n7A-n20A |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |

5.30.1.3	∆TIB,c and ∆RIB,c values

Requirements for CA_n1-n7-n20, for the TIB,c and RIB,c values are given in the tables below following CA_n1-n7, CA_n1-n20 and Ca_n7-n20  values.

Table 5.30.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n7-n20 | 0.5 | 0.6 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.30.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n7-n20 | - | 0.5 | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.30.2	Specific for 2 bands UL CA

5.30.2.1	UE co-existence studies

5.30.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.30.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n7A-n20A with UL CA_n1A-n7A.

Table 5.30.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 650 – 520 |  | 4420 - 4550 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270- 1460 |  | 3020 - 3220 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 - 6530 |  | 6920 - 7120 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3190- 3440 |  | 5508 - 6150 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1300 - 1040 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8260 - 8510 |  | 9420 - 9100 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5504 - 5684 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7088- 6818 |  | 1528 - 1348 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 - 3540 |  | 940 - 620 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4276 - 4036 |  | 1254 - 1464 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7424 - 7664 |  | 6336 - 6546 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.30.2.1.1-1, 5th order IMD from Band n1 and Band n7 may fall into Rx frequencies of band n20.

Table 5.30.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n7A-n20A with UL CA_n1A-n20A.

Table 5.30.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1148 - 1058 |  | 2752 - 2842 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 316 - 196 |  | 2978 - 3128 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3584 - 3704 |  | 4672 - 4822 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 516 - 666 |  | 4898 - 5108 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2296 - 2116 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4416 - 4566 |  | 6592 - 6802 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11440 - 10560 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13280 - 11220 |  | 4620 - 3880 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4230 - 3528 |  | 948 - 380 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7560 - 5940 |  | 660 - 1840 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 - 15360 |  | 12360 - 13540 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.30.2.1.1-2, there is no IMD impact from UL CA_n1A-n20A into Rx frequencies of band n7.

Table 5.30.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n7A-n20A with UL CA_n7A-n20A.

Table 5.30.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1738 - 1638 |  | 3332 - 3432 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 906 - 776 |  | 4138 - 4308 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4164 - 4294 |  | 5832 - 6002 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 74 - 86 |  | 6638 - 6878 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3476 - 3276 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4996 - 5156 |  | 8332 - 8572 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6664 - 6864 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9448 - 9138 |  | 948 - 758 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6046 - 5776 |  | 2414 - 2644 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10832 - 11142 |  | 5828 - 6018 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9164 - 9434 |  | 7496 - 7726 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.30.2.1.1-3, there is no IMD impact from UL CA_n7A-n20A into Rx frequencies of band n1.

From the tables above it is found that:

-  Band n20 may be subject to IMD5.

5.30.2.2	REFSENS requirements

MSD values have been taken from CA_n1A-n7A-n28A.

Table 5.30.2.2-1: MSD due to IMD issue

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n7-n20 | n1 | 1940 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 800 | 4.5 | FDD | IMD5 |

5.31	CA_n1-n3-n75

5.31.1	Common for 1 band UL and 2 bands UL CA

5.31.1.1	Operating bands for CA

Table 5.31.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL19 |

5.31.1.2	Channel bandwidths per operating band for CA

Table 5.31.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n3A-n75A | CA_n1A-n3A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |

5.31.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n3-n75, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.31.2	Specific for 2 bands UL CA

5.31.2.1	UE co-existence studies

5.31.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.31.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n3A-n75A with UL CA_n1A-n3A.

Table 5.31.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135–270 |  | 3630–3765 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055–2250 |  | 1440–1650 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550–5745 |  | 5340–5550 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3975–4230 |  | 3150–3435 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 270–540 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7470–7725 |  | 7050–7335 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7260–7530 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 4860–5220 |  | 5895–6210 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1170–1515 |  | 2190–2520 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760–9120 |  | 9390–9705 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970–9315 |  | 9180–9510 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 3rd and 5th order IMD generated by UL CA_n1A-n3A may fall into own Rx of Band n75.

5.31.2.2	REFSENS requirements

We analyze the IMD using the following component linearity assumptions:

Table 5.31.2.2-1 General linearity parameters

| Component | IP3 (dBm) |
| --- | --- |
| Antenna switch | 68 |
| Diplexer | 86 |
| Triplexer | 82 |
| PA forward mixing | 30 |
| PA reverse mixing | 28 |
| LNA | -6 |

Based on these parameters we get the following MSD.

Table 5.31.2.2-2: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n75 | n1 | 1960 | 5 | 25 | 2150 | N/A | FDD | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1480 | 24.2 | SDL | IMD34 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.32	CA_n1-n7-n78

5.32.1	Common for 1 band UL and 2 bands UL CA

5.32.1.1	Operating bands for CA

Table 5.32.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |

5.32.1.2	Channel bandwidths per operating band for CA

Table 5.32.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n7A-n78A | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |

5.32.1.3	∆TIB,c and ∆RIB,c values

For CA_n1A-n7A-n78A, the TIB,c and RIB,c values have already been specified in TS 38.101-1.

5.32.2	Specific for 2 bands UL CA

5.32.2.1	UE co-existence studies

5.32.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.32.2.1.1-1, Table 5.32.2.1.1-2 and Table 5.32.2.1.1-3 provide the two UL bands with one CC per band IMD interference analysis for CA_n1A-n7A-n78A with UL CA_n1A-n7A, UL CA_n1A-n78A and UL CA_n7A-n78A.

Table 5.32.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520–650 |  | 4420–4550 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270–1460 |  | 3020–3220 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340–5745 |  | 6920–7120 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3190–3440 |  | 5520–5790 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1040–1300 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8260–8510 |  | 9420–9690 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8840–9100 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8020–8360 |  | 5110–5420 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3540–3870 |  | 620–940 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920–12260 |  | 10180–10490 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340–11670 |  | 10760–11080 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 4th and 5th order IMD generated by UL CA_n1A-n7A may fall into own Rx of Band n78

Table 5.32.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320–1880 |  | 5520–5780 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 40–660 |  | 4620–5680 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140–7760 |  | 8520–9580 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960–2640 |  | 7920–79480 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2640–3760 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060–9740 |  | 11820–13380 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10440–11560 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11220–13280 |  | 3880–4620 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5940–7560 |  | 660–1840 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120–17180 |  | 10980–11720 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740–15360 |  | 12360–13540 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 4th order IMD generated by UL CA_n1A-n78A may fall into own Rx of Band n7.

Table 5.32.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730–1300 |  | 5800–6370 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1200–1840 |  | 4030–5100 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300–8940 |  | 9100–10170 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3700–4410 |  | 7330–8900 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1460–2600 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800–11510 |  | 12400–13970 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11600–12740 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10630–12700 |  | 6200–6980 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4760–6400 |  | 100–1110 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700–17770 |  | 13300–14080 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900–16540 |  | 14100–15310 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 4th order IMD generated by UL CA_n7A-n78A may fall into own Rx of Band n1.

5.32.2.2	REFSENS requirements

For CA_n1-n7-n78, the MSD values have already been specified in TS 38.101-1. However，the IMD5 from CA_n1A-n7A to Rx n78 is missing. Here IMD5 to n78 is added.

Table 5.32.2.2-1: MSD for the CA configuration

| CA_n1-n7-n78 | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n7 | N/A | 5 | N/A | 2627.5 | 9.1 | FDD | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 8.7 | FDD | IMD4 |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3390 | 10.1 | TDD | IMD4 |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3770 | 1.3 | TDD | IMD5 |

5.33	CA_n7-n25-n29

5.33.1	Common for 1 band UL and 2 bands UL CA

5.33.1.1	Operating bands for CA

Table 5.33.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n25 | 1850 MHz – 1915 MHz | 1930 MHz – 1995 MHz | FDD |
| n29 | N/A | 717 MHz – 728 MHz | SDL |

5.33.1.2	Channel bandwidths per operating band for CA

Table 5.33.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n7A-n25A-n29A | CA_n7A-n25A | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n29 | n29 channel bandwidths in Table 5.3.5-1 |  |

5.33.1.3	∆TIB,c and ∆RIB,c values

For CA_n7-n25-n29, the TIB,c and RIB,c values are given in the tables below.

Table 5.33.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n7-n25-n29 | 0.5 | 0.5 | N/A |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.33.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n7-n25-n29 | - | - | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.33.2	Specific for 2 bands UL CA

5.33.2.1	UE co-existence studies

5.33.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.33.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n7A-n25A-n29A with UL CA_n7A-n25A.

Table 5.33.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 585–720 |  | 4350–4485 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3085–3290 |  | 1130–1330 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6850–7055 |  | 6200–6400 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5585–5860 |  | 2980–3245 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1170–1440 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9350–9625 |  | 8050–8315 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8700–8970 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 4830–5160 |  | 8085–8430 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 410–745 |  | 3670–4010 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9900–10230 |  | 11850–12195 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10550–10885 |  | 11200–11540 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, 2nd and 5th order IMD generated by UL CA_n7A-n25A may fall into own Rx of Band n29.

5.33.2.2	REFSENS requirements

Based on the co-existence studies, there is a need to define MSD values. MSD values are specified as following.

Table 5.33.2.2-3: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n25-n29 | n7 | 2567.5 | 5 | 25 | 2687.5 | N/A | FDD | N/A |
|  | n25 | 1852.5 | 5 | 25 | 1932.5 | N/A | FDD | N/A |
|  | n29 | N/A | 5 | N/A | 719.5 | 26 | SDL | IMD2 |
|  | n7 | 2503 | 5 | 25 | 2623 | N/A | FDD | N/A |
|  | n25 | 1910 | 5 | 25 | 1990 | N/A | FDD | N/A |
|  | n29 | N/A | 5 | N/A | 724 | 4.5 | SDL | IMD5 |

5.34	CA_n7-n29-n66

5.34.1	Common for 1 band UL and 2 bands UL CA

5.34.1.1	Operating bands for CA

Table 5.34.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n29 | N/A | 717 MHz – 728 MHz | SDL |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |

5.34.1.2	Channel bandwidths per operating band for CA

Table 5.34.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n7A-n29A-n66A | CA_n7A-n66A | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n29 | n29 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |

5.34.1.3	∆TIB,c and ∆RIB,c values

For CA_n7-n29-n66, the TIB,c and RIB,c values are given in the tables below.

Table 5.34.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n7-n29-n66 | 0.5 | N/A | 0.5 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.34.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n7-n29-n66 | 0.5 | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.34.2	Specific for 2 bands UL CA

5.34.2.1	UE co-existence studies

5.34.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.34.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n7A-n29A-n66A with UL CA_n7A-n66A.

Table 5.34.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 720–860 |  | 4210–4350 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3220–3430 |  | 850–1060 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6710–6920 |  | 5920–6130 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5720–6000 |  | 2560–2840 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1440–1720 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9210–9490 |  | 7630–7910 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8420–8700 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 4270–4620 |  | 8220–8570 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10–340 |  | 3940–4290 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9340–9690 |  | 11710–12060 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10130–10480 |  | 10920–11270 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, 2nd order IMD generated by UL CA_n7A-n66A may fall into own Rx of Band n29.

5.34.2.2	REFSENS requirements

Based on the co-existence studies, there is a need to define MSD values. MSD values from DC_12-66_n7 are reused as below.

Table 5.34.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n29-n66 | n7 | 2502.5 | 5 | 25 | 2622.5 | N/A | FDD | N/A |
|  | n29 | N/A | 5 | N/A | 725 | 31 | SDL | IMD2 |
|  | n66 | 1777.5 | 5 | 25 | 2197.5 | N/A | FDD | N/A |

5.35	CA_n7-n29-n77

5.35.1	Common for 1 band UL and 2 bands UL CA

5.35.1.1	Operating bands for CA

Table 5.35.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n29 | N/A | 717 MHz – 728 MHz | SDL |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

5.35.1.2	Channel bandwidths per operating band for CA

Table 5.35.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n7A-n29A-n77A | CA_n7A-n77A | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n29 | n29 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7A-n29A-n77(2A) | CA_n7A-n77A CA_n77(2A) | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n29 | n29 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n7A-n29A-n77(3A) | CA_n7A-n77A CA_n77(2A) | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n29 | n29 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(3A)_BCS4 and 5 |  |

5.35.1.3	∆TIB,c and ∆RIB,c values

For CA_n7-n29-n77 the TIB,c and RIB,c values are given in the tables below, reusing from CA_n7-n12-n77.

Table 5.35.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n7-n29-n77 | 0.5 | N/A | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.35.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n7-n29-n77 | 0.2 | 0.5 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.35.2	Specific for 2 bands UL CA

5.35.2.1	UE co-existence studies

5.35.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.35.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n7A-n29A-n77A with UL CA_n7A-n77A.

Table 5.35.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730–1700 |  | 5800–6770 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 800–1840 |  | 4030–5900 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300–9340 |  | 9100–10970 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3300–4410 |  | 7330–10100 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1460-3400 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800–11910 |  | 12400–15170 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11600–13540 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10630–14300 |  | 5800–6980 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4760–7600 |  | 690–1110 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700–19370 |  | 13300–14480 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900–17740 |  | 14100–16110 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, IMD5 generated by UL CA_n7A-n77A fall into own Rx of Band n29.

In addition, there is no additional MSD due to UL CA_n77(2A) needs to be specified for three bands DL configuration.

5.35.2.2	REFSENS requirements

Based on the co-existence studies, MSD for IMD5 is introduced as following, the values are reused from DC_7A-29A_n78A.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n29-n77 | n7 | 2540 | 5 | 25 | 2660 | N/A | FDD | N/A |
|  | n29 | N/A | 5 | N/A | 720 | 3.0 | SDL | IMD5 |
|  | n77 | 3450 | 10 | 50 | 3450 | N/A | TDD | N/A |

  - 5.36		CA_n3-n40-n79

    - 5.36.1	Common for 1 band UL and 2 bands UL CA

      - 5.36.1.1		Operating bands for CA

Table 5.36.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880MHz | FDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

      - 5.36.1.2	Channel bandwidths per operating band for CA

Table 5.36.1.2-1: Supported bandwidths per CA band combination of band nX+nY+nZ

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n40A-n79A | CA_n3A-n40A CA_n3A-n79A CA_n40A-n79A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |

      - 5.36.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n40-n79, the TIB,c and RIB,c values are proposed in the following tables:

Table 5.36.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n3-n40-n79 | 0.5 | 0.5 | 0.8 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.36.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n3-n40-n79 | - | - | 0.5 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n3 the band order from left to right is n1, n3 and n3. |  |  |  |

    - 5.36.2	Specific for 2 bands UL CA

      - 5.36.2.1	UE co-existence studies

The co-existence for the fallback 2DL/2UL of CA_n3A-n40A, CA_n3A-n79A and CA_n40A-n79A have already been analyzed. In terms of the co-existence studies of corresponding fallbacks, it can be observed:

IMD5 issue caused by n3+n40, i.e.  fn40-4*fn3, fall into the its own band n79 Rx;

IMD5 issue caused by n3+n79, i.e.  fn79-4*fn3, fall into the its own band n40 Rx;

IMD5 issue caused by n40+n79, i.e.  2*fn79-3*fn40,  fall into the its own band n3 Rx;

      - 5.36.2.2	REFSENS requirements

Based on co-existence studies additional MSD is needed to be defined. The MSD values are proposed in table 5.36.2.2-1.

Table 5.36.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3-n40-n79 | n3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
|  | n79 | N/A | 40 | N/A | 4820 | 3.3 | IMD5 |
|  | n3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |
|  | n40 | N/A | 5 | N/A | 2310 | 5.9 | IMD5 |
|  | n79 | 4820 | 40 | 216 | 4820 | N/A | N/A |
|  | n3 | N/A | 5 | N/A | 1850 | 4.7 | IMD5 |
|  | n40 | 2350 | 5 | 25 | 2350 | N/A | N/A |
|  | n79 | 4450 | 40 | 216 | 4450 | N/A | N/A |

5.37	CA_n3-n7-n77

5.37.1	Common for 1 band UL and 2 bands UL CA

5.37.1.1	Operating bands for CA

Table 5.37.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n7 | 2500-2570 | 2620-2690 | FDD |
| n77 | 3300-4200 | 3300-4200 | TDD |

5.37.1.2	Channel bandwidths per operating band for CA

Table 5.37.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n7A-n77A | CA_n3A-n7A CA_n3A-n77A CA_n7A-n77A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n77(2A) | CA_n77(2A) CA_n3A-n7A CA_n3A-n77A CA_n7A-n77A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |

5.37.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n7-n77, the TIB,c and RIB,c values are given in the tables below as taken from CA_n3-n7-n78.

Table 5.37.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n7-n77 | 0.6 | 0.6 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5 |  |  |  |

Table 5.37.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n7-n77 | 0.2 | 0.2 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.37.2	Specific for 2 bands UL CA

5.37.2.1	UE co-existence studies

5.37.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.37.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n7A-n77A with UL CA_n3A-n7A.

Table 5.37.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 - 860 |  | 4210 - 4355 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 - 1070 |  | 3215 - 3430 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 - 6140 |  | 6710 - 6925 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2560 - 2855 |  | 5715 - 6000 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1720 - 1430 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7630 - 7925 |  | 9210 - 9495 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8420 - 8710 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 - 8215 |  | 4640 - 4270 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 - 3930 |  | 355 - 10 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 - 12065 |  | 9340 - 9710 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 - 11280 |  | 10130 - 10495 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.37.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n7A-n77A with UL CA_n3A-n77A.

Table 5.37.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515 - 2490 |  | 5010 - 5985 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 780 - 270 |  | 4815 - 6690 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 - 7770 |  | 8310 - 10185 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 930 - 2055 |  | 8115 - 10890 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4980 - 3030 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 - 9555 |  | 11610 - 14385 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10020 - 11970 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15090 - 11415 |  | 3840 - 2640 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9180 - 6330 |  | 1245 - 3270 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 - 18585 |  | 10140 - 11340 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 - 16170 |  | 11730 - 13755 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.37.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n7A-n77A with UL CA_n7A-n77A.

Table 5.37.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730 - 1700 |  | 5800 - 6770 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 800 - 1840 |  | 4030 - 5900 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300 - 9340 |  | 9100 - 10970 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3300 - 4410 |  | 7330 - 10100 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3400 - 1460 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800 - 11910 |  | 12400 - 15170 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11600 - 13540 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14300 - 10630 |  | 6980 - 5800 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7600 - 4760 |  | 1110 - 900 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 - 19370 |  | 13300 - 14480 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 - 17740 |  | 14100 - 16110 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.37.2.1.1-1 there is IMD3 and IMD5 from n3 and n7 into RX band n77.

Based on the analysis in Table 5.37.2.1.1-2 there is IMD5 from n3 and n77 into RX band n7.

Based on the analysis in Table 5.37.2.1.1-3 there is IMD3 and IMD4 from n7 and n77 into RX band n3.

5.37.2.2	REFSENS requirements

The MSD requirements are based on CA_n3-n7-n78 and DC_3-7_n77.

Table 5.37.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n7-n77 | n3 | N/A | 5 | N/A | 1820 | 17.6 | FDD | IMD3 |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1820 | 8.6 | FDD | IMD4 |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n77 | 3475 | 10 | 50 | 3475 | N/A | TDD | N/A |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2685 | 3.4 | FDD | IMD5 |
|  | n77 | 4175 | 10 | 50 | 4175 | N/A | TDD | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3390 | 16.1 | TDD | IMD31 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.38	CA_n1-n20-n41

5.38.1	Common for 1 band UL and 2 bands UL CA

5.38.1.1	Operating bands for CA

Table 5.38.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n20 | 832-862 | 791-821 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |

5.38.1.2	Channel bandwidths per operating band for CA

Table 5.38.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n20A-n41A | CA_n1A-n20A CA_n1A-n41A CA_n20A-n41A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |

5.38.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n20-n41, the TIB,c values are reused from CA_n1-n18-n41 which are given in the tables below.

Table 5.38.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n20-n41 | 0.5 | 0.3 | 0.5 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

RIB,c values are reused from CA_n1-n28-n41 which are given in the tables below.

Table 5.38.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n20-n41 | - | 0.2 | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.38.2	Specific for 2 bands UL CA

5.38.2.1	UE co-existence studies

5.38.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.38.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n41A with UL CA_n1A-n20A.

Table 5.38.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1148 - 1058 |  | 2752 - 2842 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2978 - 3128 |  | 316 - 196 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4672 - 4822 |  | 3584 - 3704 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4898 - 5108 |  | 516 - 666 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2116 - 2296 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6592 - 6802 |  | 4416 - 4566 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5504 - 5684 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1528 - 1348 |  | 7088 - 6818 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1254 - 1464 |  | 4276 - 4036 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5248 - 5428 |  | 8512 - 8782 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6336 - 6546 |  | 7424 - 7664 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.38.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n41A with UL CA_n1A-n41A.

Table 5.38.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 516 - 770 |  | 4416 - 4670 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1150 - 1464 |  | 3012 - 3460 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6336 - 6650 |  | 6912 - 7360 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3070 - 3444 |  | 5508 - 6150 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1540 - 1032 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8256 - 8630 |  | 9408 - 10050 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8832 - 9340 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8840 - 8004 |  | 5424 - 4990 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4230 - 3528 |  | 948 - 380 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11904 - 12740 |  | 10176 - 10610 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11328 - 12030 |  | 10752 - 11320 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.38.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n41A with UL CA_n20A-n41A.

Table 5.38.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1634 - 1858 |  | 3328 - 3552 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1026 - 772 |  | 4130 - 4548 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4160 - 4414 |  | 5824 - 6242 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 194 - 90 |  | 6626 - 7238 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3716 - 3268 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4992 - 5276 |  | 8320 - 8932 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6656 - 7104 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9928 - 9122 |  | 952 - 638 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6406 - 5764 |  | 2406 - 2884 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10816 - 11622 |  | 5824 - 6138 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9152 - 9794 |  | 7488 - 7966 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.38.2.1.1-2 there is IMD5 from n1 and n41 into RX band n20.

5.38.2.2	REFSENS requirements

The MSD value has been taken from CA_n1-n18-n41.

Table 5.38.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n20-n41 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 810 | 3.3 | FDD | IMD5 |
|  | n41 | 2550 | 10 | 50 | 2550 | N/A | TDD | N/A |

5.39	CA_n1-n20-n77

5.39.1	Common for 1 band UL and 2 bands UL CA

5.39.1.1	Operating bands for CA

Table 5.39.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n20 | 832-862 | 791-821 | FDD |
| n77 | 3300-4200 | 3300-4200 | TDD |

5.39.1.2	Channel bandwidths per operating band for CA

Table 5.39.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n20A-n77A | CA_n1A-n20A CA_n1A-n77A CA_n20A-n77A | n1 | 5,10,15,20,25,30,40,45,50 | 4 and 5 |
|  |  | n20 | 5,10,15,20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n20A-n77(2A) | CA_n1A-n20A CA_n1A-n77A CA_n20A-n77A | n1 | 5,10,15,20,25,30,40,45,50 | 4 and 5 |
|  |  | n20 | 5,10,15,20 |  |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |

5.39.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n20-n77, the TIB,c and RIB,c values are re-used from CA_n1-n20-n78 as given in the tables below.

Table 5.39.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n20-n77 | 0.3 | 0.6 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.39.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n20-n77 | - | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.39.2	Specific for 2 bands UL CA

5.39.2.1	UE co-existence studies

5.39.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.39.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n77A with UL CA_n1A-n20A.

Table 5.39.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1148 - 1058 |  | 2752 - 2842 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2978 - 3128 |  | 316 - 196 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4672 - 4822 |  | 3584 - 3704 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4898 - 5108 |  | 516 - 666 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2116 - 2296 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6592 - 6802 |  | 4416 - 4566 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5504 - 5684 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1528 - 1348 |  | 7088 - 6818 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1254 - 1464 |  | 4276 - 4036 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5248 - 5428 |  | 8512 - 8782 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6336 - 6546 |  | 7424 - 7664 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.39.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n77A with UL CA_n1A-n77A.

Table 5.39.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320 - 2280 |  | 5220 - 6180 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 360 - 660 |  | 4620 - 6480 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 - 8160 |  | 8520 - 10380 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1560 - 2640 |  | 7920 - 10680 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4560 - 2640 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 - 10140 |  | 11820 - 14580 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10440 - 12360 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14880 - 11220 |  | 4620 - 3480 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8760 - 5940 |  | 660 - 2640 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 - 18780 |  | 10980 - 12120 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 - 16560 |  | 12360 - 14340 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.39.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n20A-n77A with UL CA_n20A-n77A.

Table 5.39.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2438 - 3368 |  | 4132 - 5062 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2536 - 1576 |  | 5738 - 7568 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4964 - 5924 |  | 7432 - 9262 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1704 - 714 |  | 9038 - 11768 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 6736 - 4876 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 - 6786 |  | 10732 - 13462 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8264 - 10124 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15968 - 12338 |  | 148 - 872 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10936 - 8176 |  | 4014 - 5904 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 - 17662 |  | 6628 - 7648 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 - 14324 |  | 9096 - 10986 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.39.2.1.1-1 there are IMD3 and IMD5 from n1 and n20 into RX band n77.

Based on the analysis in Table 5.39.2.1.1-2 there is IMD5 from n1 and n77 into RX band n20.

Based on the analysis in Table 5.39.2.1.1-3 there is IMD3 from n20 and n77 into RX band n1.

5.39.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n1-n18-n77.

Table 5.39.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n20-n77 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3644 | 15.7 | TDD | IMD31 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 3.5 | FDD | IMD5 |
|  | n77 | 3328 | 10 | 50 | 3328 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 16.4 | FDD | IMD3 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n77 | 3834 | 10 | 50 | 3834 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.40	CA_n1-n71-n77

5.40.1	Common for 1 band UL and 2 bands UL CA

5.40.1.1	Operating bands for CA

Table 5.40.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n71 | 663-698 | 617-652 | FDD |
| n77 | 3300-4200 | 3300-4200 | TDD |

5.40.1.2	Channel bandwidths per operating band for CA

Table 5.40.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n71A-n77A | CA_n1A-n71A CA_n1A-n77A CA_n71A-n77A | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n77 | 10,15,20,25,30,40,50,60,70,80,90,100 |  |
| CA_n1A-n71A-n77(2A) | CA_n1A-n71A CA_n1A-n77A CA_n71A-n77A | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |

5.40.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n71-n77, the TIB,c and RIB,c values are re-used from CA_n1-n78-n105 and are given in the tables below.

Table 5.40.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n71-n77 | 0.3 | 0.6 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.40.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n71-n77 | - | 0.2 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.40.2	Specific for 2 bands UL CA

5.40.2.1	UE co-existence studies

5.40.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.40.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n71A-n77A with UL CA_n1A-n71A.

Table 5.40.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1317 - 1222 |  | 2583 - 2678 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3142 - 3297 |  | 654 - 524 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4503 - 4658 |  | 3246 - 3376 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5062 - 5277 |  | 9 - 174 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2444 - 2634 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6423 - 6638 |  | 3909 - 4074 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5166 - 5356 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 872 - 672 |  | 7257 - 6982 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1746 - 1971 |  | 4614 - 4364 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4572 - 4772 |  | 8343 - 8618 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5829 - 6054 |  | 7086 - 7336 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.40.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n71A-n77A with UL CA_n1A-n77A.

Table 5.40.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320 - 2280 |  | 5220 - 6180 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 360 - 660 |  | 4620 - 6480 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 - 8160 |  | 8520 - 10380 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1560 - 2640 |  | 7920 - 10680 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4560 - 2640 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 - 10140 |  | 11820 - 14580 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10440 - 12360 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14880 - 11220 |  | 4620 - 3480 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8760 - 5940 |  | 660 - 2640 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 - 18780 |  | 10980 - 12120 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 - 16560 |  | 12360 - 14340 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.40.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n71A-n77A with UL CA_n71A-n77A.

Table 5.40.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2602 - 3537 |  | 3963 - 4898 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2874 - 1904 |  | 5902 - 7737 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4626 - 5596 |  | 7263 - 9098 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2211 - 1206 |  | 9202 - 11937 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 7074 - 5204 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5289 - 6294 |  | 10563 - 13298 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7926 - 9796 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16137 - 12502 |  | 508 - 1548 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11274 - 8504 |  | 4506 - 6411 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13863 - 17498 |  | 5952 - 6992 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11226 - 13996 |  | 8589 - 10494 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.40.2.1.1-1 there is IMD3 and IMD4 from n1 and n71 into RX band n77.

Based on the analysis in Table 5.40.2.1.1-2 there is IMD3 from n1 and n77 into RX band n71.

Based on the analysis in Table 5.40.2.1.1-3 there is IMD3 and IMD4 from n71 and n77 into RX band n1.

5.40.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n1-n78-n105.

Table 5.40.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n71-n77 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n71 | N/A | 5 | N/A | 635 | 15.2 | FDD | IMD3 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n71 | 686 | 5 | 25 | 640 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3342 | 15.7 | TDD | IMD3 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3992 | 9.4 | TDD | IMD4 |
|  | n1 | N/A | 5 | N/A | 2160 | 15.7 | FDD | IMD3 |
|  | n71 | 686 | 5 | 25 | 640 | N/A | FDD | N/A |
|  | n77 | 3532 | 10 | 50 | 3532 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2141 | 10.1 | FDD | IMD4 |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n77 | 4181 | 10 | 50 | 4181 | N/A | TDD | N/A |

5.41	CA_n3-n20-n41

5.41.1	Common for 1 band UL and 2 bands UL CA

5.41.1.1	Operating bands for CA

Table 5.41.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n20 | 832-862 | 791-821 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |

5.41.1.2	Channel bandwidths per operating band for CA

Table 5.41.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n20A-n41A | CA_n3A-n20A CA_n3A-n41A CA_n20A-n41A | n3 | 5, 10, 15, 20, 25, 30, 45, 40, 45, 50 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |

5.41.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n20-n41, the TIB,c and RIB,c values are re-used from CA_n3-n18-n41 as given in the tables below.

Table 5.41.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n20-n41 | 0.5 | 0.3 | 0.31 / 0.82 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. |  |  |  |

Table 5.41.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n20-n41 | - | - | 01 / 0.52 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |  |

5.41.2	Specific for 2 bands UL CA

5.41.2.1	UE co-existence studies

5.41.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.41.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n20A-n41A with UL CA_n3A-n20A.

Table 5.41.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 953 - 848 |  | 2542 - 2647 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2558 - 2738 |  | 121 - 14 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4252 - 4432 |  | 3374 - 3509 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4268 - 4523 |  | 711 - 876 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1696 - 1906 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5962 - 6217 |  | 4206 - 4371 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5084 - 5294 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1738 - 1543 |  | 6308 - 5978 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 834 - 1074 |  | 3691 - 3406 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5038 - 5233 |  | 7672 - 8002 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5916 - 6156 |  | 6794 - 7079 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.41.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n20A-n41A with UL CA_n3A-n41A.

Table 5.41.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 711 - 980 |  | 4206 - 4475 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 - 1074 |  | 3207 - 3670 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 - 6260 |  | 6702 - 7165 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2440 - 2859 |  | 5703 - 6360 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1960 - 1422 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7626 - 8045 |  | 9198 - 9855 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8412 - 8950 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9050 - 8199 |  | 4644 - 4150 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4650 - 3918 |  | 363 - 250 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 - 12545 |  | 9336 - 9830 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 - 11640 |  | 10122 - 10735 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.41.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n20A-n41A with UL CA_n20A-n41A.

Table 5.41.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1634 - 1858 |  | 3328 - 3552 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1026 - 772 |  | 4130 - 4548 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4160 - 4414 |  | 5824 - 6242 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 194 - 90 |  | 6626 - 7238 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3716 - 3268 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4992 - 5276 |  | 8320 - 8932 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6656 - 7104 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9928 - 9122 |  | 952 - 638 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6406 - 5764 |  | 2406 - 2884 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10816 - 11622 |  | 5824 - 6138 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9152 - 9794 |  | 7488 - 7966 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.41.2.1.1-2 there are IMD2 and IMD3 from n3 and n20 into RX band n41.

Based on the analysis in Table 5.41.2.1.1-2 there are IMD2 and IMD3 from n3 and n41 into RX band n20.

Based on the analysis in Table 5.41.2.1.1-3 there is IMD2 from n20 and n41 into RX band n3.

5.41.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n3-n18-n41, excepts for IMD2 from n3 and n20 into RX band n41 where MSD requirement is re-used from CA_n5-n41-n66.

Table 5.41.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n20-n41 | n3 | 1735 | 5 | 25 | 1830 | N/A | FDD | N/A |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2623 | 29 | TDD | IMD2 |
|  | n3 | 1747 | 5 | 25 | 1842 | N/A | FDD | N/A |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2594 | 16 | TDD | IMD3 |
|  | n3 | 1747 | 5 | 25 | 1842 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 28.9 | FDD | IMD2 |
|  | n41 | 2553 | 10 | 50 | 2553 | N/A | TDD | N/A |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n20 | N/A | 5 | N/A | 806 | 19 | FDD | IMD3 |
|  | n41 | 2624 | 10 | 50 | 2624 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1831 | 28.9 | FDD | IMD2 |
|  | n20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |
|  | n41 | 2678 | 10 | 50 | 2678 | N/A | TDD | N/A |

5.42	CA_n3-n71-n77

5.42.1	Common for 1 band UL and 2 bands UL CA

5.42.1.1	Operating bands for CA

Table 5.42.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n71 | 663-698 | 617-652 | FDD |
| n77 | 3300-4200 | 3300-4200 | TDD |

5.42.1.2	Channel bandwidths per operating band for CA

Table 5.42.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n71A-n77A | CA_n3A-n71A CA_n3A-n77A CA_n71A-n77A | n3 | 5,10,15,20,25,30,35,40,45,50 | 0 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n77 | 10,15,20,25,30,40,50,60,70,80,90,100 |  |
| CA_n3A-n71A-n77(2A) | CA_n3A-n71A CA_n3A-n77A CA_n71A-n77A | n3 | 5,10,15,20,25,30,35,40,45,50 | 0 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n77 | CA_n77(2A)_BCS_4 and 5 |  |

5.42.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n71-n77, the TIB,c and RIB,c values are given in the tables below(Re-use CA_n3-n78-n105).

Table 5.42.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n71-n77 | 0.6 | 0.6 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.42.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n71-n77 | 0.2 | 0.3 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.42.2	Specific for 2 bands UL CA

5.42.2.1	UE co-existence studies

5.42.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.42.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n71A-n77A with UL CA_n3A-n71A.

Table 5.42.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1122 - 1012 |  | 2373 - 2483 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2722 - 2907 |  | 459 - 314 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4083 - 4268 |  | 3036 - 3181 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4432 - 4692 |  | 204 - 384 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2024 - 2244 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5793 - 6053 |  | 3699 - 3879 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 4746 - 4966 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1082 - 867 |  | 6477 - 6142 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1326 - 1581 |  | 4029 - 3734 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4362 - 4577 |  | 7503 - 7838 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5409 - 5664 |  | 6456 - 6751 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.42.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n71A-n77A with UL CA_n3A-n77A.

Table 5.42.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515 - 2490 |  | 5010 - 5985 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 780 - 270 |  | 4815 - 6690 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 - 7770 |  | 8310 - 10185 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 930 - 2055 |  | 8115 - 10890 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4980 - 3030 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 - 9555 |  | 11610 - 14385 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10020 - 11970 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15090 - 11415 |  | 3840 - 2640 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9180 - 6330 |  | 1245 - 3270 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 - 18585 |  | 10140 - 11340 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 - 16170 |  | 11730 - 13755 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.42.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n71A-n77A with UL CA_n71A-n77A.

Table 5.42.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2602 - 3537 |  | 3963 - 4898 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2874 - 1904 |  | 5902 - 7737 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4626 - 5596 |  | 7263 - 9098 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2211 - 1206 |  | 9202 - 11937 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 7074 - 5204 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5289 - 6294 |  | 10563 - 13298 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7926 - 9796 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16137 - 12502 |  | 508 - 1548 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11274 - 8504 |  | 4506 - 6411 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13863 - 17498 |  | 5952 - 6992 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11226 - 13996 |  | 8589 - 10494 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.42.2.1.1-1 there is IMD3, IMD4 and IMD5 from n3 and n71 into RX band n77.

Based on the analysis in Table 5.42.2.1.1-2 there is IMD3 from n3 and n77 into RX band n71.

Based on the analysis in Table 5.42.2.1.1-3 there is IMD4 from n71 and n77 into RX band n3.

5.42.2.2	REFSENS requirements

The MSD values are re-used from CA_n66-n71-n77 for IMD3 into n77 and n71 and DC_71A_n25A-n77A for the IMD4 into n3.

Table 5.42.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n71-n77 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 4140 | 15.9 | TDD | IMD31 |
|  | n3 | 1747 | 5 | 25 | 1842 | N/A | FDD | N/A |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3787 | 10.1 | TDD | IMD4 |
|  | n3 | 1748 | 5 | 25 | 1843 | N/A | FDD | N/A |
|  | n71 | N/A | 5 | N/A | 632 | 15.3 | FDD | IMD3 |
|  | n77 | 4128 | 10 | 50 | 4128 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1843 | 12.5 | FDD | IMD4 |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n77 | 3883 | 10 | 50 | 3883 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.43	CA_n1-n41-n78

5.43.1	Common for 1 band UL and 2 bands UL CA

5.43.1.1	Operating bands for CA

Table 5.43.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.43.1.2	Channel bandwidths per operating band for CA

Table 5.43.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n41A-n78C | CA_n78C CA_n1A-n41A CA_n1A-n78A CA_n1A-n78C CA_n41A-n78A CA_n41A-n78C | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n41 | 5,10,15,20,25,30,35,40,45,50,60,70,80,90,100 |  |
|  |  | n78 | CA_n78C_BSC4 & 5 |  |

5.43.1.3	∆TIB,c and ∆RIB,c values

Already completed in R4-2414365 of RAN4#112.

5.43.2	Specific for 2 bands UL CA

5.43.2.1	UE co-existence studies

5.43.2.1.1	Co-existence studies for 2UL band with 1CC per band

Already completed in R4-2414365 of RAN4#112.

5.43.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.43.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n41 interference analysis for CA_n1A-n78C with n1A/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.43.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n1, n78) | 1920-1980 |  | 3300-3800 |  |
| fDL (n41) | 2496-2690 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1720-1920 |  | 1980-2180 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.43.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n1 interference analysis for CA_n41A-n78C with n41A/78C transmitting with a 200 MHz maximum instantaneous bandwidth

Table 5.43.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n41, n78) | 2496-2690 |  | 3300-3800 |  |
| fDL (n1) | 2110-2170 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 2296-2496 |  | 2690-2890 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.43.2.2	REFSENS requirements

No additional Refsens requirements needed.

5.44	CA_n1-n71-n78

5.44.1	Common for 1 band UL and 2 bands UL CA

5.44.1.1	Operating bands for CA

Table 5.44.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n71 | 663-698 | 617-652 | FDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.44.1.2	Channel bandwidths per operating band for CA

Table 5.44.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n71A-n78A | CA_n1A-n71A CA_n1A-n78A CA_n71A-n78A | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n78 | 10,15,20,25,30,40,50,60,70,80,90,100 |  |
| CA_n1A-n71A-n78C | CA_n78C CA_n1A-n71A CA_n1A-n78A CA_n1A-n78C CA_n71A-n78A CA_n71A-n78C | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n78 | CA_n78C_BCS 4 and 5 |  |

5.44.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n71-n78, the TIB,c and RIB,c values are re-used from CA_n1-n78-n105 given in the tables below.

Table 5.44.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n71-n78 | 0.3 | 0.6 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.44.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n71-n78 | - | 0.2 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.44.2	Specific for 2 bands UL CA

5.44.2.1	UE co-existence studies

5.44.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.44.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n71A-n78A with UL CA_n1A-n71A.

Table 5.44.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1317 - 1222 |  | 2583 - 2678 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3142 - 3297 |  | 654 - 524 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4503 - 4658 |  | 3246 - 3376 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5062 - 5277 |  | 9 - 174 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2444 - 2634 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6423 - 6638 |  | 3909 - 4074 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5166 - 5356 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 872 - 672 |  | 7257 - 6982 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1746 - 1971 |  | 4614 - 4364 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4572 - 4772 |  | 8343 - 8618 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5829 - 6054 |  | 7086 - 7336 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.44.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n71A-n78A with UL CA_n1A-n78A.

Table 5.44.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320 - 1880 |  | 5220 - 5780 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 40 - 660 |  | 4620 - 5680 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 - 7760 |  | 8520 - 9580 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960 - 2640 |  | 7920 - 9480 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3760 - 2640 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 - 9740 |  | 11820 - 13380 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10440 - 11560 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13280 - 11220 |  | 4620 - 3880 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7560 - 5940 |  | 660 - 1840 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 - 17180 |  | 10980 - 11720 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 - 15360 |  | 12360 - 13540 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.44.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n71A-n78A with UL CA_n71A-n78A.

Table 5.44.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2602 - 3137 |  | 3963 - 4498 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2474 - 1904 |  | 5902 - 6937 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4626 - 5196 |  | 7263 - 8298 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1811 - 1206 |  | 9202 - 10737 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 6274 - 5204 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5289 - 5894 |  | 10563 - 12098 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7926 - 8996 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14537 - 12502 |  | 508 - 1148 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10074 - 8504 |  | 4506 - 5611 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13863 - 15898 |  | 5952 - 6592 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11226 - 12796 |  | 8589 - 9694 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.44.2.1.1-1 there is IMD3 from n1 and n71 into RX band n78.

Based on the analysis in Table 5.44.2.1.1-2 there is IMD3 from n1 and n78 into RX band n71.

Based on the analysis in Table 5.44.2.1.1-3 there is IMD3 from n71 and n78 into RX band n1.

5.44.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.44.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n71 interference analysis for CA_n1A-n78C with n1/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.44.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n1, n78) | 1920-1980 |  | 3300-3800 |  |
| fDL (n71) | 617-652 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1720-1920 |  | 1980-2180 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.44.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n1 interference analysis for CA_n71A -n78C with n71/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.44.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n71, n78) | 663-698 |  | 3300-3800 |  |
| fDL (n1) | 2110-2170 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 463-663 |  | 698-898 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.44.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n1-n78-n105.

Table 5.44.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n71-n78 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n71 | N/A | 5 | N/A | 635 | 15.2 | FDD | IMD3 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n71 | 686 | 5 | 25 | 640 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3342 | 15.7 | TDD | IMD3 |
|  | n1 | N/A | 5 | N/A | 2160 | 15.7 | FDD | IMD3 |
|  | n71 | 686 | 5 | 25 | 640 | N/A | FDD | N/A |
|  | n78 | 3532 | 10 | 50 | 3532 | N/A | TDD | N/A |

5.45	CA_n1-n8-n78

5.45.1	Common for 1 band UL and 2 bands UL CA

5.45.1.1	Operating bands for CA

Table 5.45.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n8 | 880-915 | 925-960 | FDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.45.1.2	Channel bandwidths per operating band for CA

Table 5.45.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n8A-n78C | CA_n78C CA_n1A-n78C CA_n8A-n78C | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 0 |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78C_BCS 4 and 5 |  |

5.45.2	Specific for 2 bands UL CA

5.45.2.1	UE co-existence studies

5.45.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.45.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n8 interference analysis for CA_n1A -n78C with n1/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.45.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n1, n78) | 1920-1980 |  | 3300-3800 |  |
| fDL (n8) | 925-960 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1720-1920 |  | 1980-2180 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.45.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n1 interference analysis for CA_n8A-n78C with n8/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.45.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n8, n78) | 880-915 |  | 3300-3800 |  |
| fDL (n1) | 2110-2170 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 680-880 |  | 915-1115 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.45.2.2	REFSENS requirements

No requirements since the bands are of different band groups [1].

5.46	CA_n3-n41-n78

5.46.1	Common for 1 band UL and 2 bands UL CA

5.46.1.1	Operating bands for CA

Table 5.46.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.46.1.2	Channel bandwidths per operating band for CA

Table 5.46.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n41A-n78C | CA_n78C CA_n3A-n41A CA_n3A-n78A CA_n3A-n78C CA_n41A-n78A CA_n41A-n78C | n3 | 5,10,15,20,25,30,35,40,45,50 | 4 and 5 |
|  |  | n41 | 5,10,15,20,25,30,35,40,45,50,60,70,80,90,100 |  |
|  |  | n78 | CA_n78C_BSC4 & 5 |  |
| CA_n3(2A)-n41A-n78C | CA_n3A-n41A CA_n3A-n78A CA_n41A-n78A | n3 | CA_n3(2A)_BSC0 | 4 and 5 |
|  |  | n41 | 5,10,15,20,25,30,35,40,45,50,60,70,80,90,100 |  |
|  |  | n78 | CA_n78C_BSC4 & 5 |  |

5.46.1.3	∆TIB,c and ∆RIB,c values

Already captured in fallback.

5.46.2	Specific for 2 bands UL CA

5.46.2.1	UE co-existence studies

5.46.2.1.1	Co-existence studies for 2UL band with 1CC per band

Already captured in fallback.

5.46.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.46.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n41 interference analysis for CA_n3A-n78C with n3/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.46.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n3, n78) | 1710-1785 |  | 3300-3800 |  |
| fDL (n41) | 2496-2690 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1510-1710 |  | 1785-1985 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.46.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n3 interference analysis for CA_n41A-n78C with n41/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.46.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n41, n78) | 2496-2690 |  | 3300-3800 |  |
| fDL(n3) | 1805-1880 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 2296-2496 |  | 2690-2890 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.46.2.2	REFSENS requirements

No additional Refsens requirements needed.

5.47	CA_n3-n71-n78

5.47.1	Common for 1 band UL and 2 bands UL CA

5.47.1.1	Operating bands for CA

Table 5.47.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n71 | 663-698 | 617-652 | FDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.47.1.2	Channel bandwidths per operating band for CA

Table 5.47.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n71A-n78A | C A_n3A-n71A CA_n3A-n78A CA_n71A-n78A | n3 | 5,10,15,20,25,30,35,40,45,50 | 4 and 5 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n78 | 10,15,20,25,30,40,50,60,70,80,90,100 |  |
| CA_n3(2A)-n71A-n78A | C A_n3A-n71A CA_n3A-n78A CA_n71A-n78A | n3 | CA_n3(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n78 | 10,15,20,25,30,40,50,60,70,80,90,100 |  |
| CA_n3A-n71A-n78C | CA_n78C CA_n3A-n71A CA_n3A-n78A CA_n3A-n78C CA_n71A-n78A CA_n71A-n78C | n3 | 5,10,15,20,25,30,35,40,45,50 | 4 and 5 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n78 | CA_n78C_BCS 4 and 5 |  |

5.47.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n71-n78, the TIB,c and RIB,c values are given in the tables below(Re-use CA_n3-n78-n105).

Table 5.47.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n71-n78 | 0.6 | 0.6 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.47.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n71-n78 | 0.2 | 0.3 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.47.2	Specific for 2 bands UL CA

5.47.2.1	UE co-existence studies

5.47.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.47.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n71A-n78A with UL CA_n3A-n71A.

Table 5.47.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1122 - 1012 |  | 2373 - 2483 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2722 - 2907 |  | 459 - 314 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4083 - 4268 |  | 3036 - 3181 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4432 - 4692 |  | 204 - 384 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2024 - 2244 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5793 - 6053 |  | 3699 - 3879 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 4746 - 4966 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1082 - 867 |  | 6477 - 6142 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1326 - 1581 |  | 4029 - 3734 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4362 - 4577 |  | 7503 - 7838 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5409 - 5664 |  | 6456 - 6751 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.47.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n71A-n78A with UL CA_n3A-n78A.

Table 5.47.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515 - 2090 |  | 5010 - 5585 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 380 - 270 |  | 4815 - 5890 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 - 7370 |  | 8310 - 9385 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330 - 2055 |  | 8115 - 9690 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4180 - 3030 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 - 9155 |  | 11610 - 13185 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10020 - 11170 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13490 - 11415 |  | 3840 - 3040 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7980 - 6330 |  | 1245 - 2470 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 - 16985 |  | 10140 - 10940 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 - 14970 |  | 11730 - 12955 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.47.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n3A-n71A-n78A with UL CA_n71A-n78A.

Table 5.47.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2602 - 3137 |  | 3963 - 4498 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2474 - 1904 |  | 5902 - 6937 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4626 - 5196 |  | 7263 - 8298 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1811 - 1206 |  | 9202 - 10737 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 6274 - 5204 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5289 - 5894 |  | 10563 - 12098 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7926 - 8996 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14537 - 12502 |  | 508 - 1148 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10074 - 8504 |  | 4506 - 5611 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13863 - 15898 |  | 5952 - 6592 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11226 - 12796 |  | 8589 - 9694 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.47.2.1.1-1 there is IMD4 and IMD5 from n3 and n71 into RX band n78.

Based on the analysis in Table 5.47.2.1.1-3 there is IMD4 from n71 and n78 into RX band n3.

5.47.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.47.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n71 interference analysis for CA_n3A-n78C with n3/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.47.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n3, n78) | 1710-1785 |  | 3300-3800 |  |
| fDL (n71) | 617-652 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1510-1710 |  | 1785-1985 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.47.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n3 interference analysis for CA_n71A-n78C with n71/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.47.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n71, n78) | 663-698 |  | 3300-3800 |  |
| fDL (n3) | 1805-1880 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 463-663 |  | 698-898 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.47.2.2	REFSENS requirements

The MSD values are re-used from CA_n3-n78-n105, where the IMD4 into n3 has been omitted, since the test frequencies cannot form an interferer inside the full band n3.

Table 5.47.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n71-n78 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n71 | 670 | 5 | 25 | 624 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3740 | 17.3 | TDD | IMD41 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.48	CA_n3-n8-n78

5.48.1	Common for 1 band UL and 2 bands UL CA

5.48.1.1	Operating bands for CA

Table 5.48.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710-1785 | 1805-1880 | FDD |
| n8 | 880-915 | 925-960 | FDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.48.1.2	Channel bandwidths per operating band for CA

Table 5.48.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n8A-n78C | CA_n78C CA_n3A-n8A CA_n3A-n78A CA_n3A-n78C CA_n8A-n78A CA_n8A-n78C | n3 | 5,10,15,20,25,30,35,40,45,50 | 4 and 5 |
|  |  | n8 | 5,10,15,20 |  |
|  |  | n78 | CA_n78C_BCS 4 and 5 |  |

5.48.2	Specific for 2 bands UL CA

5.48.2.1	UE co-existence studies

5.48.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.48.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n8 interference analysis for CA_n3A -n78C with n3/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.48.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n3, n78) | 1710-1785 |  | 3300-3800 |  |
| fDL (n8) | 925-960 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1510-1710 |  | 1785-1985 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.48.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n3 interference analysis for CA_n8A-n78C with n8/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.48.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n8, n78) | 880-915 |  | 3300-3800 |  |
| fDL (n3) | 1805-1880 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 680-880 |  | 915-1115 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.48.2.2	REFSENS requirements

There are no additional requirements.

5.49	CA_n41-n71-n78

5.49.1	Common for 1 band UL and 2 bands UL CA

5.49.1.1	Operating bands for CA

Table 5.49.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n41 | 2496-2690 | 2496-2690 | TDD |
| n71 | 663-698 | 617-652 | FDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.49.1.2	Channel bandwidths per operating band for CA

Table 5.49.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n41A-n71A-n78C | CA_n78C CA_n41A-n71A CA_n41A-n78A CA_n41A-n78C CA_n71A-n78A CA_n71A-n78C | n41 | 5,10,15,20,25,30,35,40,45,50,60,70,80,90,100 | 4 and 5 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n78 | CA_n78C_BCS 4 and 5 |  |

5.49.1.3	∆TIB,c and ∆RIB,c values

Already provided in fallback.

5.49.2	Specific for 2 bands UL CA

5.49.2.1	UE co-existence studies

5.49.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.49.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n71 interference analysis for CA_n41A -n78C with n41/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.49.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n41, n78) | 2496-2690 |  | 3300-3800 |  |
| fDL (n71) | 617-652 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 2296-2496 |  | 2690-2890 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.49.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n41 interference analysis for CA_n71A-n78C with n71/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.49.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n71, n78) | 663-698 |  | 3300-3800 |  |
| fDL (n41) | 2496-2690 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 463-663 |  | 698-898 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.49.2.2	REFSENS requirements

There are no additional Refsens requirements.

5.50	CA_n8-n41-n78

5.50.1	Common for 1 band UL and 2 bands UL CA

5.50.1.1	Operating bands for CA

Table 5.50.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n8 | 880-915 | 925-960 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.50.1.2	Channel bandwidths per operating band for CA

Table 5.50.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n8A-n41A-n78A | CA_n8A-n41A CA_n8A-n78A CA_n41A-n78A | n8 | 5, 10, 15, 20 | 4 and 5 |
|  |  | n41 | 5,10,15,20,25,30,35,40,45,50,60,70,80,90,100 |  |
|  |  | n78 | 10,15,20,25,30,40,50,60,70,80,90,100 |  |
| CA_n8A-n41A-n78C | CA_n78C CA_n8A-n41A CA_n8A-n78A CA_n8A-n78C CA_n41A-n78A CA_n41A-n78C | n8 | 5, 10, 15, 20 | 4 and 5 |
|  |  | n41 | 5,10,15,20,25,30,35,40,45,50,60,70,80,90,100 |  |
|  |  | n78 | CA_n78C_BCS 4 and 5 |  |

5.50.1.3	∆TIB,c and ∆RIB,c values

For CA_n8-n41-n78, the TIB,c and RIB,c values are given in the tables below, as they are not found in current fallback.

Table 5.50.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n8-n41-n78 | 0.6 | 0.3 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.50.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n8-n41-n78 | 0.2 | 0.4 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.50.2	Specific for 2 bands UL CA

5.50.2.1	UE co-existence studies

5.50.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.50.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n8A-n41A-n78A with UL CA_n8A-n41A.

Table 5.50.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1581 - 1810 |  | 3376 - 3605 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 930 - 666 |  | 4077 - 4500 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4256 - 4520 |  | 5872 - 6295 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 50 - 249 |  | 6573 - 7190 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3620 - 3162 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5136 - 5435 |  | 8368 - 8985 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6752 - 7210 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9880 - 9069 |  | 1164 - 830 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6310 - 5658 |  | 2247 - 2740 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10864 - 11675 |  | 6016 - 6350 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9248 - 9900 |  | 7632 - 8125 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.50.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n8A-n41A-n78A with UL CA_n8A-n78A.

Table 5.50.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2385 - 2920 |  | 4180 - 4715 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2040 - 1470 |  | 5685 - 6720 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5060 - 5630 |  | 7480 - 8515 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1160 - 555 |  | 8985 - 10520 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 5840 - 4770 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5940 - 6545 |  | 10780 - 12315 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8360 - 9430 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14320 - 12285 |  | 360 - 280 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9640 - 8070 |  | 3855 - 4960 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14080 - 16115 |  | 6820 - 7460 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11660 - 13230 |  | 9240 - 10345 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.50.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n8A-n41A-n78A with UL CA_n41A-n78A.

Table 5.50.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 610 - 1304 |  | 5796 - 6490 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1192 - 2080 |  | 3910 - 5104 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8292 - 9180 |  | 9096 - 10290 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3688 - 4770 |  | 7210 - 8904 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2608 - 1220 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10788 - 11870 |  | 12396 - 14090 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11592 - 12980 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12704 - 10510 |  | 7460 - 6184 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6408 - 4520 |  | 1470 - 112 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15696 - 17890 |  | 13284 - 14560 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14892 - 16780 |  | 14088 - 15670 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

For Table 5.50.2.1.1-1 there is IMD2 and IMD4 into band n78

For Table 5.50.2.1.1-2 there is IMD2 into band n41

For Table 5.50.2.1.1-3 there is IMD2 and IMD5 into band n8

5.50.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.50.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n41 interference analysis for CA_n8A -n78C with n8/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.50.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n78B/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n8, n78) | 880-915 |  | 3300-3800 |  |
| fDL (n41) | 2496-2690 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 680-880 |  | 915-1115 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.50.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n8 interference analysis for CA_n41A-n78C with n41/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.50.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n78B/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n41, n78) | 2496-2690 |  | 3300-3800 |  |
| fDL (n8) | 925-960 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 2296-2496 |  | 2690-2890 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.50.2.2	REFSENS requirements

The MSD values are taken from DC_8-41_n78 and DC_7A_n8A-n78A.

Table 5.50.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n8-n41-n78 | n8 | N/A | 10 | N/A | 950 | 29.1 | FDD | IMD2 |
|  | n41 | 2630 | 10 | 50 | N/A | N/A | TDD | N/A |
|  | n78 | 3580 | 10 | 50 | N/A | N/A | TDD | N/A |
|  | n8 | N/A | 10 | N/A | 940 | 4.0 | FDD | IMD5 |
|  | n41 | 2680 | 10 | 50 | N/A | N/A | TDD | N/A |
|  | n78 | 3550 | 10 | 50 | N/A | N/A | TDD | N/A |
|  | n8 | 895 | 5 | 25 | 940 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2650 | 28.0 | TDD | IMD2 |
|  | n78 | 3545 | 10 | 50 | N/A | N/A | TDD | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |
|  | n41 | 2555 | 10 | 50 | N/A | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3455 | 28.5 | TDD | IMD22 |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.51	CA_n1-n3-n78

5.51.1	Common for 1 band UL and 2 bands UL CA

5.51.1.1	Operating bands for CA

Table 5.51.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

5.51.1.2	Channel bandwidths per operating band for CA

Table 5.51.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n3A-n78C | CA_n78C CA_n1A-n3A CA_n1A-n78A CA_n1A-n78C CA_n3A-n78A CA_n3A-n78C | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 4 and 5 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78C_BCS4 & 5 |  |
| CA_n1A-n3(2A)-n78A | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 4 and 5 |
|  |  | n3 | CA_n3(2A)_BSC0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3(2A)-n78C | CA_n78C CA_n1A-n3A CA_n1A-n78A CA_n1A-n78C CA_n3A-n78A CA_n3A-n78C | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 4 and 5 |
|  |  | n3 | CA_n3(2A)_BSC0 |  |
|  |  | n78 | CA_n78C_BCS4 & 5 |  |

5.51.1.3	∆TIB,c and ∆RIB,c values

Already in spec. Only triple beat analysis is required.

5.51.2	Specific for 2 bands UL CA

Only triple beat analysis is required.

5.51.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.51.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n3 interference analysis for CA_n1A-n78C with n1/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.51.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n1, n78) | 1920-1980 |  | 3300-3800 |  |
| fDL (n3) | 1805-1880 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1720-1920 |  | 1980-2180 |  |
| Analysis | There is an issue of IMD3 |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.51.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n1 interference analysis for CA_n3A -n78C with n3/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.51.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n78C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n3, n78) | 1710-1785 |  | 3300-3800 |  |
| fDL (n1) | 2110-2170 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1510-1710 |  | 1785-1985 |  |
| Analysis | There is no overlap |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.51.2.2	REFSENS requirements

Refsens requirements are provided in Table 5.51.2.2-1.

Table 5.51.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n78 | n1 | 1925 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | 25 | 1875 | N/A | 15.3 | IMD3 |
|  | n78 | 3300 | 100 | 1 (RBstart=203) | 3300 | N/A | TDD | N/A |
|  |  | 3400 | 100 | 1 (RBstart=67) | 3400 |  |  |  |

5.52	CA_n1-n40-n41

5.52.1	Common for 1 band UL and 2 bands UL CA

5.52.1.1	Operating bands for CA

Table 5.52.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |

5.52.1.2	Channel bandwidths per operating band for CA

Table 5.52.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n40A-n41A | CA_n1A-n40A CA_n1A-n41A CA_n40A-n41A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |

5.52.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n40-n41, the TIB,c values are given in the tables below, which are the existing requirements for DC_1-7_n40. Reasons are:

-	With existing requirements for CA_n1-n40 and CA_n1-n41, all bands are allowed to have 0.5dB relaxation, respectively.

-	It is noticeable the requirements for DC_1-7_n40 provide 0.6dB relaxation for band 1, 0.8dB relaxation for band 7 and 0.9dB relaxation for band n40.

Table 5.52.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n40-n41 | 0.6 | 0.9 | 0.8 |

As for the RIB,c values, the existing requirements for CA_n1-n7-n40 can be reused here.

Table 5.52.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n40-n41 | - | 0.8 | 0.3 |
| NOTE *: “-” denotes ΔRIB,c = 0. |  |  |  |

5.52.2	Specific for 2 bands UL CA

5.52.2.1	UE co-existence studies

5.52.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.52.2.1.1-1~3 provide the two UL bands with one CC per band IMD interference analysis for CA_n1A-n40A-n41A with UL CA_n1A-n40A, CA_n1A-n41A and CA_n40A-n41A respectively.

Table 5.52.2.1.1-1: UL CA_n1A-n40A IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 320 – 480 |  | 4220 – 4380 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 – 1660 |  | 2620 – 2880 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6140 – 6360 |  | 6520 – 6780 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3360 – 3640 |  | 4920 – 5280 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 640 – 840 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8060 – 8340 |  | 8820 – 9180 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8440 – 8760 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7220 – 7680 |  | 5280 – 5620 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2940 – 3360 |  | 960 – 1340 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11120 – 11580 |  | 9980 –10320 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10740 – 11160 |  | 10360 – 10740 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above analysis, 3rd order IMD generated by UL CA_n1A-n40A may fall into the Rx of Band n41.

Table 5.52.2.1.1-2: UL CA_n1A-n41A IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 516 – 770 |  | 4416 – 4670 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1150 – 1464 |  | 3012 – 3460 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6336 – 6650 |  | 6912 – 7360 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3070 – 3444 |  | 5508 – 6150 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1032 – 1420 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8256 – 8630 |  | 9408 – 10050 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8832 - 9340 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8004 – 8840 |  | 4990 – 5424 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3528 – 4230 |  | 380 – 948 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11904 – 12740 |  | 10176 –10610 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11328 – 12030 |  | 10752 – 11320 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above analysis, no IMD generated by UL CA_n1A-n41A would fall into the Rx of concerned bands.

Table 5.52.2.1.1-3: UL CA_n40A-n41A IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 96 –390 |  | 4796 – 5090 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1910 – 2304 |  | 2592 – 3080 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7096 – 7490 |  | 7292 – 7780 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4210 – 4704 |  | 5088 – 5770 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 192 – 580 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9396 – 9890 |  | 9788 – 10470 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 9592 - 10180 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7584 - 8460 |  | 6510 – 7104 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2688 – 3470 |  | 1520 – 2208 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 12284 – 13160 |  | 11696 –12290 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12088 – 12870 |  | 11892 – 12580 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above analysis, 3rd order IMD and 5th order IMD generated by UL CA_n40A-n41A may fall into the Rx of Band n1.

5.52.2.2	REFSENS requirements

Based on the co-existence studies, there is a need to define MSD. The defined requirements for DC_1A_7A-n40A and DC_7A_n1A-n40A are taken into consideration here in conjunction with worst duplexer performance that would be expected from band n41 (comparing with the one for band 7).

Table 5.52.2.2-3: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n40-n41 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | TDD | N/A |
|  | n41 | N/A | 5 | N/A | 2630 | 23 | TDD | IMD3 |
|  | n1 | N/A | 5 | N/A | 2130 | 25.2 | FDD | IMD3* |
|  | n40 | 2335 | 5 | 25 | 2335 | N/A | TDD | N/A |
|  | n41 | 2540 | 5 | 25 | 2660 | N/A | TDD | N/A |
| Note *: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.53	CA_n3-n77-n79

5.53.1			Common for 1 band UL and 2 bands UL CA

5.53.1.1	Operating bands for CA

Table 5.53.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

5.53.1.2	Channel bandwidths per operating band for CA

Table 5.53.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n77(2A)-n79A4 | n777,9 n797,9 CA_n77(2A) CA_n3A-n77A7 CA_n3A-n79A7 CA_n77A-n79A7 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n77(3A)-n79A4 | CA_n77(2A) CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |

5.53.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n77-n79, the ∆TIB,c and ∆RIB,c values have already been specified in TS 38.101-1.

5.53.2	Specific for 2 bands UL CA

5.53.2.1	UE co-existence studies

5.53.2.1.1	Co-existence studies for 2UL band with 1CC per band

Based on Reference sensitivity exceptions due to intermodulation interference due to 2UL CA for CA in TS38.101-1[2], no MSD is needed for UL CA_n77(2A).

#### 5.53.2.2	REFSENS requirements

Based on co-existence studies on 5.53.2.1, no need to define exceptional REFSENS requirements.

5.54	CA_n28-n77-n79

5.54.1			Common for 1 band UL and 2 bands UL CA

5.54.1.1	Operating bands for CA

Table 5.54.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

5.54.1.2	Channel bandwidths per operating band for CA

Table 5.54.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n28A-n77(2A)-n79A4 | n777,9 n797,9 CA_n77(2A) CA_n28A-n77A7 CA_n28A-n79A7 CA_n77A-n79A7 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n77(3A)-n79A4 | CA_n77(2A) CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |

5.54.1.3	∆TIB,c and ∆RIB,c values

For CA_n28-n77-n79, the ∆TIB,c and ∆RIB,c values have already been specified in TS 38.101-1.

5.54.2	Specific for 2 bands UL CA

5.54.2.1	UE co-existence studies

5.54.2.1.1	Co-existence studies for 2UL band with 1CC per band

Based on Reference sensitivity exceptions due to intermodulation interference due to 2UL CA for CA in TS38.101-1[2], no MSD is needed for UL CA_n77(2A).

5.54.2.2	REFSENS requirements

Based on co-existence studies on 5.54.2.1, no need to define exceptional REFSENS requirements.

5.55	CA_n40-n78-n79

5.55.1	Common for 1 band UL and 2 bands UL CA

5.55.1.1	Operating bands for CA

Table 5.55.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

5.55.1.2	Channel bandwidths per operating band for CA

Table 5.55.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n40A-78A-n79A | CA_n40A-n78A CA_n40A-n79A CA_n78A-n79A | n40 | n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |

5.55.1.3	∆TIB,c and ∆RIB,c values

For CA_n40-78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 5.55.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_ n40-78-n79 | 0.3 | 0.5 / 1.58 | 0.5 / 1.58 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. NOTE 8: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |

Table 5.55.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_ n40-78-n79 | 0.4 | 0.5 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.55.2	Specific for 2 bands UL CA

5.55.2.1	UE co-existence studies

5.55.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.55.2.1.1-1, Table 5.55.2.1.1-2 and Table 5.55.2.1.1-3 provide the two UL bands with one CC per band IMD interference analysis for CA_n40A-78A-n79A with UL CA_n40A-n78A, UL CA_n40A-n79A and UL CA_n78A-n79A.

Table 5.55.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 900–1500 |  | 5600–6200 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 800–1500 |  | 4200–5300 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7900–8600 |  | 8900–10000 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3100–3900 |  | 7500–9100 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1800–3000 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10200–11000 |  | 12200–13800 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11200–12400 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10800–12900 |  | 5400–6300 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5100–6800 |  | 600–700 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15500–17600 |  | 12500–13400 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14500–16200 |  | 13500–14800 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 3rd IMD generated by UL CA_n40A-n78A may fall into own Rx of Band n79.

Table 5.55.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2000–2700 |  | 6700–7400 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 400 |  | 6400–7700 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9000–9800 |  | 11100–12400 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1900–2800 |  | 10800–12700 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4000–5400 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11300–12200 |  | 15500–17400 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 13400–14800 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15200–17700 |  | 4200–5200 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8400–10400 |  | 1600–3100 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 19900–22400 |  | 13600–14600 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 17800–19800 |  | 15700–17200 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, no IMD generated by UL CA_n40A-n79A may fall into own Rx of Band n78.

Table 5.55.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 600–1700 |  | 7700–8800 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1600–3200 |  | 5000–6700 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 11000–12600 |  | 12100–13800 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4900–7000 |  | 9400–11700 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1200–3400 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 14300–16400 |  | 16500–18800 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 15400–17600 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13800–16700 |  | 8200–10800 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5600–8400 |  | 100–2600 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 20900–23800 |  | 17600–20200 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 19800–22600 |  | 18700–21400 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 3rd,4th and 5th order IMD generated by UL CA_n78A-n79A may fall into own Rx of Band n40.

5.55.2.2	REFSENS requirements

For CA_n40-78-n79, the MSD values are given in the tables below.

Table 5.55.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n78-n79 | n40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4450 | 23 | TDD | IMD3 |
|  | n40 | N/A | 5 | N/A | 2350 | 12 | TDD | IMD31 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | 4450 | 10 | 50 | 4450 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2320 | 6 | TDD | IMD4 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | 4560 | 10 | 50 | 4560 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.56	CA_n1-n40-n79

5.56.1	Common for 1 band UL and 2 bands UL CA

5.56.1.1	Operating bands for CA

Table 5.56.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

5.56.1.2	Channel bandwidths per operating band for CA

Table 5.56.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n40A-n79A | CA_n1A-n40A CA_n1A-n79A CA_n40A-n79A | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |

5.56.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n40-n79, the TIB,c and RIB,c values can reuse that of CA_n1-n41-n79, and are given in the tables below.

Table 5.56.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n40-n79 | 0.5 | 0.5 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. NOTE 8: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |

Table 5.56.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n40-n79 | - | 0.5 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.56.2	Specific for 2 bands UL CA

5.56.2.1	UE co-existence studies

5.56.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.56.2.1.1-1, Table 5.56.2.1.1-2 and Table 5.56.2.1.1-3 provide the two UL bands with one CC per band IMD interference analysis for CA_n1A-n40A-n79A with UL CA_n1A-n40A, UL CA_n1A-n79A and UL CA_n40A-n79A.

Table 5.56.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 320–480 |  | 4220–4380 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440–1660 |  | 2620–2880 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6140–6360 |  | 6520–6780 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3360–3640 |  | 4920–5280 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 640–960 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8060–8340 |  | 8820–9180 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8440–8760 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7220–7680 |  | 5280–5620 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2940–3360 |  | 960–1340 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11120–11580 |  | 9980–10320 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10740–11160 |  | 10360–10740 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 4th IMD generated by UL CA_n1A-n40A may fall into own Rx of Band n79.

Table 5.56.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2420–3080 |  | 6320–6980 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 440-1160 |  | 6820–8080 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8240–8960 |  | 10720–11980 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 760–1540 |  | 11220–13080 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4840–6160 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10160–10940 |  | 15120–16980 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 12640–13960 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15620–18080 |  | 2680–3520 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9240–11160 |  | 2860–4240 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 19520–21980 |  | 12080–12920 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 17040–18960 |  | 14560–15940 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, no IMD generated by UL CA_n1A-n79A may fall into own Rx of Band n40.

Table 5.56.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2000–2700 |  | 6700–7400 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 400 |  | 6400–7700 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9000–9800 |  | 11100–12400 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1900–2800 |  | 10800–12700 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4000–5400 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11300–12200 |  | 15500–17400 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 13400–14800 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15200–17700 |  | 4200–5200 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8400–10400 |  | 1600–3100 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 19900–22400 |  | 13600–14600 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 17800–19800 |  | 15700–17200 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 2nd, 4th and 5th order IMD generated by UL CA_n40A-n79A may fall into own Rx of Band n1.

5.56.2.2	REFSENS requirements

For CA_n1-n40-n79, the MSD values are given in the tables below. Among them, the n79 MSD value 19dB and n1 MSD 29.4dB reused from CA_n1-n41-n79.

Table 5.56.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n40-n79 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4960 | 19 | TDD | IMD4 |
|  | n1 | N/A | 5 | N/A | 2150 | 29.4 | FDD | IMD22 |
|  | n40 | 2380 | 5 | 25 | 2380 | N/A | TDD | N/A |
|  | n79 | 4530 | 10 | 50 | 4530 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2150 | [0] | FDD | IMD5 |
|  | n40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | n79 | 4600 | 10 | 50 | 4600 | N/A | TDD | N/A |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.57	CA_n3-n40-n78

5.57.1	Common for 1 band UL and 2 bands UL CA

5.57.1.1	Operating bands for CA

Table 5.57.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |

5.57.1.2	Channel bandwidths per operating band for CA

Table 5.57.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n40A-n78A | CA_n3A-n40A CA_n3A-n78A CA_n40A-n78A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |

5.57.1.3	∆TIB,c and ∆RIB,c values

∆TIB,c and ∆RIB,c values have already been defined in the current spec.

5.57.2	Specific for 2 bands UL CA

5.57.2.1	UE co-existence studies

5.57.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.57.2.1.1-1, Table 5.57.2.1.1-2 and Table 5.57.2.1.1-3 provide the two UL bands with one CC per band IMD interference analysis for CA_n3A-n40A-n78A with UL CA_n3A-n40A, UL CA_n3A-n78A and UL CA_n40A-n78A.

Table 5.57.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 320–480 |  | 4220–4380 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440–1660 |  | 2620–2880 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6140–6360 |  | 6520–6780 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3360–3640 |  | 4920–5280 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 640–960 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8060–8340 |  | 8820–9180 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8440–8760 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7220–7680 |  | 5280–5620 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2940–3360 |  | 960–1340 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11120–11580 |  | 9980–10320 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10740–11160 |  | 10360–10740 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 4th and 5th IMD generated by UL CA_n3A-n40A may fall into own Rx of Band n78.

Table 5.57.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515–2090 |  | 5010–5585 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 270- 380 |  | 4815–5890 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720–7370 |  | 8310–9690 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330–2055 |  | 8115–13080 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3030–4180 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430–9155 |  | 11610–13185 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10020–11170 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11415–13490 |  | 3040–3840 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6330–7980 |  | 1245–2470 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910–16985 |  | 10140–10940 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320–14970 |  | 11730–12955 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 5th IMD generated by UL CA_n3A-n78A may fall into own Rx of Band n40.

Table 5.57.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 900–1500 |  | 5600–6200 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 800-1500 |  | 4200–5300 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7900–8600 |  | 8900–10000 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3100–3900 |  | 7500–9100 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1800–3000 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10200–11000 |  | 12200–13800 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11200–12400 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10800–12900 |  | 5400–6300 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5100–6800 |  | 600–700 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15500–17600 |  | 12500–13400 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14500–16200 |  | 13500–14800 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 4th order IMD generated by UL CA_n40A-n78A may fall into own Rx of Band n3.

5.57.2.2	REFSENS requirements

For CA_n3-n40-n78, the MSD values are given in the tables below.Among them, n78 MSD value 16.8dB due to IMD4, and n40 MSD value 5.3 dB due to IMD5 are referenced with that of CA_n3-n41-n77.

Table 5.57.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n40-n78 | n3 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3440 | 16.8 | TDD | IMD4 |
|  | n3 | 1925 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3335 | 0.8 | TDD | IMD5 |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2360 | 5.3 | TDD | IMD5 |
|  | n78 | 3760 | 10 | 50 | 3760 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1860 | 14 | FDD | IMD4 |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | TDD | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |

5.58	CA_n7-n40-n79

5.58.1	Common for 1 band UL and 2 bands UL CA

5.58.1.1	Operating bands for CA

Table 5.58.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

5.58.1.2	Channel bandwidths per operating band for CA

Table 5.58.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n7A-n40A-n79A | CA_n7A-n40A CA_n7A-n79A CA_n40A-n79A | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |

5.58.1.3	∆TIB,c and ∆RIB,c values

For CA_n7-n40-n79, the TIB,c and RIB,c values are given in the tables below.

Table 5.58.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n7-n40-n79 | 0.5 | 0.6 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.58.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n7-n40-n79 | - | 0.5 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.58.2	Specific for 2 bands UL CA

5.58.2.1	UE co-existence studies

5.58.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.58.2.1.1-1, Table 5.58.2.1.1-2 and Table 5.58.2.1.1-3 provide the two UL bands with one CC per band IMD interference analysis for CA_n7A-n40A-n79A with UL CA_n7A-n40A, UL CA_n7A-n79A and UL CA_n40A-n79A.

Table 5.58.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 100–270 |  | 4800–4970 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2600–2840 |  | 2030–2300 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7300–7540 |  | 7100–7370 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5100–5410 |  | 4330–4700 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 200–540 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9800–10110 |  | 9400–9770 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 9600–9940 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6630–7100 |  | 7600–7980 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1760–2200 |  | 2700–3110 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11700–12170 |  | 12300–12680 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11900–12340 |  | 12100–12510 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 2rd and 4th IMD generated by UL CA_n7A-n40A may fall into own Rx of Band n79.

Table 5.58.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1830–2500 |  | 6900–7570 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 0- 740 |  | 6230–7500 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9400–10140 |  | 11300–12570 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2500–3310 |  | 10630–12500 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3660–5000 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11900–12710 |  | 15700–17570 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 13800–15140 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15030–17500 |  | 5000–5880 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8060–10000 |  | 1090–2500 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 20100–22570 |  | 14400–15280 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 18200–20140 |  | 16300–17710 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 2rd and 5th IMD generated by UL CA_n7A-n79A may fall into own Rx of Band n40.

Table 5.58.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2000–2700 |  | 6700–7400 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 400 |  | 6400–7700 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9000–9800 |  | 11100–12400 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1900–2800 |  | 10800–12700 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4000–5400 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11300–12200 |  | 15500–17400 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 13400–14800 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15200–17700 |  | 4200–5200 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8400–10400 |  | 1600–3100 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 19900–22400 |  | 13600–14600 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 17800–19800 |  | 15700–17200 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 2rd, 4th and 5th order IMD generated by UL CA_n40A-n79A may fall into own Rx of Band n7.

5.58.2.2	REFSENS requirements

For CA_n7-n40-n79, the MSD values are given in the tables below.

Table 5.58.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n40-n79 | n7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4830 | 30.5 | TDD | IMD22 |
|  | n7 | 2530 | 5 | 25 | 2650 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2320 | 25 | TDD | IMD2 |
|  | n79 | 4850 | 10 | 50 | 4850 | N/A | FDD | N/A |
|  | n7 | 2510 | 5 | 25 | 2630 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2370 | 6 | TDD | IMD5 |
|  | n79 | 4950 | 10 | 50 | 4950 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2630 | 29.4 | FDD | IMD2 |
|  | n40 | 2320 | 5 | 25 | 2320 | N/A | TDD | N/A |
|  | n79 | 4950 | 10 | 50 | 4950 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2650 | 3 | FDD | IMD5 |
|  | n40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | n79 | 4850 | 10 | 50 | 4850 | N/A | TDD | N/A |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.59	CA_n7-n78-n79

5.59.1	Common for 1 band UL and 2 bands UL CA

5.59.1.1	Operating bands for CA

Table 5.59.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

5.59.1.2	Channel bandwidths per operating band for CA

Table 5.59.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n7A-n78A-n79A | CA_n7A-n78A CA_n7A-n79A CA_n78A-n79A | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |

5.59.1.3	∆TIB,c and ∆RIB,c values

For CA_n7-n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 5.59.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n7-n78-n79 | 0.5 | 0.5 / 1.58 | 0.5 / 1.58 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. NOTE 8: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |

Table 5.59.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n7-n78-n79 | 0.5 | 0.5 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.59.2	Specific for 2 bands UL CA

5.59.2.1	UE co-existence studies

5.59.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.59.2.1.1-1, Table 5.59.2.1.1-2 and Table 5.59.2.1.1-3 provide the two UL bands with one CC per band IMD interference analysis for CA_n7A-n78A-n79A with UL CA_n7A-n78A, UL CA_n7A-n79A and UL CA_n78A-n79A.

Table 5.59.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730–1300 |  | 5800–6370 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1200–1840 |  | 4030–5100 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300–8940 |  | 9100–10170 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3700–4410 |  | 7330–8900 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1460–2600 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800–11510 |  | 12400–13970 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11600–12740 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10630–12700 |  | 6200–6980 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4760–6400 |  | 100–1110 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700–17770 |  | 13300–14080 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900–16540 |  | 14100–15310 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 3rd ,4th and 5th IMD generated by UL CA_n7A-n78A may fall into own Rx of Band n79.

Table 5.59.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1830–2500 |  | 6900–7570 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 0- 740 |  | 6230–7500 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9400–10140 |  | 11300–12570 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2500–3310 |  | 10630–12500 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3660–5000 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11900–12710 |  | 15700–17570 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 13800–15140 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15030–17500 |  | 5000–5880 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8060–10000 |  | 1090–2500 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 20100–22570 |  | 14400–15280 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 18200–20140 |  | 16300–17710 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 4th IMD generated by UL CA_n7A-n79A may fall into own Rx of Band n78.

Table 5.59.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 600–1700 |  | 7700–8800 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1600-3200 |  | 5000–6700 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 11000–12600 |  | 12100–13800 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4900–7000 |  | 9400–11700 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1200–3400 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 14300–16400 |  | 16500–18800 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 15400–17600 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13800–16700 |  | 8200–10800 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5600–8400 |  | 100–2600 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 20900–23800 |  | 17600–20200 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 19800–22600 |  | 18700–21400 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 3rd and 4th order IMD generated by UL CA_n78A-n79A may fall into own Rx of Band n7.

5.59.2.2	REFSENS requirements

For CA_n7-n78-n79, the MSD values are given in the tables below.

Table 5.59.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n78-n79 | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4450 | 20.6 | TDD | IMD31 |
|  | n7 | 2567.5 | 5 | 25 | 2687.5 | N/A | FDD | N/A |
|  | n78 | 3302.5 | 10 | 50 | 3302.5 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4400 | 14 | TDD | IMD4 |
|  | n7 | 2525 | 5 | 25 | 2645 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3760 | 6.4 | TDD | IMD4 |
|  | n79 | 4405 | 10 | 50 | 4405 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 1860 | 19 | FDD | IMD3 |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n79 | 4780 | 10 | 50 | 4780 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2660 | 8 | FDD | IMD4 |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | TDD | N/A |
|  | n79 | 4830 | 10 | 50 | 4830 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.60	CA_n1-n3-n77

5.60.1	Common for 1 band UL and 2 bands UL CA

5.60.1.1	Operating bands for CA

Table 5.60.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

5.60.1.2	Channel bandwidths per operating band for CA

Table 5.60.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n3A-n77(2A) | n777,9 CA_n1A-n3A CA_n1A-n77A7 CA_n3A-n77A7 CA_n77(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | CA_n77(2A) BCS1 |  |
|  | CA_n1A-n3A CA_n1A-n77A CA_n3A-n77A CA_n77(2A) | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5 3 5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n1A-n3A-n77(3A) | CA_n1A-n3A CA_n1A-n77A CA_n3A-n77A CA_n77(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Minimum requirements for Power Class 1.5 are applicable for single uplink carrier in this downlink/uplink combination. |  |  |  |  |

5.60.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n3-n77, TIB,c and RIB,c values have already been defined in TS38.101-1[2].

5.60.2	Specific for 2 bands UL CA

5.60.2.1	UE co-existence studies

5.60.2.1.1	Co-existence studies for 2UL band with 1CC per band

Based on the exception to the reference sensitivity due to intermodulation interference by 2UL CA in TS38.101-1[2], no MSD is needed for UL CA_n77(2A).

5.60.2.2	REFSENS requirements

Based on co-existence studies on 5.60.2.1, no need to define exceptional REFSENS requirements.

5.61	CA_n3-n40-n41

5.61.1	Common for 1 band UL and 2 bands UL CA

5.61.1.1		Operating bands for CA

Table 5.61.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880MHz | FDD |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |

5.61.1.2	Channel bandwidths per operating band for CA

Table 5.61.1.2-1: Supported bandwidths per CA band combination of band n3+n34+n41

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n34A-n41A | CA_n3A-n34A CA_n3A-n41A CA_n34A-n41A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n34 | See n34 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n34A-n41C | CA_n3A-n34A CA_n3A-n41A CA_n34A-n41A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n34 | See n34 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |

5.61.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n34-n41, the TIB,c and RIB,c values are proposed in the following tables:

Table 5.61.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n3-n34-n41 | 0.5 | 0.5 | 0.34 / 0.85 |
| NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.61.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n3-n34-n41 | - | - | 04 / 0.55 |
| NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n3 the band order from left to right is n1, n3 and n3. |  |  |  |

5.61.2	Specific for 2 bands UL CA

5.61.2.1	UE co-existence studies

The co-existence for the fallback 2DL/2UL of CA_n3A-n34A, CA_n3A-n41A and CA_n34A-n41A have already been analyzed. In terms of the co-existence studies of corresponding fallbacks, it can be observed:

IMD5 issue caused by n3+n34, i.e.  2*fn3-3*fn34, fall into the its own band n41 Rx;

No IMD issue caused by n34+n41 falls into the its own band n3 Rx;

No IMD issue caused by n3+n41 fallx into the its own band n34 Rx;

5.61.2.2	REFSENS requirements

Based on co-existence studies additional MSD is needed to be defined. The MSD values are proposed in table 5.61.2.2-1.

Table 5.61.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3-n34-n41 | n3 | 1742.5 | 5 | 25 | 1837.5 | N/A | N/A |
|  | n34 | 2020 | 5 | 25 | 2020 | N/A | N/A |
|  | n41 | N/A | 5 | N/A | 2550 | 1.0 | IMD5 |

5.62		CA_n3-n40-n79

5.62.1	Common for 1 band UL and 2 bands UL CA

5.62.1.1		Operating bands for CA

Table 5.62.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880MHz | FDD |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

5.62.1.2	Channel bandwidths per operating band for CA

Table 5.62.1.2-1: Supported bandwidths per CA band combination of band n3+n34+n39

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n34A-n79A | CA_n3A-n34A CA_n3A-n79A CA_n34A-n79A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n34 | See n34 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |

5.62.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n34-n79, the TIB,c and RIB,c values are proposed in the following tables:

Table 5.62.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n3-n34-n79 | 0.5 | 0.5 | 0.8 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.62.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n3-n34-n79 | - | - | 0.5 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n3 the band order from left to right is n1, n3 and n3. |  |  |  |

5.62.2	Specific for 2 bands UL CA

5.62.2.1	UE co-existence studies

The co-existence for the fallback 2DL/2UL of CA_n3A-n34A, CA_n3A-n79A and CA_n34A-n79A have already been analyzed. In terms of the co-existence studies of corresponding fallbacks, it can be observed:

IMD5 issue caused by n3+n34, i.e.  4*fn34-fn3, fall into the its own band n79 Rx;

No IMD issue caused by n34+n79 falls into the its own band n3 Rx;

IMD5 issue caused by n3+n79, i.e.  4*fn3-fn79,  falls into the its own band n34 Rx.

5.62.2.2	REFSENS requirements

Based on co-existence studies additional MSD is needed to be defined. The MSD values are proposed in table 5.62.2.2-1.

Table 5.62.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3-n34-n79 | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n34 | 2020 | 5 | 25 | 2020 | N/A | N/A |
|  | n79 | N/A | 40 | N/A | 4900 | 3.2 | IMD5 |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n79 | 4900 | 40 | 216 | 4900 | N/A | N/A |
|  | n34 | N/A | 5 | N/A | 2020 | 9.7 | IMD5 |

5.63	CA_n2-n66-n77

5.63.1	Common for 1 band UL and 2 bands UL CA

5.63.1.1	Operating bands for CA

Table 5.63.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n2 | 1850 MHz – 1910 MHz | 1930 MHz –1990 MHz | FDD |
| n66 | 1710 MHz –1780 MHz | 2110 MHz – 2200 MHz | FDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

5.63.1.2	Channel bandwidths per operating band for CA

Table 5.63.1.2-1: Supported bandwidths per CA band combination of band n2+n66+n77

| NR CA configuration |  | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n2A-n66A-n77C | n777,9 CA_n2A-n66A CA_n2A-n77A7 CA_n66A-n77A7 |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n77 | CA_n77C_BCS0 |  |
|  |  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |
|  | CA_n77C CA_n2A-n66A CA_n2A-n77A CA_n2A-n77C CA_n66A-n77A CA_n66A-n77C |  | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n2(2A)-n66A-n77C | CA_n2A-n66A CA_n2A-n77A CA_n2A-n77C CA_n66A-n77A CA_n66A-n77C CA_n77C |  | n2 | CA_n2(2A)_BCS4 and 5 | 4 and 5 |
|  |  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n2A-n66(2A)-n77C | CA_n2A-n66A CA_n2A-n77A CA_n2A-n77C CA_n66A-n77A CA_n66A-n77C CA_n77C |  | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n66 | CA_n66(2A)_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n2(2A)-n66(2A)-n77C | CA_n2A-n66A CA_n2A-n77A CA_n2A-n77C CA_n66A-n77A CA_n66A-n77C CA_n77C |  | n2 | CA_n2(2A)_BCS4 and 5 | 4 and 5 |
|  |  |  | n66 | CA_n66(2A)_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |

5.63.2	Specific for 2 bands UL CA

5.63.2.1	UE co-existence studies

5.63.2.1.1	Co-existence studies for 2UL band with 1CC per band

Co-existence study has been performed in Rel-18.

5.63.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.63.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n66 interference analysis for CA_n2A-n77C with n77C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.63.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n2, n77) | 1850 – 1910 |  | 3300 – 4200 |  |
| fDL (n66) | 2110 – 2200 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 – 200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1650 – 1850 |  | 1910 – 2110 |  |
| Analysis | No Issue |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.63.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n2 interference analysis for CA_n66A-n77C with n77C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.63.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n66, n77) | 1710 – 1780 |  | 3300 – 4200 |  |
| fDL (n2) | 1930 – 1990 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 – 200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1510 – 1710 |  | 1780 – 1980 |  |
| Analysis | Triple beat IMD may fall into band n2. |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.63.2.2	REFSENS requirements

Based on the triple beat analysis of the added ULCA, triple beat IMD of UL CA_ n66-n77 may fall into RX of band n2 and need to define new MSD value for IMD3.  MSD value for CA_n1-n3-n78 is reused.

Table 5.63.2.2-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n2-n66-n77 | n2 | N/A | 5 | 25 | 1950 | 15.3 | FDD | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2195 | N/A | FDD | N/A |
|  | n77 | 3300 | 100 | 1 (RBstart=203) | 3300 | N/A | TDD | N/A |
|  |  | 3400 | 100 | 1 (RBstart=67) | 3400 |  |  |  |

5.64	CA_n2-n48-n77

5.64.1	Common for 1 band UL and 2 bands UL CA

5.64.1.1	Operating bands for CA

.

Table 5.64.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n2 | 1850 MHz – 1910 MHz | 1930 MHz –1990 MHz | FDD |
| n48 | 3550 MHz – 3700 MHz | 3550 MHz – 3700 MHz | TDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

5.64.1.2	Channel bandwidths per operating band for CA

Table 5.64.1.2-1: Supported bandwidths per CA band combination of band n2+n48+n77

| NR CA configuration |  | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n2A-n48A-n77C | n777,9 CA_n2A-n48A CA_n2A-n77A7 CA_n77C |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  |  | n77 | CA_n77C_BCS0 |  |
|  |  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |
|  | CA_n2A-n48A CA_n2A-n77A CA_n2A-n77C CA_n77C |  | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n48 | n48 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n2A-n48(2A)-n77C | n777,9 CA_n2A-n48A CA_n2A-n77A7 |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |
|  | CA_n77C CA_n2A-n48A CA_n2A-n77A CA_n2A-n77C |  | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n48 | CA_n48(2A)_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n2A-n48B-n77A | n777,9 CA_n48B CA_n2A-n48A CA_n2A-n77A7 |  | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n48 | CA_n48B_BCS0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  |  | n48 | CA_n48B_BCS1 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n2 | 5, 10, 15, 20 | 2 |
|  |  |  | n48 | CA_n48B_BCS2 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n48B CA_n2A-n48A CA_n2A-n48B CA_n2A-n77A |  | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n48 | CA_n48B_BCS4 and 5 |  |
|  |  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2A-n48B-n77C | n777,9 CA_n48B CA_n2A-n48A CA_n2A-n77A7 |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n48 | CA_n48B_BCS2 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |
|  | CA_n48B CA_n77C CA_n2A-n48A CA_n2A-n48B CA_n2A-n77A CA_n2A-n77C |  | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n48 | CA_n48B_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n2(2A)-n48B-n77A | CA_n2A-n48A CA_n2A-n48B CA_n2A-n77A CA_n48B |  | n2 | CA_n2(2A)_BCS4 and 5 | 4 and 5 |
|  |  |  | n48 | CA_n48B_BCS4 and 5 |  |
|  |  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2(2A)-n48A-n77C | CA_n2A-n48A CA_n2A-n77A CA_n2A-n77C CA_n77C |  | n2 | CA_n2(2A)_BCS4 and 5 | 4 and 5 |
|  |  |  | n48 | n48 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n2(2A)-n48(2A)-n77C | CA_n2A-n48A CA_n2A-n77A CA_n2A-n77C CA_n77C |  | n2 | CA_n2(2A)_BCS4 and 5 | 4 and 5 |
|  |  |  | n48 | CA_n48(2A)_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n2(2A)-n48B-n77C | CA_n2A-n48A CA_n2A-n48B CA_n2A-n77A CA_n2A-n77C CA_n48B CA_n77C |  | n2 | CA_n2(2A)_BCS4 and 5 | 4 and 5 |
|  |  |  | n48 | CA_n48B_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |

5.64.2	Specific for 2 bands UL CA

5.64.2.1	UE co-existence studies

5.64.2.1.1	Co-existence studies for 2UL band with 1CC per band

Co-existence study has been performed in Rel-18.

5.64.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.64.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n48 interference analysis for CA_n2A-n77C with n77C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.64.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n2, n77) | 1850 – 1910 |  | 3300 – 4200 |  |
| fDL (n48) | 3550 – 3700 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 – 200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1650 – 1850 |  | 1910 – 2110 |  |
| Analysis | No Issue |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.64.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n77 interference analysis for CA_n2A-n48B with n48B transmitting with a 40 MHz maximum instantaneous bandwidth.

Table 5.64.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n48B |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n2, n48) | 1850 – 1910 |  | 3550 – 3700 |  |
| fDL (n77) | 3300 – 4200 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 – 150 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1810 – 1850 |  | 1910 – 1950 |  |
| Analysis | No Issue |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.64.2.2	REFSENS requirements

Based on the triple beat analysis of the added UL CA_n2A-n77C and CA_n2A-n48B, there are no triple beat issues and no need to define new MSD values.

5.65	CA_n5-n66-n77

5.65.1	Common for 1 band UL and 2 bands UL CA

5.65.1.1	Operating bands for CA

Table 5.65.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n5 | 824 MHz – 849 MHz | 869 MHz –894 MHz | FDD |
| n66 | 1710 MHz –1780 MHz | 2110 MHz – 2200 MHz | FDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

5.65.1.2	Channel bandwidths per operating band for CA

Table 5.65.1.2-1: Supported bandwidths per CA band combination of band n5+n66+n77

| NR CA configuration |  | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n5A-n66A-n77C | n777,9 CA_n5A-n66A CA_n5A-n77A7 CA_n66A-n77A7 CA_n77C |  | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n77 | CA_n77C_BCS0 |  |
|  |  |  | n5 | 5, 10, 15, 20, 251 | 1 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |
|  | CA_n5A-n66A CA_n5A-n77A CA_n5A-n77C CA_n66A-n77A CA_n66A-n77C CA_n77C |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n5A-n66(2A)-n77C | CA_n5A-n66A CA_n5A-n77A CA_n5A-n77C CA_n66A-n77A CA_n66A-n77C CA_n77C |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n66 | CA_n66(2A)_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n5B-n66A-n77C | CA_n5A-n66A CA_n5A-n77A CA_n5A-n77C CA_n5B CA_n66A-n77A CA_n66A-n77C CA_n77C |  | n5 | CA_n5B_BCS4 and 5 | 4 and 5 |
|  |  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n5B-n66(2A)-n77C | CA_n5A-n66A CA_n5A-n77A CA_n5A-n77C CA_n5B CA_n66A-n77A CA_n66A-n77C CA_n77C |  | n5 | CA_n5B_BCS4 and 5 | 4 and 5 |
|  |  |  | n66 | CA_n66(2A)_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |

5.65.2	Specific for 2 bands UL CA

5.65.2.1	UE co-existence studies

5.65.2.1.1	Co-existence studies for 2UL band with 1CC per band

Co-existence study has been performed in Rel-18.

5.65.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.65.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n66 interference analysis for CA_n5A-n77C with n77C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.65.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n5, n77) | 824 – 849 |  | 3300 – 4200 |  |
| fDL (n66) | 2110 – 2200 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 – 200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 624 – 824 |  | 849 – 1049 |  |
| Analysis | No Issue |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.65.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n5 interference analysis for CA_n66A-n77C with n77C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.65.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n66, n77) | 1710 – 1780 |  | 3300 – 4200 |  |
| fDL (n5) | 869 – 894 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 – 200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1510 – 1710 |  | 1780 – 1980 |  |
| Analysis | No Issue |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.65.2.2	REFSENS requirements

Based on the triple beat analysis of the added ULCA, there are no triple beat IMD issues and no need to define MSD values.

5.66	CA_n48-n66-n77

5.66.1	Common for 1 band UL and 2 bands UL CA

5.66.1.1	Operating bands for CA

Table 5.66.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n48 | 3550 MHz – 3700MHz | 3550 MHz – 3700MHz | TDD |
| n66 | 1710 MHz –1780 MHz | 2110 MHz – 2200 MHz | FDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

5.66.1.2	Channel bandwidths per operating band for CA

Table 5.66.1.2-1: Supported bandwidths per CA band combination of band n48+n66+n77

| NR CA configuration |  | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n48A-n66A-n77C | n777,9 CA_n48A-n66A CA_n66A-n77A7 CA_n77C |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n77 | CA_n77C_BCS0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 | 1 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |
|  | CA_n48A-n66A CA_n66A-n77A CA_n66A-n77C CA_n77C |  | n48 | n48 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n48B-n66A-n77A | n777,9 CA_n48A-n66A CA_n66A-n77A7 |  | n48 | CA_n48B_BCS0 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n48A-n66A CA_n48B-n66A CA_n66A-n77A CA_n48B |  | n48 | CA_n48B_BCS4 and 5 | 4 and 5 |
|  |  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n48B-n66A-n77C | n777,9 CA_n48A-n66A CA_n66A-n77A7 CA_n77C |  | n48 | CA_n48B_BCS2 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |
|  | CA_n48A-n66A CA_n48B-n66A CA_n66A-n77A CA_n66A-n77C CA_n48B CA_n77C |  | n48 | CA_n48B_BCS4 and 5 | 4 and 5 |
|  |  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n48(2A)-n66A-n77C | n777,9 CA_n77C CA_n48A-n66A CA_n66A-n77A7 |  | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n77 | CA_n77C_BCS0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS0 | 1 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 | 2 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n77 | CA_n77C_BCS0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 | 3 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |
|  | CA_n77C CA_n48A-n66A CA_n66A-n77A CA_n66A-n77C |  | n48 | CA_n48(2A)_BCS4 and 5 | 4 and 5 |
|  |  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n48B-n66(2A)-n77A | CA_n48B CA_n48A-n66A CA_n48B-n66A CA_n66A-n77A |  | n48 | CA_n48B_BCS4 and 5 | 4 and 5 |
|  |  |  | n66 | CA_n66(2A)_BCS4 and 5 |  |
|  |  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n48A-n66(2A)-n77C | CA_n77C CA_n48A-n66A CA_n66A-n77A CA_n66A-n77C |  | n48 | n48 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n66 | CA_n66(2A)_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n48(2A)-n66(2A)-n77C | CA_n77C CA_n48A-n66A CA_n66A-n77A CA_n66A-n77C |  | n48 | CA_n48(2A)_BCS4 and 5 | 4 and 5 |
|  |  |  | n66 | CA_n66(2A)_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n48B-n66(2A)-n77C | CA_n48B CA_n77C CA_n48A-n66A CA_n48B-n66A CA_n66A-n77A CA_n66A-n77C |  | n48 | CA_n48B_BCS4 and 5 | 4 and 5 |
|  |  |  | n66 | CA_n66(2A)_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |

5.66.2	Specific for 2 bands UL CA

5.66.2.1	UE co-existence studies

5.66.2.1.1	Co-existence studies for 2UL band with 1CC per band

Co-existence study has been performed in Rel-18.

5.66.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.66.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n48 interference analysis for CA_n66A-n77C with n77C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.66.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n66, n77) | 1710 – 1780 |  | 3300 – 4200 |  |
| fDL (n48) | 3550 – 3700 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 – 200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1510 – 1710 |  | 1780 – 1980 |  |
| Analysis | No Issue |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.66.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n48 interference analysis for CA_n48B-n66A with n48B transmitting with a 40 MHz maximum instantaneous bandwidth.

Table 5.66.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n48B |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n48, n66) | 1710 – 1780 |  | 3550 – 3700 |  |
| fDL (n77) | 3300 – 4200 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 – 150 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1670 – 1710 |  | 1780 – 1820 |  |
| Analysis | No Issue |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.66.2.2	REFSENS requirements

Based on the triple beat analysis of the added ULCA, there are no triple beat IMD issues and no need to define MSD values.

5.67	CA_n3-n78-n79

5.67.1	Common for 1 band UL and 2 bands UL CA

5.67.1.1	Operating bands for CA

Table 5.67.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

5.67.1.2	Channel bandwidths per operating band for CA

Table 5.67.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n78A-n79A5 | n787,9 CA_n3A-n78A CA_n3A-n79A CA_n78A-n79A5 | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n3A-n78A-n79A | CA_n3A-n78A CA_n3A-n79A CA_n78A-n79A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 5: Simultaneous Rx/Tx capability for TDD combinations does not apply for UEs supporting band n78 with an n77 implementation. NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Minimum requirements for Power Class 1.5 are applicable for single uplink carrier in this downlink/uplink combination. |  |  |  |  |

Note that the single uplink carrier with note 7 or 9 is already supported in the specification.

5.67.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n78-n79, the ∆TIB,c and ∆RIB,c values have already been defined in the current specification.

5.67.2	Specific for 2 bands UL CA

5.67.2.1	UE co-existence studies

5.67.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.67.2.1.1-1, Table 5.67.2.1.1-2 and Table 5.67.2.1.1-3 provide the two UL bands with one CC per band IMD interference analysis for CA_n3A-n78A-n79A with UL CA_n3A-n78A, UL CA_n3A-n79A A and UL CA_n78A-n79A.

Table 5.67.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515–2090 |  | 5010–5585 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 270–380 |  | 4815–5890 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720–7370 |  | 8310–9385 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330–2055 |  | 8115–9690 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3030–4180 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430–9155 |  | 11610–13185 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10020–11170 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11415–13490 |  | 3040–3840 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6330–7980 |  | 1245–2470 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910–16985 |  | 10140–10940 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320–14970 |  | 11730–12955 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 3rd IMD generated by UL CA_n3A-n78A may fall into own Rx of Band n79.

Table 5.67.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2615–3290 |  | 6110–6785 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 830 -1580 |  | 7015–8290 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7820–8570 |  | 10510–11785 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 130–955 |  | 11415–13290 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 5230–6580 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9530–10355 |  | 14910–16785 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 12220–13570 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15815–18290 |  | 1840–2740 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9630–11580 |  | 3445–4870 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 19310–21785 |  | 11240–12140 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 16620–18570 |  | 13930–15355 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 5th IMD generated by UL CA_n3A-n79A may fall into own Rx of Band n78.

Table 5.67.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 600–1700 |  | 7700–8800 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1600–3200 |  | 5000–6700 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 11000–12600 |  | 12100–13800 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4900–7000 |  | 9400–11700 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1200–3400 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 14300–16400 |  | 16500–18800 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 15400–17600 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13800–16700 |  | 8200–10800 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5600–8400 |  | 100–2600 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 20900–23800 |  | 17600–20200 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 19800–22600 |  | 18700–21400 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 3rd, 4th and 5th IMD generated by UL CA_n78A-n79A may fall into own Rx of Band n3.

5.67.2.2	REFSENS requirements

For CA_n3-n78-n79, the MSD values are given as follows. Among them, IMD3 MSD value of n3 is reused from that of  CA_n3-n77-n79.

Table 5.67.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n78-n79 | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n78 | 3320 | 10 | 52 | 3320 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4860 | 13.1 | TDD | IMD3 |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3480 | 1 | TDD | IMD5 |
|  | n79 | 4410 | 10 | 52 | 4410 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 15.7 | FDD | IMD31 |
|  | n78 | 3350 | 10 | 52 | 3350 | N/A | TDD | N/A |
|  | n79 | 4840 | 10 | 52 | 4840 | N/A | TDD | N/A |
|  | n3 | 1745 | 5 | 25 | 1840 | 3 | FDD | IMD4 |
|  | n78 | 3780 | 10 | 52 | 3780 | N/A | TDD | N/A |
|  | n79 | 4700 | 10 | 52 | 4700 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.68	CA_n28-n40-n71

5.68.1	Common for 1 band UL and 2 bands UL CA

5.68.1.1	Operating bands for CA

Table 5.68.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n28 | 703 MHz - 748 MHz | 758 MHz - 803 MHz | FDD |
| n40 | 2300 MHz - 2400 MHz | 2300 MHz - 2400 MHz | TDD |
| n71 | 663 MHz - 698 MHz | 617 MHz - 652 MHz | FDD |

5.68.1.2	Channel bandwidths per operating band for CA

Table 5.68.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n28A-n40A-n71A | CA_n40A-n71A CA_n28A-n40A | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4&5 |
|  |  | n40 | n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |

5.68.1.3	∆TIB,c and ∆RIB,c values

For CA_n28-n40-n71, the TIB,c and RIB,c values are given in the tables below.

Table 5.68.1.3-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n28-n40-n71 | 1.1 | 0.3 | 1.1 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |  |

Table 5.68.1.3-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n28-n40-n71 | 0.7 | - | 0.7 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |  |

5.68.2	Specific for 2 bands UL CA

5.68.2.1	UE co-existence studies

5.68.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.68.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n28A-n40A-n71A with UL CA_n40A-n71A.

Table 5.68.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1737 | 1602 | 2963 | 3098 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3902 | 4137 | 1074 | 904 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5263 | 5498 | 3626 | 3796 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6202 | 6537 | 411 | 206 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3204 | 3474 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7563 | 7898 | 4289 | 4494 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5926 | 6196 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 492 | 252 | 8937 | 8502 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2506 | 2811 | 5874 | 5504 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4952 | 5192 | 9863 | 10298 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6589 | 6894 | 8226 | 8596 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.68.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n28A-n40A-n71A with UL CA_n28A-n40A.

Table 5.68.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1552 | 1697 | 3003 | 3148 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 994 | 804 | 3852 | 4097 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3706 | 3896 | 5303 | 5548 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 291 | 56 | 6152 | 6497 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3394 | 3104 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4409 | 4644 | 7603 | 7948 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6006 | 6296 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8897 | 8452 | 692 | 412 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5794 | 5404 | 2356 | 2691 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9903 | 10348 | 5112 | 5392 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8306 | 8696 | 6709 | 7044 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.68.2.1.1-1 there is no IMD issue when both n40 and n71 are transmitting signal.

Based on the analysis in Table 5.68.2.1.1-2 there is IMD5 from n28 and n40 into RX band n71.

5.68.2.2	REFSENS requirements

The MSD values are proposed below based on the existing CA_n18-n28-n41.

Table 5.68.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n40-n71 | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | n71 | N/A | 5 | N/A | 622 | 3.9 | FDD | IMD5 |

5.69	CA_n28-n71-n77

5.69.1	Common for 1 band UL and 2 bands UL CA

5.69.1.1	Operating bands for CA

Table 5.69.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n28 | 703 MHz - 748 MHz | 758 MHz - 803 MHz | FDD |
| n71 | 663 MHz - 698 MHz | 617 MHz - 652 MHz | FDD |
| n77 | 3300 MHz - 4200 MHz | 3300 MHz - 4200 MHz | TDD |

5.69.1.2	Channel bandwidths per operating band for CA

Table 5.69.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n28A-n71A-n77A | CA_n28A-n77A CA_n71A-n77A | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4&5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n71A-n77(2A) | CA_n28A-n77A CA_n71A-n77A | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4&5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |

5.69.1.3	∆TIB,c and ∆RIB,c values

For CA_n28-n71-n77, the TIB,c and RIB,c values are given in the tables below.

Table 5.69.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n28-n71-n77 | 1.1 | 1.1 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |  |

Table 5.69.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n28-n71-n77 | 0.7 | 0.7 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |  |

5.69.2	Specific for 2 bands UL CA

#### 5.69.2.1	UE co-existence studies

5.69.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.69.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n28A-n71A-n77A with UL CA_n28A-n77A.

Table 5.69.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2552 | 3497 | 4003 | 4948 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2794 | 1804 | 5852 | 7697 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4706 | 5696 | 7303 | 9148 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2091 | 1056 | 9152 | 11897 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 6994 | 5104 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 6444 | 10603 | 13348 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8006 | 9896 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16097 | 12452 | 308 | 1388 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11194 | 8404 | 4356 | 6291 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13903 | 17548 | 6112 | 7192 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11306 | 14096 | 8709 | 10644 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.69.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n28A-n71A-n77A with UL CA_n71A-n77A.

Table 5.69.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2602 | 3537 | 3963 | 4898 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2874 | 1904 | 5902 | 7737 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4626 | 5596 | 7263 | 9098 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2211 | 1206 | 9202 | 11937 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 7074 | 5204 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5289 | 6294 | 10563 | 13298 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7926 | 9796 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16137 | 12502 | 508 | 1548 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11274 | 8504 | 4506 | 6411 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13863 | 17498 | 5952 | 6992 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11226 | 13996 | 8589 | 10494 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.69.2.1.1-1 there is IMD5 from n28 and n77 into RX band n71.

Based on the analysis in Table 5.69.2.1.1-2 there is IMD5 from n71 and n77 into RX band n28.

#### 5.69.2.2	REFSENS requirements

The MSD values are proposed below based on the existing CA_n5-n28-n78 and CA_n5-n12-n77

Table 5.69.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n71-n77 | n28 | 713 | 5 | 25 | 768 | N/A | FDD | N/A |
|  | n71 | N/A | 5 | N/A | 628 | 4.7 | FDD | IMD5 |
|  | n77 | 3480 | 10 | 50 | 3480 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 778 | 4.3 | FDD | IMD5 |
|  | n71 | 688 | 5 | 25 | 642 | N/A | FDD | N/A |
|  | n77 | 3530 | 10 | 50 | 3530 | N/A | TDD | N/A |

5.70	CA_n40-n71-n77

5.70.1	Common for 1 band UL and 2 bands UL CA

5.70.1.1	Operating bands for CA

Table 5.70.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n40 | 2300 MHz - 2400 MHz | 2300 MHz - 2400 MHz | TDD |
| n71 | 663 MHz - 698 MHz | 617 MHz - 652 MHz | FDD |
| n77 | 3300 MHz - 4200 MHz | 3300 MHz - 4200 MHz | TDD |

5.70.1.2	Channel bandwidths per operating band for CA

Table 5.70.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n40A-n71A-n77A | CA_n40A-n71A CA_n40A-n77A CA_n71A-n77A | n40 | n40 channel bandwidths in Table 5.3.5-1 | 4&5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n40A-n71A-n77(2A) | CA_n40A-n71A CA_n40A-n77A CA_n71A-n77A | n40 | n40 channel bandwidths in Table 5.3.5-1 | 4&5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |

5.70.1.3	∆TIB,c and ∆RIB,c values

For CA_n40-n71-n77, the TIB,c and RIB,c values are given in the tables below.

Table 5.70.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n40-n71-n77 | 0.3 | 0.5 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |  |

Table 5.70.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n40-n71-n77 | - | 0.2 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |  |

5.70.2	Specific for 2 bands UL CA

5.70.2.1	UE co-existence studies

5.70.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.70.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n40A-n71A-n77A with UL CA_n40A-n71A.

Table 5.70.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1737 | 1602 | 2963 | 3098 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3902 | 4137 | 1074 | 904 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5263 | 5498 | 3626 | 3796 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6202 | 6537 | 411 | 206 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3204 | 3474 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7563 | 7898 | 4289 | 4494 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5926 | 6196 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 492 | 252 | 8937 | 8502 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2506 | 2811 | 5874 | 5504 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4952 | 5192 | 9863 | 10298 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6589 | 6894 | 8226 | 8596 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.70.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n40A-n71A-n77A with UL CA_n40A-n77A.

Table 5.70.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 900 | 1900 | 5600 | 6600 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 400 | 1500 | 4200 | 6100 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7900 | 9000 | 8900 | 10800 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2700 | 3900 | 7500 | 10300 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3800 | 1800 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10200 | 11400 | 12200 | 15000 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11200 | 13200 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14500 | 10800 | 6300 | 5000 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8000 | 5100 | 600 | 1500 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15500 | 19200 | 12500 | 13800 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14500 | 17400 | 13500 | 15600 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.70.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n40A-n71A-n77A with UL CA_n71A-n77A.

Table 5.70.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2602 | 3537 | 3963 | 4898 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2874 | 1904 | 5902 | 7737 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4626 | 5596 | 7263 | 9098 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2211 | 1206 | 9202 | 11937 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 7074 | 5204 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5289 | 6294 | 10563 | 13298 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7926 | 9796 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16137 | 12502 | 508 | 1548 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11274 | 8504 | 4506 | 6411 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13863 | 17498 | 5952 | 6992 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11226 | 13996 | 8589 | 10494 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.70.2.1.1-1 there are IMD3 and IMD4 from n40 and n71 into RX band n77.

Based on the analysis in Table 5.70.2.1.1-2 there are IMD3 and IMD5 from n40 and n77 into RX band n71.

Based on the analysis in Table 5.70.2.1.1-3 there is IMD3 from n71 and n77 into RX band n40.

5.70.2.2	REFSENS requirements

The MSD values are proposed below based on the existing CA_n41-n71-n77.

Table 5.70.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n71-n77 | n40 | 2320 | 5 | 25 | 2320 | N/A | TDD | N/A |
|  | n71 | N/A | 5 | N/A | 630 | 18.7 | FDD | IMD32 |
|  | n77 | 3580 | 10 | 50 | 3580 | N/A | TDD | N/A |
|  | n40 | 2330 | 5 | 25 | 2330 | N/A | TDD | N/A |
|  | n71 | 688 | 5 | 25 | 642 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3706 | 16.3 | TDD | IMD31 |
|  | n40 | N/A | 5 | N/A | 2354 | 15.5 | TDD | IMD3 |
|  | n71 | 688 | 5 | 25 | 642 | N/A | FDD | N/A |
|  | n77 | 3730 | 10 | 50 | 3530 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.71	CA_n8-n78-n79

5.71.1	Common for 1 band UL and 2 bands UL CA

5.71.1.1	Operating bands for CA

Table 5.71.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

5.71.1.2	Channel bandwidths per operating band for CA

Table 5.71.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n8A-n78A-n79A | CA_n8A-n78A CA_n8A-n79A CA_n78A-n79A | n8 | n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |

5.71.1.3	∆TIB,c and ∆RIB,c values

For CA_n8-n78-n79, the ∆TIB,c and ∆RIB,c values have already been defined in the current specification.

5.71.2	Specific for 2 bands UL CA

5.71.2.1	UE co-existence studies

5.71.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.71.2.1.1-1, Table 5.71.2.1.1-2 and Table 5.71.2.1.1-3 provide the two UL bands with one CC per band IMD interference analysis for CA_n8A-n78A-n79A with UL CA_n8A-n78A, UL CA_n8A-n79A A and UL CA_n78A-n79A.

Table 5.71.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2385–2920 |  | 4180–4715 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1470–2040 |  | 5685–6720 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5060–5630 |  | 7480–8515 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 555–1160 |  | 8985–10520 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4770–5840 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5940–6545 |  | 10780–12315 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8360–9430 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12285–14320 |  | 280–360 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8070–9640 |  | 3855–4960 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14080–16115 |  | 6820–7460 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11660–13230 |  | 9240–10345 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 2nd,4th and 5th IMD generated by UL CA_n8A-n78A may fall into own Rx of Band n79.

Table 5.71.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3485–4120 |  | 5280–5915 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2570 -3240 |  | 7885–9120 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6160–6830 |  | 9680–10915 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1655–2360 |  | 12285–14120 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 6970–8240 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7040–7745 |  | 14080–15915 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10560–11830 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16685–19120 |  | 740–1480 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11370–13240 |  | 6055–7360 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 18480–20915 |  | 7920–8660 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14960–16830 |  | 11440–12745 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 2nd IMD generated by UL CA_n8A-n79A may fall into own Rx of Band n78.

Table 5.71.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 600–1700 |  | 7700–8800 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1600–3200 |  | 5000–6700 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 11000–12600 |  | 12100–13800 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4900–7000 |  | 9400–11700 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1200–3400 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 14300–16400 |  | 16500–18800 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 15400–17600 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13800–16700 |  | 8200–10800 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5600–8400 |  | 100–2600 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 20900–23800 |  | 17600–20200 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 19800–22600 |  | 18700–21400 |  |
| NOTE: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 2nd and 5th IMD generated by UL CA_n78A-n79A may fall into own Rx of Band n8.

5.71.2.2	REFSENS requirements

For CA_n8-n78-n79, the MSD values are given as follows.

Table 5.71.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n8-n78-n79 | n8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |
|  | n78 | 3700 | 10 | 52 | 3700 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4600 | 25 | TDD | IMD22 |
|  | n8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |
|  | n78 | 3700 | 10 | 52 | 3700 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4700 | 2 | TDD | IMD5 |
|  | n8 | 827 | 5 | 25 | 872 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3593 | 25 | TDD | IMD2 |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n8 | N/A | 5 | N/A | 930 | 20.3 | FDD | IMD2 |
|  | n78 | 3600 | 10 | 50 | 3600 | N/A | TDD | N/A |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | TDD | N/A |
|  | n8 | N/A | 5 | N/A | 940 | 3 | FDD | IMD5 |
|  | n78 | 3600 | 10 | 50 | 3600 | N/A | TDD | N/A |
|  | n79 | 4930 | 40 | 216 | 4930 | N/A | TDD | N/A |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.72	CA_n2-n5-n77

5.72.1	Common for 1 band UL and 2 bands UL CA

5.72.1.1	Operating bands for CA

Table 5.72.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n2 | 1850 MHz – 1910 MHz | 1930 MHz –1990 MHz | FDD |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

5.72.1.2	Channel bandwidths per operating band for CA

Table 5.72.1.2-1: Supported bandwidths per CA band combination of band n2+n5+n77

| NR CA configuration |  | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n2A-n5A-n77C | n777,9 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 CA_n77C |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  |  | n77 | CA_n77C_BCS0 |  |
|  |  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |
|  | CA_n2A-n5A CA_n2A-n77A CA_n2A-n77C CA_n5A-n77A CA_n5A-n77C CA_n77C |  | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n2A-n5B-n77C | CA_n2A-n5A CA_n2A-n77A CA_n2A-n77C CA_n5B CA_n5A-n77A CA_n5A-n77C CA_n77C |  | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n5 | CA_n5B_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n2(2A)-n5A-n77C | CA_n2A-n5A CA_n2A-n77A CA_n2A-n77C CA_n5A-n77A CA_n5A-n77C CA_n77C |  | n2 | CA_n2(2A)_BCS4 and 5 | 4 and 5 |
|  |  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |

5.72.2	Specific for 2 bands UL CA

5.72.2.1	UE co-existence studies

5.72.2.1.1	Co-existence studies for 2UL band with 1CC per band

Co-existence study has been performed in Rel-18.

5.72.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.72.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n5 interference analysis for CA_n2A-n77C with n77C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.72.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n2, n77) | 1850 – 1910 |  | 3300 – 4200 |  |
| fDL (n5) | 869 – 894 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 – 200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1650 – 1850 |  | 1910 – 2110 |  |
| Analysis | No Issue |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.72.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n2 interference analysis for CA_n5A-n77C with n77C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.72.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n5, n77) | 824 – 849 |  | 3300 – 4200 |  |
| fDL (n2) | 1930 – 1990 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 – 200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 624 – 824 |  | 849 – 1049 |  |
| Analysis | No Issue |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.72.2.2	REFSENS requirements

Based on the triple beat analysis of the added ULCA, there are no triple beat IMD issue and no need to define MSD values.

5.73	CA_n5-n48-n77

5.73.1	Common for 1 band UL and 2 bands UL CA

5.73.1.1	Operating bands for CA

Table 5.73.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| n48 | 3550 MHz – 3700 MHz | 3550 MHz – 3700 MHz | TDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

5.73.1.2	Channel bandwidths per operating band for CA

Table 5.73.1.2-1: Supported bandwidths per CA band combination of band n5+n48+n77

| NR CA configuration |  | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n5A-n48A-n77C | n777,9 CA_n5A-n48A CA_n5A-n77A7 CA_n77C |  | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  |  | n77 | CA_n77C_BCS0 |  |
|  |  |  | n5 | 5, 10, 15, 20, 251 | 1 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |
|  | CA_n5A-n48A CA_n5A-n77A CA_n5A-n77C CA_n77C |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n48 | n48 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n5A-n48B-n77A | n777,9 CA_n5A-n48A CA_n5A-n77A7 |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n48 | CA_n48B_BCS0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  |  | n48 | CA_n48B_BCS1 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  |  | n48 | CA_n48B_BCS2 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n5A-n48A CA_n5A-n48B CA_n5A-n77A CA_n48B |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n48 | CA_n48B_BCS4 and 5 |  |
|  |  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5A-n48B-n77C | n777,9 CA_n5A-n48A CA_n5A-n77A7 CA_n77C |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n48 | CA_n48B_BCS0 |  |
|  |  |  | n77 | CA_n77C_BCS0 |  |
|  |  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  |  | n48 | CA_n48B_BCS0 |  |
|  |  |  | n77 | CA_n77C BCS1 |  |
|  |  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  |  | n48 | CA_n48B_BCS1 |  |
|  |  |  | n77 | CA_n77C BCS0 |  |
|  |  |  | n5 | 5, 10, 15, 20 | 3 |
|  |  |  | n48 | CA_n48B_BCS1 |  |
|  |  |  | n77 | CA_n77C BCS1 |  |
|  | CA_n5A-n48A CA_n5A-n48B CA_n5A-n77A CA_n5A-n77C CA_n48B CA_n77C |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n48 | CA_n48B_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n5A-n48(2A)-n77C | n777,9 CA_n5A-n48A CA_n5A-n77A7 CA_n77C |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  |  | n77 | CA_n77C_BCS0 |  |
|  |  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |
|  |  |  | n5 | 5, 10, 15, 20 | 2 |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  |  | n77 | CA_n77C_BCS0 |  |
|  |  |  | n5 | 5, 10, 15, 20 | 3 |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |
|  | CA_n5A-n48A CA_n5A-n77A CA_n5A-n77C CA_n77C |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n48 | CA_n48(2A)_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n5B-n48(2A)-n77C | CA_n5A-n48A CA_n5A-n77A CA_n5A-n77C CA_n5B CA_n77C |  | n5 | CA_n5B_BCS4 and 5 | 4 and 5 |
|  |  |  | n48 | CA_n48(2A)_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n5B-n48A-n77C | CA_n5A-n48A CA_n5A-n77A CA_n5A-n77C CA_n5B CA_n77C |  | n5 | CA_n5B_BCS4 and 5 | 4 and 5 |
|  |  |  | n48 | n48 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n5B-n48B-n77A | CA_n5A-n48A CA_n5A-n48B CA_n5A-n77A CA_n5B CA_n48B |  | n5 | CA_n5B_BCS4 and 5 | 4 and 5 |
|  |  |  | n48 | CA_n48B_BCS4 and 5 |  |
|  |  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5B-n48B-n77C | CA_n5A-n48A CA_n5A-n48B CA_n5A-n77A CA_n5A-n77C CA_n5B CA_n48B CA_n77C |  | n5 | CA_n5B_BCS4 and 5 | 4 and 5 |
|  |  |  | n48 | CA_n48B_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |

5.73.2	Specific for 2 bands UL CA

5.73.2.1	UE co-existence studies

5.73.2.1.1	Co-existence studies for 2UL band with 1CC per band

Co-existence study has been performed in Rel-18.

5.73.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.73.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n48 interference analysis for CA_n5A-n77C with n77C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.73.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n5, n77) | 824 – 849 |  | 3300 – 4200 |  |
| fDL (n48) | 3550 – 3700 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 – 200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 624 – 824 |  | 849 – 1049 |  |
| Analysis | No Issue |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.73.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n48 interference analysis for CA_n5A-n48B with n48B transmitting with a 40 MHz maximum instantaneous bandwidth.

Table 5.73.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | Single Carrier |  | CA_n48B |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n5, n48) | 824 – 849 |  | 3550 – 3700 |  |
| fDL (n77) | 3300 – 4200 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 – 150 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 784 – 824 |  | 849 – 889 |  |
| Analysis | No Issue |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.73.2.2	REFSENS requirements

Based on the triple beat analysis of the added UL CA_n5A-n77C and CA_n5A-n48B, there are no triple beat issues and no need to define new MSD values.

5.74	CA_n28-n74-n77

5.74.1	Common for 1 band UL and 2 bands UL CA

5.74.1.1	Operating bands for CA

Table 5.74.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n74 | 1427 MHz – 1470 MHz | 1475 MHz – 1518 MHz | FDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

5.74.1.2	Channel bandwidths per operating band for CA

Table 5.74.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n28A-n74A-n77A | CA_n28A-n77A CA_n74A-n77A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n74 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |

5.74.1.3	∆TIB,c and ∆RIB,c values

The TIB,c and RIB,c values are given in the tables below.

Table 5.74.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n28-n74-n77 | 0.6 | 0.4 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.74.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n28-n74-n77 | 0.2 | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.74.2	Specific for 2 bands UL CA

5.74.2.1	UE co-existence studies

5.74.2.1.1	Co-existence studies for 2UL band with 1CC per band

Based on co-existence calculation,

- IMD4 due to UL CA_n28A-n77A falls into n74 DL

- IMD3/4 due to UL CA_n74A-n77A falls into n28 DL

5.74.2.2	REFSENS requirements

Based on clause 5.74.2.1.1, the MSD values are given as follows, which are reused from DC_19-21_n77.

Table 5.74.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n74-n77 | n28 | 730 | 5 | 25 | 785 | N/A | FDD | N/A |
|  | n74 | N/A | 5 | 25 | 1500 | 9.0 | FDD | IMD4 |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | 25 | 785 | 18.7 | FDD | IMD3 |
|  | n74 | 1452 | 5 | 25 | 1500 | N/A | FDD | N/A |
|  | n77 | 3689 | 10 | 50 | 3689 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | 25 | 785 | 13.2 | FDD | IMD4 |
|  | n74 | 1458 | 5 | 25 | 1506 | N/A | FDD | N/A |
|  | n77 | 3589 | 10 | 50 | 3589 | N/A | TDD | N/A |

5.75	CA_n28-n41-n74

5.75.1	Common for 1 band UL and 2 bands UL CA

5.75.1.1	Operating bands for CA

Table 5.75.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n41 | 2496 MHz – 2690MHz | 2496 MHz – 2690MHz | TDD |
| n74 | 1427 MHz – 1470 MHz | 1475 MHz – 1518 MHz | FDD |

5.75.1.2	Channel bandwidths per operating band for CA

Table 5.75.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n28A-n41A-n74A | CA_n28A-n41A CA_n41A-n74A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n74 | 5, 10, 15, 20 |  |

5.75.1.3	∆TIB,c and ∆RIB,c values

The TIB,c and RIB,c values are given in the tables below.

Table 5.75.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n28-n41-n74 | 0.6 | 0.3 | 0.4 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.75.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n28-n41-n74 | 0.2 | - | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.75.2	Specific for 2 bands UL CA

5.75.2.1	UE co-existence studies

5.75.2.1.1	Co-existence studies for 2UL band with 1CC per band

Based on co-existence calculation,

- No IMD due to UL CA_n28A-n41A falls into 74 DL

- IMD5 due to UL CA_n41A-n74A falls into n28 DL

5.75.2.2	REFSENS requirements

Based on clause 5.75.2.1.1, the MSD values are given as follows.

Table 5.75.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n41-n74 | n28 | N/A | 5 | N/A | 782 | 3.5 | FDD | IMD5 |
|  | n41 | 2569 | 10 | 50 | 2569 | N/A | TDD | N/A |
|  | n74 | 1452 | 5 | 25 | 1500 | N/A | FDD | N/A |

5.76	CA_n41-n74-n77

5.76.1	Common for 1 band UL and 2 bands UL CA

5.76.1.1	Operating bands for CA

Table 5.76.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n74 | 1427 MHz – 1470 MHz | 1475 MHz – 1518 MHz | FDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

5.76.1.2	Channel bandwidths per operating band for CA

Table 5.76.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n41A-n74A-n77A | CA_n41A-n74A CA_n41A-n77A CA_n74A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n74 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |

5.76.1.3	∆TIB,c and ∆RIB,c values

The TIB,c and RIB,c values are given in the tables below.

Table 5.76.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n41-n74-n77 | 0.3 | 0.4 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.76.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n41-n74-n77 | - | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.76.2	Specific for 2 bands UL CA

5.76.2.1	UE co-existence studies

5.76.2.1.1	Co-existence studies for 2UL band with 1CC per band

Based on co-existence calculation,

- IMD2/3/5 due to UL CA_n41A-n74A falls into n77 DL

- IMD2/3/4 due to UL CA_n41A-n77A falls into n74 DL

- IMD2/5 due to UL CA_n74A-n77A falls into n41 DL

5.76.2.2	REFSENS requirements

Based on clause 5.76.2.1.1, the MSD values are given as follows.

Table 5.76.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n74-n77 | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n74 | N/A | 5 | N/A | 1500 | 29.2 | FDD | IMD22 |
|  | n77 | 4150 | 10 | 50 | 4150 | NA | TDD | N/A |
|  | n41 | 2525 | 10 | 50 | 2525 | N/A | TDD | N/A |
|  | n74 | N/A | 5 | N/A | 1500 | 15 | FDD | IMD3 |
|  | n77 | 3550 | 10 | 50 | 3550 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2650 | 28 | TDD | IMD2 |
|  | n74 | 1450 | 5 | 25 | 1500 | N/A | FDD | N/A |
|  | n77 | 4100 | 10 | 50 | 4150 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2518 | 3 | TDD | IMD5 |
|  | n74 | 1454 | 5 | 25 | 1502 | N/A | FDD | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n74 | 1450 | 5 | 25 | 1500 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 4100 | 27 | TDD | IMD2 |
|  | n41 | 2525 | 10 | 50 | 2525 | N/A | TDD | N/A |
|  | n74 | 1450 | 5 | 25 | 1498 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3600 | 14 | TDD | IMD31 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.77	CA_n8-n40-n78

5.77.1	Common for 1 band UL and 2 bands UL CA

5.77.1.1	Operating bands for CA

Table 5.77.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |

5.77.1.2	Channel bandwidths per operating band for CA

Table 5.77.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n8A-n40A-n78A | CA_n8A-n40A CA_n8A-n78A CA_n40A-n78A | n8 | n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |

5.77.1.3	∆TIB,c and ∆RIB,c values

For CA_n8-n40-n78, the TIB,c and RIB,c values have already been defined in the current spec.

5.77.2	Specific for 2 bands UL CA

5.77.2.1	UE co-existence studies

5.77.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.77.2.1.1-1, Table 5.77.2.1.1-2 and Table 5.77.2.1.1-3 provide the two UL bands with one CC per band IMD interference analysis for CA_n8A-n40A-n78A with UL CA_n8A-n40A, UL CA_n8A-n78A and UL CA_n40A-n78A.

Table 5.77.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1385 | 1520 | 3180 | 3315 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 640 | 470 | 3685 | 3920 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4060 | 4230 | 5480 | 5715 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 240 | 445 | 5985 | 6320 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3040 | 2770 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4940 | 5145 | 7780 | 8115 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6360 | 6630 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8720 | 8285 | 1360 | 1120 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5440 | 5070 | 1855 | 2160 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10080 | 10515 | 5820 | 6060 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8660 | 9030 | 7240 | 7545 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 2nd and 3rd IMD generated by UL CA_n8A-n40A may fall into own Rx of Band n78.

Table 5.77.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2385 | 2920 | 4180 | 4715 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2040 | 1470 | 5685 | 6720 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5060 | 5630 | 7480 | 8515 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1160 | 555 | 8985 | 10520 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 5840 | 4770 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5940 | 6545 | 10780 | 12315 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8360 | 9430 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14320 | 12285 | 360 | 280 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9640 | 8070 | 3855 | 4960 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14080 | 16115 | 6820 | 7460 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11660 | 13230 | 9240 | 10345 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 2nd IMD generated by UL CA_n8A-n78A may fall into own Rx of Band n40.

Table 5.77.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 900 | 1500 | 5600 | 6200 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 800 | 1500 | 4200 | 5300 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7900 | 8600 | 8900 | 10000 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3100 | 3900 | 7500 | 9100 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3000 | 1800 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10200 | 11000 | 12200 | 13800 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 11200 | 12400 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12900 | 10800 | 6300 | 5400 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6800 | 5100 | 600 | 700 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15500 | 17600 | 12500 | 13400 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14500 | 16200 | 13500 | 14800 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 2nd and 3rd order IMD generated by UL CA_n40A-n78A may fall into own Rx of Band n8.

5.77.2.2	REFSENS requirements

For CA_n8-n40-n78, the MSD values are already in the current spec for BCS 0, considering the smallest CBW of n40 introduced in BCS 4 and 5, the modified MSD are given in the tables below. Note that IM2 from n8 and n40 to n78, the n40 channel bandwidth is modified from 10MHz to 5MHz.And compared with BCS 0, MSD of IM3 from n8 and n40 to n78 is added.

Table 5.77.2.2-3: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n8-n40-n78 | n8 | N/A | 5 | N/A | 950 | 30.5 | FDD | IMD2 |
|  | n40 | 2380 | 5 | 25 | 2380 | N/A | TDD | N/A |
|  | n78 | 3330 | 10 | 50 | 3330 | N/A | TDD | N/A |
|  | n8 | N/A | 5 | N/A | 935 | 19.8 | FDD | IMD3 |
|  | n40 | 2320 | 5 | 25 | 2320 | N/A | TDD | N/A |
|  | n78 | 3705 | 10 | 50 | 3705 | N/A | TDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2395 | 28 | TDD | IMD2 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3305 | 28.8 | TDD | IMD2 |
|  | n8 | 890 | 5 | 25 | 935 | N/A | FDD | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3730 | 16.8 | TDD | IMD3 |

5.78	CA_n28-n40-n79

5.78.1	Common for 1 band UL and 2 bands UL CA

5.78.1.1	Operating bands for CA

Table 5.78.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

5.78.1.2	Channel bandwidths per operating band for CA

Table 5.78.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n28A-n40A-n79A | CA_n28A-n40A CA_n28A-n79A CA_n40A-n79A | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |

5.78.1.3	∆TIB,c and ∆RIB,c values

For CA_n28-n40-n79, the TIB,c and RIB,c values have already been defined in the current spec.

5.78.2	Specific for 2 bands UL CA

5.78.2.1	UE co-existence studies

5.78.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.78.2.1.1-1, Table 5.78.2.1.1-2 and Table 5.78.2.1.1-3 provide the two UL bands with one CC per band IMD interference analysis for CA_n28A-n40A-n79A with UL CA_n28A-n40A, UL CA_n28A-n79A and UL CA_n40A-n79A.

Table 5.78.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1552 | 1697 | 3003 | 3148 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 994 | 804 | 3852 | 4097 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3706 | 3896 | 5303 | 5548 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 291 | 56 | 6152 | 6497 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3394 | 3104 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4409 | 4644 | 7603 | 7948 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6006 | 6296 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8897 | 8452 | 692 | 412 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5794 | 5404 | 2356 | 2691 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9903 | 10348 | 5112 | 5392 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8306 | 8696 | 6709 | 7044 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 4th IMD generated by UL CA_n28A-n40A may fall into own Rx of Band n79.

Table 5.78.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3652 | 4297 | 5103 | 5748 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3594 | 2904 | 8052 | 9297 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5806 | 6496 | 9503 | 10748 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2891 | 2156 | 12452 | 14297 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 8594 | 7304 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6509 | 7244 | 13903 | 15748 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10206 | 11496 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 19297 | 16852 | 1408 | 2188 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 13594 | 11704 | 6556 | 7891 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 18303 | 20748 | 7212 | 7992 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14606 | 16496 | 10909 | 12244 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 4th IMD generated by UL CA_n28A-n79A may fall into own Rx of Band n40.

Table 5.78.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2000 | 2700 | 6700 | 7400 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 400 | 400 | 6400 | 7700 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9000 | 9800 | 11100 | 12400 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1900 | 2800 | 10800 | 12700 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 5400 | 4000 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11300 | 12200 | 15500 | 17400 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 13400 | 14800 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 17700 | 15200 | 5200 | 4200 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10400 | 8400 | 1600 | 3100 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 19900 | 22400 | 13600 | 14600 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 17800 | 19800 | 15700 | 17200 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, there is no IMD issue generated by UL CA_n40A-n79A to Band n8.

5.78.2.2	REFSENS requirements

For CA_n8-n40-n78, the MSD values are already in the current spec for BCS 0, considering the smallest CBW of n40 and n79 introduced in BCS 4 and 5, the modified MSD are given in the tables below.

Table 5.78.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n40-n79 | n28 | 730 | 5 | 25 | 785 | N/A | FDD | N/A |
|  | n40 | 2350 | 5 | 5025 | 2350 | N/A | TDD | N/A |
|  | n79 | N/A | 4010 | N/A | 4540 | 10.7 | TDD | IMD4 |
|  | n28 | 720 | 5 | 25 | 775 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2340 | 9.2 | TDD | IMD4 |
|  | n79 | 4500 | 10 | 21650 | 4500 | N/A | TDD | N/A |

5.79	CA_n28-n78-n79

5.79.1	Common for 1 band UL and 2 bands UL CA

5.79.1.1	Operating bands for CA

Table 5.79.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

5.79.1.2	Channel bandwidths per operating band for CA

Table 5.79.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n28A-n78A-n79A | CA_n28A-n78A CA_n28A-n79A CA_n78A-n79A | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |

5.79.1.3	∆TIB,c and ∆RIB,c values

For CA_n28-n78-n79, the TIB,c and RIB,c values have already been defined in the current spec.

5.79.2	Specific for 2 bands UL CA

5.79.2.1	UE co-existence studies

5.79.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.79.2.1.1-1, Table 5.79.2.1.1-2 and Table 5.79.2.1.1-3 provide the two UL bands with one CC per band IMD interference analysis for CA_n28A-n78A-n79A with UL CA_n28A-n78A, UL CA_n28A-n79A and UL CA_n78A-n79A.

Table 5.79.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2552 | 3097 | 4003 | 4548 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2394 | 1804 | 5852 | 6897 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4706 | 5296 | 7303 | 8348 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1691 | 1056 | 9152 | 10697 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 6194 | 5104 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 6044 | 10603 | 12148 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8006 | 9096 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14497 | 12452 | 308 | 988 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9994 | 8404 | 4356 | 5491 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13903 | 15948 | 6112 | 6792 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11306 | 12896 | 8709 | 9844 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 2nd, 3rd and 5th IMD generated by UL CA_n28A-n78A may fall into own Rx of Band n79.

Table 5.79.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3652 | 4297 | 5103 | 5748 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3594 | 2904 | 8052 | 9297 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5806 | 6496 | 9503 | 10748 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2891 | 2156 | 12452 | 14297 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 8594 | 7304 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6509 | 7244 | 13903 | 15748 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10206 | 11496 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 19297 | 16852 | 1408 | 2188 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 13594 | 11704 | 6556 | 7891 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 18303 | 20748 | 7212 | 7992 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14606 | 16496 | 10909 | 12244 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 2nd and 3rd IMD generated by UL CA_n28A-n79A may fall into own Rx of Band n78.

Table 5.79.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 600 | 1700 | 7700 | 8800 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1600 | 3200 | 5000 | 6700 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 11000 | 12600 | 12100 | 13800 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4900 | 7000 | 9400 | 11700 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3400 | 1200 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 14300 | 16400 | 16500 | 18800 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 15400 | 17600 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16700 | 13800 | 10800 | 8200 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8400 | 5600 | 2600 | 100 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 20900 | 23800 | 17600 | 20200 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 19800 | 22600 | 18700 | 21400 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the above table, the 2nd and 5th IMD generated by UL CA_n78A-n79A may fall into own Rx of Band n28.

5.79.2.2	REFSENS requirements

For CA_n28-n78-n79, the MSD values are already in the current spec for BCS 0, considering the smallest CBW of n79 introduced in BCS 4 and 5, the modified MSD are given in the tables below.

Table 5.79.2.2-3: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n78-n79 | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
|  | n79 | N/A | 4010 | N/A | 4440 | 26.2 | TDD | IMD21,3,4 |
|  | n28 | 708 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4721 | 5.1 | TDD | IMD31 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3700 | 26.9 | TDD | IMD23,4 |
|  | n79 | 4440 | 4010 | 21650 | 4440 | N/A | TDD | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3310 | 4.6 | TDD | IMD3 |
|  | n79 | 4790 | 10 | 50 | 4790 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 800 | 16.2 | FDD | IMD21 |
|  | n78 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
|  | n79 | 4420 | 4010 | 21650 | 4420 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |

6	Dual Connectivity: Specific Band Combination Part

6.0	DC_nX-nY-nZ

Editor’s note: The texts for NR DC can only be added associated with the texts for the corresponding inter-band 2 bands UL CA above, which means pure TP to TR to included NR DC configuration is not allowed.

6.0.1	Configurations for DC_nX-nY-nZ

Table 6.0.1-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_nXA-nYA-nZ | DC_nXA-nYA DC_nXA-nZA DC_nYA-nZA |
| DC_nXA/B/C/(2A)-nYA/B/C/(2A)- nZA/B/C/(2A) | DC_nXA/B/C-nYA DC_nXA/B/C-nZA DC_nYA/B/C-nZA |

Annex <A> (normative):
Band group definition

For three DL and 2UL/3CC triple beat MSD to be considered, the victim band should be part of the same or adjacent band group than one of the UL band as defined in Table A-1.

Table A-1: Band group definition for same or adjacent band-group criterion

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-a | FR1-b | FR1-c | FR1-d | FR1-e |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5150-7125 |
| Duplex mode | Mostly FDD | Mostly FDD | FDD and TDD | TDD only | TDD only |

Annex <B> (normative):
Valid UL configurations

For CA_nX-nY-nZ three band DL inter band combinations with two UL bands, the following UL configurations are applicable:

-	Two band UL with one CC per band: CA_nXA-nYA.

-	Two band UL with two CC in one band: CA_nXB-nYA, CA_nXC-nYA, CA_nXA-nYB, CA_nXA-nYC.

-	The following three UL cluster cases are not supported: CA_nX(2A)-nYA, CA_nXA-nY(2A).

-	Combinations with four UL CCs are not supported.

Annex <C> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | RAN4 #112 | R4-2411825 |  |  |  | TR skeleton for TR 38.719-03-01 | 0.0.0 |
| 2024-08 | RAN4 #112 | R4-2411826 |  |  |  | (1) R4-2414340, TP for TR38.719-03-01_CA_n28A-n41A/C-n79A/C, ZTE Corporation, Sanechips. (2) R4-2414355, TP to TR 38.719-03-01 include CA_n8A-n20A-n75A, Huawei, HiSilicon, DT. (3) R4-2414356, TP to TR 38.719-03-01 include CA_n8A-n28A-n75A, Huawei, HiSilicon, DT. (4) R4-2414357, TP to TR 38.719-03-01 include CA_n20A-n28A-n75A, Huawei, HiSilicon, DT. (5) R4-2414358, TP to TR 38.719-03-01 include CA_n3A-n7A-n75A, Huawei, HiSilicon, DT. (6) R4-2414359, TP to TR 38.719-03-01 include CA_n3A-n75A-n78A, Huawei, HiSilicon, DT. (7) R4-2414360, TP to TR 38.719-03-01 include CA_n7A-n75A-n78A, Huawei, HiSilicon, DT. (8) R4-2414361, TP to TR 38.719-03-01 include CA_n1A-n7A-n75A, Huawei, HiSilicon, DT. (9) R4-2414362, TP to TR 38.719-03-01 include CA_n1A-n3A-n7A, Huawei, HiSilicon, DT. (10) R4-2414364, TP for 38.719-03-01 to add UL CA_n41C-n71A and CA_n41C-n66A for CA_n41C-n66-n71A, Ericsson, T-Mobile USA. (11) R4-2414365, TP to TR 38.719-03-01 Addition of CA_n1A-n41A-n78A, Nokia, Etisalat. (12) R4-2414366, TP to TR 38.719-03-01 Addition of CA_n1A-n5A-n8A, Nokia. (13) R4-2414367, TP to TR 38.719-03-01 Addition of CA_n3A-n5A-n8A, Nokia. | 0.1.0 |
| 2024-10 | RAN4 #112bis | R4-2416096 |  |  |  | (1) R4-2414961, TP for TR 38.719-03-01 CA_n25-n29-n77, Samsung, TELUS, Bell Mobility (2) R4-2414972, TP to TR 38.719-03-01 to add CA_n1A-n75A-n78A, Huawei, Hisilicon, Deutsche Telekom (3) R4-2415400 TP to TR 38.719-03-01 CA_n1-n28-n40, Nokia, STC (4) R4-2415404 TP to TR 38.719-03-01 CA_n1-n20-n71, Nokia, Mobily (5) R4-2415406, TP to TR 38.719-03-01 CA_n1-n20-n78, Nokia, Mobily (6) R4-2415408, TP to TR 38.719-03-01 CA_n20-n41-n71, Nokia, Mobily (7) R4-2415409, TP to TR 38.719-03-01 CA_n20-n41-n77, Nokia, Mobily (8) R4-2415410, TP to TR 38.719-03-01 CA_n20-n41-n78, Nokia, Mobily (9) R4-2415411, TP to TR 38.719-03-01 CA_n20-n71-n78, Nokia, Mobily (10) R4-2415413, TP to TR 38.719-03-01 CA_n3-n20-n71, Nokia, Mobily (11) R4-2415414, TP to TR 38.719-03-01 CA_n3-n20-n77, Nokia, Mobily (12) R4-2415419, TP to TR 38.719-03-01 CA_n1-n3-n71, Nokia, Etisalat UAE (13) R4-2415420, TP to TR 38.719-03-01 CA_n1-n3-n8, Nokia, Etisalat UAE (14) R4-2415421, TP to TR 38.719-03-01 CA_n1-n41-n71, Nokia, Etisalat UAE (15) R4-2415424, TP to TR 38.719-03-01 CA_n1-n8-n41, Nokia, Mobily (16) R4-2415426, TP to TR 38.719-03-01 CA_n3-n41-n71, Nokia, Etisalat UAE, Mobily (17) R4-2416219, TP for 38.719-03-01 to add CA_n1A-n7A-n20A, Ericsson, BT plc (18) R4-2417132, TP to TR 38.719-03-01 include CA_n1A-n3A-n75A, Huawei, Hisilicon, Deutsche Telekom (19) R4-2417133, TP to TR 38.719-03-01 include CA_n1A-n7A-n78A, Huawei, Hisilicon, Deutsche Telekom (20) R4-2417134, TP for TR 38.719-03-01 CA_n7-n25-n29, Samsung, TELUS, Bell Mobility, Qualcomm, Skyworks (21) R4-2417135, TP for TR 38.719-03-01 CA_n7-n29-n66, Samsung, TELUS, Bell Mobility (22) R4-2417136, TP for TR 38.719-03-01 CA_n7-n29-n77, Samsung, TELUS, Bell Mobility (23) R4-2417154, TP for TR38.719-03-01_3DL_2UL CA_n3A-n40A-n79A, ZTE Corporation, Sanechips (24) R4-2417137, Corrections of TR 38.719-03-01, Nokia (25) R4-2417138, TP to TR 38.719-03-01 CA_n3-n7-n77, Nokia, BT plc (26) R4-2417154, TP for TR38.719-03-01_3DL_2UL CA_n3A-n40A-n79A, ZTE Corporation, Sanechips (27) R4-2417140, TP to TR 38.719-03-01 CA_n1-n20-n41, Nokia, BT plc (28) R4-2417141, TP to TR 38.719-03-01 CA_n1-n20-n77, Nokia, Mobily (29) R4-2417142, TP to TR 38.719-03-01 CA_n1-n71-n77, Nokia, Mobily (30) R4-2417143, TP to TR 38.719-03-01 CA_n3-n20-n41, Nokia, Mobily (31) R4-2417144, TP to TR 38.719-03-01 CA_n3-n71-n77, Nokia, Mobily (32) R4-2417145 TP to TR 38.719-03-01 CA_n1-n41-n78 w ULCA, Nokia, Etisalat UAE (33) R4-2417146, TP to TR 38.719-03-01 CA_n1-n71-n78 w ULCA, Nokia, Etisalat UAE (34) R4-2417147, TP to TR 38.719-03-01 CA_n1-n8-n78 w ULCA, Nokia, Etisalat UAE (35) R4-2417148, TP to TR 38.719-03-01 CA_n3-n41-n78 w ULCA, Nokia, Etisalat UAE (36) R4-2417149, TP to TR 38.719-03-01 CA_n3-n71-n78 w ULCA, Nokia, Etisalat UAE (37) R4-2417150, TP to TR 38.719-03-01 CA_n3-n8-n78 w ULCA, Nokia, Etisalat UAE (38) R4-2417151, TP to TR 38.719-03-01 CA_n41-n71-n78 w ULCA, Nokia, Etisalat UAE (40) R4-2417152, TP to TR 38.719-03-01 CA_n8-n41-n78 w ULCA, Nokia, Etisalat UAE (41) R4-2417153, TP to TR 38.719-03-01 Uplink additions of CA_n1A-n3A-n78C, Nokia, Etisalat UAE | 0.2.0 |
| 2024-11 | RAN4 #113 | R4-2419559 |  |  |  | (1) R4-2419571, TP for TR 38.719-03-01 on corrections for two UL band with intra-band ULCA triple beat analysis table, ZTE Corporation, Sanechips (2) R4-2420421, TP to TR 38.719-03-01 to add CA_n1A-n40A-n41A, Huawei, Hisilicon | 0.3.0 |
| 2025-02 | RAN4 #114 | R4-2501626 |  |  |  | (1) R4-2502907, TP for TR38.719-03-01 Support of CA_n3-n77-n79 and CA_n28_n77_n79, Softbank (2) R4-2502908, TP to TR 38.719-03-01 include CA_n40A-78A-n79A, Huawei, Hisilicon (3) R4-2502909, TP to TR 38.719-03-01 to include CA_n1-n40-n79, Huawei, Hisilicon (4) R4-2502910, TP to TR 38.719-03-01 to include CA_n3-n40-n78, Huawei, Hisilicon (5) R4-2502912, TP to TR 38.719-03-01 to include CA_n7-n40-n79, Huawei, Hisilicon (6) R4-2502913, TP to TR 38.719-03-01 to include CA_n7-n78-n79, Huawei, Hisilicon | 0.4.0 |
| 2025-04 | RAN4 #114bis | R4-2504486 |  |  |  | (1) R4-2503211, TP for TR38.719-03-01 addition of UL CA_n77(2A) to CA_n1-n3-n77, Softbank (2) R4-2503978, TP for TR38.719-03-01_3DL_xUL CA_n3A-n34A-n41A, ZTE Corporation, Sanechips (3) R4-2503979, TP for TR38.719-03-01_3DL_xUL CA_n3A-n34A-n79A, ZTE Corporation, Sanechips (4) R4-2504272, TP for TR 38.719-03-01 to add UL CA_n2A-n77C and CA_n66A-n77C for CA_n2A-n66A-n77C, Ericsson, Verizon (5) R4-2504273, TP for TR 38.719-03-01 to add UL CA_n2A-n77C for CA_n2-n48-n77, Ericsson, Verizon (6) R4-2504274, TP for TR 38.719-03-01 to add UL CA_n5A-n77C and CA_n66A-n77C for CA_n5-n66-n77, Ericsson, Verizon (7) R4-2504276, TP for TR 38.719-03-01 to add UL CA_n66A-n77C for CA_n48-n66-n77, Ericsson, Verizon (8) R4-2505176, draft TP to TR 38.719-03-01 to include CA_n3A-n78A-n79A, Huawei, Hisilicon, SoftBank, LG Electronics (9) R4-2505178, TP for TR 38.719-03-01 on introduction of CA_n28A-n40A-n71A, Huawei, HiSilicon (10) R4-2505179, TP for TR 38.719-03-01 on introduction of CA_n28A-n71A-n77A CA_n28A-n71A-n77(2A), Huawei, HiSilicon (11) R4-2505180, TP for TR 38.719-03-01 on introduction of CA_n40A-n71A-n77A CA_n40A-n71A-n77(2A), Huawei, HiSilicon (12) R4-2505182, draft TP to TR 38.719-03-01 to include CA_n8A-n78A-n79A, Huawei, HiSilicon (13) R4-2505184, TP for TR 38.719-03-01 to add UL CA_n2AC-n77C and CA_n5A-n77C for CA_n2-n5-n77, Ericsson, Verizon (14) R4-2505185, TP for TR 38.719-03-01 to add UL CA_n5A-n77C for CA_n5-n48-n77, Ericsson, Verizon | 0.5.0 |
| 2025-05 | RAN4 #115 | R4-2507398 |  |  |  | (1) R4-2507398, TR 38.719-03-01 v0.5.1 on Rel-19 NR Inter-band CA DC configurations including inter band CA for 3 different bands DL with x different bands UL (x=1,2), ZTE | 0.5.1 |
| 2025-05 | RAN4 #115 | R4-2507399 |  |  |  | (1) R4-2506271, TP to TR 38.719-03-01 to include CA_n28A-n74A-n77A, Samsung, KDDI, Qualcomm (2) R4-2506272, TP to TR 38.719-03-01 to include CA_n28A-n41A-n74A, Samsung, KDDI, Qualcomm (3) R4-2507978, TP for TR 38.719-03-01 to add UL CA_n2A-n77C and CA_n2A-n48B for CA_n2-n48-n77, Ericsson, Verizon (4) R4-2507979, TP for TR 38.719-03-01 to add UL CA_n5A-n77C and CA_n5A-n48B for CA_n5-n48-n77, Ericsson, Verizon (5) R4-2507980, TP for TR 38.719-03-01 to add UL CA_n66A-n77C and CA_n48B-n66A for CA_n48-n66-n77, Ericsson, Verizon | 0.6.0 |
| 2025-08 | RAN4 #116 | R4-2510059 |  |  |  | (1) R4-2510046, TP to TR 38.719-03-01 to include CA_n41A-n74A-n77A, Samsung, KDDI, Qualcomm (2) R4-2512834, Draft TP to TR 38.719-03-01 to include CA_n8-n40-n78, Huawei, Hisilicon (3) R4-2512835, Draft TP to TR 38.719-03-01 to include CA_n28-n40-n79, Huawei, Hisilicon (4) R4-2512836, Draft TP to TR 38.719-03-01 to include CA_n28-n78-n79, Huawei, Hisilicon | 0.7.0 |