3GPP TSG-RAN WG4 Meeting #116	R4-2510078

Bangalore, India, 25th - 29th Aug, 2025

Source:	KDDI, Qualcomm, Samsung

Title:	TP to TR 38.746 for adding CA_n18A-n77(2A) with PC2 UL CA_n77(2A)

Agenda item:			6.8.3

Document for:	Approval

- Introduction

This a TP to TR 38.746 for adding CA_n18A-n77(2A) with PC2 UL CA_n77(2A). The revised basket WID was approved in RAN#108[2].

CA_n18A-n77(2A) with n77(2A) PC3 is submitted in this meeting in TP R4-2510054.

- References

[1] 3GPP TR 38.746 v0.4.0 (2025-05),

[2] RP-251450, Revised WID: Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary uplink (SUL)

[3] 3GPP TS 38.101-1 V19.2.0(2025-06)

Text Proposal

<Start of Text Proposal>

5.13	CA_n18-n77

5.13.1	Configurations

Table 5.X.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n18A-n77(2A) | n77(2A)8 | n18 | 5, 10, 15 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n18 | See n18 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |

5.13.2	Maximum output power

It is omitted here because it is already defined in clause 5.X.2 of [1]..

5.13.3	REFSENS requirements

Unaffected parts, such as single band and inter-band cases, are omitted.

5.13.3.x	Power class 2 for uplink CA_n77(2A)

Based on co-existence studies on TP R4-2510054, IMD2/4 of ULCA_n77(2A) falls into the n18DL, and the MSD requirements are defined as following. The MSD for IMD2 is N/A as 600MHz is max CC. MSD corresponding to IMD4 is borrowed from CA_n5-n77(2A) in Table 7.3A.5-1a of [3].

Table 5.x.1.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n18-n77 | n18 | N/A | N/A | N/A | N/A | N/A | FDD | IMD27 |
|  | n7712 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 870 | 18.5 | FDD | IMD4 |
|  | n7712 | 3480 | 10 | 1 (RBSTART=25) | 3480 | N/A | TDD | N/A |
|  |  | 3915 | 10 | 1 (RBSTART=25) | 3800 |  |  |  |
| NOTE 7: In current release the maximum separation bandwidth class is 600MHz, therefore, no IMD MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

5.13.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

<End of Text Proposal>