# 3GPP TSG-RAN WG4 Meeting #116	R4-2510081

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.14.3.1

Source:	China Telecom

Title:	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

In RAN4 #115 meeting, FR2-1 SSB based L3 measurement delay reduction for connected mode was discussed and the WF was approved [1].

In this paper, we present our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor captured in the WF [1] and summary [2].

- Discussion

RRM core requirements for FR2-1 L3 measurement delay by optimizing Rx BSF

Status in the WF [1]:

### Issue 1-1-1: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” Agreement: For the quitting condition of FBS: - Introduce a new UAI indicating “overheating” issue: - RAN4 will not discuss the UE behaviour after reporting UAI - Introduce a timer-based solution: - UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition - Further discuss the candidate numbers for T1. Way forward: For re-evaluation mechanism for scenario 1/2/3/4: - Check whether the following mechanism is agreeable: - UE re-evaluates the activation condition after the timer expires, where the timer starts after UE quits from Multi-Rx mode.

For re-evaluation mechanism for scenario 1/2/3/4, timer-based solution can be used, UE re-evaluates the activation condition after the timer expires, where the timer starts after UE quits from Multi-Rx mode. In order to avoid ping-pong, the timer for re-evaluation mechanism need to be considered coordinately with T1 for quitting mechanism.

Proposal 1: For re-evaluation mechanism for scenario 1/2/3/4, UE re-evaluates the activation condition after the timer expires, where the timer starts after UE quits from Multi-Rx mode.

- Conclusion

The contribution presents our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor, with the following proposals:

Proposal 1: For re-evaluation mechanism for scenario 1/2/3/4, UE re-evaluates the activation condition after the timer expires, where the timer starts after UE quits from Multi-Rx mode.

- References

- R4-2508266, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT, RAN WG4 Meeting #115, 19 – 23 May, 2025.

- R4-2505562, Topic summary for [115][213] NR_RRM_Ph5_Part2, CATT, RAN WG4 Meeting #115, 19 – 23 May, 2025.